3GPP TSG-RAN5 Meeting #80	R5-184332

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0016 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint Corporation, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-08-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For EN-DC test cases, the LTE test frequency needs to be specified, for inter-band and intra-band configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added information about test frequencies for NR and LTE for EN-DC inter-band configuration Added information about LTE test frequencies for EN-DC intra-band contiguous configuration Based on disciussions in the RAN5#80 meeting, the selected approach has been to keep the NR carrier at the edge of the band and put the NR LTE carrier above the NR carrier for low range and mid range, and below the NR carrier for high range. For EN-DC intra-band non-contiguous configuration, the LTE test frequency is calculated as follows: Low range: FLow_NR + (BWNR + BWLTE)/2, aligned with the LTE frequency raster Mid range: FMid_NR + (BWNR + BWLTE)/2, aligned with the LTE frequency raster High range: FHigh_NR - (BWNR + BWLTE)/2, aligned with the LTE frequency raster |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Many EN-DC test cases will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | NR band n41 test frequencies are aligned with values in R5-184464 |  |  |  |  |  |  |  |  |

## << Beginning of changes>>

4.3.1.3.2	EN-DC (two bands)

4.3.1.3.2.0	Default reference test frequencies for EN-DC combinations (two bands)

For inter-band EN-DC configurations as listed in Table 4.3.1.3.2.0-1, the following apply:

For the E-UTRA band, test frequencies as specified in TS 36.508 [2] Table 4.3.1.1.1-1 are used.

For the NR band, test frequencies as specified in clause 4.3.1 are used.

Table 4.3.1.3.2.0-1: Inter-band EN-DC configurations (FR1, two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n28A | DC_1A_n28A | 1 | n28A |
| DC_1A_n77A | DC_1A_n77A | 1A | n77A |
| DC_1A_n78A | DC_1A_n78A | 1A | n78A |
| DC_1A_n79A | DC_1A_n79A | 1A | n79A |
| DC_3A_n77A | DC_3A_n77A | 3A | n77A |
| DC_3A_n78A | DC_3A_n78A | 3A | n78A |
| DC_3A_n79A | DC_3A_n79A | 3A | n79A |
| DC_19A_n77A | DC_19A_n77A | 19A | n77A |
| DC_19A_n78A | DC_19A_n78A | 19A | n78A |
| DC_19A_n79A | DC_19A_n79A | 19A | n79A |
| DC_20A_n78A | DC_20A_n78A | 20A | n78A |
| DC_21A_n77A | DC_21A_n77A | 21A | n77A |
| DC_21A_n78A | DC_21A_n78A | 21A | n78A |
| DC_21A_n79A | DC_21A_n79A | 21A | n79A |
| DC_25A_n41A | DC_25A_n41A | 25 | n41A |
| DC_28A_n77A | DC_28A_n77A | 28A | n77A |
| DC_28A_n78A | DC_28A_n78A | 28A | n78A |
| DC_28A_n79A | DC_28A_n79A | 28A | n79A |

Table 4.3.1.3.2.0-2: Inter-band EN-DC configurations (FR1, three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-3: Inter-band EN-DC configurations (FR1, four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-5: Inter-band EN-DC configurations (FR1, five bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-6: Inter-band EN-DC configurations (FR1, six bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-7: Inter-band EN-DC configurations (FR2, two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n257A | DC_1A_n257A | 1A | n257A |
| DC_3A_n257A | DC_3A_n257A | 3A | n257A |
| DC_5A-n260A | DC_5A_n260A |  | 5 |  | n260A |  |
| DC_5A_n261A | DC_5A_n261A |  | 5 |  | n261A |  |
| DC_13A_n257A | DC_13A_n257A | 13 | n257A |
| DC_19A_n257A | DC_19A_n257A | 19A | n257A |
| DC_21A_n257A | DC_21A_n257A | 21A | n257A |
| DC_66A_n260A | DC_66A_n260A |  | 66 |  | n260A |  |
| DC_66A-n261A | DC_66A_n261A |  | 66 |  | n261A |  |

Table 4.3.1.3.2.0-8: Inter-band EN-DC configurations (FR2, three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-9: Inter-band EN-DC configurations (FR2, four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-10: Inter-band EN-DC configurations (FR2, five bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

Table 4.3.1.3.2.0-11: Inter-band EN-DC configurations (FR2, six bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| FFS | FFS | FFS | FFS |

4.3.1.3.2.1	Reference test frequencies for EN-DC combinations beginning with B1

Table 4.3.1.3.2.1-28: EN-DC combination DC_1_n28

| Range | FFS |  |  | E-UTRA |  |  |  |  | NR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | FFS | fUL [MHz] | FFS | fDL [MHz] |
| Low | FFS |  |  |  |  |  |  |  |  |  |  |
|  | FFS |  |  |  |  |  |  |  |  |  |  |
| Mid | FFS |  |  |  |  |  |  |  |  |  |  |
|  | FFS |  |  |  |  |  |  |  |  |  |  |
| High | FFS |  |  |  |  |  |  |  |  |  |  |
|  | FFS |  |  |  |  |  |  |  |  |  |  |

4.3.1.3.2.2 - 4.3.1.3.2.40 FFS

4.3.1.3.2.41	Reference test frequencies for EN-DC combinations beginning with B41

Table 4.3.1.3.2.41-1: EN-DC combination DC_(n)41, intra-band contiguous, SCS 15 kHz

| Range |  |  | E-UTRA |  |  |  | NR |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW [MHz] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [MHz] | fUL [MHz] | fDL [MHz] |
| Low | 5 | 39776 | 2508,6 | 39776 | 2508,6 | 10 | 2501,01 | 2501,01 |
| Mid | 5 | 40696 | 2600,6 | 40696 | 2600,6 | 10 | 2593,005 | 2593,005 |
| High | 5 | 41465 | 2677,5 | 41465 | 2677,5 | 10 | 2685 | 2685 |
| Low | 5 | 39825 | 2513,5 | 39825 | 2513,5 | 15 | 2503,5 | 2503,5 |
| Mid | 5 | 40721 | 2603,1 | 40721 | 2603,1 | 15 | 2593,005 | 2593,005 |
| High | 5 | 41414 | 2672,4 | 41414 | 2672,4 | 15 | 2682,495 | 2682,495 |
| Low | 5 | 39876 | 2518,6 | 39876 | 2518,6 | 20 | 2506,005 | 2506,005 |
| Mid | 5 | 40746 | 2605,6 | 40746 | 2605,6 | 20 | 2593,005 | 2593,005 |
| High | 5 | 41364 | 2667,4 | 41364 | 2667,4 | 20 | 2679,99 | 2679,99 |
| Low | 5 | 40076 | 2538,6 | 40076 | 2538,6 | 40 | 2516,01 | 2516,01 |
| Mid | 5 | 40847 | 2615,7 | 40847 | 2615,7 | 40 | 2593,125 | 2593,125 |
| High | 5 | 41165 | 2647,5 | 41165 | 2647,5 | 40 | 2670 | 2670 |
| Low | 5 | 40176 | 2548,6 | 40176 | 2548,6 | 50 | 2521,005 | 2521,005 |
| Mid | 5 | 40896 | 2620,6 | 40896 | 2620,6 | 50 | 2593,005 | 2593,005 |
| High | 5 | 41064 | 2637,4 | 41064 | 2637,4 | 50 | 2664,99 | 2664,99 |
| NOTE 1: NR test frequencies are defined in sub-clause 4.3.1.1.1.41 |  |  |  |  |  |  |  |  |

Table 4.3.1.3.2.41-2: EN-DC combination DC_(n)41, intra-band contiguous, SCS 30 kHz

| Range |  |  | E-UTRA |  |  |  | NR |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW [MHz] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [MHz] | fUL [MHz] | fDL [MHz] |
| Low |  | 5 |  | 39776 |  | 2508,6 |  | 39776 |  | 2508,6 |  | 10 |  | 2501,01 |  | 2501,01 |  |
| Mid |  | 5 |  | 40695 |  | 2600,5 |  | 40695 |  | 2600,5 |  | 10 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 41465 |  | 2677,5 |  | 41465 |  | 2677,5 |  | 10 |  | 2685 |  | 2685 |  |
| Low |  | 5 |  | 39825 |  | 2513,5 |  | 39825 |  | 2513,5 |  | 15 |  | 2503,5 |  | 2503,5 |  |
| Mid |  | 5 |  | 40720 |  | 2603 |  | 40720 |  | 2603 |  | 15 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 41414 |  | 2672,4 |  | 41414 |  | 2672,4 |  | 15 |  | 2682,48 |  | 2682,48 |  |
| Low |  | 5 |  | 39876 |  | 2518,6 |  | 39876 |  | 2518,6 |  | 20 |  | 2506,02 |  | 2506,02 |  |
| Mid |  | 5 |  | 40745 |  | 2605,5 |  | 40745 |  | 2605,5 |  | 20 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 41364 |  | 2667,4 |  | 41364 |  | 2667,4 |  | 20 |  | 2679,99 |  | 2679,99 |  |
| Low |  | 5 |  | 40076 |  | 2538,6 |  | 40076 |  | 2538,6 |  | 40 |  | 2516,01 |  | 2516,01 |  |
| Mid |  | 5 |  | 40845 |  | 2615,5 |  | 40845 |  | 2615,5 |  | 40 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 41165 |  | 2647,5 |  | 41165 |  | 2647,5 |  | 40 |  | 2670 |  | 2670 |  |
| Low |  | 5 |  | 40176 |  | 2548,6 |  | 40176 |  | 2548,6 |  | 50 |  | 2521,02 |  | 2521,02 |  |
| Mid |  | 5 |  | 40895 |  | 2620,5 |  | 40895 |  | 2620,5 |  | 50 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 41064 |  | 2637,4 |  | 41064 |  | 2637,4 |  | 50 |  | 2664,99 |  | 2664,99 |  |
| Low |  | 5 |  | 40275 |  | 2558,5 |  | 40275 |  | 2558,5 |  | 60 |  | 2526 |  | 2526 |  |
| Mid |  | 5 |  | 40945 |  | 2625,5 |  | 40945 |  | 2625,5 |  | 60 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 40964 |  | 2627,4 |  | 40964 |  | 2627,4 |  | 60 |  | 2659,98 |  | 2659,98 |  |
| Low |  | 5 |  | 40476 |  | 2578,6 |  | 40476 |  | 2578,6 |  | 80 |  | 2536,02 |  | 2536,02 |  |
| Mid |  | 5 |  | 41045 |  | 2635,5 |  | 41045 |  | 2635,5 |  | 80 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 40764 |  | 2607,4 |  | 40764 |  | 2607,4 |  | 80 |  | 2649,99 |  | 2649,99 |  |
| Low |  | 5 |  | 40575 |  | 2588,5 |  | 40575 |  | 2588,5 |  | 90 |  | 2541 |  | 2541 |  |
| Mid |  | 5 |  | 41095 |  | 2640,5 |  | 41095 |  | 2640,5 |  | 90 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 40664 |  | 2597,4 |  | 40664 |  | 2597,4 |  | 90 |  | 2644,98 |  | 2644,98 |  |
| Low |  | 5 |  | 40676 |  | 2598,6 |  | 40676 |  | 2598,6 |  | 100 |  | 2546,01 |  | 2546,01 |  |
| Mid |  | 5 |  | 41145 |  | 2645,5 |  | 41145 |  | 2645,5 |  | 100 |  | 2592,99 |  | 2592,99 |  |
| High |  | 5 |  | 40565 |  | 2587,5 |  | 40565 |  | 2587,5 |  | 100 |  | 2640 |  | 2640 |  |
| NOTE 1: NR test frequencies are defined in sub-clause 4.3.1.1.1.41 |  |  |  |  |  |  |  |  |

Table 4.3.1.3.2.41-3: EN-DC combination DC_(n)41, intra-band contiguous, SCS 60 kHz

| Range |  |  | E-UTRA |  |  |  | NR |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW [MHz] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [MHz] | fUL [MHz] | fDL [MHz] |
| Low | 5 | 39776 | 2508,6 | 39776 | 2508,6 | 10 | 2501,01 | 2501,01 |
| Mid | 5 | 40696 | 2600,6 | 40696 | 2600,6 | 10 | 2593,005 | 2593,005 |
| High | 5 | 41465 | 2677,5 | 41465 | 2677,5 | 10 | 2685 | 2685 |
| Low | 5 | 39825 | 2513,5 | 39825 | 2513,5 | 15 | 2503,5 | 2503,5 |
| Mid | 5 | 40721 | 2603,1 | 40721 | 2603,1 | 15 | 2593,005 | 2593,005 |
| High | 5 | 41414 | 2672,4 | 41414 | 2672,4 | 15 | 2682,495 | 2682,495 |
| Low | 5 | 39876 | 2518,6 | 39876 | 2518,6 | 20 | 2506,005 | 2506,005 |
| Mid | 5 | 40746 | 2605,6 | 40746 | 2605,6 | 20 | 2593,005 | 2593,005 |
| High | 5 | 41364 | 2667,4 | 41364 | 2667,4 | 20 | 2679,99 | 2679,99 |
| Low | 5 | 40076 | 2538,6 | 40076 | 2538,6 | 40 | 2516,01 | 2516,01 |
| Mid | 5 | 40846 | 2615,6 | 40846 | 2615,6 | 40 | 2593,005 | 2593,005 |
| High | 5 | 41165 | 2647,5 | 41165 | 2647,5 | 40 | 2670 | 2670 |
| Low | 5 | 40176 | 2548,6 | 40176 | 2548,6 | 50 | 2521,005 | 2521,005 |
| Mid | 5 | 40896 | 2620,6 | 40896 | 2620,6 | 50 | 2593,005 | 2593,005 |
| High | 5 | 41064 | 2637,4 | 41064 | 2637,4 | 50 | 2664,99 | 2664,99 |
| Low | 5 | 40275 | 2558,5 | 40275 | 2558,5 | 60 | 2526 | 2526 |
| Mid | 5 | 40946 | 2625,6 | 40946 | 2625,6 | 60 | 2593,005 | 2593,005 |
| High | 5 | 40964 | 2627,4 | 40964 | 2627,4 | 60 | 2659,995 | 2659,995 |
| Low | 5 | 40476 | 2578,6 | 40476 | 2578,6 | 80 | 2536,005 | 2536,005 |
| Mid | 5 | 41046 | 2635,6 | 41046 | 2635,6 | 80 | 2593,005 | 2593,005 |
| High | 5 | 40764 | 2607,4 | 40764 | 2607,4 | 80 | 2649,99 | 2649,99 |
| Low | 5 | 40575 | 2588,5 | 40575 | 2588,5 | 90 | 2541 | 2541 |
| Mid | 5 | 41096 | 2640,6 | 41096 | 2640,6 | 90 | 2593,005 | 2593,005 |
| High | 5 | 40664 | 2597,4 | 40664 | 2597,4 | 90 | 2644,995 | 2644,995 |
| Low | 5 | 40676 | 2598,6 | 40676 | 2598,6 | 100 | 2546,01 | 2546,01 |
| Mid | 5 | 41146 | 2645,6 | 41146 | 2645,6 | 100 | 2593,005 | 2593,005 |
| High | 5 | 40565 | 2587,5 | 40565 | 2587,5 | 100 | 2640 | 2640 |
| NOTE 1: NR test frequencies are defined in sub-clause 4.3.1.1.1.41 |  |  |  |  |  |  |  |  |

## << End of changes>>