3GPP TSG-RAN WG5 Meeting #80	R5-184350

Gothenburg, Sweden, August 20-24th 2018

# Source:	Qualcomm Incorporated

# Title:	Multi-PDN Handling in EN-DC

# Document for:	Endorsement

Agenda Item:	6.3.15.19

## 1.	Introduction

Based on the Outcome\Way Forward of R5-183241  from RAN5#79 –

- Continue PDNs handling for EN-DC test cases with same approach as used for LTE test cases, with QCI value suggested as per RAN2 LS.

- Scope out the changes needed for allowing 2 default PDN’s to be retained in EN-DC test cases body. Investigate their handling in various procedures and test case prose in EN-DC including level of effort. Based on the scoping, come up with optimized way of handling required changes. Combination of IMS and Internet PDN (IMS+ Internet (any order) and Internet only) in EN-DC shall be handled allowing both PDN’s to be active and EN-DC test cases to use Internet PDN. Timeline to complete this activity is by RAN5#80 (Aug-2018). Interested companies to collaborate, discuss and resolve issues prior to next RAN5 meeting.

- To avoid immediate TTCN implementation schedule, continue with current TTCN development and be flexible to change based on outcome of #2 above

Item 1 and 3 were handled in RAN5#79 itself, this document discusses high level scoping of Item#2, proposed solution and RAN5 spec changes.

## 2.	Discussion

### 2.1 New Requirements for Multi-PDN design

Below new requirements are considered as basis for proposing new solution -

- Allow 2 default EPS bearers to exist during the duration of test.

- No deactivation of any of the above two bearers in preamble or test case execution.

- Default IMS and Internet bearer to stay on MCG (LTE) and dedicated bearer (non QCI =1) of Internet to be used for EN-DC Multi-PDN testing.

- Handling of IMS and Internet PDN’s as well as Internet only to cover below scenarios

  - Internet (attach) + IMS

  - IMS (attach) +Internet

  - Internet Only (attach)

  - IMS Only (Attach) – Considered, however not a typical use case

- No interruption to LTE test cases and test environment.

- Test Case to handle multiple bearers in test case prose of all test cases.

Highlighted above are key new requirement.

### 2.2 Proposed solution- Mapping of EPS, DRB and QCI mapping

EPS, DRB and QCI Mapping of Default Bearer (E-UTRA) for Internet and IMS PDN is defined in Table-1 along with Dedicated bearer on NR.

|  | Default Bearer Internet (LTE) |  |  | Default Bearer IMS (LTE) |  |  | Dedicated Bearer (Internet for first three case, IMS only for last case) (NR) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EPS | DRB | QCI | EPS | DRB | QCI | EPS | DRB | QCI |
| Internet + IMS | 5 | 1 | 9 | 12 | 8 | 5 | 6 | 2 | 8 |
| IMS + Internet | 5 | 1 | 9 | 12 | 8 | 5 | 6 | 2 | 8 |
| Internet Only | 5 | 1 | 9 | NA | NA | NA | 6 | 2 | 8 |
| IMS only | NA | NA | NA | 5 | 1 | 5 | ,6 | 2 | 8 |

Table-1

### 2.3 Proposed Solution - No PDN Deactivation

Below call flow in figure-1 compares legacy E-UTRA behavior and proposed behavior in EN-DC test cases. There is no release of internet PDN thus allowing it to be used of setting of NR dedicated bearer for all EN-DC TC

Figure-1

### 2.4 Proposed Solution - RAN5 spec changes

To address above requirements in 2.1 and proposed changes in 2.2 and 2.3, below RAN5 spec needs to be changed

- Spec changes for TS 36.508 –

  - Modification of EPS & Data Radio Bearer ID mapping for EN-DC Test cases

  - Add MCG(s)+SCG and MCG(a)+Split in EN-DC conditions in description of specific messages

- Spec changes for TS 38.508-1 (Preambles and Generic Procedure Parameter) –

  - RRC Idle Mode Procedure, remove deactivation of PDN’s and adjust preamble for EN-DC case.

  - Update Generic Procedure Parameter to MCG(s) +SCG when Multi PDN is true. (s=1,2)

  - RRC Connected Procedure– Address DRB bring up of IMS PDN when Multi_PDN is true

- Spec changes for TS 38.509 –

  - None

- Spec changes for TS 38.523-1 –

  - Update Generic Section of L2 TC to handle Multi_PDN

  - Introduce Generic section in EN-DC RRC TC to mention that multiple MCG's will be present when Multi PDN is true

- Spec changes for TS 38.523-3 –

  - Absorb above  changes for EN-DC test modelling

## 3.	Way Forward

RAN5 spec changes, in 2.4, are introduced in R5-184346, R5-184347, R5-184348, R5-184349, R5-184613, R5-184613 and R5-184977 for approval in RAN5#80.

## 4.	References

[1] 		R5-181169 - Adding UE’s IP Stack Check for 5G NR”

[2]	R5-150645, “Test Case limitations on handling IMS signaling for Non-IMS E-UTRA Test Cases”. Source: Qualcomm Incorporated, Anritsu Ltd., Anite, Rohde & Schwarz.

[3]	R5-155779, “Test Case limitations on handling IMS signalling (Single-PDN/ Multi-PDN) for Non-IMS E-UTRA and Inter-RAT Test Cases”. Source: Qualcomm Incorporated.

[4]	R5-153969, “Testing of Loopback mode A and B activated test cases with IMS enabled UE”. Source: Qualcomm Incorporated, Anritsu Ltd., Anite.

[5]	R5-153970, “Update to Loopback data procedure with IMS signalling”. Source: Anritsu Ltd., Qualcomm Incorporated, Samsung, MCC TF160.

[6]	R5-162144, “Handling of multi-PDNs-configured UEs”. Source: MCC TF160.

[7]	R2-1804034 - Summary of ASN.1 review session on part 7 (UE capability), Intel

[8]	R2-1802941 - Open issues on EN-DC capabilities on LTE side (I044, I089), Intel

[9]	R5-182174 – Response LS on IMS Voice over NR in EN-DC

[10]	R5-182088 - Handling of IMS PDN in 5G NR NSA mode

[11]	R5-183241 - Handling of IMS PDN in 5G EN-DC mode