3GPP TSG-RAN5 Meeting #80	R5-184733

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4453 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TC7.9G Spurious emissions for V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SL_V2V-UEConTest |  |  |  |  |  | Date: |  |  | 2018-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TC7.9G Spurious emissions for V2X is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of TC7.9G Spurious emissions for V2X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC7.9G Spurious emissions for V2X will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9G.1, Annex F.1.3, Annex F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

7.9G	Spurious emissions for V2X Communication

7.9G.1	Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

7.9G.1.1	Test Purpose

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

7.9G.1.2	Test Applicability

This test applies to all types of UE that support V2X Sidelink Communication and Band 47.

7.9G.1.3	Minimum Conformance Requirements

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9G.1.3-1.

Table 7.9G.1.3-1: General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 1 |
| 12.75 GHz – 26GHz | 1 MHz | -47dBm | 3 |
| NOTE 1: Applies only for Band 22, Band 42, Band 43 and Band 48 NOTE 2: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH_RA/RB as defined in Annex C.3.1. NOTE 3: Applies for Band 47 |  |  |  |

In addition, for a V2X UE operating in Region 1, the power of any spurious emission shall not exceed the levels specified in Table 7.9G.1.3-2.

Table 7.9G.1.3-2: Additional RX spurious emissions limits in Region 1

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 5795 MHz f < 5815MHz | [-65 dBm] | 1 MHz |  |
| NOTE: The additional Rx spurious emission limits, however, are specified in EIRP in Region 1. Furthermore, the requirement varies based on antenna installation. It is FFS how to convert this to conducted requirements. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 7.9.

7.9G.1.4	Test Description

7.9G.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2G.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.9G.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 7.9G.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | RB allocation |
| 10MHz | QPSK |  | [48] |
| 20MHz | QPSK |  | [96] |

1.	Connect the SS to the UE antenna connectors and connect the GNSS simulator to the UE GNSS RX antenna connector as shown in TS 36.508 [7] Annex A, Figure A.89.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 6.8.2. Message content exceptions are defined in clause 7.9G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 7.9G.1.4.1-1.

3a.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

6.	Trigger the UE to reset UTC time. (NOTE: The UTC time reset may be performed by MMI or AT command (+CUTCR).)

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

7.9G.1.4.2	Test Procedure

1.	Sweep the spectrum analyzer (or equivalent equipment) over a frequency range and measure the average power of spurious emission.

2.	Repeat step 1 for all E-UTRA Rx antennas of the UE.

7.9G.1.4.3	Message Contents

Message contents are according to TS 36.508 [7] subclause 4.6.

7.9G.1.5	Test Requirement

The measured spurious emissions derived in step 1), shall not exceed the maximum level specified in Table 7.9G.1.5-1

Table 7.9G.1.5-1: General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 1 |
| 12.75 GHz – 26GHz | 1 MHz | -47dBm | 3 |
| NOTE 1: Applies only for Band 22, Band 42, Band 43 and Band 48 NOTE 2: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH_RA/RB as defined in Annex C.3.1. NOTE 3: Applies for Band 47 |  |  |  |

In addition, for a V2X UE operating in Region 1, the measured spurious emissions derived in step 1) shall not exceed the levels specified in Table 7.9G.1.5-2.

Table 7.9G.1.5-2: Additional RX spurious emissions limits in Region 1

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 5795 MHz f < 5815MHz | [-65 dBm] | 1 MHz |  |
| NOTE: The additional Rx spurious emission limits, however, are specified in EIRP in Region 1. Furthermore, the requirement varies based on antenna installation. It is FFS how to convert this to conducted requirements. |  |  |  |

<<Unchanged sections omitted >>

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| # < Unchanged rows omitted > |  |  |
| 7.9A Spurious emissions for CA | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |
| 7.9E Spurious emissions for UE category 0 | Same as 7.9 |  |
| 7.9F Spurious emissions for category NB1 | Same as 7.9 |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |

<<Unchanged sections omitted >>

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| # < Unchanged rows omitted > |  |  |  |
| 7.9E Spurious emissions for UE category 0 | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9EA Spurious emissions for UE category M1 | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9F Spurious emissions for category NB1 | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz 12.75GHz f 26 GHz: -47dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |

<< End of Changes >>