3GPP TSG-RAN5 Meeting #80	R5-184935

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4485 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cat1bis new RF test case: Occupied bandwidth for UE category 1bis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UE_cat_1RX-UEConTest |  |  |  |  |  | Date: |  |  | 2018-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Cat1bis workplan identifies the need to add occupied bandwidth test case for UE category 1bis which is currently missing in the 36.521-1 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 6.6.1EB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cat 1bis WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1EB (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 6.6.1EA.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1EA.5-1.

Table 6.6.1EA.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | N/A | N/A | 1.4 | N/A | N/A | N/A |

<Start of changes>

### 6.6.1EB	Occupied bandwidth for UE category 1bis

#### 6.6.1EB.1	Test purpose

Same test purpose as in clause 6.6.1.1.

#### 6.6.1EB.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

#### 6.6.1EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.1.3.

#### 6.6.1EB.4	Test description

##### 6.6.1EB.4.1	Initial conditions

Same initial conditions as in clause 6.6.1.4.1 with the following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

##### 6.6.1EB.4.2	Test procedure

Same test procedure as in clause 6.6.1.4.2

##### 6.6.1EB.4.3	Message contents

Same message contents as in clause 6.6.1.4.3

#### 6.6.1EB.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1EB.5-1.

Table 6.6.1E.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

<End changes>