3GPP TSG RAN WG5 Meeting #80	R5-184957

Gothenburg, Sweden, 20th – 24th August 2018

Source:		Rohde & Schwarz

Title:	Discussion on test point selection for In-band Emissions test case in FR1

Agenda Item:	5.3.13.17

Document for:	Endorsement

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the In-band Emissions test case is analyzed.

# Discussion

The In-band Emissions are unwanted signals present in non-allocated RBs and are defined in section 6.4.2.3 and Annex F Transmit Modulation of TS 38.101-1 [1].

The minimum requirement is specified as follows:

6.4.2.3	In-band emissions

The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain, however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.

The average of the basic in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3-1.

Table 6.4.2.3-1: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the carrier leakage frequency if is odd, or in the two RBs immediately adjacent to the carrier leakage frequency if is even but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.3.3). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.3.3). NOTE 8: is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power normalized by the number of allocated RBs, measured in dBm. |  |  |  |  |

## Test Environment

In LTE, In-band Emissions are tested under extreme conditions. However, since In-band Emissions are not a regulatory requirement, it is proposed to apply normal condition as test environment in order to save test time.

Proposal 1: Define test environment as Normal for In-band Emissions in FR1.

## Test Subcarrier Spacing

It is not expected that in-band emissions depend on the choice of subcarrier spacing.

Proposal 2: Select smallest SCS for In-band Emissions in FR1.

## Test Frequencies

In LTE, In-band Emissions are tested for Low range, Mid range, and High range. Due to the presence of filters in the FR1 UEs it is reasonable to follow the LTE approach for NR.

Proposal 3: Test In-band Emissions for Low range, Mid range, and High range frequencies in FR1.

## Test Channel Bandwidth

In LTE, In-band Emissions are tested for Lowest, 5 MHz and Highest channel bandwidths. Due to the presence of filters in the FR1 UEs it is reasonable to follow the LTE approach for NR.

Proposal 4: Test In-band Emissions for Lowest, Mid and Highest channel bandwidth in FR1.

## Test Modulations and waveforms

In LTE In-band Emissions are tested for test points without MPR. Therefore, DFT-s-OFDM QPSK should be selected as modulation and waveform as it does not have any MPR.
Moreover, CP OFDM has a higher CREST factor than DFT-s-OFDM and may result in higher in-band emissions although an MPR is applied. Therefore, both modulations should be tested.

Proposal 5: Test In-band Emissions with DFT-s-OFDM QPSK and CP-OFDM QPSK.

## RB allocation

In LTE, In-band Emissions are tested with highest allocation without MPR where the partial allocation is located on the lower and upper channel edge. In order to achieve a reasonable MU for the large channel bandwidths of NR, a low allocation is beneficial. Taking this into account the natural choices are Inner_1RB_Left or Inner_1RB_Right as they keep the allocation to a minimum while having no MPR.

Proposal 6: Test In-band Emissions with Inner_1RB_Left and Inner_1RB_Right.

# Conclusion

To sum up, the following selection of test points is proposed for In-band Emissions in FR1:

Proposal 1: Define test environment as Normal for In-band Emissions in FR1.

Proposal 2: Select smallest SCS for In-band Emissions in FR1.

Proposal 3: Test In-band Emissions for Low range, Mid range, and High range frequencies in FR1.

Proposal 4: Test In-band Emissions for Lowest, Mid and Highest channel bandwidth in FR1.

Proposal 5: Test In-band Emissions with DFT-s-OFDM QPSK and CP-OFDM QPSK.

Proposal 6: Test In-band Emissions with Inner_1RB_Left and Inner_1RB_Right.

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 3 | 1 | 4 | 36 |

# References

- TS 38.101-1 V15.2.0 , 3GPP TSG RAN WG4, June 2018