3GPP TSG-RAN5 Meeting #80	R5-184980

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 0766 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cat1bis test cases applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UE_cat_1RX-UEConTest |  |  |  |  |  | Date: |  |  | 2018-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Cat 1bis test cases for multiple test cases were added to the 36.521-1 and 36.521-3 spec as per the Cat1 bis work plan. The test case applicability for these cases are currently missing in the 36.521-2 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added applicability for multiple Cat 1bis test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These Cat1 bis test cases cannot be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, FGI support in combined or fdd-Add-UE-EUTRA-Capabilities or tdd-Add-UE-EUTRA-Capabilities is taken into account.

Table 4.1-1: Applicability of RF conformance test cases, ref. TS 36.521-1 [1]

| Clause |  | Title |  |  |  |  | Release |  |  | Applicability |  |  |  |  |  |  |  |  |  |  |  | Tested Bands / CA-Configurations Selection |  |  |  | Branch |  | Additional Information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Condition |  |  |  |  |  | Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Transmitter Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.2 |  | UE Maximum Output Power |  |  |  |  | Rel-8 |  |  | C186 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.2.2_1 |  | UE Maximum Output Power for HPUE |  |  |  |  | Rel-10 |  |  | C39 |  |  |  |  |  | UE supporting E-UTRA Power Class 1 or Power Class 2 |  |  |  |  |  | D04 |  |  |  | FDD, TDD |  |  |  |
| 6.2.2A.1 |  | UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.2.2A.2 |  | UE Maximum Output Power for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  | FDD, TDD |  |  |  |
| 6.2.2A.3 |  | UE Maximum Output Power for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD, TDD |  |  |  |
| 6.2.2B |  | UE Maximum Output Power for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 and UL-MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.2.2B_1 |  | HPUE Maximum Output Power for UL-MIMO |  |  |  |  | Rel-10 |  |  | C202 |  |  |  |  |  | UE supporting E-UTRA Power Class 2 and UL-MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.2.2E |  | UE Maximum Output Power for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.2.2EA |  | UE Maximum Output Power for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.2.2EB |  | UE Maximum Output Power for UE category 1bis |  |  |  |  | Rel-13 |  |  | C122c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.2F |  | UE Maximum Output Power for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 6.2.2G.1 |  | UE Maximum Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.2.3EB |  | Maximum Power Reduction (MPR) for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.4EB |  | Additional Maximum Power Reduction (A-MPR) for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.5 |  | Configured UE transmitted Output Power |  |  |  |  | Rel-8 |  |  | C186 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.2.5_1 |  | Configured UE transmitted Output Power for HPUE |  |  |  |  | Rel-10 |  |  | C39 |  |  |  |  |  | UE supporting E-UTRA Power Class 1 or Power Class 2 |  |  |  |  |  | D04 |  |  |  | FDD, TDD |  |  |  |
| 6.2.5A.1 |  | Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.2.5A.3 |  | Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.2.5A.4 |  | Additional Maximum Power Reduction (A-MPR) for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.2.5B |  | Configured transmitted power for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.2.5E |  | Configured transmitted power for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.2.5EA |  | Configured UE transmitted Power for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.2.5EB |  | Configured UE transmitted Power for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.5F |  | Configured UE transmitted Output Power for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.2.5G.1 |  | Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.3.1 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2 |  | Minimum Output Power |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.2A.1 |  | Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.2A.2 |  | Minimum Output Power for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.2A.3 |  | Minimum Output Power for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD |  |  |  |
| 6.3.2B |  | Minimum Output Power for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.3.2E |  | Minimum Output Power for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.2EA |  | Minimum Output Power for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.2EB |  | Minimum Output Power for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2F |  | Minimum Output Power for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.3.2G.1 |  | Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.3.3 |  | Transmit OFF Power |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.3A.1 |  | Transmit OFF Power for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.3A.2 |  | UE Transmit OFF power for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.3A.3 |  | Transmit OFF Power for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.3.3B |  | UE Transmit OFF power for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.3.3E |  | UE Transmit OFF power for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.3EA |  | UE Transmit OFF power for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.3EB |  | UE Transmit OFF power for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.3G.1 |  | UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.3.4.1 |  | General ON/OFF time mask |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4.2.1 |  | PRACH time mask |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4.2.2 |  | SRS time mask |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4A.1.1 |  | General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4A.1.2 |  | General ON/OFF time mask for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4A.1.3 |  | General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4B.1 |  | ON/OFF time mask for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4C.1 |  | General ON/OFF time mask for Dual Connectivity |  |  |  |  | Rel-12 |  |  | C224 |  |  |  |  |  | UE supporting Dual Connectivity |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4C.1_1 |  | General ON/OFF time mask for asynchronous Dual Connectivity |  |  |  |  | Rel-12 |  |  | C225 |  |  |  |  |  | UE supporting asynchronous Dual Connectivity |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.4E.1 |  | General ON/OFF time mask for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.4E.2 |  | Prach and SRC ON/OFF time mask for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.4EA.1 |  | General ON/OFF time mask for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.4EA.2.1 |  | PRACH time mask for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.4EA.2.2 |  | SRS time mask for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.4EB.1 |  | General ON/OFF time mask for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4EB.2.1 |  | PRACH time mask for UE category 1bis |  |  |  |  | Rel13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4EB.2.2 |  | SRS time mask for UE category 1bis |  |  |  |  | Rel13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4F.1 |  | General ON/OFF time mask for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.3.4F.2 |  | NPRACH time mask for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.3.4G.1 |  | General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.3.4G.4 |  | PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.3.5.1 |  | Power Control Absolute Power Tolerance |  |  |  |  | Rel-8 |  |  | C186 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5.2 |  | Power Control Relative Power Tolerance |  |  |  |  | Rel-8 |  |  | C186 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5.3 |  | Aggregate Power Control Tolerance |  |  |  |  | Rel-8 |  |  | C186 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.1.1 |  | Power Control Absolute Power Tolerance for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.1.2 |  | Power Control Absolute Power Tolerance for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5A.1.3 |  | Power Control Absolute Power Tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.2.1 |  | Power Control Relative Power Tolerance for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.2.2 |  | Power Control Relative Power Tolerance for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5A.2.3 |  | Power Control Relative Power Tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.3.1 |  | Aggregate Power Control Tolerance for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.3.2 |  | Aggregate Power Control Tolerance for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5A.3.3 |  | Aggregate Power Control Tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5B.1 |  | Power Control Absolute power tolerance for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5B.2 |  | Power Control Relative power tolerance for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5B.3 |  | Aggregate power control tolerance for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5C.2 |  | Power Control Relative power tolerance for Dual Connectivity |  |  |  |  | Rel-12 |  |  | C224 |  |  |  |  |  | UE supporting Dual Connectivity |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5C.2_1 |  | Power Control Relative power tolerance for asynchronous Dual Connectivity |  |  |  |  | Rel-12 |  |  | C225 |  |  |  |  |  | UE supporting asynchronous Dual Connectivity |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.3.5E.1 |  | Power Control Absolute power tolerance for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.5E.2 |  | Power Control Relative power tolerance for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.5E.3 |  | Aggregate power control tolerance for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.5EA.1 |  | Power control for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.5EA.2 |  | Power Control Relative power tolerance for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.5EA.3 |  | Aggregate power control tolerance for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.3.5EA.3_1 |  | Aggregate power control tolerance for UE category M1 (CE Mode B) |  |  |  |  | Rel-13 |  |  | C156c |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D02 |  |  |  |  | HD-FDD |  |  |
| 6.3.5EB.1 |  | Power Control Absolute power tolerance for UE category 1bis |  |  |  |  | Rel13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5EB.2 |  | Power Control Relative power tolerance for UE category 1bis |  |  |  |  | Rel13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5EB.3 |  | Aggregate power control tolerance for UE category 1bis |  |  |  |  | Rel13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5F.1 |  | Power Control Absolute power tolerance for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  |  | HD-FDD |  |  |
| 6.3.5F.2 |  | Power Control Relative power tolerance for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  |  | HD-FDD |  |  |
| 6.3.5F.3 |  | Aggregate power control tolerance for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  |  | HD-FDD |  |  |
| 6.3.5_1.1 |  | Power Control Absolute Power Tolerance for HPUE |  |  |  |  | Rel-10 |  |  | C39 |  |  |  |  |  | UE supporting E-UTRA Power Class 1 or Power Class 2 |  |  |  |  |  | D04 |  |  |  |  | FDD |  |  |
| 6.3.5_1.2 |  | Power Control Relative Power Tolerance for HPUE |  |  |  |  | Rel-10 |  |  | C39 |  |  |  |  |  | UE supporting E-UTRA Power Class 1 or Power Class 2 |  |  |  |  |  | D04 |  |  |  |  | FDD |  |  |
| 6.3.5_1.3 |  | Aggregate Power Control Tolerance for HPUE |  |  |  |  | Rel-10 |  |  | C39 |  |  |  |  |  | UE supporting E-UTRA Power Class 1 or Power Class 2 |  |  |  |  |  | D04 |  |  |  |  | FDD |  |  |
| 6.3.5G.1 |  | Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.5.1 |  | Frequency Error |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1A.1 |  | Frequency Error for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1A.2 |  | Frequency error for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1A.3 |  | Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1B |  | Frequency Error for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1D.1 |  | Frequency error for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1D.2 |  | Frequency error for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 6.5.1E |  | Frequency Error for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.1EA |  | Frequency Error for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.1EA_1 |  | Frequency Error for UE category M1 (CEmodeB) |  |  |  |  | Rel-13 |  |  | C156c |  |  |  |  |  | UE supporting E-UTRA FDD and (UE category M1 and CE Mode B) |  |  |  |  |  | D02 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.1F |  | Frequency Error for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.5.1G.1 |  | Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.5.2.1 |  | Error Vector Magnitude (EVM) |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2.1_1 |  | Error Vector Magnitude (EVM) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C147 |  |  |  |  |  | UE supporting E-UTRA and UL 64QAM |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.5.2.1_2 |  | Error Vector Magnitude (EVM) for UL 256QAM |  |  |  |  | Rel-14 |  |  | C301 |  |  |  |  |  | UE supporting E-UTRA and UL 256QAM |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2.1A |  | PUSCH-EVM with exclusion period |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2.1E.1 |  | Error Vector Magnitude for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.1E.2 |  | PUSCH-EVM with exclusion period for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.1EA.1 |  | Error Vector Magnitude (EVM) for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.1EA.2 |  | PUSCH-EVM with exclusion period for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.1F.1 |  | Error Vector Magnitude (EVM) for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.5.2.1G.1 |  | Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.5.2.2 |  | Carrier leakage |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2.2E |  | Carrier leakage for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.2EA |  | Carrier leakage for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.2F |  | Carrier leakage for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.5.2.2G.1 |  | Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.5.2.3 |  | In-band emissions for non allocated RB |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2.3E |  | In-band emissions for non allocated RB for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.3EA |  | In-band emissions for non allocated RB for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.3F |  | In-band emissions for non allocated RB for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.5.2.3G.1 |  | In-band emissions for non-allocated RB for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.5.2.4 |  | EVM equalizer spectrum flatness |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2.4E |  | EVM equalizer spectrum flatness for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.4EA |  | EVM equalizer spectrum flatness for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.5.2.4G.1 |  | Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.5.2A.1.1 |  | Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.1.1_1 |  | EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM |  |  |  |  | Rel-13 |  |  | C148 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA and UL 64QAM. |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.5.2A.1.1_2 |  | EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM |  |  |  |  | Rel-14 |  |  | C302 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA and UL 256QAM. |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.1.2 |  | Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.2A.1.2_1 |  | Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C160 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA and UL 64QAM |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.5.2A.1.2_2 |  | Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM |  |  |  |  | Rel-14 |  |  | C303 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA and UL 256QAM. |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.1.3 |  | Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.1.3_1 |  | Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C185 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA and UL 64QAM |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.1.3_2 |  | Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  |  |  |  | Rel-14 |  |  | C304 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA and UL 256QAM |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.2.1 |  | Carrier leakage for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.2.2 |  | Carrier leakage for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.2.3 |  | Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.3.1 |  | In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.3.2 |  | In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2A.3.3 |  | In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2B.1 |  | Error Vector Magnitude for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2B.2 |  | Carrier leakage for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2B.3 |  | In-band emissions for non allocated RB for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.5.2B.4 |  | EVM equalizer spectrum flatness for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.6.1 |  | Occupied bandwidth |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.1A.1 |  | Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.1A.2 |  | 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.1A.3 |  | Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.1B |  | Occupied bandwidth for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.6.1E |  | Occupied bandwidth for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.1EA |  | Occupied bandwidth for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.1EB |  | Occupied bandwidth for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.1F |  | Occupied bandwidth for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.1G.1 |  | Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.6.2.1 |  | Spectrum Emission Mask |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1_1 |  | Spectrum Emission Mask for Multi-cluster PUSCH |  |  |  |  | Rel-10 |  |  | C100 |  |  |  |  |  | UE supporting E-UTRA and Multi-Cluster PUSCH |  |  |  |  |  | D07 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1A.1 |  | Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1A.2 |  | Spectrum Emission Mask for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1A.3 |  | Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1B |  | Spectrum Emission Mask for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1E |  | Spectrum Emission Mask for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.2.1EA |  | Spectrum Emission Mask for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.2.1EB |  | Spectrum Emission Mask for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.1F |  | Spectrum Emission Mask for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 6.6.2.1G.1 |  | Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.6.2.2 |  | Additional Spectrum Emission Mask |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.2_1 |  | Additional Spectrum Emission Mask for UL 64QAM |  |  |  |  | Rel-13 |  |  | C147 |  |  |  |  |  | UE supporting E-UTRA and UL 64QAM |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.2A.1 |  | Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.2A.2 |  | Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.2A.3 |  | Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.2A.1_1 |  | Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C148 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA and UL 64QAM. |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.2A.2_1 |  | Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C160 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA and UL 64QAM |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.2B |  | Additional Spectrum Emission Mask for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.2E |  | Additional Spectrum Emission Mask for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.2.2EA |  | Additional Spectrum Emission Mask for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.2.2EB |  | Additional Spectrum Emission Mask for UE Category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.2G.1 |  | Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.6.2.3 |  | Adjacent Channel Leakage power Ratio |  |  |  |  | Rel-8 |  |  | C186 |  |  |  |  |  | UE supporting E-UTRA Power Class 3 |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3_1 |  | Adjacent Channel Leakage power Ratio for HPUE |  |  |  |  | Rel-10 |  |  | C39 |  |  |  |  |  | UE supporting E-UTRA Power Class 1 or Power Class 2 |  |  |  |  |  | D04 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3_2 |  | Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH |  |  |  |  | Rel-10 |  |  | C159 (Note 2) |  |  |  |  |  | UE supporting E-UTRA and Multi-Cluster PUSCH |  |  |  |  |  | D07 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3A.1 |  | Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3A.2 |  | Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.2.3A.3 |  | Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3A.1_1 |  | Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C148 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA and UL 64QAM |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.3A.2_1 |  | Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C160 |  |  |  |  |  | UE supporting E-UTRA and inter band DL CA and UL CA and UL 64QAM |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.3A.3_1 |  | Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C161 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA and UL 64QAM |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.3B |  | Adjacent Channel Leakage power Ratio for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3E |  | Adjacent Channel Leakage power Ratio for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.2.3EA |  | Adjacent Channel Leakage power Ratio for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.2.3EB |  | Adjacent Channel Leakage power Ratio for UE Category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.3F |  | Adjacent Channel Leakage power Ratio for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 6.6.2.3G.1 |  | Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.6.2.3_3 |  | Adjacent Channel Leakage power Ratio for UL 64QAM |  |  |  |  | Rel-13 |  |  | C147 |  |  |  |  |  | UE supporting E-UTRA and UL 64QAM |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  | Note 1 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 1 |  |
| 6.6.2.3_4 |  | Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM |  |  |  |  | Rel-13 |  |  | C149 |  |  |  |  |  | UE supporting E-UTRA and Multi-Cluster PUSCH and UL 64QAM |  |  |  |  |  | D07 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.2.3_5 |  | Adjacent Channel Leakage power Ratio for UL 256QAM |  |  |  |  | Rel-14 |  |  | C301 |  |  |  |  |  |  |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.2.4 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.3.1 |  | Transmitter Spurious emissions |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.1_1 |  | Transmitter Spurious emissions for Multi-Cluster PUSCH |  |  |  |  | Rel-10 |  |  | C100 |  |  |  |  |  | UE supporting E-UTRA and Multi-Cluster PUSCH |  |  |  |  |  | D07 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.1A.1 |  | Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.1A.2 |  | Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.3.1A.3 |  | Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3F.1 |  | Transmitter Spurious emissions for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 6.6.3F.2 |  | Spurious emission band UE co-existence for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 6.6.3.2 |  | Spurious emission band UE co-existence |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.2A.1 |  | Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.2A.2 |  | Spurious emission band UE co-existence for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.2A.3 |  | Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.3 |  | Additional spurious emissions |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.3_1 |  | Additional spurious emissions for UL 64QAM |  |  |  |  | Rel-13 |  |  | C147 |  |  |  |  |  | UE supporting E-UTRA and UL 64QAM |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.3.3A.1 |  | Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3.3A.1_1 |  | Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C148 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA and UL 64QAM. |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.3.3A.2 |  | Additional spurious emissions for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  |  |  | Rel-14 |  |  | C305 |  |  |  |  |  | UE supporting E-UTRA and eLAA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.3.3A.2_1 |  | Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM |  |  |  |  | Rel-13 |  |  | C160 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA and UL 64QAM |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  | Note 1 |  |
| 6.6.3.3A.3 |  | Additional spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3B.2 |  | Spurious emission band UE co-existence for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3E.1 |  | Transmitter Spurious emissions for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.3E.2 |  | Transmitter Spurious Band UE co-existence for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.3EA.1 |  | Transmitter Spurious emissions for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.3EA.2 |  | Spurious emission band UE co-existence for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.6.3EA.3 |  |  | Additional spurious emissions for UE category M1 |  |  |  |  |  | Rel-13 |  |  |  | C112a |  |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  |  | D01 |  |  |  |  | FDD, HD-FDD, TDD |  |  |  |  |  |
| 6.6.3EB.1 |  |  | Transmitter Spurious emissions for UE category 1bis |  |  |  |  |  | Rel-13 |  |  |  | C112c |  |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  |  | D01 |  |  |  |  | FDD, TDD |  |  |  |  |  |
| 6.6.3EB.2 |  | Spurious emission band UE co-existence for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3EB.3 |  | Additional spurious emissions for UE category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.6.3E.3 |  |  | Additional spurious emissions for UE category 0 |  |  |  |  |  | Rel-12 |  |  |  | C112 |  |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  |  | D01 |  |  |  |  | FDD, HD-FDD, TDD |  |  |  |  |  |
| 6.6.3G.1 |  | Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.6.3G.1_1 |  | Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.7 |  | Transmit intermodulation |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.7A.1 |  | Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD, TDD |  |  |  |
| 6.7A.2 |  | Transmit intermodulation for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD, TDD |  |  |  |
| 6.7B |  | Transmit intermodulation for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 6.7E |  | Transmit intermodulation for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.7EA |  | Transmit intermodulation for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 6.7EB |  | Transmit intermodulation for UE Category 1bis |  |  |  |  | Rel-13 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 6.7F |  | Transmit intermodulation for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting Category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 6.7G.1 |  | Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 6.8B |  | Time alignment between transmitter branches for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
|  |  |  |  |  | Receiver Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3 |  | Reference sensitivity level |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D01 |  |  |  | FDD, TDD |  |  |  |
| 7.3_1 |  | Reference sensitivity level with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.3A.1 |  | Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.3A.2 |  | Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E08 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.3A.3 |  | Reference sensitivity level for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E10 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3A.4 |  | Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.3A.5 |  | Reference sensitivity level for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.3A.6 |  | Reference sensitivity level for CA (inter-band DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C116 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and UL CA |  |  |  |  |  | E03 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.3A.7 |  | Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) |  |  |  |  | Rel-11 |  |  | C115 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA |  |  |  |  |  | E02 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.3A.9 |  | Reference sensitivity level for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.3A.10 |  | Reference sensitivity level for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | C223 |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  |  |  |  |  |  |
| 7.3E |  | Reference sensitivity level for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.3EA |  | Reference sensitivity level for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.3EB |  | Reference sensitivity level for UE category 1bis |  |  |  |  | Rel-14 |  |  | C112c |  |  |  |  |  | UE supporting E-UTRA and UE category 1bis |  |  |  |  |  | D01 |  |  |  |  |  | FDD, HD-FDD, TDD |  |
| 7.3B |  | Reference sensitivity level for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.3F.1 |  | Reference sensitivity level without repetitions for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 7.3F.2 |  | Reference sensitivity level with repetitions for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D11 |  |  |  | HD-FDD |  |  |  |
| 7.3G.1 |  | Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.4 |  | Maximum input level |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.4_1 |  | Maximum input level with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C168 |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports but not 256QAM in DL |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.4_H |  | 7.4_H Maximum input level for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C113h |  |  |  |  |  | UE supporting E-UTRA and 256QAM in DL |  |  |  |  |  | D01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.1 |  | Maximum input level for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.1_H |  | Maximum input level for CA (intra-band contiguous DL CA and UL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C19h |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA and 256QAM in DL |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.2 |  | Maximum input level for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E08 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.2_H |  | Maximum input level for CA (intra-band contiguous DL CA without UL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C20h |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and 256QAM in DL |  |  |  |  |  | E08 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.3 |  | Maximum input level for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E10 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4A.3_H |  | Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C21h |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA and 256QAM in DL |  |  |  |  |  | E10 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.4 |  | Maximum input level for CA (intra band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.4_H |  | Maximum input level for CA (intra band non-contiguous DL CA without UL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C43h |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA and 256QAM in DL |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.4A.5 |  | Maximum input level for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.4A.5_H |  | Maximum input level for 3DL CA for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C122h |  |  |  |  |  | UE supporting E-UTRA and 3DL CA and 256QAM in DL |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with FDD-TDD CA under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with TDD-TDD CA under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.4A.7 |  | Maximum input level for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.4A.8 | Maximum input level for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E15 |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4B |  | Maximum input level for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.4D.1 |  | Maximum input level for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.4D.2 |  | Maximum input level for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.4E |  | Maximum input level for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.4EA |  | Maximum input level for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.4EC |  | Maximum input level for UE category M2 |  |  |  |  | Rel-14 |  |  | C112d |  |  |  |  |  | UE supporting E-UTRA and UE category M2 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.4F |  | Maximum input level for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 7.4G.1 |  | Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.5 |  | Adjacent Channel Selectivity (ACS) |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.5_1 |  | Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.5A.1 |  | Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.5A.2 |  | Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E11 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.5A.3 |  | Adjacent Channel Selectivity (ACS) for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E12 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5A.4 |  | Adjacent Channel Selectivity (ACS) for CA (intra band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.5A.5 |  | Adjacent Channel Selectivity (ACS) for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.5A.7 |  | Adjacent Channel Selectivity (ACS) for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.5A.8 | Adjacent Channel Selectivity (ACS) for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5B |  | Adjacent Channel Selectivity (ACS)for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.5D.1 |  | Adjacent Channel Selectivity (ACS) for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.5D.2 |  | Adjacent Channel Selectivity (ACS) for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.5E |  | Adjacent Channel Selectivity (ACS) for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.5EA |  | Adjacent Channel Selectivity (ACS) for category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.5F |  | Adjacent Channel Selectivity (ACS) for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 7.5G.1 |  | Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.6.1 |  | In-band blocking |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.6.1_1 |  | In-band blocking with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.6.1A.1 |  | In-band blocking for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.1A.2 |  | In-band blocking for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E11 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.1A.3 |  | In-band blocking for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E12 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  | E12 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.1A.4 |  | In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.1A.5 |  | In-band blocking for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.6.1A.7 |  | In-band blocking for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.6.1A.8 | In-band blocking for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.1B |  | In-band blocking for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.6.1D.1 |  | In-band blocking for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.6.1D.2 |  | In-band blocking for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.6.1E |  | In-band blocking for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.6.1EA |  | In-band blocking for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.6.1F |  | In-band blocking for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 7.6.1G.1 |  | In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.6.2 |  | Out of-band blocking |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.6.2_1 |  | Out of-band blocking with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.6.2A.1 |  | Out of-band blocking for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.2A.2 |  | Out of-band blocking for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E08 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.2A.3 |  | Out of-band blocking for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E10 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  | E12 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.2A.4 |  | Out of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.2A.5 |  | Out-of-band blocking for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.6.2A.7 |  | Out-of-band blocking for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.6.2A.8 | Out-of-band blocking for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.2B |  | Out-of-band blocking for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.6.2D.1 |  | Out-of-band blocking for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.6.2D.2 |  | Out-of-band blocking for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.6.2E |  | Out of-band blocking for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.6.2EA |  | Out of-band blocking for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.6.2F |  | Out-of-band blocking for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 7.6.2G.1 |  | Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.6.3 |  | Narrow band blocking |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.6.3_1 |  | Out of-band blocking with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.6.3A.1 |  | Narrow band blocking for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.3A.2 |  | Narrow band blocking for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E08 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.3A.3 |  | Narrow band blocking for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E10 |  |  |  | FDD_2Rx. FDD_4Rx. TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band DL CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.3A.4 |  | Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.6.3A.5 |  | Narrow band blocking for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.6.3A.7 |  | Narrow band blocking for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | TBD |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.6.3A.8 | Narrow band blocking for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.3B |  | Narrow band blocking for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.6.3D.1 |  | Narrow band blocking for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.6.3D.2 |  | Narrow band blocking for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.6.3E |  | Narrow band blocking for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.6.3EA |  | Narrow band blocking for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.7 |  | Spurious response |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.7_1 |  | Spurious response with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.7A.1 |  | Spurious response for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.7A.2 |  | Spurious response for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E08 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.7A.3 |  | Spurious response for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E10 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  | E12 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7A.4 |  | Spurious response for CA (intra-band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.7A.5 |  | Spurious response for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.7A.8 | Spurious response for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7A.7 |  | Spurious response for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.7B |  | Spurious response for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.7D.1 |  | Spurious response for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.7D.2 |  | Spurious response for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.7E |  | Spurious response for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.7EA |  | Spurious response for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.7F |  | Spurious response for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 7.7G.1 |  | Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.8.1 |  | Wide band Intermodulation |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.8.1_1 |  | Wide band Intermodulation with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.8.1A.1 |  | Wide band Intermodulation for CA (intra-band contiguous DL CA and UL CA) |  |  |  |  | Rel-10 |  |  | C19 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  |  |  |  |  | E01 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.8.1A.2 |  | Wide band Intermodulation for CA (intra-band contiguous DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C20 |  |  |  |  |  | UE supporting E-UTRA and intra-band contiguous DL CA |  |  |  |  |  | E11 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.8.1A.3 |  | Wide band Intermodulation for CA (inter-band DL CA without UL CA) |  |  |  |  | Rel-10 |  |  | C21 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA |  |  |  |  |  | E12 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C146 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C207 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with FDD-TDD inter-band CA under FS3 |  |  |  |  |  | E12 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C208 |  |  |  |  |  | UE supporting E-UTRA and 2DL CA with TDD-TDD inter-band CA under FS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.8.1A.4 |  | Wide band Intermodulation for CA (intra-band non-contiguous DL CA without UL CA) |  |  |  |  | Rel-11 |  |  | C43 |  |  |  |  |  | UE supporting E-UTRA and intra-band non-contiguous DL CA but no UL CA |  |  |  |  |  | E09 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.8.1A.5 |  | Wideband intermodulation for 3DL CA |  |  |  |  | Rel-10 |  |  | C121 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C122 |  |  |  |  |  | UE supporting E-UTRA and 3DL with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C123 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C268 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-13 |  |  | C269 |  |  |  |  |  | UE supporting E-UTRA and 3DL CA with CA configurations in Table 4.1-3 under FS3 |  |  |  |  |  | E07 |  |  |  |  |  |  |  |
| 7.8.1A.7 |  | Wideband intermodulation for 4DL CA |  |  |  |  | Rel-11 |  |  | C187 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |
|  |  |  |  |  |  |  | Rel-11 |  |  | C211 |  |  |  |  |  | UE supporting E-UTRA and 4DL with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Rel-12 |  |  | C188 |  |  |  |  |  | UE supporting E-UTRA and 4DL CA with CA configurations in Table 4.1-4 |  |  |  |  |  | E14 |  |  |  |  |  |  |  |
| 7.8.1A.8 | Wideband intermodulation for 5DL CA |  |  |  |  | Rel-11 |  |  | C221 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL with CA configurations in Table 4.1-5 |  |  |  |  |  | E15 |  |  | FDD 2Rx, FDD 4Rx, TDD 2Rx, TDD 4Rx, FDD-TDD_2Rx, FDD-TDD_4Rx |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C222 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C223 |  |  |  |  |  |  | UE supporting E-UTRA and 5DL CA with CA configurations in Table 4.1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.8.1B |  | Wide band intermodulation for UL-MIMO |  |  |  |  | Rel-10 |  |  | C07 |  |  |  |  |  | UE supporting E-UTRA and UL_MIMO |  |  |  |  |  | D05 |  |  |  | FDD, TDD |  |  |  |
| 7.8.1D.1 |  | Wide band Intermodulation for ProSe Direct Discovery |  |  |  |  | Rel-12 |  |  | C163 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct discovery |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.8.1D.2 |  | Wide band Intermodulation for ProSe Direct Communication |  |  |  |  | Rel-12 |  |  | C162 |  |  |  |  |  | UE supporting E-UTRA and ProSe direct communication |  |  |  |  |  | D10 |  |  |  | FDD, TDD |  |  |  |
| 7.8.1E |  | Wide band Intermodulation for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.8.1EA |  | Wide band Intermodulation for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.8.1F |  | Wide band Intermodulation for category NB1 |  |  |  |  | Rel-13 |  |  | C112b |  |  |  |  |  | UE supporting category NB1 |  |  |  |  |  | D12, D13 |  |  |  | HD-FDD |  |  |  |
| 7.8.1G.1 |  | Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
| 7.9 |  | Spurious emissions |  |  |  |  | Rel-8 |  |  | C113 |  |  |  |  |  | UE supporting E-UTRA |  |  |  |  |  | D15 |  |  |  | FDD, TDD |  |  |  |
| 7.9_1 |  | Spurious emissions with 4 Rx antenna ports |  |  |  |  | Rel-10 |  |  | C113a |  |  |  |  |  | UE supporting E-UTRA with 4Rx antenna ports |  |  |  |  |  | D09 |  |  |  | FDD, TDD |  |  |  |
| 7.9A |  | Spurious emissions for CA |  |  |  |  | Rel-10 |  |  | C120 |  |  |  |  |  | UE supporting E-UTRA and inter-band DL CA with a DL-only band |  |  |  |  |  | E13 |  |  |  | FDD_2Rx, FDD_4Rx, TDD_2Rx, TDD_4Rx |  |  |  |
| 7.9E |  | Spurious emissions for UE category 0 |  |  |  |  | Rel-12 |  |  | C112 |  |  |  |  |  | UE supporting E-UTRA (UE category 0) |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.9EA |  | Spurious emissions for UE category M1 |  |  |  |  | Rel-13 |  |  | C112a |  |  |  |  |  | UE supporting E-UTRA and UE category M1 |  |  |  |  |  | D01 |  |  |  | FDD, HD-FDD, TDD |  |  |  |
| 7.9G.1 |  | Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  | Rel-14 |  |  | C313 |  |  |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  | D14 |  |  |  | TDD |  |  |  |
|  | Performance Requirement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.1.1 | FDD PDSCH Single Antenna Port Performance |  |  |  |  | Rel-8 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.1 or 8.2.1.1.1_A.2 is executed. |  |
| 8.2.1.1.1_1 | FDD PDSCH Single Antenna Port Performance (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C31 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE categories 1, 2) |  |  |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.1.1.1_A.1 or 8.2.1.1.1_A.2 is executed. |  |
| 8.2.1.1.1_A.1 | FDD PDSCH Single Antenna Port Performance for CA (2 DL CA) |  |  |  |  | Rel-10 |  |  | C102 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 2) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.2 or 8.2.1.1.1_A.3 or 8.2.1.1.1_A.4 or 8.13.1.2.2 or 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C103 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Downlink Intra-band non-contiguous CA (UE Category >= 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.1.1_A.2 | FDD PDSCH Single Antenna Port Performance for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C124 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.1.1_A.3 or 8.2.1.1.1_A.4 or 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C125 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.1.1.1_A. 3 | FDD PDSCH Single Antenna Port Performance for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C214 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 4DL CA configurations in Table 4.1-4 (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.2.1.1.1_A.4 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |  |
| 8.2.1.1.1_A. 4 | FDD PDSCH Single Antenna Port Performance for CA (5DL CA) |  |  |  |  | Rel-11 |  |  | C215 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.1.2.5 is executed. |  |
|  |  |  |  |  |  | Rel-12 |  |  | C216 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.1.1.2 | FDD PDSCH Single Antenna Port Performance with 1 PRB in presence of MBSFN |  |  |  |  | Rel-8 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.1 | FDD PDSCH Transmit Diversity 2x2 |  |  |  |  | Rel-8 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.1_1 | FDD PDSCH Transmit Diversity 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C15 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 1) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.2 | FDD PDSCH Transmit Diversity 4x2 |  |  |  |  | Rel-8 |  |  | C09 |  |  |  |  |  |  | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.2_1 | FDD PDSCH Transmit Diversity 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.3_C.1 | FDD PDSCH Transmit diversity 2x2 for eICIC (non-MBFSN ABS) |  |  |  |  | Rel-10 |  |  | C29 |  |  |  |  |  |  | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.3_E.1 | FDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) |  |  |  |  | Rel-11 |  |  | C77 |  |  |  |  |  |  | UE supporting E-UTRA FDD and CRS interference handling (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.4 | FDD PDSCH Transmit Diversity 2x2 with TM3 Interference Model -- Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C44 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.5 | FDD PDSCH Transmit Diversity 2x2 with TM2 Interference Model -- Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C150 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.2.6 | FDD PDSCH Transmit Diversity 2x2 with TM9 Interference Model -- Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C150 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 |  |  |  |  | Rel-8 |  |  | C13b |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE categories >=2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.1.3.1_A.1 or 8.2.1.3.1_A.2 is executed. |  |
| 8.2.1.3.1_1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 (Release 11 and forward) |  |  |  |  | Rel-11 |  |  | C13b |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE categories >=2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.1.3.1_A.1 or 8.2.1.3.1_A.2 is executed. |  |
| 8.2.1.3.1_A.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2 DL CA) |  |  |  |  | Rel-10 |  |  | C101 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >=2) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | If 8.2.1.3.1_A.2 is executed for a CA capability, test execution is not necessary for that CA capability. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C90 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.1_A.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C124 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  | Rel-11 |  |  | C125 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.1.3.1_A.3 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C214 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 4DL CA configurations in Table 4.1-4 (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.1_A.4 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) |  |  |  |  | Rel-11 |  |  | C215 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C216 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL CA configurations in Table 4.1-5 (UE Category 8, >= 11) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.1A_A.1 | FDD Soft buffer management test for CA (2 DL CA) |  |  |  |  | Rel-10 |  |  | C104 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE category 3 and 4) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  | Rel-11 |  |  | C106 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Downlink Intra-band non-contiguous CA (UE categories 3 and 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.1B | FDD PDSCH Open Loop Spatial Multiplexing 2x2 -- Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C142 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.1C | FDD PDSCH Open Loop Spatial Multiplexing 2x2 with TM1 Interference -- Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C142 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each ""Test Number"" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.2 | FDD PDSCH Open Loop Spatial Multiplexing 4x2 |  |  |  |  | Rel-8 |  |  | C13 b |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE categories >=2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.3_C.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (non-MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C29 |  |  |  |  |  |  | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.3_C.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C29 |  |  |  |  |  |  | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.3.3_E.1 | FDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C77 |  |  |  |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 |  |  |  |  | Rel-8 only |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.1_1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.1_E.1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C77 |  |  |  |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.1_H | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C01h |  |  |  |  |  |  | UE supporting E-UTRA FDD and 256QAM in DL |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.2 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 |  |  |  |  | Rel-8 only |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.1.4.2_A.1 or 8.2.1.4.2_A.2 is executed. |  |
| 8.2.1.4.2_1 | FDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.1.4.2_A.1 or 8.2.1.4.2_A.2 is executed. |  |
| 8.2.1.4.2_A.1 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (2 DL CA) |  |  |  |  | Rel-10 |  |  | C102 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 2) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.4.2_A.2 or 8.2.1.4.2_A.3 or 8.2.1.4.2_A.4 or 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C103 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.2_A.2 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C124 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.4.2_A.3 or 8.2.1.4.2_A.4 or 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C125 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.2_A.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C212 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >=≥ 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.1.4.2_A.4 or 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |  |
| 8.2.1.4.2_A.4 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) |  |  |  |  | Rel-11 |  |  | TBD |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >=≥ 11) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.13.1.1.1.2 or 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |  |
|  |  |  |  |  |  | Rel-12 |  |  | TBD |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= ≥ 11) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.2A | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 - Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C142 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | 8.2.1.4.2A |  |
| 8.2.1.4.3 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference model - Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C44 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.4.3A | FDD PDCSH Closed Loop Multi-Layer Spatial Multiplexing 4X2 for Dual Connectivity |  |  |  |  | Rel-12 |  |  | C169 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Dual Connectivity (UE Category >= 3) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.1.4.4 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C150 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.1.7_A.1 | FDD Carrier aggregation with power imbalance (intra-band contiguous DL CA) |  |  |  |  | Rel-10 |  |  | C22 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.1.9 | FDD PDSCH in HST-SFN scenario |  |  |  |  | Rel-14 |  |  | C299 |  |  |  |  |  |  | UEs supporting E-UTRA FDD and high speed enhancement for measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.1 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.1.1 | TDD PDSCH Single Antenna Port Performance |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.2.1.1_A.1 or 8.2.2.1.1_A.2 is executed. |  |
| 8.2.2.1.1_1 | TDD PDSCH Single Antenna Port Performance (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C54 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE categories 1, 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.2.1.1_A.1 or 8.2.2.1.1_A.2 is executed. |  |
| 8.2.2.1.1_A.1 | TDD PDSCH Single Antenna Port Performance for CA (2DL CA) |  |  |  |  | Rel-10 |  |  | C110 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or interband DL CA (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.2.1.1_A.2 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C109 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Intra-band non-contiguous DL CA(UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.1.1_A.2 | TDD PDSCH Single Antenna Port Performance for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C128 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with i CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  | Rel-11 |  |  | C129 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.2.1.1_A.3 | TDD PDSCH Single Antenna Port Performance for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C194 |  |  |  |  |  |  | CA configurations in Table 4.1-4 (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.2.1.1_A.4 | TDD PDSCH Single Antenna Port Performance for CA (5DL CA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.1.2 | TDD PDSCH Single Antenna Port Performance with 1PRB in the presence of MBSFN |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.1 | TDD PDSCH Transmit Diversity 2x2 |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.1_1 | TDD PDSCH Transmit Diversity 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C16 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 1) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.2 | TDD PDSCH Transmit Diversity 4x2 |  |  |  |  | Rel-8 |  |  | C10 |  |  |  |  |  |  | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.2_1 | TDD PDSCH Transmit Diversity 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.3_C.1 | TDD PDSCH Transmit diversity 2x2 for eICIC (non-MBFSN ABS) |  |  |  |  | Rel-10 |  |  | C30 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.3_E.1 | TDD PDSCH Transmit diversity 2x2 for feICIC (non-MBFSN ABS) |  |  |  |  | Rel-11 |  |  | C78 |  |  |  |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.4 | TDD PDSCH Transmit Diversity 2x2 with TM3 Interference Model - Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C45 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.5 | TDD PDSCH Transmit Diversity 2x2 for eIMTA (when EIMTA-MainConfigServCell-r12 is configured) |  |  |  |  | Rel-12 |  |  | C274 |  |  |  |  |  |  | UE supporting E-UTRA TDD and eIMTA TDD UL-DL reconfiguration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.6 | TDD PDSCH Transmit Diversity 2x2 with TM2 Interference Model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C151 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.7 | TDD PDSCH Transmit Diversity 2x2 with TM9 Interference Model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C151 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.3 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.2.3.1_A.1 or .2 is executed. |  |
| 8.2.2.3.1_1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 (Release 11 and forward) |  |  |  |  | Rel-11 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.2.3.1_A.1 or .2 is executed. |  |
| 8.2.2.3.1_A.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (2DL CA) |  |  |  |  | Rel-10 |  |  | C110 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or interband DL CA (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | If 8.2.2.3.1_A.2 is executed for a CA capability, test execution is not necessary for that CA capability |  |
|  |  |  |  |  |  | Rel-11 |  |  | C109 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1_A.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C128 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  | Rel-11 |  |  | C129 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.2.3.1_A.3 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA(4DL CA) |  |  |  |  | Rel-11 |  |  | C194 |  |  |  |  |  |  | UE supporting 4DL CA configurations in Table 4.1-4.(UE category >=8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1_A.4 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for CA (5DL CA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1A_A.1 | TDD Soft buffer management for CA (2 DL CA) |  |  |  |  | Rel-10 |  |  | C105 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE category 3 and 4) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | TBD |  |
|  |  |  |  |  |  | Rel-11 |  |  | C72 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE category 3 and 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1B | TDD PDSCH Open Loop Spatial Multiplexing 2x2 - Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C143 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.1C | TDD PDSCH Open Loop Spatial Multiplexing 2x2 with TM1 Interference - Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C143 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.2 | TDD PDSCH Open Loop Spatial Multiplexing 4x2 |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.3_C.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (non-MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C30 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.3_C.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for eICIC (MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C30 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.3.3_E.1 | TDD PDSCH Open Loop Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C78 |  |  |  |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.2.2.4 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.1 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 |  |  |  |  | Rel-8 only |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.1_1 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.1_E.1 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C78 |  |  |  |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.1_H | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C02h |  |  |  |  |  |  | UE supporting E-UTRA TDD and 256QAM in DL |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.2 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 |  |  |  |  | Rel-8 only |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.2.4.2_A.1 or 8.2.2.4.2_A.2 is executed. |  |
| 8.2.2.4.2_1 | TDD PDSCH Closed Loop Single/Multi Layer Spatial Multiplexing 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | Test execution not necessary if 8.2.2.4.2_A.1 or 8.2.2.4.2_A.2 is executed. |  |
| 8.2.2.4.2_A.1 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (2DL CA) |  |  |  |  | Rel-10 |  |  | C110 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.2.4.2_A.2 or 8.2.2.4.2_A.3 or 8.13.2.1.1.2 or 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C109 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Intra-band non-contiguous DL CA(UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.2_A.2 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C128 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.2.2.4.2_A.3 or 8.13.2.1.1.2 or 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C129 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.2_A.3 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C194 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 4DL CA configurations in Table 4.1-4 (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.13.2.1.1.2 or 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |  |
| 8.2.2.4.2_A.4 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for CA (5DL CA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.2A | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 2x2 - Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C143 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.3 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model - Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C45 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.4.4 | TDD PDSCH Closed Loop Multi-Layer Spatial Multiplexing 4x2 for Dual Connectivity |  |  |  |  | Rel-12 |  |  | C170 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Dual Connectivity (UE Category >= 5) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.2.4.5 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x2 with TM4 Interference Model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C151 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.2.7_A.1 | TDD Carrier aggregation with power imbalance (intra-band contiguous DL CA) |  |  |  |  | Rel-10 |  |  | C24 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.2.9 | TDD PDSCH in HST-SFN scenario |  |  |  |  | Rel-14 |  |  | C300 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and high speed enhancement for measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.3.1.1.1 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD Pcell (2DL CA) |  |  |  |  | Rel-12 |  |  | C154 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.1.1.2 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C133 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.1.1.3 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C133a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.2.3.1.1.4 | TDD FDD CA PDSCH Single Antenna Port Performance for FDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C133b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category8, and Category 11 and onwards) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.3.1.2.1 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell(2DL CA) |  |  |  |  | Rel-12 |  |  | C155 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.1.2.2 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C135 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.1.2.3 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C135a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.2.3.1.2.4 | TDD FDD CA PDSCH Single Antenna Port Performance for TDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C135b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.2.3.2.1.1 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C154 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.2.1.2 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C133 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.2.1.3 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C133a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.3.2.1.4 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for FDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C133b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.3.2.1A | TDD FDD CA PDSCH Soft buffer management test for FDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C136 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE categories 3 and 4) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.2.2.1 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C155 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.2.2.2 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C135 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.2.2.3 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell(4DL CA) |  |  |  |  | Rel-12 |  |  | C135a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.3.2.2.4 | TDD FDD CA PDSCH Open Loop Spatial Multiplexing 2x2 for TDD PCell(5DL CA) |  |  |  |  | Rel-12 |  |  | C135b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.2.3.2.2A | TDD FDD CA PDSCH Soft buffer management test for TDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C137 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD PCell (UE categories 3 and 4) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.1.1 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C154 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.1.2 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C133 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.2.3.3.1.3 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C133a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.1.4 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for FDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C133b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.2.1 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C155 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.2.2 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C135 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.2.3 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C135a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.3.3.2.4 | TDD FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x2 for TDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C135b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.2.4.1.1 | LAA PDSCH CA Closed Loop Spatial Multiplexing Performance-4 Tx Antenna port with FDD as Pcell |  |  |  |  | Rel-13 |  |  | C209 |  |  |  |  |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.2.4.1.2 | LAA PDSCH CA Closed Loop Spatial Multiplexing Performance-4 Tx Antenna port with TDD as Pcell |  |  |  |  | Rel-13 |  |  | C210 |  |  |  |  |  |  | UE supporting E-UTRA TDD and downlink LAA |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.1_D | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO |  |  |  |  | Rel-10 |  |  | C25 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.1_H | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C25h |  |  |  |  |  |  | UE supporting E-UTRA FDD and eDL-MIMO and 256QAM in DL and Feature Group Indicator 103 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.2_D | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with a simultaneous transmission for eDL-MIMO |  |  |  |  | Rel-10 |  |  | C25 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.3 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C40 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indictor 103 and supporting the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.4 | FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C262 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.6 | FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C150 |  |  |  |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.1.7 | FDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM10 serving cell configuration and TM9 interference model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C175 |  |  |  |  |  |  | UE supporting E-UTRA FDD, enhanced performance requirements type B and PDSCH Transmission mode 10 for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.2.1_D | FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO |  |  |  |  | Rel-10 |  |  | C25 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.2.1_D_1 | FDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO (Release 11 and forward) |  |  |  |  | Rel-11 |  |  | C25 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.2.2 | FDD PDSCH Dual-layer Spatial Multiplexing - Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C144 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.3.1_F | FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and single NZP CSI-RS resource for CoMP |  |  |  |  | Rel-11 |  |  | C50 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.3.2_F | FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP |  |  |  |  | Rel-11 |  |  | C52 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.1.3.3_F | FDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS resource for CoMP |  |  |  |  | Rel-11 |  |  | C117 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 8 and forward) |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.1_1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 5 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C16 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 1) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.2 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 without a simultaneous transmission |  |  |  |  | Rel-9 only |  |  | C34 |  |  |  |  |  |  | UE supporting E-UTRA TDD and supporting enhanced dual layer TDD. |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-10 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD. |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.2_D | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO |  |  |  |  | Rel-10 |  |  | C26 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.2_H | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 without a simultaneous transmission for eDL-MIMO for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C26h |  |  |  |  |  |  | UE supporting E-UTRA TDD and 256QAM in DL and Feature Group Indicator 104 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.3 | TDD PDSCH Single-layer Spatial Multiplexing on antenna port 7 or 8 with a simultaneous transmission |  |  |  |  | Rel-9 only |  |  | C34 |  |  |  |  |  |  | UE supporting E-UTRA TDD and supporting enhanced dual layer TDD. |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-10 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD. |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.3_D | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with a simultaneous transmission for eDL-MIMO |  |  |  |  | Rel-10 |  |  | C26 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.4 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C41 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Feature Group Indictor 103 and supporting the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.5 | TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C263 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.7 | TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM3 interference model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C151 |  |  |  |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.1.8 | TDD PDSCH Closed Loop Single-layer Spatial Multiplexing on antenna ports 7 or 8 with TM10 serving cell configuration and TM9 interference model - Enhanced Performance Requirement Type B |  |  |  |  | Rel-12 |  |  | C176 |  |  |  |  |  |  | UE supporting E-UTRA TDD, enhanced performance requirements type B and PDSCH Transmission mode 10 for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.2.1 | TDD PDSCH Dual-layer Spatial Multiplexing |  |  |  |  | Rel-9 only |  |  | C34 |  |  |  |  |  |  | UE supporting E-UTRA TDD and supporting enhanced dual layer TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-10 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.2.1_D | TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO |  |  |  |  | Rel-10 |  |  | C25a |  |  |  |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.2.1_D_1 | TDD PDSCH Dual-layer Spatial Multiplexing for eDL-MIMO (Release 11 and forward) |  |  |  |  | Rel-11 |  |  | C25a |  |  |  |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.3.2.2.2 | TDD PDSCH Dual-layer Spatial Multiplexing - Enhanced Performance Requirement Type C |  |  |  |  | Rel-12 |  |  | C143 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Enhanced Performance Requirement TypeC for LTE (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.4.1_F | TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and single NZP CSI-RS resource for CoMP |  |  |  |  | Rel-11 |  |  | C51 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.4.2_F | TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Same Cell ID and multiple NZP CSI-RS resources for CoMP |  |  |  |  | Rel-11 |  |  | C53 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.2.4.3_F | TDD PDSCH Performance with DCI format 2D, non Quasi Co-located Antenna Ports, Different Cell ID, Colliding CRS and single NZP CSI-RS resource for CoMP |  |  |  |  | Rel-11 |  |  | C118 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.3.1.1 | LAA Dual-Layer Spatial Multiplexing with DM-RS with FDD as PCell |  |  |  |  | Rel-13 |  |  | C264 |  |  |  |  |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and TM9 on LAA cells |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.3.3.1.2 | LAA Dual-Layer Spatial Multiplexing with DM-RS with TDD as Pcell |  |  |  |  | Rel-13 |  |  | C265 |  |  |  |  |  |  | UE supporting E-UTRA TDD and downlink LAA and TM9 on LAA cells |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.1 | FDD PCFICH/PDCCH Single-antenna Port Performance |  |  |  |  | Rel-8 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 |  |  |  |  | Rel-8 only |  |  | C09 |  |  |  |  |  |  | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.1_1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.2 | FDD PCFICH/PDCCH Transmit Diversity 4x2 |  |  |  |  | Rel-8 only |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.3_E.1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C77 |  |  |  |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.3_E.2 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C77 |  |  |  |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.2_1 | FDD PCFICH/PDCCH Transmit Diversity 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.3_C.1 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C29 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.3_C.2 | FDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C29 |  |  |  |  |  |  | UEs supporting E-UTRA FDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.5 | FDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port under Asynchronous Network |  |  |  |  | Rel-13 |  |  | C285 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.6 | FDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C285 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.7 | FDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C286 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.1.2.8 | FDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C286 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.2.1 | TDD PCFICH/PDCCH Single-antenna Port Performance |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 |  |  |  |  | Rel-8 only |  |  | C10 |  |  |  |  |  |  | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.1_1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.2 | TDD PCFICH/PDCCH Transmit Diversity 4x2 |  |  |  |  | Rel-8 only |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.2_1 | TDD PCFICH/PDCCH Transmit Diversity 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.3_C.1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C30 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.3_C.2 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for eICIC (MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C30 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.3_E.1 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C78 |  |  |  |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.3_E.2 | TDD PCFICH/PDCCH Transmit Diversity 2x2 for feICIC (MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C78 |  |  |  |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.5 | TDD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C287 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.6 | DD Enhanced Downlink Control Channel Performance Type A for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C287 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.7 | TDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C288 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.2.2.8 | TDD Enhanced Downlink Control Channel Performance Type B for PCFICH/PDCCH, 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C288 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4.3.1.1 | LAA PCFICH/PDCCH Transmit Diversity 2x2 with FDD as Pcell |  |  |  |  | Rel-13 |  |  | C209 |  |  |  |  |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.4.3.1.2 | LAA PCFICH/PDCCH Transmit Diversity 2x2 with TDD as Pcell |  |  |  |  | Rel-13 |  |  | C217 |  |  |  |  |  |  | UE supporting E-UTRA TDD and downlink LAA |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.1 | FDD PHICH Single-antenna Port Performance |  |  |  |  | Rel-8 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.1 | FDD PHICH Transmit Diversity 2x2 |  |  |  |  | Rel-8 only |  |  | C09 |  |  |  |  |  |  | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.1_1 | FDD PHICH Transmit Diversity 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.2 | FDD PHICH Transmit Diversity 4x2 |  |  |  |  | Rel-8 only |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.2_1 | FDD PHICH Transmit Diversity 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C01 |  |  |  |  |  |  | UE supporting E-UTRA FDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.3_C.1 | FDD PHICH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C29 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.3_E.1 | FDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C77 |  |  |  |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.5 | FDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH- 2 Tx Antenna Ports under Asynchronous Network |  |  |  |  | Rel-13 |  |  | C285 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.6 | FDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C285 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.7 | FDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C286 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.8 | FDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C286 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.2.1 | TDD PHICH Single-antenna Port Performance |  |  |  |  | Rel-8 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2 | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.1 | TDD PHICH Transmit Diversity 2x2 |  |  |  |  | Rel-8 only |  |  | C10 |  |  |  |  |  |  | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.1_1 | TDD PHICH Transmit Diversity 2x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.2 | TDD PHICH Transmit Diversity 4x2 |  |  |  |  | Rel-8 only |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.2_1 | TDD PHICH Transmit Diversity 4x2 (Release 9 and forward) |  |  |  |  | Rel-9 |  |  | C02 |  |  |  |  |  |  | UE supporting E-UTRA TDD |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.3_C.1 | TDD PHICH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) |  |  |  |  | Rel-10 |  |  | C30 |  |  |  |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.3_E.1 | TDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) |  |  |  |  | Rel-11 |  |  | C78 |  |  |  |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.5 | TDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH- 2 Tx Antenna Ports with Colliding CRS Dominant Interfere |  |  |  |  | Rel-13 |  |  | C287 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.6 | TDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C287 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.7 | TDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH- 2 Tx Antenna Ports with Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C288 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.8 | TDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH- 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  |  |  |  | Rel-13 |  |  | C288 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.1.1 | FDD sustained data rate performance (Rel-9 and forward) |  |  |  |  | Rel-9 |  |  | C76 |  |  |  |  |  |  | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories from1 to 4) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1 or 8.7.3.1 is executed. |  |
| 8.7.1.1_1 | FDD sustained data rate performance (Rel-10 and forward) |  |  |  |  | Rel-10 |  |  | C42 |  |  |  |  |  |  | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories 6, 7) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1or 8.7.3.1 is executed. |  |
| 8.7.1.1_2 | FDD sustained data rate performance for UE category 1bis |  |  |  |  | Rel-13 |  |  | C145d |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.7.1.1_A.1 | FDD Sustained data rate performance for CA (2 DL CA ) |  |  |  |  | Rel-10 |  |  | C107 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.2 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C93 |  |  |  |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.1.1_A.2 | FDD Sustained data rate performance for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C126 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.4 is executed |  |
|  |  |  |  |  |  | Rel-10 |  |  | C126a |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-11 |  |  | C127 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-11 |  |  | C127a |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.7.1.1_A.4 | FDD Sustained data rate performance for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C189 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and not supporting 256QAM in DL (UE category 11 and 12) |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_A.5 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C189a |  |  |  |  |  |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting at most 60MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 9 and 10) |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_A.5 is executed. |  |
| 8.7.1.1_A.5 | FDD Sustained data rate performance for CA (5DL CA) |  |  |  |  | Rel-11 |  |  | C266 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Rel-12 |  |  | C267 |  |  |  |  |  |  | UE supporting E-UTRA FDD and 5DL CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -8.7.1.1_H.1 | FDD sustained data rate performance (Single Carrier) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C42h |  |  |  |  |  |  | UE supporting E-UTRA FDD and 256QAM and UE DL category 13 |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_H.2 is executed |  |
| 8.7.1.1_H.2 | FDD Sustained data rate performance for CA (2DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C107h |  |  |  |  |  |  | UE supporting E-UTRA FDD and 2DL CA and 256QAM in DL (UE DL category 11, 12 and 13) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.3 is executed |  |
| 8.7.1.1_H.3 | FDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C126h |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL (UE DL category 11, 12 and 15) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.4 is executed |  |
|  |  |  |  |  |  | Rel-12 |  |  | C126ha |  |  |  |  |  |  | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL and supporting at most 40MHz aggregated bandwidth (UE DL category 13) |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_H.4 is executed |  |
| 8.7.2.1 | TDD sustained data rate performance (Rel-9 and forward) |  |  |  |  | Rel-9 |  |  | C111 |  |  |  |  |  |  | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE categories from 1 to 4) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1 or 8.7.4.1 is executed. |  |
| 8.7.2.1_1 | TDD sustained data rate performance (Rel-10 and forward) |  |  |  |  | Rel-10 |  |  | C73 |  |  |  |  |  |  | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE category 6 and 7) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1or 8.7.4.1 is executed. |  |
| 8.7.2.1_2 | TDD sustained data rate performance for UE category 1bis |  |  |  |  | Rel-13 |  |  | C156f |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.7.2.1_A.1 | TDD sustained data rate performance for CA (2DL CA) |  |  |  |  | Rel-10 |  |  | C74 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.2 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C75 |  |  |  |  |  |  | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.2.1_A.2 | TDD Sustained data rate performance for CA (3DL CA) |  |  |  |  | Rel-10 |  |  | C130 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.3 is executed. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C131 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.7.2.1_A.3 | TDD Sustained data rate performance for CA (4DL CA) |  |  |  |  | Rel-11 |  |  | C213 |  |  |  |  |  |  | UE supporting E-UTRA TDD and 4DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.2.1_A.4 is executed. |  |
| 8.7.2.1_H.1 | TDD sustained data rate performance (Single Carrier) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C73h |  |  |  |  |  |  | UE supporting E-UTRA TDD and 256QAM in DL and UE DL category 13 |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.2.1_H.2 is executed. |  |
| 8.7.2.1_H.2 | TDD sustained data rate performance for CA (2DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C74h |  |  |  |  |  |  | UE supporting E-UTRA TDD and 2DL CA, and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.3 is executed. |  |
| 8.7.2.1_H.3 | TDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C130h |  |  |  |  |  |  | UE supporting E-UTRA TDD and 3DL CA and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.4 is executed. |  |
| 8.7.3.1 | FDD sustained data rate performance for EPDCCH scheduling |  |  |  |  | Rel-11 |  |  | C55 |  |  |  |  |  |  | UE supporting E-UTRA FDD and EPDCCH |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.7.4.1 | TDD sustained data rate performance for EPDCCH scheduling |  |  |  |  | Rel-11 |  |  | C56 |  |  |  |  |  |  | UE supporting E-UTRA TDD and EPDCCH |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.7.5.1.1 | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C138 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.2 is executed. |  |
| 8.7.5.1.2 | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C139 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.3 is executed. |  |
| 8.7.5.1.3 | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C139a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  |  |  | TBD |  |  |  |  |  |  |  | Test execution not necessary if 8.7.5.1.4 is executed. |  |
| 8.7.5.1.4 | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C139b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.7.5.1_H.1 | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C138h |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.2 is executed. |  |
| 8.7.5.1_H.2 | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C139h |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.3 is executed. |  |
| 8.7.5.1_H.3 | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C139ha |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |  |  |  |  |  |  | Test execution not necessary if 8.7.5.1_H.4 is executed. |  |
| 8.7.5.1_H.4 | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C139hb |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.5.2.1 | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) |  |  |  |  | Rel-12 |  |  | C140 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.2 is executed. |  |
| 8.7.5.2.2 | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) |  |  |  |  | Rel-12 |  |  | C141 |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  | TBD |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.3 is executed. |  |
| 8.7.5.2.3 | TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) |  |  |  |  | Rel-12 |  |  | C141a |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  |  |  | TBD |  |  |  |  |  |  |  | Test execution not necessary if 8.7.5.2.4 is executed. |  |
| 8.7.5.2.4 | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) |  |  |  |  | Rel-12 |  |  | C141b |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  |  |  | TBD |  |  |  |  |  |  |  |  |  |
| 8.7.5.2_H.1 | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C140h |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 13) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.2 is executed. |  |
| 8.7.5.2_H.2 | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) for 256QAM in DL |  |  |  |  | Rel-12 |  |  | C141h |  |  |  |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  |  |  |  |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.3 is executed. |  |
| 8.7.6.1 | FDD sustained data rate performance for Dual Connectivity 64QAM |  |  |  |  | Rel-12 |  |  | C171 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 3, 4, 6, 7, 9, and 10) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.7.6.2 | FDD sustained data rate performance for Dual Connectivity 256QAM |  |  |  |  | Rel-12 |  |  | C173 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Dual Connectivity and supporting 256QAM in DL |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.7.7.1 | TDD sustained data rate performance for Dual Connectivity 64QAM |  |  |  |  | Rel-12 |  |  | C172 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 6, 7, 9, and 10) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.7.7.2 | TDD sustained data rate performance for Dual Connectivity 256QAM |  |  |  |  | Rel-12 |  |  | C174 |  |  |  |  |  |  | UE supporting E-UTRA TDD and Dual Connectivity and supporting 256QAM in DL |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 8.7.9.2 | FDD sustained data rate performance for 4 layer MIMO (single carrier) |  |  |  |  | Rel-10 |  |  | C226 |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  |  |  | One "Test Number" to be performed. The selected band shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.9.3 | FDD sustained data rate performance for 4 layer MIMO (2DL CA) |  |  |  |  | Rel-10 |  |  | C227 |  |  |  |  |  |  | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  | One "Test Number" to be performed. The selected CA configuration shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C228 |  |  |  |  |  |  | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.9.4 | FDD sustained data rate performance for 4 layer MIMO (3DL CA) |  |  |  |  | Rel-10 |  |  | C229 |  |  |  |  |  |  | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C230 |  |  |  |  |  |  | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 and 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.9.5 | FDD sustained data rate performance for 4 layer MIMO (4DL CA) |  |  |  |  | Rel-11 |  |  | C236 |  |  |  |  |  |  | UE supporting 4DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 15, 16, 18 and 19) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.9.6 | FDD sustained data rate performance for 4 layer MIMO (5DL CA) |  |  |  |  | Rel-11 |  |  | C237 |  |  |  |  |  |  | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  |  |  | Rel-12 |  |  | C238 |  |  |  |  |  |  | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.10.2 | TDD sustained data rate performance for 4 layer MIMO (single carrier) |  |  |  |  | Rel-10 |  |  | C239 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.10.3 | TDD sustained data rate performance for 4 layer MIMO (2DL CA) |  |  |  |  | Rel-10 |  |  | C240 |  |  |  |  |  |  | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C241 |  |  |  |  |  |  | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.10.4 | TDD sustained data rate performance for 4 layer MIMO (3DL CA) |  |  |  |  | Rel-10 |  |  | C242 |  |  |  |  |  |  | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  |  |  | Rel-11 |  |  | C243 |  |  |  |  |  |  | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.10.5 | TDD sustained data rate performance for 4 layer MIMO (4DL CA) |  |  |  |  | Rel-11 |  |  | C244 |  |  |  |  |  |  | UE supporting 4DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 16, 18 and 19) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.10.6 | TDD sustained data rate performance for 4 layer MIMO (5DL CA) |  |  |  |  | Rel-11 |  |  | C245 |  |  |  |  |  |  | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  |  |  | Rel-12 |  |  | C246 |  |  |  |  |  |  | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7.11.2 | TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) |  |  |  |  | Rel-12 |  |  | C247 |  |  |  |  |  |  | UE supporting 2DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.11.3 | TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) |  |  |  |  | Rel-12 |  |  | C248 |  |  |  |  |  |  | UE supporting 3DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.11.4 | TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) |  |  |  |  | Rel-12 |  |  | C249 |  |  |  |  |  |  | UE supporting 4DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 15, 16, 18 and 19) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.11.5 | TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) |  |  |  |  | Rel-12 |  |  | C250 |  |  |  |  |  |  | UE supporting 5DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 16, 18 and 19) |  |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  |  |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.8.1.1 | FDD distributed EPDCCH performance |  |  |  |  | Rel-11 |  |  | C55 |  |  |  |  |  |  | UE supporting E-UTRA FDD and EPDCCH |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.8.1.2 | TDD distributed EPDCCH performance |  |  |  |  | Rel-11 |  |  | C56 |  |  |  |  |  |  | UE supporting E-UTRA TDD and EPDCCH |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.8.2.1 | FDD localized EPDCCH performance with TM9 |  |  |  |  | Rel-11 |  |  | C91 |  |  |  |  |  |  | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.8.2.2 | TDD localized EPDCCH performance with TM9 |  |  |  |  | Rel-11 |  |  | C92 |  |  |  |  |  |  | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.8.3.1 | FDD localized EPDCCH transmission with TM10 Type B quasi co-location type |  |  |  |  | Rel-11 |  |  | C57 |  |  |  |  |  |  | UE supporting E-UTRA FDD and EPDCCH and Multiple CSI processes on a component carrier within a band with PDSCH transmission mode 10 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.8.3.2 | TDD localized EPDCCH transmission with TM10 Type B quasi co-location type |  |  |  |  | Rel-11 |  |  | C58 |  |  |  |  |  |  | UE supporting E-UTRA TDD and EPDCCH and Multiple CSI processes on a component carrier within a band with PDSCH transmission mode 10 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.8.4.1 | FDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Localized Transmission with CRS Interference Model |  |  |  |  | Rel-13 |  |  | C289 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8.4.2 | TDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Localized Transmission with CRS Interference Model |  |  |  |  | Rel-13 |  |  | C290 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8.5.1 | TDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Distributed Transmission with TM9 Interference Model |  |  |  |  | Rel-13 |  |  | C290 |  |  |  |  |  |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8.6.1 | FDD Enhanced Downlink Control Channel Performance Type A for EDPCCH- Distributed Transmission with TM3 Interference Model |  |  |  |  | Rel-13 |  |  | C289 |  |  |  |  |  |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.9.1.1.1 | Transmit diversity performance for UE category 0 (Cell-Specific Reference Symbols) |  |  |  |  | Rel-12 |  |  | C145 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 0) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.1.1_1 | FDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  |  |  |  | Rel-13 |  |  | C145d |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.1.2 | FDD closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) |  |  |  |  | Rel-12 |  |  | C145 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 0) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.1.2_1 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  |  |  |  | Rel-13 |  |  | C145d |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.1.3 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 |  |  |  |  | Rel-12 |  |  | C157 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 0) and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.1.3_1 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis |  |  |  |  | Rel-13 |  |  | C157a |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 1bis) and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.2.1 | TDD PDSCH Transmit Diversity for UE category 0 |  |  |  |  | Rel-12 |  |  | C156 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 0) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.2.1_1 | TDD PDSCH Transmit Diversity for UE category 1bis |  |  |  |  | Rel-13 |  |  | C156f |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.2.2 | TDD closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) |  |  |  |  | Rel-12 |  |  | C145 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 0) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.2.2_1 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  |  |  |  | Rel-13 |  |  | C156f |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.2.3 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 |  |  |  |  | Rel-12 |  |  | C158 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 0) and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.1.2.3_1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis |  |  |  |  | Rel-13 |  |  | C158a |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 1bis) and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.2.1.1 | FDD PHICH Transmit Diversity for UE category 0 |  |  |  |  | Rel-12 |  |  | C145 |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 0) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.2.1.1_1 | FDD PHICH Transmit Diversity for UE category 1bis |  |  |  |  | Rel-13 |  |  | C145d |  |  |  |  |  |  | UE supporting E-UTRA FDD (UE category 1bis) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.9.2.2.1 | TDD PHICH Transmit Diversity for UE category 0 |  |  |  |  | Rel-12 |  |  | C156 |  |  |  |  |  |  | UE supporting E-UTRA TDD (UE category 0) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.1 | FDD PDSCH Transmit Diversity 2x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.3 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C113d |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.4 | FDD PDSCH Closed Loop Spatial Multiplexing 4x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.5 | FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C113e |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.6 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) |  |  |  |  | Rel-10 |  |  | C113c |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 and 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.7 | FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  |  |  |  | Rel-10 |  |  | C220 |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and 3-layer spatial multiplexing |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.8 | FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  |  |  |  | Rel-10 |  |  | C220 |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.1.9 | FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  |  |  |  | Rel-10 |  |  | C113c |  |  |  |  |  |  | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 and 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.1 | TDD PDSCH Transmit Diversity 2x4 |  |  |  |  | Rel-10 |  |  | C198 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x4 |  |  |  |  | Rel-10 |  |  | C198 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.3 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C198 a |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.4 | TDD PDSCH Closed Loop Spatial Multiplexing 4x4 |  |  |  |  | Rel-10 |  |  | C198 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.5 | TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A |  |  |  |  | Rel-11 |  |  | C198 c |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.6 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) |  |  |  |  | Rel-10 |  |  | C198 b |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and Feature Group Indicator 103 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.7 | TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  |  |  |  | Rel-10 |  |  | C235 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and 3-layer spatial multiplexing |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.8 | TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  |  |  |  | Rel-10 |  |  | C235 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.1.2.9 | TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  |  |  |  | Rel-10 |  |  | C183 |  |  |  |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 and 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.2.1.1 | FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.2.1.2 | FDD PCFICH/PDCCH Transmit Diversity Performance 2x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.2.1.3 | FDD PCFICH/PDCCH Transmit Diversity Performance 4x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.2.2.1 | TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  |  |  |  | Rel-10 |  |  | C184 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.2.2.2 | TDD PCFICH/PDCCH Transmit Diversity Performance 2x4 |  |  |  |  | Rel-10 |  |  | C184 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.2.2.3 | TDD PCFICH/PDCCH Transmit Diversity Performance 4x4 |  |  |  |  | Rel-10 |  |  | C184 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.3.1.1 | FDD PHICH Single-antenna Port Performance 1x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.3.1.2 | FDD PHICH Transmit Diversity Performance 2x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.3.1.3 | FDD PHICH Transmit Diversity Performance 4x4 |  |  |  |  | Rel-10 |  |  | C113b |  |  |  |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.3.2.1 | TDD PHICH Single-antenna Port Performance 1x4 |  |  |  |  | Rel-10 |  |  | C184 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.3.2.2 | TDD PHICH Transmit Diversity Performance 2x4 |  |  |  |  | Rel-10 |  |  | C184 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.3.2.3 | TDD PHICH Transmit Diversity Performance 4x4 |  |  |  |  | Rel-10 |  |  | C184 |  |  |  |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.4.1.1 | FDD distributed EPDCCH performance 2x4 |  |  |  |  | Rel-10 |  |  | C164 |  |  |  |  |  |  | UE supporting E-UTRA FDD and EPDCCH with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.4.1.2 | TDD distributed EPDCCH performance 2x4 |  |  |  |  | Rel-10 |  |  | C165 |  |  |  |  |  |  | UE supporting E-UTRA TDD and EPDCCH with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.4.2.1 | FDD localized EPDCCH performance with TM9 2x4 |  |  |  |  | Rel-10 |  |  | C166 |  |  |  |  |  |  | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.10.4.2.2 | TDD localized EPDCCH performance with TM9 2x4 |  |  |  |  | Rel-10 |  |  | C167 |  |  |  |  |  |  | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |
| 8.11.1.1.1 | FDD and half-duplex FDD Closed-loop spatial multiplexing performance for UE category M1 |  |  |  |  |  |  | Rel-13 |  |  |  | C145x |  |  |  | UE supporting E-UTRA FDD and UE category M1 and TM6 in CE Mode A |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.1.2 | FDD and half-duplex FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1 |  |  |  |  |  |  | Rel-13 |  |  |  | C145d |  |  |  | UE supporting E-UTRA FDD and UE category M1 and TM9 in CE Mode A |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.1.3.1 | FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 |  |  |  |  |  |  | Rel-13 |  |  |  | C145a |  |  |  | UE supporting E-UTRA FDD and UE category M1 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.1.3.1_1 | FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEmodeB) |  |  |  |  |  |  | Rel-13 |  |  |  | C156c |  |  |  | UE supporting E-UTRA FDD and (UE category M1 and CE Mode B) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.2.1 | TDD Closed-loop spatial multiplexing performance for UE category M1 (Cell-Specific Reference Symbols) |  |  |  |  |  |  | Rel-13 |  |  |  | C156x |  |  |  | UE supporting E-UTRA TDD and UE category M1 and TM6 in CE Mode A |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.2.2 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category M1 |  |  |  |  |  |  | Rel-13 |  |  |  | C156e |  |  |  | UE supporting E-UTRA TDD and UE category M1 and TM9 in CE Mode A |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.2.3.1 | TDD PDSCH Transmit Diversity for UE category M1 |  |  |  |  |  |  | Rel-13 |  |  |  | C156b |  |  |  | UE supporting E-UTRA TDD and UE category M1 |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.1.2.3.1_1 | TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) |  |  |  |  |  |  | Rel-13 |  |  |  | C156d |  |  |  | UE supporting E-UTRA TDD and UE category M1 and CE Mode B |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
| 8.11.2.1.1 | FDD demodulation of MPDCCH in CE Mode A |  |  |  |  |  |  | Rel-13 |  |  |  | C145b |  |  |  | UE supporting E-UTRA FDD and (UE category M1 or CE Mode A) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-14 |  |  |  | C314 |  |  |  | UE supporting E-UTRA FDD and (UE category M2 or CE Mode A) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.11.2.1.2 | FDD and half-duplex FDD demodulation of MPDCCH in CE Mode B |  |  |  |  |  |  | Rel-13 |  |  |  | C156c |  |  |  | UE supporting E-UTRA FDD and (UE category M1 and CE Mode B) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-14 |  |  |  | C315 |  |  |  | UE supporting E-UTRA FDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.11.2.2.1 | TDD demodulation of MPDCCH in CE Mode A |  |  |  |  |  |  | Rel-13 |  |  |  | C156b |  |  |  | UE supporting E-UTRA TDD and (UE category M1 or UE category M2 or CE Mode A) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-14 |  |  |  | C316 |  |  |  | UE supporting E-UTRA TDD and (UE category M2 or CE Mode A) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.11.2.2.2 | TDD demodulation of MPDCCH in CE Mode B |  |  |  |  |  |  | Rel-13 |  |  |  | C156d |  |  |  | UE supporting E-UTRA TDD and (UE category M1 or UE category M2 and CE Mode B) |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-14 |  |  |  | C317 |  |  |  | UE supporting E-UTRA TDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12.1.1.1 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for Category NB1 |  |  |  |  |  |  | Rel-13 |  |  |  | C112b |  |  |  | UE supporting category NB1 |  |  |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |  |  |  |  |  |  |
| 8.12.1.1.2 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 |  |  |  |  |  |  | Rel-13 |  |  |  | C112b |  |  |  | UE supporting category NB1 |  |  |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |  |  |  |  |  |  |
| 8.12.1.1.3 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 |  |  |  |  |  |  | Rel-14 |  |  |  | C298 |  |  |  | UE supporting category NB2 |  |  |  | Each “Test Number” to be performed once, in a chosen band |  |  |  |  |  |  |  |  |  |
| 8.12.2.1.1 | Demodulation of NPDCCH single-antenna performance for category NB1 |  |  |  |  |  |  | Rel-13 |  |  |  | C112b |  |  |  | UE supporting category NB1 |  |  |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |  |  |  |  |  |  |
| 8.12.2.1.2 | Demodulation of NPDCCH in In-band mode Transmit Diversity performance for Category NB1 |  |  |  |  |  |  | Rel-13 |  |  |  | C112b |  |  |  | UE supporting category NB1 |  |  |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |  |  |  |  |  |  |
| 8.13.1.1.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C278 |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and 4 Rx antenna ports and 256QAM in DL (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C279 |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and 4 Rx antenna ports and 256QAM in DL (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.1.4 | FDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C280 |  |  |  | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.1.1.2 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C253 |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C254 |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.1.1.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C255 |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C256 |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.1.1.4 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C257 |  |  |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.1.1.1.5 is executed. |  |
| 8.13.1.1.1.5 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C258 |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C259 |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.2.2 |  |  | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA) |  |  |  |  |  |  |  | Rel-10 |  | C253 |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  | Test execution not necessary if 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |  |
|  |  |  |  |  |  |  |  |  |  |  | Rel-11 |  | C254 |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.2.3 |  |  | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA) |  |  |  |  |  |  |  | Rel-10 |  | C255 |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  | Test execution not necessary if 8.13.1.2.4 or 8.13.1.2.5 is executed. |  |
|  |  |  |  |  |  |  |  |  |  |  | Rel-11 |  | C256 |  |  |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.2.4 |  |  | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA) |  |  |  |  |  |  |  | Rel-11 |  | C257 |  |  |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  | Test execution not necessary if 8.13.1.2.5 is executed. |  |
| 8.13.1.2.5 |  |  | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA) |  |  |  |  |  |  |  | Rel-11 |  | C258 |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Rel-12 |  | C259 |  |  |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.3.1 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C281 |  |  |  | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1.4.1 | FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C282 |  |  |  | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.1.1.2 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C291 |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C292 |  |  |  | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.1.1.3 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C293 |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C294 |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.1.1.4 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C295 |  |  |  | UE supporting E-UTRA TDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.2.1.1.5 is executed. |  |
| 8.13.2.1.1.5 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C296 |  |  |  | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C297 |  |  |  | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.2.2 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C291 |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.2.2.3 or 8.13.2.2.4 or 8.13.2.2.5 is executed. |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C292 |  |  |  | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.2.3 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA) |  |  |  |  |  |  | Rel-10 |  |  |  | C293 |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.2.2.4 or 8.13.2.2.5 is executed. |  |
|  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C294 |  |  |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.2.4 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C295 |  |  |  | UE supporting E-UTRA TDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.2.2.5 is executed. |  |
| 8.13.2.2.5 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C296 |  |  |  | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C297 |  |  |  | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.3.1 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C283 |  |  |  | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.2.4.1 | TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) |  |  |  |  |  |  | Rel-11 |  |  |  | C284 |  |  |  | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.3.1.1.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C306 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.3.1.1.3 or 8.13.3.1.1.4 or 8.13.3.1.1.5 is executed. |  |
| 8.13.3.1.1.3 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (3DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C307 |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.3.1.1.4 or 8.13.3.1.1.5 is executed. |  |
| 8.13.3.1.1.4 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (4DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C308 |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.3.1.1.5 is executed. |  |
| 8.13.3.1.1.5 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (5DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C309 |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.13.3.1.2.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C310 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.3.1.2.3 or 8.13.3.1.2.4 or 8.13.3.1.2.5 is executed. |  |
| 8.13.3.1.2.3 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (3DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C311 |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.3.1.2.4 or 8.13.3.1.2.5 is executed. |  |
| 8.13.3.1.2.4 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (4DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C312 |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  | Test execution not necessary if 8.13.3.1.2.5 is executed. |  |
| 8.13.3.1.2.5 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (5DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | CXX8 |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) and 4Rx antenna ports |  |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  |  |  |  |  |  |  |
| 8.13.3.3.1 | TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C231 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and FDD as PCell (UE Category >=5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.3.3.2 | TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C232 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and TDD as PCell (UE Category >=5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.3.4.1 | TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A for FDD PCell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C233 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indictor 103 and the enhanced performance requirements type A for LTE and FDD as PCell (UE Category >=5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.3.4.2 | TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A for TDD PCell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C234 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indictor 103 and the enhanced performance requirements type A for LTE and TDD as PCell (UE Category >=5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.3.6.1 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for FDD PCell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C251 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and 256QAM in DL and FDD as Pcell(UE Category >=5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.3.6.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for TDD PCell (2DL CA) |  |  |  |  |  |  | Rel-12 |  |  |  | C252 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and 256QAM in DL and TDD as Pcell(UE Category >=5) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Reporting of Channel State Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C01 |  |  |  | UE supporting E-UTRA FDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.1.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C02 |  |  |  | UE supporting E-UTRA TDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.1.3_C.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C29 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.1.4_C.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C30 |  |  |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.1.5_E.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C77 |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.1.6_E.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C78 |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.1.7 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for 256QAM in DL |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C01h |  |  |  | UE supporting E-UTRA FDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) |  |  |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 9.2.1.8 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for 256QAM in DL |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C02h |  |  |  | UE supporting E-UTRA TDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) |  |  |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 9.2.2.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C13 b |  |  |  | UE supporting E-UTRA FDD (UE categories >=2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.2.2.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C02 |  |  |  | UE supporting E-UTRA TDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.2.3.1_D | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.3.2_D | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C26 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.4.1_F | FDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C117 |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.2.4.2_F | TDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C118 |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.2.5 | TDD CQI Reporting under AWGN conditions – PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C275 |  |  |  | UE supporting E-UTRA TDD and eIMTA TDD UL-DL reconfiguration and Rel-12 CSI subframe sets (UE Category >= 2) |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.6.1 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-0) |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C209 |  |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.6.2 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-0) |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C217 |  |  |  | UE supporting E-UTRA TDD and downlink LAA with TDD as Pcell |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.7.1 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-1) |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C218 |  |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and TM9 on LAA cells |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.2.7.2 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-1) |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C219 |  |  |  | UE supporting E-UTRA TDD and downlink LAA and TM9 on LAA cells |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.1.1.1 | FDD CQI Reporting under fading conditions - PUSCH 3-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C01 |  |  |  | UE supporting E-UTRA FDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.1.1.2 | TDD CQI Reporting under fading conditions - PUSCH 3-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C02 |  |  |  | UE supporting E-UTRA TDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.1.1.5 | TDD CQI Reporting under fading conditions – PUCCH 3-0 (when csi-SubframeSet –r12 is configured) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C276 |  |  |  | UE supporting E-UTRA TDD and Rel-12 CSI subframe sets (UE category >= 1) |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.1.2.1_D | FDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.1.2.2_D | TDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25a |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.1.2.3 | FDD CQI Reporting under fading conditions - PUSCH 3-1 for 256QAM in DL |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C260 |  |  |  | UE supporting E-UTRA FDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) and Feature Group Indicator 103 |  |  |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.1.2.4 | TDD CQI Reporting under fading conditions - PUSCH 3-1 for 256QAM in DL |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C261 |  |  |  | UE supporting E-UTRA TDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) and Feature Group Indicator 103 |  |  |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.1.2.6 | TDD CQI Reporting under fading conditions – PUCCH 3-1 (when csi-SubframeSet –r12 is configured with one CSI process) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C277 |  |  |  | UE supporting E-UTRA TDD and Rel-12 CSI subframe sets and TM10 ( UE Category >= 1). |  |  |  |  |  |  |  | 2Rx |  |  |  |
| 9.3.1.3.1_E.1 | FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C79 |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.1.3.2_E.1 | TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C80 |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.2.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C13 b |  |  |  | UE supporting E-UTRA FDD (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.2.1.1_1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 (Release 9 and forward) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C15 |  |  |  | UE supporting E-UTRA FDD (UE category 1) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.2.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C14 |  |  |  | UE supporting E-UTRA TDD (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.2.1.2_1 | TDD CQI Reporting under fading conditions - PUCCH 1-0 (Release 9 and forward) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C16 |  |  |  | UE supporting E-UTRA TDD (UE category 1) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.2.2.1_D | FDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25x |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.2.2.2_D | TDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C28y |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.3.1.1 | FDD CQI Reporting under fading conditions and frequency-selective interference - PUSCH 3-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C01 |  |  |  | UE supporting E-UTRA FDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.3.1.2 | TDD CQI Reporting under fading conditions and frequency-selective interference - PUSCH 3-0 |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C02 |  |  |  | UE supporting E-UTRA TDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.4.1.1 | FDD CQI Reporting under fading conditions - PUSCH 2-0 |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C32 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 1 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.4.1.2 | TDD CQI Reporting under fading conditions - PUSCH 2-0 |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C37 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 1 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.4.2.1 | FDD CQI Reporting under fading conditions - PUCCH 2-0 |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C36 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 2 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.4.2.2 | TDD CQI Reporting under fading conditions - PUCCH 2-0 |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C38 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 2 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.5.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C44 |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.5.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C45 |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.5.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C44z |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.5.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C45i |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.6.1_F.1 | FDD CQI Reporting under fading conditions with Single CSI process for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C50a |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.6.1_F.2 | FDD CQI Reporting under fading conditions with Three CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C96 |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.6.1_F.3 | FDD CQI Reporting under fading conditions with Four CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C97 |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.6.2_F.1 | TDD CQI Reporting under fading conditions with Single CSI process for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C51a |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.6.2_F.2 | TDD CQI Reporting under fading conditions with Three CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C98 |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.6.2_F.3 | TDD CQI Reporting under fading conditions with Four CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C99 |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.7.1 | FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  | 9.3.7.1 |  |
| 9.3.7.2 | TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C25a |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.3.8.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TM4 - Enhanced Receiver Type B |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C152 |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.8.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TM4 - Enhanced Receiver Type B |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C153 |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.8.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C152 |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.8.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C153 |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.8.3.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C152 |  |  |  | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.3.8.3.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C153 |  |  |  | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.4.1.1.1 | FDD PMI Reporting - PUSCH 3-1 (Single PMI) |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C01 |  |  |  | UE supporting E-UTRA FDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.1.2 | TDD PMI Reporting - PUSCH 3-1 (Single PMI) |  |  |  |  |  |  |  |  | Rel-8 |  |  |  | C02 |  |  |  | UE supporting E-UTRA TDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.2.1 | FDD PMI Reporting - PUCCH 2-1 (Single PMI) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C36 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 2 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.2.2 | TDD PMI Reporting - PUCCH 2-1 (Single PMI) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C38 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 2 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.3.1_D | FDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.3.2_D | TDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C26 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.4.1 | FDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.1.4.2 | TDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C25a |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.1.1 | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) |  |  |  |  |  |  |  |  | Rel-8 only |  |  |  | C11 |  |  |  | UE supporting E-UTRA FDD and operating bands supporting 20 MHz Bandwidth (UE categories 2, 3, 4, 5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.1.1_1 | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) (Release 9 and forward) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C01 |  |  |  | UE supporting E-UTRA FDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.1.2 | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) |  |  |  |  |  |  |  |  | Rel-8 only |  |  |  | C12 |  |  |  | UE supporting E-UTRA TDD and operating bands supporting 20 MHz Bandwidth (UE categories 2, 3, 4, 5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.1.2_1 | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) (Release 9 and forward) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C02 |  |  |  | UE supporting E-UTRA TDD |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.2.1 | FDD PMI Reporting - PUSCH 2-2 (Multiple PMI) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C32 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 1 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.2.2 | TDD PMI Reporting - PUSCH 2-2 (Multiple PMI) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  | C33 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 1 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.3.1_D | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.3.2_D | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C26 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.4.2.3.3 | FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C25 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.4.2.3.4 | TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C25a |  |  |  | UE supporting E-UTRA TDD and eDL-MIMO Enhancement and Feature Group Indicator 103 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.5.1.1 | FDD RI Reporting - PUCCH 1-1 |  |  |  |  |  |  |  |  | Rel-8 and Rel-9 only |  |  |  | C13a |  |  |  | UE supporting E-UTRA FDD (UE Category 2-5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.1.1_1 | FDD RI Reporting - PUCCH 1-1 (Release 10) |  |  |  |  |  |  |  |  | Rel-10 only |  |  |  | C13 |  |  |  | UE supporting E-UTRA FDD (UE Category 2-8) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.1.1_2 | FDD RI Reporting- PUCCH 1-1 (Release 11) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C13b |  |  |  | UE supporting E-UTRA FDD (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.1.2 | TDD RI Reporting - PUSCH 3-1 |  |  |  |  |  |  |  |  | Rel-8 and Rel-9 only |  |  |  | C14a |  |  |  | UE supporting E-UTRA TDD (UE Category 2-5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.1.2_1 | TDD RI Reporting - PUSCH 3-1 (Release 10) |  |  |  |  |  |  |  |  | Rel-10 only |  |  |  | C14 |  |  |  | UE supporting E-UTRA TDD (UE Category 2-8) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.1.2_2 | TDD RI Reporting- PUSCH 3-1 (Release 11) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C14b |  |  |  | UE supporting E-UTRA TDD (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.2.1_D | FDD RI Reporting - PUCCH 1-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25x |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 103 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.2.2_D | TDD RI Reporting - PUCCH 1-1 for eDL-MIMO |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C25y |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.3.1_C.1 | FDD RI Reporting - PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C29 |  |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.3.2_C.1 | TDD RI Reporting - PUCCH 1-0 for eICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C30 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.4.1_E.1 | FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C77 |  |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.4.2_E.1 | TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C78 |  |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.5.5.1_F.1 | FDD RI Reporting with Single CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C50 |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.5.5.1_F.2 | FDD RI Reporting with Multiple CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C52 |  |  |  | UE supporting E-UTRA FDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.5.5.2_F.1 | TDD RI Reporting with Single CSI process for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C51 |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.5.5.2_F.2 | TDD RI Reporting with Multiple CSI processes for CoMP |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C53 |  |  |  | UE supporting E-UTRA TDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.1_A.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (2 DL CA) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C108 |  |  |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 3) |  |  |  |  | Refer to 36.521-1 9.1.1.2 |  |  | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.2 is executed. |  |
|  |  |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C103 |  |  |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA(UE Category >= 3) |  |  |  |  |  |  |  |  |  |  |  |
| 9.6.1.1_A.2 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (3 DL CA) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C124 |  |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA (UE Category >= 5) |  |  |  |  | Refer to 36.521-1 9.1.1.2 |  |  | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.3 is executed. |  |
|  |  |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C125 |  |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 5) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.3 is executed. |  |
| 9.6.1.1_A.3 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (4DL CA) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C192 |  |  |  | UE supporting E-UTRA FDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA (UE Category >= 8) |  |  |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C193 |  |  |  | UE supporting E-UTRA FDD and 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 8) |  |  |  |  |  |  |  |  |  |  |  |
| 9.6.1.1_A.4 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (5DL CA) |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C192a |  |  |  | UE supporting E-UTRA FDD and 5DL with Intra-band contiguous and Inter-band CA or 5DL with Intra-band non-contiguous and Inter-band CA or 5DL with Intra-band non-contiguous and Intra-band contiguous CA (UE Category 8 and >=≥11) |  |  |  |  |  |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C193a |  |  |  | UE supporting E-UTRA FDD and 5DL Inter-band CA (UE Category 8 and>= ≥11) |  |  |  |  |  |  |  |  |  |  |  |
| 9.6.1.2_A.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (2DL CA) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C114 |  |  |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA (UE Category >=3) |  |  |  |  | Refer to 36.521-1 9.1.1.2 |  |  | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.2_A.2 is executed. |  |
| 9.6.1.2_A.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (3 DL CA) |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C128 |  |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA (UE Category >= 5) |  |  |  |  | Refer to 36.521-1 9.1.1.2 |  |  | 2Rx, 4Rx |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C129 |  |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 5) |  |  |  |  |  |  |  |  |  |  |  |
| 9.6.1.2_A.3 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (4 DL CA) |  |  |  |  |  |  |  | Rel-11 |  |  |  |  | C270 |  |  |  | UE supporting E-UTRA TDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA (UE Category >= 8) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Rel-11 |  |  |  |  | C271 |  |  |  | UE supporting E-UTRA TDD and 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 8) |  |  |  |  |  |  |  |  |  |  |  |
| 9.6.1.2_A.4 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (5 DL CA) |  |  |  |  |  |  |  | Rel-11 |  |  |  |  | C272 |  |  |  | UE supporting E-UTRA TDD and 5DL with intra-band contiguous CA, or 5DL with inter-band CA, or 5DL with intra-band contiguous and inter-band CA (UE Category 8 and >=11) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Rel-11 |  |  |  |  | C273 |  |  |  | UE supporting E-UTRA TDD and 5DL with intra-band non-contiguous and inter-band CA, or 5DL with intra-band non-contiguous and intra-band contiguous CA (UE Category 8 and >=11) |  |  |  |  |  |  |  |  |  |  |  |
| 9.6.1.3.1 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (2DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C132 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 3) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.3.2 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (3DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C133 |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.3.3 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (4DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C133a |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.3.4 | TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C133b |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.4.1 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (2DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C134 |  |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 3) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.4.2 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (3DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C135 |  |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.4.3 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (4DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C135a |  |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.6.1.4.4 | TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C133b |  |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  |  |  |  | TBD |  |  | 2Rx, 4Rx |  |  |  |
| 9.7.1.1 | FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C145 |  |  |  | UE supporting E-UTRA FDD (UE category 0) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.7.1.2 | TDD CQI reporting definition under AWGN conditions for UE category 0 |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C119 |  |  |  | UE supporting E-UTRA TDD (UE category 0) |  |  |  |  |  |  |  |  |  |  |  |
| 9.7.2.1 | FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C145 |  |  |  | UE supporting E-UTRA FDD (UE category 0) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.7.2.2 | TDD CQI reporting definition under fading conditions for UE category 0 |  |  |  |  |  |  |  |  | Rel-12 |  |  |  | C156 |  |  |  | UE supporting E-UTRA TDD (UE category 0) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.8.1.1 | FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C145a |  |  |  | UE supporting E-UTRA FDD and UE category M1 |  |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 9.8.1.2 | TDD CQI reporting definition under AWGN conditions for UE category M1 |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C156a |  |  |  | UE supporting E-UTRA TDD and UE category M1 |  |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 9.8.2.1 | FDD and Half-duplex FDD UE-selected subband CQI for UE category M1 |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C145a |  |  |  | UE supporting E-UTRA FDD and UE category M1 |  |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 9.8.2.2 | TDD UE-selected subband CQI for UE category M1 |  |  |  |  |  |  |  |  | Rel-13 |  |  |  | C156a |  |  |  | UE supporting E-UTRA TDD and UE category M1 |  |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 9.9.1.1.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 with Rank 1 1x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C113b |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.1.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 with Rank 1 1x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C177 |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.2.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 2 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C178 |  |  |  | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.2.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 2 8x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C179 |  |  |  | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.3.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 4 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C180 |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.3.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 4 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C181 |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.4.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 3 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C182 |  |  |  | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.1.4.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 3 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C183 |  |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 5) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.2.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C197 |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  |  |  |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.2.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C198 |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  |  |  |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.2.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A2 x4 |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C199 |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  |  |  |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.2.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A2 x4 |  |  |  |  |  |  |  |  | Rel-11 |  |  |  | C200 |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  |  |  |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.3.1.1 | TDD PMI Reporting - PUSCH 3-1 (Single PMI) 8x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C179 |  |  |  | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 9.9.4.1.1 | FDD RI Reporting- PUCCH 1-1 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C203 |  |  |  | UE supporting E-UTRA FDD with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.4.1.2 | TDD RI Reporting- PUSCH 3-1 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C204 |  |  |  | UE supporting E-UTRA TDD with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.4.2.1 | FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C205 |  |  |  | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
| 9.9.4.2.2 | TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 4x4 |  |  |  |  |  |  |  |  | Rel-10 |  |  |  | C206 |  |  |  | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |  |  |  |
|  |  |  |  | MBMS Performance Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | FDD MBMS performance (Fixed Reference Channel) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  |  | C03 |  |  |  | UE supporting E-UTRA FDD and MBMS |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 10.1_1 | FDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) |  |  |  |  |  |  |  |  | Rel-13 |  |  |  |  | C03 |  |  |  | UE supporting E-UTRA FDD and MBMS |  |  |  | Performed once |  |  |  |  |  |  |
| 10.2 | TDD MBMS performance (Fixed Reference Channel) |  |  |  |  |  |  |  |  | Rel-9 |  |  |  |  | C04 |  |  |  | UE supporting E-UTRA TDD and MBMS |  |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |  |
| 10.2_1 | TDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) |  |  |  |  |  |  |  |  | Rel-13 |  |  |  |  | C04 |  |  |  | UE supporting E-UTRA TDD and MBMS |  |  |  | Performed once |  |  |  |  |  |  |
| V2X Sidelink Performance Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.2 | Demodulation of PSSCH / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  |  |  |  |  | Rel-14 |  |  |  |  | C313 |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |  |  |  |
| 14.3 | Demodulation of PSCCH / Non-concurrent with E-UTRA uplink transmissions |  |  |  |  |  |  |  |  | Rel-14 |  |  |  |  | C313 |  |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |  |  |  |
| Note 1: Due to UE capability signalling for UL 64QAM is introduced from Rel-12, this test case can optionally be executed with a Rel-12 UE. Note 2: For a transition period until RAN5#72, this condition in version 13.0.0 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Note 3: Equivalent aggregated bandwidth is defined as: . Where is number of CCs, and is MIMO layer and bandwidth of CC . The number of MIMO layer for CC in each CA configuration is according to Table A.4.6.1-3, Table A.4.6.2-3, Table A.4.6.3-3, Table A.4.6.3-4 or Table A.4.6.3-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.1-1a: Applicability of RF conformance test cases Conditions

C01 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 THEN R ELSE N/A

C01h IF (A.4.1-1/1 AND A.4.5-1/18) THEN R ELSE N/A

C02 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 THEN R ELSE N/A

C02h IF (A.4.1-1/2 AND A.4.5-1/18) THEN R ELSE N/A

C03 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/1) THEN R ELSE N/A

C04 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/1) THEN R ELSE N/A

C05 Void

C06 Void

C07 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-3b/2 AND A.4.2-1/3) THEN R ELSE N/A

C08 Void

C09 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-3a/1) THEN R ELSE N/A

C10 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-3a/1) THEN R ELSE N/A

C11 IF A.4.1-1/1 AND A.4.3-3a/6 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5) THEN R ELSE N/A

C12 IF A.4.1-1/2 AND A.4.3-3a/6 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5) THEN R ELSE N/A

C13 IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C13a IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5)) THEN R ELSE N/A

C13b IF ((A.4.1-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C14 IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C14a IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5)) THEN R ELSE N/A

C14b IF ((A.4.1-1/2) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C15 IF (A.4.1-1/1 AND A.4.3-4/1) THEN R ELSE N/A

C16 IF (A.4.1-1/2 AND A.4.3-4/1) THEN R ELSE N/A

C17 Void

C18 Void

C19 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2) THEN R ELSE N/A

C19h IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/18) THEN R ELSE N/A

C20 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2) ) THEN R ELSE N/A

C20h IF ((A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2) AND A.4.5-1/18) THEN R ELSE N/A

C21 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1) THEN R ELSE N/A

C21h IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1) AND A.4.5-1/18 THEN R ELSE N/A

C22 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.6.1-1/2) THEN R ELSE N/A

C23 Void

C24 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.6.1-1/2) THEN R ELSE N/A

C25 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.4-3a/103) THEN R ELSE N/A

C25a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.4-3b/103) THEN R ELSE N/A

C25h IF (A.4.1-1/1 AND A.4.4-3a/103 AND A.4.5-1/18) THEN R ELSE N/A

C25x IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C25y IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C26 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/104)) THEN R ELSE N/A

C26h IF (((A.4.1-1/1 AND A.4.4-3a/104) OR (A.4.1-1/2 AND A.4.4-3b/104)) AND A.4.5-1/18) THEN R ELSE N/A

C27 Void

C28

C28y IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3a/104 AND A.4.4-3a/110) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C29 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-3a/115) THEN R ELSE N/A

C30 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-3b/115) THEN R ELSE N/A

C31 IF (A.4.1-1/1 AND (A.4.3-4/1 OR A.4.3-4/2)) THEN R ELSE N/A

C32 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-1a/1) THEN R ELSE N/A

C33 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-1b/1) THEN R ELSE N/A

C34 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/5) THEN R ELSE N/A

C35 Void

C36 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-1a/2 THEN R ELSE N/A

C37 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-1b/1 THEN R ELSE N/A

C38 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-1b/2 THEN R ELSE N/A

C39 IF(NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3-3b/1 OR A.4.3-3b/4)) THEN R ELSE N/A

C40 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.4-3a/103 AND A.4.3-7/1) THEN R ELSE N/A

C41 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.4-3b/103 AND A.4.3-7/1) THEN R ELSE N/A

C42 IF ((A.4.1-1/1) AND (NOT A.4.5-1/18) AND (A.4.3-4/6 OR A.4.3-4/7)) THEN R ELSE N/A

C42h IF ((A.4.1-1/1) AND (A.4.3-4/6 OR A.4.3-4/7) AND A.4.5-1/18 AND A.4.3-4a/8) THEN R ELSE N/A

C43 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND NOT A.4.6.2-2/1) THEN R ELSE N/A

C43h IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND NOT A.4.6.2-2/1 AND A.4.5-1/18) THEN R ELSE N/A

C44 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/1) THEN R ELSE N/A

C44z IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND A.4.4-3a/103) THEN R ELSE N/A

C45 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/1) THEN R ELSE N/A

C45i IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND A.4.4-3b/103) THEN R ELSE N/A

C46 Void

C47 Void

C48 Void

C49 Void

C50 IF (A.4.1-1/1 AND A.4.5-1/8 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C50a IF (A.4.1-1/1 AND A.4.5-1/8) THEN R ELSE N/A

C51 IF (A.4.1-1/2 AND A.4.5-1/8 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C51a IF (A.4.1-1/2 AND A.4.5-1/8) THEN R ELSE N/A

C52 IF (A.4.1-1/1 AND (A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C53 IF (A.4.1-1/2 AND (A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C54 IF (A.4.1-1/2 AND (A.4.3-4/1 OR A.4.3-4/2)) THEN R ELSE N/A

C55 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6) THEN R ELSE N/A

C56 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6) THEN R ELSE N/A

C57 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6 AND (A.4.5-1/11 OR A.4.5-1/12)) THEN R ELSE N/A

C58 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6 AND (A.4.5-1/11 OR A.4.5-1/12)) THEN R ELSE N/A

C59 Void

C60 Void

C61 Void

C62 void

C63 void

C64 Void

C65 Void

C66 Void

C67 Void

C68 Void

C69 void

C70 void

C71 Void

C72 IF ((A.4.1-1/2) AND (A.4.6.2-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C73 IF ((A.4.1-1/2) AND (NOT A.4.5-1/18) AND (A.4.3-4/6 OR A.4.3-4/7)) THEN R ELSE N/A

C73h IF ((A.4.1-1/2) AND A.4.5-1/18 AND A.4.3-4a/8) THEN R ELSE N/A

C74 IF ((A.4.1-1/2) AND (NOT A.4.5-1/18) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C74h IF ((A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/8)) THEN R ELSE N/A

C75 IF ((A.4.1-1/2) AND (NOT A.4.5-1/18) AND (A.4.6.2-1/1) AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C76 IF A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4) THEN R ELSE N/A

C77 IF (A.4.1-1/1 AND A.4.5-2/1 AND A.4.4-3a/115 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C78 IF (A.4.1-1/2 AND A.4.5-2/1 AND A.4.5-2/2 AND A.4.4-3b/115 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C79 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.5-2/1 AND A.4.4-3a/115) THEN R ELSE N/A

C80 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.5-2/1 AND A.4.5-2/2 AND A.4.4-3b/115) THEN R ELSE N/A

C81 void

C82 void

C83 IF ((A.4.1-1/2) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7) AND (A.4.6.3-1/1)) THEN R ELSE N/A

C84 void

C85 Void

C86 Void

C87 void

C88 Void

C89 Void

C90 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C91 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/6 AND A.4.4-3a/103) THEN R ELSE N/A

C92 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/6 AND A.4.4-3b/103) THEN R ELSE N/A

C93 IF ((A.4.1-1/1) AND (NOT A.4.5-1/18) AND (A.4.6.2-1/1) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C94 Void

C95 void

C96 IF (A.4.1-1/1 AND A.4.5-1/11) THEN R ELSE N/A

C97 IF (A.4.1-1/1 AND A.4.5-1/12) THEN R ELSE N/A

C98 IF (A.4.1-1/2 AND A.4.5-1/11) THEN R ELSE N/A

C99 IF (A.4.1-1/2 AND A.4.5-1/12) THEN R ELSE N/A

C100 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/13) THEN R ELSE N/A

C101 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 or A.4.6.3-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C102 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C103 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C104 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C105 IF ((A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C106 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C107 IF ((A.4.1-1/1) AND (NOT A.4.5-1/18) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10)) THEN R ELSE N/A

C107h IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/8)) THEN R ELSE N/A

C108 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C109 IF (A.4.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND (A.4.6.2-1/1 OR A.4.6.3-1/1)) THEN R ELSE N/A

C110 IF (A.4.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8) AND (A.4.6.1-1/1 OR A.4.6.1-1/2)) THEN R ELSE N/A

C111 IF A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4) THEN R ELSE N/A

C112 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4a/1) THEN R ELSE N/A

C112a IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4aa/1) THEN R ELSE N/A

C112b IF (A.4.1-1/8 AND A.4.3-4 c/1) THEN R ELSE N/A

C112c IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4a/1a) THEN R ELSE N/A

C112d IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-4aa/1 AND A.4.3-4aa/2) THEN R ELSE N/A

C113 IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT(A.4.3-4a/1 OR A.4.3-4aa/1) THEN R ELSE N/A

C113a IF (A.4.5-1/22) THEN R ELSE N/A

C113b IF (A.4.1-1/1 AND A.4.5-1/37) THEN R ELSE N/A

C113c IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.4-3a/103) THEN R ELSE N/A

C113d IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.3-7/1) THEN R ELSE N/A

C113e IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND A.4.4-3a/103) THEN R ELSE N/A

C113h IF (A.4.5-1/18) THEN R ELSE N/A

C114 IF (A.4.1-1/2 AND A.4.6.1-1/2) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10) THEN R ELSE N/A

C115 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1) THEN R ELSE N/A

C116 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.6.3-2/1) THEN R ELSE N/A

C117 IF (A.4.1-1/1 AND (A.4.5-1/8 OR A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C118 IF (A.4.1-1/2 AND (A.4.5-1/8 OR A.4.5-1/11 OR A.4.5-1/12) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8)) THEN R ELSE N/A

C119 IF A.4.1-1/2 AND A.4.3-4a/1 THEN R ELSE N/A

C120 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/5) THEN R ELSE N/A

C121 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4)) THEN R ELSE N/A

C122 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/2 OR A.4.6.2-1/2)) THEN R ELSE N/A

C122h IF ((A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4 OR A.4.6.3-1/2 OR A.4.6.2-1/2)) AND A.4.5-1/18 THEN R ELSE N/A

C123 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2) THEN R ELSE N/A

C124 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C125 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C126 IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C126a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/6 OR A.4.3-4/7) AND A.4.3-3a/9) THEN R ELSE N/A

C126h IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C126ha IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/18 AND ( A.4.3-4a/8) AND A.4.3-3a/9) THEN R ELSE N/A

C127 IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C127a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/6 OR A.4.3-4/7) AND A.4.3-3a/9) THEN R ELSE N/A

C128 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C129 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C130 IF (A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C130h IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C131 IF (A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C132 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C133 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C133a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C133b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C134 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C135 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C135a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C135b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C136 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C137 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.3-4/3 OR A.4.3-4/4)) THEN R ELSE N/A

C138 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) THEN R ELSE N/A

C138h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C139 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) THEN R ELSE N/A

C139a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C139b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND A.4.3-4a/10) THEN R ELSE N/A

C139h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C139ha IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C139hb IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.3-4a/11)) THEN R ELSE N/A

C140 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) THEN R ELSE N/A

C140h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/8)) THEN R ELSE N/A

C141 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) THEN R ELSE N/A

C141a IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12) THEN R ELSE N/A

C141b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND A.4.3-4a/10 THEN R ELSE N/A)

C141h IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND A.4.5-1/18 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/10)) THEN R ELSE N/A

C142 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/3) THEN R ELSE N/A

C143 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/3) THEN R ELSE N/A

C144 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/3 AND A.4.4-3a/103) THEN R ELSE N/A

C145 IF A.4.1-1/1 AND A.4.3-4a/1 THEN R ELSE N/A

C145a IF A.4.1-1/1 AND A.4.3-4aa/1 THEN R ELSE N/A

C145b IF A.4.1-1/1 AND (A.4.3-4aa/1 OR A.4.5-1/25) THEN R ELSE N/A

C145c Void

C145d IF A.4.1-1/1 AND A.4.3-4a/1a THEN R ELSE N/A

C145x IF A.4.1-1/1 AND A.4.3-4aa/1 AND A.4.5-1/63 THEN R ELSE N/A

C146 IF (NOT(A.4.3-4a/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6.3-1/1) THEN R ELSE N/A

C147 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/17) THEN R ELSE N/A

C148 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/17) THEN R ELSE N/A

C149 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.5-1/13 AND A.4.5-1/17) THEN R ELSE N/A

C150 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/4) THEN R ELSE N/A

C151 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/4) THEN R ELSE N/A

C152 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.3-7/4) THEN R ELSE N/A

C153 IF (NOT(A.4.3-4/1 OR A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.3-7/4) THEN R ELSE N/A

C154 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C155 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C156 IF A.4.1-1/2 AND A.4.3-4a/1 THEN R ELSE N/A

C156a IF A.4.1-1/2 AND A.4.3-4aa/1 THEN R ELSE N/A

C156b IF A.4.1-1/2 AND (A.4.3-4aa/1 OR A.4.5-1/25) THEN R ELSE N/A

C156c IF A.4.1-1/1 AND (A.4.3-4aa/1 AND A.4.5-1/26 THEN R ELSE N/A

C156d IF A.4.1-1/2 AND (A.4.3-4aa/1 AND A.4.5-1/26 THEN R ELSE N/A

C156e IF A.4.1-1/2 AND (A.4.3-4aa/1 OR A.4.5-1/25) AND A.4.5-1/51 THEN R ELSE N/A

C156f IF A.4.1-1/2 AND A.4.3-4a/1a THEN R ELSE N/A

C156x IF A.4.1-1/2 AND (A.4.3-4aa/1 OR A.4.5-1/25) AND A.4.5-1/63 THEN R ELSE N/A

C157 IF A.4.1-1/1 AND A.4.3-4a/1 AND A.4.4-3a/103 THEN R ELSE N/A

C157a IF A.4.1-1/1 AND A.4.3-4a/1a AND A.4.4-3a/103 THEN R ELSE N/A

C158 IF A.4.1-1/2 AND A.4.3-4a/1 AND A.4.4-3b/103 THEN R ELSE N/A

C158a IF A.4.1-1/2 AND A.4.3-4a/1a AND A.4.4-3b/103 THEN R ELSE N/A

C159 IF (NOT(A.4.3-4a/1 OR A.4.5-1/17 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/13) THEN R ELSE N/A

C160 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.6.3-2/1 AND A.4.5-1/17) THEN R ELSE N/A

C161 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1 AND A.4.5-1/17) THEN R ELSE N/A

C162 IF A.4.5-1/23 THEN R ELSE N/A

C163 IF A.4.5-1/24 THEN R ELSE N/A

C164 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.5-1/37) THEN R ELSE N/A

C165 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.5-1/38) THEN R ELSE N/A

C166 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.4-3a/103 AND A.4.5-1/37) THEN R ELSE N/A

C167 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.2-1/6 AND A.4.4-3a/103 AND A.4.5-1/38) THEN R ELSE N/A

C168 IF (A.4.5-1/22 AND NOT A.4.5-1/18) THEN R ELSE N/A

C169 IF A.4.1-1/1 AND A.4.2-1/8 AND NOT (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4a/1) THEN R ELSE N/A

C170 IF A.4.1-1/2 AND A.4.2-1/8 AND NOT (A.4.3-4/1 OR A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4a/1) THEN R ELSE N/A

C171 IF A.4.1-1/1 AND A.4.2-1/8 AND (A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) AND (NOT A.4.5-1/18) THEN R ELSE N/A

C172 IF A.4.1-1/2 AND A.4.2-1/8 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/9 OR A.4.3-4/10) AND (NOT A.4.5-1/18) THEN R ELSE N/A

C173 IF A.4.1-1/1 AND A.4.2-1/8 AND A.4.5-1/18 THEN R ELSE N/A

C174 IF A.4.1-1/2 AND A.4.2-1/8 AND A.4.5-1/18 THEN R ELSE N/A

C175 IF (NOT(A.4.3-4a/1) AND A.4.1-1/1 AND A.4.5-1/12 AND A.4.3-7/4) THEN R ELSE N/A

C176 IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND A.4.5-1/12 AND A.4.3-7/4) THEN R ELSE N

C177 IF (A.4.5-1/38) THEN R ELSE N/A

C178 IF (A.4.4-3a/103 AND A.4.5-1/37 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C179 IF (A.4.4-3a/104 AND A.4.5-1/38 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C180 IF (A.4.5-1/37 AND A.4.5-1/46 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C181 IF (A.4.5-1/38 AND A A.4.5-1/46 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C182 IF (A.4.4-3a/103 AND A.4.5-1/37 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C183 IF (A.4.4-3a/103 AND A.4.5-1/38 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C184 IF (A.4.5-1/38) THEN R ELSE N/A

C185 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1 AND A.4.5-1/17) THEN R ELSE N/A

C186 IF A.4.3-3b/2 AND NOT(A.4.3-4a/1 OR A.4.3-4aa/1) THEN R ELSE N/A

C187 IF ((A.4.1-1/1 OR A.4.1-1/2) AND ((A.4.1-1/2 AND A.4.6.1-1/4) OR A.4.6.3-1/6 OR A.4.6.3-1/7)) THEN R ELSE N/A

C188 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 THEN R ELSE N/A

C189 IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6-1/3) AND (A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C189a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6-1/3) AND (A.4.3-4/9 OR A.4.3-4/10) AND A.4.3-3a/10) THEN R ELSE N/A

C189a2 Void

C189b Void

C190 Void

C191 Void

C192 IF (A.4.1-1/1 AND (A.4.6.1-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C192a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND (A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C193 IF (A.4.1-1/1 AND (A.4.6.3-1/10 OR A.4.6.2-1/3) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C193a IF (A.4.1-1/1 AND (NOT A.4.5-1/18) AND A.4.6.3-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C194 IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.2-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3 OR A.4.6.3-1/11)) THEN R ELSE N/A

C195 Void

C196 Void

C197 IF (A.4.5-1/37 AND A.4.3-7/1) THEN R ELSE N/A

C198 IF (A.4.5-1/38 AND A.4.3-7/1) THEN R ELSE N/A

C198a IF A.4.1-1/2 AND A.4.5-1/38 AND A.4.3-7/1 THEN R ELSE N/A

C198b IF A.4.1-1/2 AND A.4.5-1/38 THEN R ELSE N/A

C198c IF A.4.1-1/2 AND A.4.5-1/38 AND A.4.3-7/1 AND A.4.4-3b/103 THEN R ELSE N/A

C199 IF A.4.5-1/37 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.4-3a/103 THEN R ELSE N/A

C200 IF A.4.5-1/38 AND A.4.3-7/1 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.4-3b/103 THEN R ELSE N/A

C201 Void

C202 IF ((NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3-3b/4 AND A.4.2-1/3) THEN R ELSE N/A

C203 IF ((A.4.5-1/37) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C204 IF ((A.4.5-1/38) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 )) THEN R ELSE N/A

C205 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103 AND A.4.5-1/37) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C206 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/103 AND A.4.5-1/38) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C207 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.5-1/32) THEN R ELSE N/A

C208 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.6.3-1/1 AND A.4.5-1/32) THEN R ELSE N/A

C209 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 THEN R ELSE N/A

C210 IF A.4.1-1/2 AND A.4.5-1/14 AND A.4.5-1/32 THEN R ELSE N/A

C211 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/12 OR A.4.6.2-1/3 OR A.4.6.3-1/11) THEN R ELSE N/A

C212 IF (A.4.1-1/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3 OR A.4.6.3-1/11)) THEN R ELSE N/A

C213 IF (A.4.1-1/2 AND (NOT A.4.5-1/18) AND (A.4.6-1/3 OR A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C214 IF (A.4.1-1/1 AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/10 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C215 IF (A.4.1-1/1 AND (A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C216 IF (A.4.1-1/1 AND A.4.6.3-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C217 IF A.4.1-1/2 AND A.4.5-1/15 AND A.4.5-1/32 THEN R ELSE N/A

C218 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C219 IF A.4.1-1/2 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C220 IF A.4.1-1/1 AND (A.4.5-1/37 AND A.4.5-1/46) THEN R ELSE N/A

C221 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8) THEN R ELSE N/A

C222 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/14) OR (A.4.1-1/2 AND A.4.6.1-1/5) THEN R ELSE N/A

C223 IF (A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 THEN R ELSE N/A

C224 IF A.4.2-1/8 THEN R ELSE N/A

C225 IF (A.4.2-1/8 AND A.4.5-1/27) THEN R ELSE N/A

C226 IF (A.4.1-1/1 AND A.4.5-1/37 AND A.4.5-1/46 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C227 IF (A.4.5-8/1 AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/4 OR A.4.3-4a/5 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C228 IF (A.4.5-8/1 AND A.4.6.2-1/1 AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/4 OR A.4.3-4a/5 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C229 IF (A.4.5-8/4 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C230 IF (A.4.5-8/4 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C231 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C232 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C233 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.4-3a/103 AND A.4.4-3b/103) AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C234 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.4-3a/103 AND A.4.4-3b/103) AND (A.4.5-1/37 OR A.4.5-1/38) AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C235 IF A.4.1-1/2 AND (A.4.5-1/38 AND A.4.5-1/46) THEN R ELSE N/A

C236 IF (A.4.5-8/7 AND (A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C237 IF (A.4.5-8/10 AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C238 IF (A.4.5-8/10 AND A.4.6.3-1/14 AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C239 IF (A.4.1-1/2 AND A.4.5-1/38 AND A.4.5-1/46 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C240 IF (A.4.5-8/2 AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1 OR A.4.6.3-1/5) AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C241 IF (A.4.5-8/2 AND A.4.6.1-2/1 AND (A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4a/8)) THEN R ELSE N/A

C242 IF (A.4.5-8/5 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C243 IF (A.4.5-8/5 AND (A.4.6. 2-1/2 OR A.4.6.3-1/2) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C244 IF (A.4.5-8/8 AND (A.4.6.1-1/4 OR A.4.6.2-1/4 OR A.4.6.2-1/5 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12) AND (A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C245 IF (A.4.5-8/11 AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND (A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C246 IF (A.4.5-8/11 AND A.4.6.3-1/14 AND (A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C247 IF (A.4.5-8/3 AND (A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/4 OR A.4.3-4a/5 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10)) THEN R ELSE N/A

C248 IF (A.4.5-8/6 AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13)) THEN R ELSE N/A

C249 IF (A.4.5-8/9 AND (A.4.3-4a/10 OR A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C250 IF (A.4.5-8/12 AND (A.4.3-4a/11 OR A.4.3-4a/13 OR A.4.3-4a/14)) THEN R ELSE N/A

C251 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND A.4.5-1/18 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C252 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND A.4.5-1/18 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C253 IF ((A.4.1-1/1) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C254 IF ((A.4.1-1/1) AND (A.4.6.2-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C255 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C256 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C257 IF (A.4.1-1/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3 OR A.4.6.3-1/11) AND A.4.5-1/39) THEN R ELSE N/A

C258 IF (A.4.1-1/1 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3) AND A.4.5-1/39) THEN R ELSE N/A

C259 IF (A.4.1-1/1 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/14) AND A.4.5-1/39) THEN R ELSE N/A

C260 IF (A.4.1-1/1 AND A.4.5-1/18 AND A.4.4-3a/103) THEN R ELSE N/A

C261 IF (A.4.1-1/2 AND A.4.5-1/18 AND A.4.4-3b/103) THEN R ELSE N/A

C262 IF (NOT(A.4.3-4a/1) AND A.4.1-1/1 AND A.4.3-7/4 AND A.4.4-3a/103) THEN R ELSE N/A

C263 IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND A.4.3-7/4 AND A.4.4-3b/103) THEN R ELSE N/A

C264 IF A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C265 IF A.4.1-1/2 AND A.4.5-1/14 AND A.4.5-1/32 AND A.4.5-1/56 THEN R ELSE N/A

C266 IF (A.4.1-1/1 AND A.4.2-1/2 AND NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.1-1/5 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) THEN R ELSE N/A

C267 IF A.4.1-1/1 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND (A.4.6.1-1/5 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8 OR A.4.6.3-1/8 OR A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) THEN R ELSE N/A

C268 IF (NOT(A.4.3-4a/1) AND (A.4.1-1/1 AND A.4.1-1/2) AND (A.4.6.3-1/2 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/32) THEN R ELSE N/A

C269 IF (NOT(A.4.3-4a/1) AND A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.5-1/32) THEN R ELSE N/A

C270 IF (A.4.1-1/2 AND (A.4.6.1-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C271 IF (A.4.1-1/2 AND (A.4.6.3-1/10 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C272 IF (A.4.1-1/2 AND (A.4.6.1-1/5 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16) AND (A.4.3-4/8 AND (A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C273 IF (A.4.1-1/2 AND (A.4.6.3-1/17 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8) AND (A.4.3-4/8 AND (A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C274 IF A.4.1-1/2 AND A.4.5-1/54 THEN R ELSE N/A

C275 IF A.4.1-1/2 AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) AND A.4.5-1/54 AND A.4.5-1/55 THEN R ELSE N/A

C276 IF A.4.1-1/2 AND A.4.5-1/55 THEN R ELSE N/A

C277 IF A.4.1-1/2 AND A.4.5-1/8 AND A.4.5-1/55 THEN R ELSE N/A

C278 IF (A.4.1-1/1 AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND A.4.5-1/18 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C279 IF (A.4.1-1/1 AND A.4.6.2-1/1 AND A.4.5-1/18 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C280 IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.5-1/46 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C281 IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C282 IF (A.4.1-1/1 AND A.4.6-1/1 AND A.4.5-1/37 AND A.4.3-7/1 AND A.4.4-3a/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C283 IF (A.4.1-1/2 AND A.4.6-1/1 AND A.4.5-1/38 AND A.4.3-7/1 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C284 IF (A.4.1-1/2 AND A.4.6-1/1 AND A.4.5-1/38 AND A.4.3-7/1 AND A.4.4-3b/103 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C285 IF ((A.4.1-1/1) AND (A.4.3-7/6)) THEN R ELSE N/A

C286 IF ((A.4.1-1/1) AND (A.4.3-7/7)) THEN R ELSE N/A

C287 IF ((A.4.1-1/2) AND (A.4.3-7/6)) THEN R ELSE N/A

C288 IF ((A.4.1-1/2) AND (A.4.3-7/7)) THEN R ELSE N/A

C289 IF ((A.4.1-1/1) AND (A.4.3-7/6) AND (A.4.2-1/6)) THEN R ELSE N/A

C290 IF ((A.4.1-1/2) AND (A.4.3-7/6) AND (A.4.2-1/6)) THEN R ELSE N/A

C291 IF ((A.4.1-1/2) AND (A.4.6.1-1/1 OR A.4.6.1-1/2 OR A.4.6.3-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C292 IF ((A.4.1-1/2) AND (A.4.6.2-1/1) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C293 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C294 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND A.4.5-1/39) THEN R ELSE N/A

C295 IF (A.4.1-1/2 AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3 OR A.4.6.3-1/11) AND A.4.5-1/39) THEN R ELSE N/A

C296 IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.2-1/3) AND A.4.5-1/39) THEN R ELSE N/A

C297 IF (A.4.1-1/2 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12) AND (A.4.6.3-1/14) AND A.4.5-1/39) THEN R ELSE N/A

C298 IF (A.4.1-1/8 AND A.4.3-4c/2) THEN R ELSE N/A

C299 IF (A.4.1-1/1 AND A.4.3-7/5) THEN R ELSE N/A

C300 IF (A.4.1-1/2 AND A.4.3-7/5) THEN R ELSE N/A

C301 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/58) THEN R ELSE N/A

C302 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.1-1/2 AND A.4.6.1-2/2 AND A.4.5-1/58) THEN R ELSE N/A

C303 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.6.3-2/1 AND A.4.5-1/58) THEN R ELSE N/A

C304 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND (A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.2-1/1 AND A.4.6.2-2/1 AND A.4.5-1/58) THEN R ELSE N/A

C305 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.6.3-1/1 AND A.4.6.3-2/1 AND A.4.5-1/62) THEN R ELSE N/A

C306 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C307 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C308 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C309 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C310 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C311 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C312 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (A.4.5-1/37 OR A.4.5-1/38) AND (A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C313 IF (A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7/1) THEN R ELSE N/A

C314 IF A.4.1-1/1 AND (A.4.3-4aa/2 OR A.4.5-1/25) THEN R ELSE N/A

C315 IF A.4.1-1/1 AND (A.4.3-4aa/2 AND A.4.5-1/26 THEN R ELSE N/A

C316 IF A.4.1-1/2 AND (A.4.3-4aa/2 OR A.4.5-1/25) THEN R ELSE N/A

C317 IF A.4.1-1/2 AND (A.4.3-4aa/2 AND A.4.5-1/26 THEN R ELSE N/A

C318 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1/13 AND A.4.5-1/58) THEN R ELSE N/A

Note 1: Cxxxh applicability is defined for small cell enhancements for physical layer related test.

<unchanged sections skipped>

## 4.2	RRM conformance test cases

Table 4.2-1: Applicability of RRM conformance test cases, ref. TS 36.521-3 [2]

NOTE:	To determine applicability of a test case, FGI support in combined or fdd-Add-UE-EUTRA-Capabilities or tdd-Add-UE-EUTRA-Capabilities is taken into account.

| Clause | Title | Release | Applicability |  |  |  |  | Additional Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition |  |  | Comments |  | Number of TC Executions |  | Release on other RAT |  | Branch |  |
| E-UTRAN RRC_IDLE State Mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.1 | E-UTRAN FDD - FDD cell re-selection intra frequency case | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.2 | E-UTRAN TDD - TDD cell re-selection intra frequency case | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.3 | E-UTRAN FDD - FDD cell re-selection inter frequency case | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.4 | E-UTRAN FDD - TDD cell re-selection inter frequency case | Rel-9 | C03 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.5 | E-UTRAN TDD - FDD cell re-selection inter frequency case | Rel-9 | C03 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.6 | E-UTRAN TDD - TDD cell re-selection inter frequency case | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.7 | E-UTRAN FDD - FDD Inter frequency case in the existence of non-allowed CSG cell | Rel-9 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.8 | E-UTRAN TDD - TDD Inter frequency case in the existence of non-allowed CSG cell | Rel-9 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.2.9 | E-UTRAN FDD-FDD intra-frequency Cell Re-selection case for 5MHz bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 4.2.12 | E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE category M1 |  |  |  |  |  |  |  |
| 4.2.13 | E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE category M1 |  |  |  |  |  |  |  |
| 4.2.14 | E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in normal coverage | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE category M1 |  |  |  |  |  |  |  |
| 4.2.15 | E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Rel-13 | C94e |  |  | UE supporting E-UTRA FD-FDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 4.2.16 | E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Rel-13 | C94f |  |  | UE supporting E-UTRA HD-FDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 4.2.17 | E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage | Rel-13 | C93e |  |  | UE supporting E-UTRA TDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 4.2.18 | HD-FDD Cell Re-selection Intra frequency case for Category NB1 UE In-Band mode under Normal Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 4.2.19 | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 4.2.20 | E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis | Rel-13 | C194 |  |  | UE supporting E-UTRA FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 4.2.21 | E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis | Rel-13 | C195 |  |  | UE supporting E-UTRA TDD and UE Category 1bis |  |  |  |  |  |  |  |
| 4.2.22 | E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Rel-14 |  |  | C196 |  | UEs supporting E-UTRA FDD and high speed enhancement for measurement |  |  |  |  |  |  |
| 4.2.23 | E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Rel-14 |  |  | C197 |  | UEs supporting E-UTRA TDD and high speed enhancement for measurement |  |  |  |  |  |  |
| 4.2.24 | HD – FDD Inter frequency case for UE Category NB1 In-Band mode in normal coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 4.3.1.1 | E-UTRA FDD - UTRAN FDD cell re-selection | Rel-8 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.3.1.2 | E-UTRA FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority | Rel-8 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.3.1.3 | E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority | Rel-8 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.3.1.4 | E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz bandwidth | Rel-8 | C53 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 and UTRA FDD |  |  |  |  |  |  |  |
| 4.3.2 | E-UTRAN FDD - UTRAN TDD cell re-selection | Rel-8 | C06 |  |  | UE supporting E-UTRA FDD and UTRA TDD |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 4.3.3 | E-UTRAN TDD - UTRAN FDD cell re-selection | Rel-8 | C07 |  |  | UE supporting E-UTRA TDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.3.4.1 | E-UTRA TDD - UTRAN TDD cell re-selection | Rel-8 | C05 |  |  | UE supporting E-UTRA TDD and UTRA TDD |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 4.3.4.2 | E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of lower priority | Rel-8 | C05 |  |  | UE supporting E-UTRA TDD and UTRA TDD |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 4.3.4.3 | EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | Rel-8 | C05 |  |  | UE supporting E-UTRA TDD and UTRA TDD |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 4.4.1 | E-UTRAN FDD - GSM cell re-selection | Rel-8 | C08 |  |  | UE supporting E-UTRA FDD and GSM |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.4.2 | E-UTRAN TDD - GSM cell re-selection | Rel-8 | C09 |  |  | UE supporting E-UTRA TDD and GSM |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.5.1.1 | E-UTRAN FDD - HRPD Cell re-selection: HRPD is of lower priority | Rel-8 | C10 |  |  | UE supporting E-UTRA FDD and cdma2000 HRPD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.5.2.1 | E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority | Rel-9 | C34 |  |  | UE supporting E-UTRA TDD and cdma2000 HRPD |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.6.1.1 | E-UTRAN FDD - cdma2000 1xRTT Cell re-selection: cdma2000 1x is of lower priority | Rel-8 | C11 |  |  | UE supporting E-UTRA FDD and cdma2000 1xRTT |  |  |  |  |  | 2Rx, 4Rx |  |
| 4.6.2.1 | E-UTRAN TDD-cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority | Rel-9 | C35 |  |  | UE supporting E-UTRA TDD and cdma2000 1xRTT |  |  |  |  |  | 2Rx, 4Rx |  |
| E-UTRAN RRC_CONNECTED State Mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1.1 | E-UTRAN FDD - FDD Handover intra frequency case | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.2 | E-UTRAN TDD - TDD Handover intra frequency case | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.3 | E-UTRAN FDD - FDD Handover inter frequency case | Rel-8 | C01d |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 5, 13 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.4 | E-UTRAN TDD - TDD Handover inter frequency case | Rel-8 | C02d |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 5, 13 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.5 | E-UTRAN FDD - FDD inter frequency handover: unknown target cell | Rel-8 | C01a |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 13 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.6 | E-UTRAN TDD-TDD inter frequency handover: unknown target cell | Rel-8 | C02a |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 13 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.7 | E-UTRAN FDD - TDD handover inter frequency case | Rel-9 | C21 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 5, 25 and 30 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.8 | E-UTRAN TDD - FDD handover inter frequency case | Rel-9 | C21 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 5, 25 and 30 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.1.9 | E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 5.1.10 | E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Rel-12 | C94 |  |  | UE supporting E-UTRA FD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 5.1.11 | E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Rel-12 | C110 |  |  | UE supporting E-UTRA HD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 5.1.12 | E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Rel-12 | C93 |  |  | UE supporting E-UTRA TDD and UE Category 0 |  |  |  |  |  |  |  |
| 5.1.13 | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 5.1.14 | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 5.1.15 | E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 5.1.16 | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Rel-13 | C94e |  |  | UE supporting E-UTRA FD-FDD and (UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 5.1.17 | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Rel-13 | C94f |  |  | UE supporting E-UTRA HD-FDD and (UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 5.1.18 | E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB | Rel-13 | C93e |  |  | UE supporting E-UTRA TDD and (UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 5.1.19 | E-UTRAN FDD – FDD Intra frequency handover for UE Category 1bis | Rel-13 | C194 |  |  | UE supporting E-UTRA FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 5.1.20 | E-UTRAN TDD – TDD Intra frequency handover for UE Category 1bis | Rel-13 | C195 |  |  | UE supporting E-UTRA TDD and UE Category 1bis |  |  |  |  |  |  |  |
| 5.2.1 | E-UTRAN FDD - UTRAN FDD handover | Rel-8 | C04a |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicators 8 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.2 | E-UTRAN TDD - UTRAN FDD handover | Rel-8 | C07a |  |  | UE supporting E-UTRA TDD and UTRA FDD and Feature Group Indicators 8 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.3 | E-UTRAN FDD - GSM handover | Rel-8 | C08e |  |  | UE supporting E-UTRA FDD and GSM and inter-RAT PS handover to GERAN and Feature Group Indicators 9, 15 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.4 | E-UTRAN TDD - UTRAN TDD handover | Rel-8 | C05a |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 8 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 5.2.5 | E-UTRAN FDD - UTRAN TDD handover | Rel-8 | C06a |  |  | UE supporting E-UTRA FDD and UTRA TDD and Feature Group Indicators 8 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 5.2.6 | E-UTRA TDD - GSM handover | Rel-8 | C09f |  |  | UE supporting E-UTRA TDD and GSM and inter-RAT PS handover to GERAN and Feature Group Indicators 9, 15 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.7 | E-UTRAN FDD - UTRAN FDD handover: unknown target cell | Rel-8 | C04a |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicators 8 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.8 | E-UTRAN FDD - GSM handover: unknown target cell | Rel-8 | C08a |  |  | UE supporting E-UTRA FDD and GSM and inter-RAT PS handover to GERAN and inter-RAT PS handover to GERAN and Feature Group Indicators 9 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.9 | E-UTRAN TDD - GSM handover: unknown target cell | Rel-8 | C09b |  |  | UE supporting E-UTRA TDD and GSM and Feature Group Indicators 9 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.2.10 | E-UTRAN TDD - UTRAN TDD handover: unknown target cell | Rel-8 | C05a |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 8 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 5.2.11 | E-UTRAN FDD - UTRAN FDD handover for 5MHz Bandwidth | Rel-8 | C54 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 and UTRA FDD and Feature Group Indicators 8 and 22 |  |  |  |  |  |  |  |
| 5.3.1 | E-UTRAN FDD - HRPD Handover | Rel-8 | C10a |  |  | UE supporting E-UTRA FDD and cdma2000 HRPD and Feature Group Indicators 12 and 26 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.3.2 | E-UTRAN FDD - cdma2000 1xRTT handover | Rel-8 | C11a |  |  | UE supporting E-UTRA FDD and cdma2000 1xRTT and Feature Group Indicators 11 and 24 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.3.3 | E-UTRAN FDD - HRPD handover: unknown target cell | Rel-8 | C10a |  |  | UE supporting E-UTRA FDD and cdma2000 HRPD and Feature Group Indicators 12 and 26 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.3.4 | E-UTRAN FDD - cdma2000 1xRTT handover: unknown target cell | Rel-8 | C11a |  |  | UE supporting E-UTRA FDD and cdma2000 1xRTT and Feature Group Indicators 11 and 24 |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.3.5 | E-UTRAN TDD-HRPD Handover | Rel-9 | C36 |  |  | UE supporting E-UTRA TDD and cdma2000 HRPD and Feature Group Indicators 12 and 26. |  |  |  |  |  | 2Rx, 4Rx |  |
| 5.3.6 | E-UTRAN TDD-cdma2000 1X Handover | Rel-9 | C37 |  |  | UE supporting E-UTRA TDD and cdma2000 1xRTT and Feature Group Indicators 11 and 24. |  |  |  |  |  | 2Rx, 4Rx |  |
| RRC Connection Mobility Control |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1 | E-UTRAN FDD Intra-frequency RRC Re-establishment | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.1.1_2 | E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis | Rel-13 | C01m |  |  | UE supporting E-UTRA FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 6.1.2 | E-UTRAN FDD Inter-frequency RRC Re-establishment | Rel-8 | C01b |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.1.2_2 | E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis | Rel-13 | C01n |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 6.1.3 | E-UTRAN TDD Intra-frequency RRC Re-establishment | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.1.3_2 | E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis | Rel-13 | C02m |  |  | UE supporting E-UTRA TDD and UE Category 1bis |  |  |  |  |  |  |  |
| 6.1.4 | E-UTRAN TDD Inter-frequency RRC Re-establishment | Rel-8 | C02b |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.1.4_2 | E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis | Rel-13 | C02n |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 6.1.5 | E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 6.1.6 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0 | Rel-12 | C94 |  |  | UE supporting E-UTRA FD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 6.1.7 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0 | Rel-12 | C107 |  |  | UE supporting E-UTRA HD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 6.1.8 | E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 | Rel-12 | C93 |  |  | UE supporting E-UTRA TDD and UE Category 0 |  |  |  |  |  |  |  |
| 6.1.9 | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 6.1.10 | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 6.1.11 | E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 6.1.15 | HD-FDD Intra-frequency RRC Re-establishment for category NB1 UE in In-Band mode under normal coverage | Rel-13 | C162 |  |  | UE supporting category NB1 and User plane CIoT |  |  |  |  |  |  |  |
| 6.1.16 | HD-FDD Inter-frequency RRC Re-establishment for category NB1 UE in In-Band mode under Enhanced Coverage | Rel-13 | C162 |  |  | UE supporting category NB1 and User plane CIoT |  |  |  |  |  |  |  |
| 6.2.1 | E-UTRAN FDD - Contention Based Random Access Test | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.2 | E-UTRAN FDD - Non-Contention Based Random Access Test | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.3 | E-UTRAN TDD - Contention Based Random Access Test | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.4 | E-UTRAN TDD - Non-Contention Based Random Access Test | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.5 | E-UTRAN FDD - Contention Based Random Access Test for 5MHz Bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.6 | E-UTRAN FDD - Non-Contention Based Random Access Test for 5MHz Bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.7 | E-UTRAN FDD - Non-Contention Based Random Access Test For SCell in sTAG | Rel-12 | C61 |  |  | UE supporting E-UTRA FDD and Uplink Carrier Aggregation and multiple timing advances |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.8 | E-UTRAN TDD - Non-Contention Based Random Access Test For SCell in sTAG | Rel-12 | C62 |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advances |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.2.10 | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 6.2.11 | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 6.2.12 | E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE category M1 |  |  |  |  |  |  |  |
| 6.2.13 | E-UTRAN FDD - Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Rel-13 | C94e |  |  | U supporting E-UTRA FD-FDD and( UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 6.2.14 | E-UTRAN HD-FDD - Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Rel-13 | C94f |  |  | UE supporting E-UTRA HD-FDD and( UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 6.2.15 | E-UTRAN TDD - Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Rel-13 | C93e |  |  | UE supporting E-UTRA TDD and( UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 6.2.16 | Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 6.2.17 | Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 6.2.18 | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Rel-14 | C202 |  |  | UE supporting category NB1 and supporting NPRACH on non-anchor carrier |  |  |  |  |  |  |  |
| 6.3.1 | Redirection from E-UTRAN FDD to UTRAN FDD | Rel-9 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.2 | Redirection from E-UTRAN TDD to UTRAN FDD | Rel-9 | C07 |  |  | UE supporting E-UTRA TDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.3 | Redirection from E-UTRAN FDD to GERAN when System Information is provided | Rel-9 | C27 |  |  | UE supporting E-UTRA FDD and GERAN |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.4 | Redirection from E-UTRAN TDD to GERAN when System Information is provided | Rel-9 | C28 |  |  | UE supporting E-UTRA TDD and GERAN |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.5 | E-UTRA TDD RRC connection release redirection to UTRA TDD | Rel-9 | C26 |  |  | UE supporting E-UTRA TDD and UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.6 | E-UTRA FDD RRC connection release redirection to UTRA TDD | Rel-9 | C25 |  |  | UE supporting E-UTRA FDD and UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.7 | E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided | Rel-9 | C26 |  |  | UE supporting E-UTRA TDD and UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.8 | E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided | Rel-9 | C25 |  |  | UE supporting E-UTRA FDD and UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.9 | Redirection from E-UTRAN FDD to UTRAN FDD without System Information | Rel-9 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.10 | Redirection from E-UTRAN FDD to GERAN when System Information is not provided | Rel-9 | C27 |  |  | UE supporting E-UTRA FDD and GERAN |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.11 | Redirection from E-UTRAN TDD to GERAN when System Information is not provided | Rel-9 | C28 |  |  | UE supporting E-UTRA TDD and GERAN |  |  |  |  |  | 2Rx, 4Rx |  |
| 6.3.12 | E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided | Rel-9 | C07 |  |  | UE supporting E-UTRA TDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| Timing and Signalling Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1 | E-UTRAN FDD - UE Transmit Timing Accuracy | Rel-8 | C01c |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 |  |  |  |  |  | 2Rx, 4Rx |  |
| 7.1.1_1 | E-UTRAN FDD - UE Transmit Timing Accuracy (Non DRx UE) | Rel-8 only | C23 |  |  | UE supporting E-UTRA FDD but not supporting Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.1_2 | E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis | Rel-13 | C206 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 7.1.2 | E-UTRAN TDD - UE Transmit Timing Accuracy | Rel-8 | C02c |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 |  |  |  |  |  | 2Rx, 4Rx |  |
| 7.1.2_1 | E-UTRAN TDD - UE Transmit Timing Accuracy (Non DRx UE) | Rel-8 only | C24 |  |  | UE supporting E-UTRA TDD but not supporting Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.2_2 | E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis | Rel-13 | C207 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 7.1.3 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell | Rel-11 | C57 |  |  | UE supporting E-UTRA FDD and Uplink Carrier Aggregation and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.3_1 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Rel-12 | C57 |  |  | UE supporting E-UTRA FDD and Uplink Carrier Aggregation and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.4 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell | Rel-11 | C58 |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and Feature Group Indicator 5 |  | Either TC 7.1.4 or TC 7.1.4A shall be executed. (Note 1) |  |  |  |  |  |
| 7.1.4A | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth | Rel-11 | C58a |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and Feature Group Indicator 5 |  | Either TC 7.1.4 or TC 7.1.4A shall be executed. (Note 1) |  |  |  |  |  |
| 7.1.4_1 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Rel-12 | C58 |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.5 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth | Rel-8 | C56 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.6 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Rel-11 | C63 |  |  | UE supporting E-UTRA FDD and Uplink Carrier Aggregation and multiple timing advances and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.7 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Rel-11 | C64 |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advance and Feature Group Indicator 5 |  | Either TC 7.1.7 or TC 7.1.7A or TC 7.1.7B shall be executed. (Note 1) |  |  |  |  |  |
| 7.1.7A | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth | Rel-11 | C64a |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advance and Feature Group Indicator 5 |  | Either TC 7.1.7 or TC 7.1.7A or TC 7.1.7B shall be executed. (Note 1) |  |  |  |  |  |
| 7.1.7B | E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth | Rel-11 | C64b |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advance and Feature Group Indicator 5 |  | Either TC 7.1.7 or TC 7.1.7A or TC 7.1.7B shall be executed. (Note 1) |  |  |  |  |  |
| 7.1.10 | E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Rel-13 | C94b |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.11 | E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Rel-13 | C107c |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.12 | E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Rel-13 | C93c |  |  | UE supporting E-UTRA TDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.14 | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Rel-13 | C94h |  |  | UE supporting E-UTRA FD-FDD and (UE category M1 and CE Mode B) and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.15 | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Rel-13 | C94i |  |  | UE supporting E-UTRA HD-FDD and (UE category M1 and CE Mode B) and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.16 | E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Rel-13 | C93k |  |  | UE supporting E-UTRA TDD and (UE category M1 and CE Mode B) and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.1.17 | HD-FDD Transmit Timing Accuracy Test for Category NB1 UE In-Band mode under Normal Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 7.1.18 | HD-FDD Transmit Timing Accuracy Test for Category NB1 UE In-band mode under Enhanced Coverage | Rel-13 | C155 |  |  | UE supporting category NB1 and Feature Group Indicators 5 |  |  |  |  |  |  |  |
| 7.2.1 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 7.2.1_2 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Rel-13 | C194 |  |  | UE supporting E-UTRA FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 7.2.2 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy | Rel-8 | C02 |  |  | UE supporting E-UTRA TDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 7.2.2_2 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Rel-13 | C195 |  |  | UE supporting E-UTRA TDD and UE Category 1bis |  |  |  |  |  |  |  |
| 7.2.3 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 7.2.4 | E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | Rel-12 | C61 |  |  | UE supporting E-UTRA FDD and Uplink Carrier Aggregation and multiple timing advances |  |  |  |  |  |  |  |
| 7.2.5 | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test For SCell in sTAG | Rel-11 | C62 |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advances |  | Either TC 7.2.5 or TC 7.2.5A or TC 7.2.5B shall be executed. (Note 1) |  |  |  |  |  |
| 7.2.5A | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth | Rel-11 | C62a |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advances |  | Either TC 7.2.5 or TC 7.2.5A or TC 7.2.5B shall be executed. (Note 1) |  |  |  |  |  |
| 7.2.5B | E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth | Rel-11 | C62b |  |  | UE supporting E-UTRA TDD and Uplink Carrier Aggregation and multiple timing advances |  | Either TC 7.2.5 or TC 7.2.5A or TC 7.2.5B shall be executed. (Note 1) |  |  |  |  |  |
| 7.2.6 | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.2.7 | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.2.8 | E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.2.9 | HD-FDD UE Timing Advance Adjustment Accuracy Test for Category NB1 UE in Standalone Mode under Enhance Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 7.2.10 | E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Rel-13 | C94e |  |  | U supporting E-UTRA FD-FDD and (UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 7.2.11 | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Rel-13 | C94f |  |  | UE supporting E-UTRA HD-FDD and (UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 7.2.12 | E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Rel-13 | C93e |  |  | UE supporting E-UTRA TDD and (UE Category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 7.3.1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-Sync | Rel-8 | C01i |  |  | UE supporting E-UTRA FDD but not 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.1_1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Rel-10 | C140 |  |  | UE supporting E-UTRA FDD and 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.2 | E-UTRAN FDD Radio Link Monitoring Test for In-Sync | Rel-8 | C01i |  |  | UE supporting E-UTRA FDD but not 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.2_1 | E-UTRAN FDD Radio Link Monitoring Test for In-Sync with 4 Rx antenna ports | Rel-10 | C140 |  |  | UE supporting E-UTRA FDD and 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.3 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-Sync | Rel-8 | C02a |  |  | UE supporting E-UTRA TDD but not 4Rx antenna ports on all supported TDD operating bands |  |  |  |  |  |  |  |
| 7.3.3_1 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Rel-10 | C143 |  |  | UE supporting E-UTRA TDD and 4Rx antenna ports on all supported TDD operating bands |  |  |  |  |  |  |  |
| 7.3.4 | E-UTRAN TDD Radio Link Monitoring Test for In-Sync | Rel-8 | C02i |  |  | UE supporting E-UTRA TDD but not 4Rx antenna ports on all supported TDD operating bands |  |  |  |  |  |  |  |
| 7.3.4_1 | E-UTRAN TDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Rel-10 | C143 |  |  | UE supporting E-UTRA TDD and 4Rx antenna ports on all supported TDD operating bands |  |  |  |  |  |  |  |
| 7.3.5 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX | Rel-8 | C01j |  |  | UE supporting E-UTRA FDD but not 4Rx antenna ports on all supported FDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.5_1 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports | Rel-10 | C181 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 and 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.6 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX | Rel-8 | C01j |  |  | UE supporting E-UTRA FDD but not 4Rx antenna ports on all supported FDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.6_1 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports | Rel-10 | C181 |  |  | UE supporting E-UTRA FDD and 4Rx antenna ports on all supported FDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.7 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX | Rel-8 | C02j |  |  | UE supporting E-UTRA TDD but not 4Rx antenna ports on all supported TDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.7_1 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports | Rel-10 | C182 |  |  | UE supporting E-UTRA TDD and 4Rx antenna ports on all supported TDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.8 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX | Rel-8 | C02j |  |  | UE supporting E-UTRA TDD but not 4Rx antenna ports on all supported TDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.8_1 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports | Rel-10 | C182 |  |  | UE supporting E-UTRA TDD and 4Rx antenna ports on all supported TDD operating bands and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.9 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.10 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.11 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.12 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.13 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.14 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.15 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.16 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.17 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Rel-11 |  | C59 |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.18 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Rel-11 |  | C60 |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.19 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non-MBSFN ABS (feICIC) | Rel-11 | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.20 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non-MBSFN ABS (feICIC) | Rel-11 | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.21 | E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | Rel-11 | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.22 | E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | Rel-11 | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 7.3.23 | E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 7.3.24 | E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 7.3.25 | E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth | Rel-8 | C56 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.26 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Rel-12 | C94 |  |  | UE supporting E-UTRA FD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.26_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Rel-13 | C194 |  |  | UE supporting E-UTRA FD-FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.27 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Rel-12 | C94 |  |  | UE supporting E-UTRA FD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.27_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Rel-13 | C194 |  |  | UE supporting E-UTRA FD-FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.28 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Rel-12 | C95 |  |  | UE supporting E-UTRA FD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.28_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Rel-13 | C206 |  |  | UE supporting E-UTRA FD-FDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.29 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Rel-12 | C95 |  |  | UE supporting E-UTRA FD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.29_2 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Rel-13 | C206 |  |  | UE supporting E-UTRA FD-FDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.30 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Rel-12 | C110 |  |  | UE supporting E-UTRA HD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.31 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Rel-12 | C110 |  |  | UE supporting E-UTRA HD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.32 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Rel-12 | C111 |  |  | UE supporting E-UTRA HD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.33 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Rel-12 | C111 |  |  | UE supporting E-UTRA HD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.34 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Rel-12 | C93 |  |  | UE supporting E-UTRA TDD and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.34_2 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Rel-13 | C195 |  |  | UE supporting E-UTRA TDD and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.35 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 | Rel-12 | C93 |  |  | UE supporting E-UTRA TDD and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.35_2 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Rel-13 | C195 |  |  | UE supporting E-UTRA TDD and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.36 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Rel-12 | C96 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.36_2 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Rel-13 | C207 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.37 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Rel-12 | C96 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 7.3.37_2 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Rel-13 | C207 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 7.3.38 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Rel-12 | C123b |  |  | UE supporting E-UTRA FDD and Dual Connectivity but not 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.38_1 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with 4 Rx antenna ports | Rel-12 | C185 |  |  | UE supporting E-UTRA FDD and Dual Connectivity and 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.39 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC | Rel-12 | C125a |  |  | UE supporting E-UTRA FDD and asynchronous Dual Connectivity but not 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.39_1 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC with 4 Rx antenna ports | Rel-12 | C186 |  |  | UE supporting E-UTRA FDD and asynchronous Dual Connectivity and 4Rx antenna ports on all supported FDD operating bands |  |  |  |  |  |  |  |
| 7.3.40 | E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Rel-12 | C124 |  |  | UE supporting E-UTRA TDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.41 | E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Rel-12 | C123 |  |  | UE supporting E-UTRA FDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.42 | E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC | Rel-12 | C125 |  |  | UE supporting E-UTRA FDD and asynchronous Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.43 | E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Rel-12 | C124 |  |  | UE supporting E-UTRA TDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.44 | E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD | Rel-12 | C123a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.45 | E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD | Rel-12 | C123a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.46 | E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD | Rel-12 | C123a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.47 | E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD | Rel-12 | C123a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Dual Connectivity |  |  |  |  |  |  |  |
| 7.3.48 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.49 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.50 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.51 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.52 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category CAT-M1 | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.53 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category CAT-M1 | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.54 | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.55 | E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.56 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.57 | E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 7.3.58 | E- UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Rel-13 | C93c |  |  | UE supporting E-UTRA TDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.59 | E- UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category M1 configured in CEMode A | Rel-13 | C93c |  |  | UE supporting E-UTRA TDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.3.60 | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Rel-13 | C155 |  |  | UE supporting category NB1 and Feature Group Indicators 5 |  |  |  |  |  |  |  |
| 7.3.61 | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in Enhanced Coverage | Rel-13 | C155 |  |  | UE supporting category NB1 and Feature Group Indicators 5 |  |  |  |  |  |  |  |
| 7.3.62 | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Rel-13 | C155 |  |  | UE supporting category NB1 and Feature Group Indicators 5 |  |  |  |  |  |  |  |
| 7.3.63 | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | Rel-13 | C155 |  |  | UE supporting category NB1 and Feature Group Indicators 5 |  |  |  |  |  |  |  |
| 7.3.64 | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 7.3.65 | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 7.3.66 | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 7.3.67 | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Rel-13 | C154 |  |  | UE supporting category NB1 |  |  |  |  |  |  |  |
| 7.4.1 | E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Rel-12 | C175 |  |  | UE supporting E-UTRA FDD, Dual Connectivity and Feature Group Indicator 5 |  | It is not necessary for DC ASYNCH UEs to be tested in this test if 7.4.3 case is executed. (Note 2) |  |  |  |  |  |
| 7.4.2 | E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Rel-12 | C136 |  |  | UE supporting E-UTRA TDD, Dual Connectivity and Feature Group Indicator 5 |  | It is not necessary for DC ASYNCH UEs to be tested in this test if 7.4.4 case is executed. (Note 2) |  |  |  |  |  |
| 7.4.3 | E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity | Rel-12 | C135 |  |  | UE supporting E-UTRA FDD, Dual Connectivity Asynch and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 7.6.1 | E-UTRAN FDD-TDD CA interruption at SRS carrier based switching | Rel-14 | C200 |  |  | UE supporting E-UTRA FDD and TDD CA with FDD as PCell and SRS switching between component carriers |  |  |  |  |  |  |  |
| 7.6.2 | E-UTRAN TDD-TDD CA interruption at SRS carrier based switching | Rel-14 | C201 |  |  | UE supporting E-UTRA TDD CA and SRS switching between component carriers |  |  |  |  |  |  |  |
| UE Measurements Procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.1 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Rel-8 | C01 |  |  | UE supporting E-UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.1.2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells | Rel-8 | C01c |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.1.3 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Rel-8 | C01c |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.1.4 | Void |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.5 | E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Rel-9 | C13 |  |  | UE supporting E-UTRA FDD and intra-frequency SI acquisition in FDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.1.6 | E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Rel-9 | C13 |  |  | UE supporting E-UTRA FDD and intra-frequency SI acquisition in FDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.1.7 | E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 8.1.8 | E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 8.1.9 | E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth | Rel-8 | C49 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 |  |  |  |  |  |  |  |
| 8.1.10 | E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz bandwidth | Rel-8 | C56 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.1.11 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | Rel-12 | C94 |  |  | UE supporting E-UTRA FD-FDD and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.11_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Rel-13 | C194 |  |  | UE supporting E-UTRA FDD and UE Category 1bis |  |  |  |  |  |  |  |
| 8.1.12 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Rel-12 | C95 |  |  | UE supporting E-UTRA FD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.12_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Rel-13 | C194a |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.1.13 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Rel-12 | C95 |  |  | UE supporting E-UTRA FD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.13_2 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Rel-13 | C194a |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.1.14 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | Rel-12 | C112 |  |  | UE supporting E-UTRA HD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.15 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Rel-12 | C112 |  |  | UE supporting E-UTRA HD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.16 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Rel-12 | C112 |  |  | UE supporting E-UTRA HD-FDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.17 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Rel-12 | C96 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.17_2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Rel-13 | C195a |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.1.18 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Rel-12 | C96 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.1.18_2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Rel-13 | C195a |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.1.19 | E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Rel-12 | C108 |  |  | UE supporting E-UTRA FD-FDD, CSG and intra-frequency SI acquisition in FDD for HO and Category 0 |  |  |  |  |  |  |  |
| 8.1.19_2 | E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis | Rel13 | C108a |  |  | UE supporting E-UTRA FDD-FDD, CSG and intra-frequency SI acquisition in FDD for HO and Category 1bis |  |  |  |  |  |  |  |
| 8.1.20 | E-UTRAN FDD – FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Rel-12 | C108 |  |  | UE supporting E-UTRA FD-FDD, CSG and intra-frequency SI acquisition in FDD for HO and Category 0 |  |  |  |  |  |  |  |
| 8.1.20_2 | E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis | Rel-13 | C108b |  |  | UE supporting E-UTRA FDD-FDD, CSG and intra-frequency SI acquisition in FDD for HO, Feature Group Indicator 5 and Category 1bis |  |  |  |  |  |  |  |
| 8.1.21 | E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Rel-12 | C109 |  |  | UE supporting E-UTRA HD-FDD, CSG and intra-frequency SI acquisition in FDD for HO and Category 0 |  |  |  |  |  |  |  |
| 8.1.22 | E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Rel-12 | C109 |  |  | UE supporting E-UTRA HD-FDD, CSG and intra-frequency SI acquisition in FDD for HO and Category 0 |  |  |  |  |  |  |  |
| 8.1.23 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.24 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Rel-13 | C94a |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.25 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Rel-13 | C94b |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.1.26 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.27 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.28 | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.29 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.30 | E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Rel-13 | C93c |  |  | UE supporting E-UTRA TDD and UE Category M1 and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.1.31 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Rel-13 | C94e |  |  | UE supporting E-UTRA FD-FDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 8.1.32 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Rel-13 | C94e |  |  | UE supporting E-UTRA FD-FDD and (UE category M1 and CE Mode B) |  |  |  |  |  |  |  |
| 8.1.33 | E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.34 | E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Rel-13 | C107a |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.35 | E-UTRAN TDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Rel-13 | C93a |  |  | UE supporting E-UTRA TDD and UE Category M1 |  |  |  |  |  |  |  |
| 8.1.36 | E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Rel-13 | C94g |  |  | UE supporting E-UTRA FD- FDD and (UE Category M1 and CE Mode B) and intra-frequency SI acquisition for HO |  |  |  |  |  |  |  |
| 8.1.37 | E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Rel-13 | C94g |  |  | UE supporting E-UTRA FD-FDD and (UE Category M1 and CE Mode B) and intra-frequency SI acquisition for HO |  |  |  |  |  |  |  |
| 8.1.40 | E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | Rel-14 | C205 |  |  | UE supporting E-UTRA FDD and highspeed measurement enhancement and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.2.1 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells | Rel-8 | C02c |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.2.2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Rel-8 | C02c |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 5 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.2.3 | E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Rel-9 | C15 |  |  | UE supporting E-UTRA TDD and intra-frequency SI acquisition in TDD for HO. |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.2.4 | E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Rel-9 | C15 |  |  | UE supporting E-UTRA TDD and intra-frequency SI acquisition in TDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.2.5 | E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 8.2.6 | E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 8.2.7 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Rel-12 | C113 |  |  | UE supporting E-UTRA TDD, CSG. inter-frequency SI acquisition in TDD for HO and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.2.8 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Rel-12 | C113 |  |  | UE supporting E-UTRA TDD, CSG. inter-frequency SI acquisition in TDD for HO and Feature Group Indicator 5 and UE Category 0 |  |  |  |  |  |  |  |
| 8.2.9 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB | Rel-13 | C93f |  |  | UE supporting E-UTRA TDD and (UE Category M1 and CE Mode B) and intra-frequency SI acquisition for HO |  |  |  |  |  |  |  |
| 8.2.10 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB | Rel-13 | C93f |  |  | UE supporting E-UTRA TDD and (UE Category M1 and CE Mode B) and intra-frequency SI acquisition for HO |  |  |  |  |  |  |  |
| 8.3.1 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Rel-8 | C01b |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 25 |  | It is not necessary for CA UEs to be tested in this test if 8.20.1 case is executed. |  |  |  | 2Rx, 4Rx |  |
| 8.3.1_2 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Rel-13 | C208 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.3.2 | E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Rel-8 | C01e |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 5 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.3.2_2 | E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis | Rel-13 | C210 |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 5 and 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.3.3 | E-UTRAN FDD-FDD inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used | Rel-8 | C01e |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 5 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.3.4 | E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Rel-9 | C14 |  |  | UE supporting E-UTRA FDD and inter-frequency SI acquisition in FDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.3.5 | E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Rel-9 | C14 |  |  | UE supporting E-UTRA FDD and inter-frequency SI acquisition in FDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.3.6 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells | Rel-10 | C47 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 25 and Measurement without gaps |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.3.7 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Rel-12 | FFS |  |  | FFS |  |  |  |  |  | FFS |  |
| 8.3.8 | FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX | Rel-12 | FFS |  |  | FFS |  |  |  |  |  | FFS |  |
| 8.3.9 | FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Rel-12 | FFS |  |  | FFS |  |  |  |  |  | FFS |  |
| 8.4.1 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Rel-8 | C02b |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 |  | It is not necessary for CA UEs to be tested in this test if 8.20.2 case is executed. |  |  |  | 2Rx, 4Rx |  |
| 8.4.1_2 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Rel-13 | C209 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.4.2 | E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Rel-8 | C02e |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 5 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.4.2_2 | E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis | Rel-13 | C211 |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 5 and 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.4.3 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used | Rel-8 | C02e |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 5 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.4.4 | E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Rel-9 | C16 |  |  | UE supporting E-UTRA TDD and inter-frequency SI acquisition in TDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.4.5 | E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Rel-9 | C16 |  |  | UE supporting E-UTRA TDD and inter-frequency SI acquisition in TDD for HO |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.4.6 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | Rel-12 | C02b |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.4.6_2 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis | Rel-13 | C209 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 and UE Category 1bis |  |  |  |  |  |  |  |
| 8.4.7 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Rel-12 | FFS |  |  | FFS |  |  |  |  |  | FFS |  |
| 8.5.1 | E-UTRAN FDD-UTRAN FDD event triggered reporting under fading propagation conditions | Rel-8 | C04g |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicators 15 and 22 |  | It is not necessary for CA UEs to be tested in this test if 8.20.3 case is executed. |  |  |  | 2Rx, 4Rx |  |
| 8.5.2 | E-UTRAN FDD-UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions | Rel-8 | C04f |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicators 5, 19 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.5.3 | E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions | Rel-8 | C04d |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicators 5, 15 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.5.4 | E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions | Rel-9 | C29 |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicator 15 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.5.6 | E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions | Rel-10 | C48 |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicator 15 and 22 and Measurement without gaps |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.5.7 | E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz bandwidth | Rel-8 | C55 |  |  | UE supporting E-UTRA FDD and only E-UTRA Band 31 and UTRA FDD and Feature Group Indicators 15 and 22 |  |  |  |  |  |  |  |
| 8.6.1 | E-UTRAN TDD-UTRAN FDD event triggered reporting under fading propagation conditions | Rel-8 | C07b |  |  | UE supporting E-UTRA TDD and UTRA FDD and Feature Group Indicators 15 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.7.1 | E-UTRAN TDD-UTRAN TDD event triggered reporting under fading propagation conditions | Rel-8 Only | C05b |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 15 and 22 |  | It is not necessary for CA UEs to be tested in this test if 8.20.4 case is executed. |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C83 |  |  | UE supporting E-UTRA TDD and UTRA TDD and not supporting UTRA FDD Feature Group Indicators 15 and 22 |  | It is not necessary for CA UEs to be tested in this test if 8.20.4 case is executed. |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C79 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 15 and 39 |  | It is not necessary for CA UEs to be tested in this test if 8.20.4 case is executed |  |  |  | 2Rx, 4Rx |  |
| 8.7.2 | E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions | Rel-8 Only | C05d |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 5, 15 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C84 |  |  | UE supporting E-UTRA TDD and UTRA TDD and not supporting UTRA FDD and Feature Group Indicators 5, 15 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C80 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 5, 15 and 39 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 8.7.3 | E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions | Rel-8 Only | C120 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 19 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C121 |  |  | UE supporting E-UTRA TDD and UTRA TDD and not supporting UTRA FDD and Feature Group Indicators 22 and 37 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C122 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 37 and 39 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 8.7.4 | E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Rel-9 | C79 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicator 15 and 39 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C31 |  |  | UE supporting E-UTRA TDD and UTRA TDD and not supporting UTRA FDD and Feature Group Indicator 15 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.8.1 | E-UTRAN FDD-GSM event triggered reporting in AWGN | Rel-8 | C08f |  |  | UE supporting E-UTRA FDD and GSM and Feature Group Indicator s 15 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.8.2 | E-UTRAN FDD - GSM event triggered reporting when DRX is used in AWGN | Rel-8 | C08d |  |  | UE supporting E-UTRA FDD and GSM and Feature Group Indicators 5, 15 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.9.1 | E-UTRAN FDD-UTRAN TDD event triggered reporting in fading propagation conditions | Rel-8 Only | C06b |  |  | UE supporting E-UTRA FDD and UTRA TDD and Feature Group Indicators 15 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C85 |  |  | UE supporting E-UTRA FDD and UTRA TDD and not supporting UTRA FDD and Feature Group Indicators 15 and 22 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C77 |  |  | UE supporting E-UTRA FDD and UTRA TDD and Feature Group Indicators 15 and 39 |  |  |  | Rel-9 UTRA TDD |  | 2Rx, 4Rx |  |
| 8.9.2 | E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Rel-9 | C78 |  |  | UE supporting E-UTRA FDD and UTRA TDD and not supporting UTRA FDD and Feature Group Indicator 15 and 22 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C77 |  |  | UE supporting E-UTRA FDD and UTRA TDD and Feature Group Indicators 15 and 39 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.10.1 | E-UTRAN TDD-GSM event triggered reporting in AWGN | Rel-8 | C09g |  |  | UE supporting E-UTRA TDD and GSM and Feature Group Indicators 15 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.10.2 | E-UTRAN TDD - GSM event triggered reporting when DRX is used in AWGN | Rel-8 | C09e |  |  | UE supporting E-UTRA TDD and GSM and Feature Group Indicators 5, 15 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.2.11 | E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | Rel-14 |  |  | C190 |  | UEs supporting E-UTRA TDD and high speed enhancement for measurement |  |  |  |  |  |  |
| 8.11.1 | Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions | Rel-8 | C01b |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.11.2 | E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions | Rel-8 | C02b |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.11.3 | E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions | Rel-8 | C04e |  |  | UE supporting E-UTRA FDD and UTRA FDD and Feature Group Indicators 22 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.11.4 | InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search | Rel-8 Only | C05e |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 22 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C86 |  |  | UE supporting E-UTRA TDD and UTRA TDD and not supporting UTRA FDD and Feature Group Indicators 22 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C82 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 25 and 39 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.11.5 | Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search; E-UTRA cells in fading; GSM cell in static propagation conditions | Rel-8 | C08b |  |  | UE supporting E-UTRA FDD and GSM and Feature Group Indicator 23 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.11.6 | Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search; E-UTRA cells in fading; GSM cell in static propagation conditions | Rel-8 | C09a |  |  | UE supporting E-UTRA TDD and GSM and Feature Group Indicator 23 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.12.1 | Void |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13.1 | Void |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.14.1 | E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Rel-9 | C22 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.14.2 | E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Rel-9 | C38 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 4 and 25 |  |  |  |  |  |  |  |
| 8.14.3 | E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Rel-9 | C39a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and inter-frequency SI acquisition in TDD for HO and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.15.1 | E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Rel-9 | C22 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.15.2 | E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Rel-9 | C38 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 4 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.15.3 | E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Rel-9 | C39 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and inter-frequency SI acquisition in FDD for HO and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.1 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX | Rel-10 | C32 |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.1 or TC 8.16.5 or TC 8.16.9 or TC 8.16.13 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.2 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX | Rel-10 | C33 |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.2 or TC 8.16.6 or TC 8.16.10 or TC 8.16.14 or TC 8.16.21 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.3 | E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Rel-10 | C32 |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.3 or TC 8.16.7 or TC 8.16.11 or TC 8.16.15 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.4 | E-UTRANTDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Rel-10 | C33 |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.4 or TC 8.16.8 or TC 8.16.12 or TC 8.16.16 or TC 8.16.22 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.5 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Rel-10 | C32c |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.1 or TC 8.16.5 or TC 8.16.9 or TC 8.16.13 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.6 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Rel-10 | C33c |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.2 or TC 8.16.6 or TC 8.16.10 or TC 8.16.14 or TC 8.16.21 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.7 | E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Rel-10 | C32c |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.3 or TC 8.16.7 or TC 8.16.11 or TC 8.16.15 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.8 | E-UTRAN TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Rel-10 | C33c |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.4 or TC 8.16.8 or TC 8.16.12 or TC 8.16.16 or TC 8.16.22 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.9 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Rel-11 | C32 |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.1 or TC 8.16.5 or TC 8.16.9 or TC 8.16.13 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.10 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Rel-11 | C33 |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.2 or TC 8.16.6 or TC 8.16.10 or TC 8.16.14 or TC 8.16.21 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.11 | E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Rel-11 | C32 |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.3 or TC 8.16.7 or TC 8.16.11 or TC 8.16.15 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.12 | E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Rel-11 | C33 |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.4 or TC 8.16.8 or TC 8.16.12 or TC 8.16.16 or TC 8.16.22 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.13 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz+5MHz | Rel-10 | C32 |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.1 or TC 8.16.5 or TC 8.16.9 or TC 8.16.13 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.14 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz+5MHz | Rel-10 | C33 |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.2 or TC 8.16.6 or TC 8.16.10 or TC 8.16.14 or TC 8.16.21 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.15 | E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5MHz+5MHz bandwidth | Rel-10 | C32 |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.3 or TC 8.16.7 or TC 8.16.11 or TC 8.16.15 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.16 | E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5MHz+5MHz bandwidth | Rel-10 | C33 |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.4 or TC 8.16.8 or TC 8.16.12 or TC 8.16.16 or TC 8.16.22 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.17 | E-UTRAN FDD activation and deactivation of known SCell in non-DRX | Rel-10 | C32b |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 25 |  | Either TC 8.16.17 or TC 8.16.17A shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.17A | E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | Rel-10 | C32a |  |  | UE supporting E-UTRA FDD and CA and Feature Group Indicator 25 |  | Either TC 8.16.17 or TC 8.16.17A shall be executed. (Note 1) |  |  |  |  |  |
| 8.16.18 | E-UTRAN TDD activation and deactivation of known SCell in non-DRX | Rel-10 | C33b |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 25 |  | Either TC 8.16.18 or TC 8.16.18A shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.18A | E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz +20MHz bandwidth | Rel-10 | C33a |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 25 |  | Either TC 8.16.18 or TC 8.16.18A shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.21 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz | Rel-10 | C33d |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.2 or TC 8.16.6 or TC 8.16.10 or TC 8.16.14 or TC 8.16.21 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.22 | E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz | Rel-10 | C33d |  |  | UE supporting E-UTRA TDD and CA and Feature Group Indicator 111 |  | Either TC 8.16.4 or TC 8.16.8 or TC 8.16.12 or TC 8.16.16 or TC 8.16.22 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.16.23 | E-UTRAN TDD-FDD CA event triggered reporting under deactivated SCell in non-DRX with PCell in FDD | Rel-12 | C67 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.24 | E-UTRAN TDD-FDD CA event triggered reporting under deactivated SCell in non-DRX with PCell in TDD | Rel-12 | C68 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.25 | E-UTRAN TDD-FDD CA event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD | Rel-12 | C67 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.26 | E-UTRAN TDD-FDD CA event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD | Rel-12 | C68 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.27 | E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in FDD | Rel-12 | C167 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.28 | E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in TDD | Rel-12 | C168 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.29 | 3DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX | Rel-10 | C163 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C164 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.30 | 3DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX | Rel-10 | C165 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C166 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.31 | E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD | Rel-12 | C167 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.32 | E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD | Rel-12 | C168 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.33 | E-UTRAN FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Rel-10 | C163 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C164 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.34 | E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Rel-10 | C165 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C166 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.35 | E-UTRAN TDD-FDD 3 DL CA activation and deactivation of known SCell in non-DRX with PCell in FDD | Rel-12 | C130 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.36 | E-UTRAN TDD-FDD 3 DL CA activation and deactivation of known SCell in non-DRX with PCell in TDD | Rel-12 | C131 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.37 | 3DL FDD CA activation and deactivation of known SCell in non-DRX | Rel-10 | C91 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | Ce92 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.38 | 3DL TDD CA activation and deactivation of known SCell in non-DRX | Rel-10 | C132 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C133 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.39 | E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD | Rel-12 | C130 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.40 | E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD | Rel-12 | C131 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.41 | 3DL FDD CA activation and deactivation of unknown SCell in non-DRX | Rel-10 | C91 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C92 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.42 | 3DL TDD CA activation and deactivation of unknown SCell in non-DRX | Rel-10 | C132 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C133 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.51 | 4DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | Rel-11 | C156 |  |  | UE supporting E-UTRA FDD and 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA, or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.52 | 4DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | Rel-11 | C161 |  |  | UE supporting E-UTRA TDD and 4DL intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA. and Feature Group Indicator 111. |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.53 | 4DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | Rel-12 | C187 |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and Feature Group Indicator 111 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.54 | 4DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | Rel-12 | C188 |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and Feature Group Indicator 111 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.55 | 4 DLFDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Rel-12 | C156 |  |  | UE supporting E-UTRA FDD and 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA, or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and Feature Group Indicator 111 |  |  |  | 2Rx, 4Rx |  | 2Rx, 4Rx |  |
| 8.16.56 | 4DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Rel-12 | C189 |  |  | UE supporting E-UTRA TDD and 4DL CA and Feature Group Indicator 111 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.57 | 4DL FDD CA activation and deactivation of know SCell in non-DRX | Rel-11 | C192 |  |  | UE supporting E-UTRA FDD and 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA, or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.58 | 4DL TDD CA activation and deactivation of know SCell in non-DRX | Rel-11 | C193 |  |  | UE supporting E-UTRA TDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA, or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA, or 4DL with intra-band contiguous and inter-band CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.59 | 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX | Rel-12 | C190 |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and Feature Group Indicator 25 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.60 | 4DL PCell in TDD CA activation and deactivation of known SCell in non-DRX | Rel-12 | C191 |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.61 | 4DL FDD CA activation and deactivation of unknown SCell in non-DRX | Rel-11 | C192 |  |  | UE supporting E-UTRA FDD and 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA, or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.62 | 4DL TDD CA activation and deactivation of unknown SCell in non-DRX | Rel-11 | C193 |  |  | UE supporting E-UTRA TDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous, or 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA, or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and inter-band CA and Feature Group Indicator 25 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.63 | 4 DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Rel-12 | C190 |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and Feature Group Indicator 25 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.64 | 4DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX | Rel-12 | C191 |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and Feature Group Indicator 25 |  |  |  |  |  |  | 2Rx, 4Rx |
| 8.16.65 | 5DL PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX | Rel-11 | C169 |  |  | UE supporting E-UTRA FDD and TDD with FDD as PCell and 5DL with Intra-band contiguous and Inter-band CA, or 5DL with Intra-band non-contiguous and Inter-band CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-12 | C170 |  |  | UE supporting E-UTRA FDD and TDD with FDD as PCell and 5DL with Inter-band CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.66 | 5DL PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX | Rel-11 | C169 |  |  | UE supporting E-UTRA FDD and TDD with TDD as PCell and 5DL with Intra-band contiguous and Inter-band CA, or 5DL with Intra-band non-contiguous and Inter-band CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-12 | C170 |  |  | UE supporting E-UTRA FDD and TDD with TDD as PCell and 5DL with Inter-band CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.67 | 5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX | Rel-12 | C190a |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.68 | 5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX | Rel-12 | C191a |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.69 | 5DL FDD CA activation and deactivation of unknown SCell in non-DRX | Rel-12 | C75c |  |  | UE supporting E-UTRA FDD and 5DL CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.70 | 5DL TDD CA activation and deactivation of unknown SCell in non-DRX | Rel-12 | C76c |  |  | UE supporting E-UTRA TDD and 5DL CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.71 | 5DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | Rel-11 | C171 |  |  | UE supporting E-UTRA FDD and 5DL with Intra-band contiguous and Inter-band CA, or 5DL with Intra-band non-contiguous and Inter-band CA, or 5DL with Intra-band non-contiguous and Intra-band contiguous CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-12 | C172 |  |  | UE supporting E-UTRA FDD and 5DL with Inter-band CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.72 | 5DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | Rel-11 | C173 |  |  | UE supporting E-UTRA TDD and 5DL with Intra-band contiguous and Inter-band CA, or 5DL with Intra-band non-contiguous and Inter-band CA, or 5DL with Intra-band non-contiguous and Intra-band contiguous CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-12 | C174 |  |  | UE supporting E-UTRA TDD and 5DL with Intra-band contiguous CA, or 5DL with Inter-band CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.73 | 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Rel-12 | C75d |  |  | UE supporting E-UTRA FDD and 5DL CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.74 | 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Rel-12 | C76d |  |  | UE supporting E-UTRA TDD and 5DL CA and Feature Group Indicator 111 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.75 | 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX | Rel-12 | C190a |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.76 | 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX | Rel-12 | C191a |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.16.77 | 5DL FDD CA activation and deactivation of known SCell in non-DRX | Rel-12 | C75c |  |  | UE supporting E-UTRA FDD and 5DL CA and Feature Group Indicator 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.18.1 | E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions | Rel-9 | C40 |  |  | UE supporting E-UTRA TDD and cdma2000 HRPD and Feature Group Indicator 15 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.19.1 | E-UTRAN TDD-CDMA2000 1X event triggered reporting under fading propagation conditions | Rel-9 | C41 |  |  | UE supporting E-UTRA TDD and cdma2000 1xRTT and Feature Group Indicator 15 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.20.1 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.20.2 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 8.20.2 or TC 8.20.2A or TC 8.20.2B shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.20.2A | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth | Rel-10 | C19a |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 8.20.2 or TC 8.20.2A or TC 8.20.2B shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.20.2B | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth | Rel-10 | C19b |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 8.20.2 or TC 8.20.2A or TC 8.20.2B shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.20.3 | E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | Rel-10 | C43 |  |  | UE supporting E-UTRA FDD, CA and UTRA FDD and Feature Group Indicator 15 |  |  |  |  |  | 2Rx, 4Rx |  |
| 8.20.4 | E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions | Rel-10 | C44 |  |  | UE supporting E-UTRA TDD, CA and UTRA TDD and Feature Group Indicator 15 |  | Either TC 8.20.4 or TC 8.20.4A or TC 8.20.4B shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.20.4A | E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth | Rel-10 | C44a |  |  | UE supporting E-UTRA TDD, CA and UTRA TDD and Feature Group Indicator 15 |  | Either TC 8.20.4 or TC 8.20.4A or TC 8.20.4B shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.20.4B | E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth | Rel-10 | C44b |  |  | UE supporting E-UTRA TDD, CA and UTRA TDD and Feature Group Indicator 15 |  | Either TC 8.20.4 or TC 8.20.4A or TC 8.20.4B shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 8.22.1 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Rel-12 | C01ch |  |  | UE supporting E-UTRA FDD and CRS based discovery signals measurement and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.22.2 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Rel-12 | C02ch |  |  | UE supporting E-UTRA TDD and CRS based discovery signals measurement and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.22.3 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Rel-12 | C01eh |  |  | UE supporting E-UTRA FDD and CRS based discovery signals measurement and Feature Group Indicators 5 and 25 |  |  |  |  |  |  |  |
| 8.22.4 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Rel-12 | C02eh |  |  | UE supporting E-UTRA TDD and CRS based discovery signals measurement and Feature Group Indicators 5 and 25 |  |  |  |  |  |  |  |
| 8.22.5 | E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Rel-12 | C97 |  |  | UE supporting E-UTRA FDD and CSI-RS based discovery signals measurement and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.22.6 | E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Rel-12 | C98 |  |  | UE supporting E-UTRA TDD and CSI-RS based discovery signals measurement and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.22.7 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Rel-12 | C99 |  |  | UE supporting E-UTRA FDD and CSI-RS based discovery signals measurement and Feature Group Indicators 5 and 25 |  |  |  |  |  |  |  |
| 8.22.8 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal | Rel-12 | C100 |  |  | UE supporting E-UTRA TDD and CSI-RS based discovery signals measurement and Feature Group Indicators 5 and 25 |  |  |  |  |  |  |  |
| 8.22.9 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Rel-12 | C126 |  |  | UE supporting E-UTRA FDD and CA and CRS based discovery signal measurement and Feature Group Indicators 111 |  |  |  |  |  |  |  |
| 8.22.10 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Rel-12 | C126 |  |  | UE supporting E-UTRA TDD and CA and CRS based discovery signal measurement and Feature Group Indicators 111 |  |  |  |  |  |  |  |
| 8.22.11 | E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Rel-12 | C118 |  |  | UE supporting E-UTRA FDD and CA and CSI-RS based discovery signal measurement |  |  |  |  |  |  |  |
| 8.22.12 | E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Rel-12 | C119 |  |  | UE supporting E-UTRA TDD and CA and CSI-RS based discovery signal measurement |  |  |  |  |  |  |  |
| 8.23.1 | E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Rel-12 | C134 |  |  | UE supporting E-UTRA FDD, Dual Connectivity and Feature Group Indicator 5 |  | It is not necessary for DC ASYNCH UEs to be tested in this test if 8.23.2 case is executed. (Note 2) |  |  |  |  |  |
| 8.23.2 | E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC | Rel-12 | C135 |  |  | UE supporting E-UTRA FDD, Dual Connectivity Asynch and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.23.3 | E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Rel-12 | C136 |  |  | UE supporting E-UTRA TDD, Dual Connectivity and Feature Group Indicator 5 |  |  |  |  |  |  |  |
| 8.23.4 | E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Rel-12 | C137 |  |  | UE supporting E-UTRA FDD, Dual Connectivity and Feature Group Indicator 5 and 25 |  | It is not necessary for DC ASYNCH UEs to be tested in this test if 8.23.5 case is executed. (Note 2) |  |  |  |  |  |
| 8.23.5 | E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC | Rel-12 | C138 |  |  | UE supporting E-UTRA FDD, Dual Connectivity Asynch and Feature Group Indicator 5 and 25 |  |  |  |  |  |  |  |
| 8.23.6 | E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Rel-12 | C139 |  |  | UE supporting E-UTRA TDD, Dual Connectivity and Feature Group Indicator 5 and 25 |  |  |  |  |  |  |  |
| 8.23.7 | E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC | Rel-12 | C176 |  |  | UE supporting E-UTRA FDD, Dual Connectivity |  | It is not necessary for DC ASYNCH UEs to be tested in this test if 8.23.2 case is executed. (Note 2) |  |  |  |  |  |
| 8.23.8 | E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC | Rel-12 | C177 |  |  | UE supporting E-UTRA FDD, Dual Connectivity Asynch |  |  |  |  |  |  |  |
| 8.23.9 | E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC | Rel-12 | C178 |  |  | UE supporting E-UTRA TDD, Dual Connectivity |  |  |  |  |  |  |  |
| 8.25.1 | E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN | Rel-13 | C179 |  |  | UE supporting E-UTRA FDD and WLAN Aggregation |  |  |  |  |  |  |  |
| 8.25.2 | E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN | Rel-13 | C180 |  |  | UE supporting E-UTRA TDD and WLAN Aggregation |  |  |  |  |  |  |  |
| 8.26.1 | E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX | Rel-13 | C144 |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and Feature Group Indicator 25 |  |  |  |  |  |  |  |
| 8.26.2 | E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX | Rel-13 | C159 |  |  | UE supporting E-UTRA TDD and downlink LAA and Feature Group Indicator 25 |  |  |  |  |  |  |  |
| 8.26.3 | E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Rel-13 | C145 |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and Feature Group Indicator 111 |  | It is not necessary for LAA UEs to execute this test if 8.26.3A case is executed (Note 3) |  |  |  |  |  |
| 8.26.3A | E-UTRAN FDD-FS3 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Rel-13 | C145a |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and Feature Group Indicator 111 |  |  |  |  |  |  |  |
| 8.26.4 | E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX | Rel-13 | C160 |  |  | UE supporting E-UTRA TDD and downlink LAA and Feature Group Indicator 111 |  | It is not necessary for LAA UEs to execute this test if 8.26.4A case is executed (Note 3) |  |  |  |  |  |
| 8.26.4A | E-UTRAN TDD-FS3 3DL Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX | Rel-13 | C160a |  |  | UE supporting E-UTRA TDD and downlink LAA on two SCells and Feature Group Indicator 111 |  |  |  |  |  |  |  |
| 8.26.5 | E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal | Rel-13 | C153 |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and CRS based discovery signals Feature Group Indicator 111 |  | It is not necessary for LAA UEs to execute this test if 8.26.5A case is executed (Note 3) |  |  |  |  |  |
| 8.26.5A | E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells | Rel-13 | C153a |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and CRS based discovery signals Feature Group Indicator 111 |  |  |  |  |  |  |  |
| 8.26.6 | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal | Rel-13 | C146 |  |  | UE supporting E-UTRA TDD and downlink LAA and CRS based discovery signals and Feature Group Indicator 111 |  | It is not necessary for LAA UEs to execute this test if 8.26.6A case is executed (Note 3) |  |  |  |  |  |
| 8.26.6A | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells | Rel-13 | C146a |  |  | UE supporting E-UTRA TDD and downlink LAA CRS based discovery signals and Feature Group Indicator 111 |  |  |  |  |  |  |  |
| 8.26.7 | E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal | Rel-13 | C198 |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and Feature Group Indicators 5 and 111 |  |  |  |  |  |  |  |
| 8.26.8 | E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal | Rel-13 | C199 |  |  | UE supporting E-UTRA TDD and downlink LAA and Feature Group Indicator 5 and 111 |  |  |  |  |  |  |  |
| 8.26.9 | E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Rel-13 | C147 |  |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and Feature Group Indicator 25 |  |  |  |  |  |  |  |
| 8.26.10 | E-UTRAN TDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Rel-13 | C148 |  |  | UE supporting E-UTRA TDD and downlink LAA and Feature Group Indicator 25 |  |  |  |  |  |  |  |
| Measurement Performance Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1 | FDD Intra Frequency Absolute RSRP Accuracy | Rel-8 to Rel-11 | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.1.1_1 | FDD Intra Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.1.1_2 | FDD Intra frequency Absolute RSRP accuracy for UE category 1bis | Rel-13 | C01k |  |  | UE supporting E-UTRA FDD and UE Category 1bis and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.1.2 | FDD Intra Frequency Relative Accuracy of RSRP | Rel-8 | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.1.2_2 | FDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis | Rel-13 | C01k |  |  | UE supporting E-UTRA FDD and UE Category 1bis and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.2.1 | TDD Intra Frequency Absolute RSRP Accuracy | Rel-8 to Rel-11 | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.2.1_1 | TDD Intra Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.2.1_2 | TDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis | Rel-13 | C02k |  |  | UE supporting E-UTRA TDD and UE Category 1bis and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.2.2 | TDD Intra Frequency Relative Accuracy of RSRP | Rel-8 | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.2.2_2 | TDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis | Rel-13 | C02k |  |  | UE supporting E-UTRA TDD and UE Category 1bis and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.3.1 | FDD - FDD Inter Frequency Absolute RSRP Accuracy | Rel-8 to Rel-11 | C01g |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.3.1_1 | FDD - FDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C01g |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.3.1_2 | FDD - FDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Rel-13 | C01l |  |  | UE supporting E-UTRA FDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.3.2 | FDD - FDD Inter Frequency Relative Accuracy of RSRP | Rel-8 to Rel-11 | C01g |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.3.2_1 | FDD - FDD Inter Frequency Relative Accuracy of RSRP (Rel-12 and forward) | Rel-12 | C01g |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.3.2_2 | FDD - FDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Rel-13 | C01l |  |  | UE supporting E-UTRA FDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.4.1 | TDD - TDD Inter Frequency Absolute RSRP Accuracy | Rel-8 to Rel-11 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.4.1_1 | TDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.4.1_2 | TDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Rel-13 | C02l |  |  | UE supporting E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.4.2 | TDD - TDD Inter Frequency Relative Accuracy of RSRP | Rel-8 to Rel-11 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.4.2_1 | TDD - TDD Inter Frequency Relative Accuracy of RSRP (Rel-12 and forward) | Rel-12 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.4.2_2 | TDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Rel-13 | C02l |  |  | UE supporting E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.5.1 | FDD - TDD Inter Frequency Absolute RSRP Accuracy | Rel-9 to Rel-11 | C42 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.5.1_1 | FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C42 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.5.1_2 | FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Rel-13 | C42a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.5.2 | FDD - TDD Inter Frequency Relative Accuracy of RSRP | Rel-9 to Rel-11 | C42 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.5.2_1 | FDD - TDD Inter Frequency Relative Accuracy of RSRP (Rel-12 and forward) | Rel-12 | C42 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.5.2_2 | FDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Rel-13 | C42a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.6.1 | FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | Rel-10 and Rel-11 only | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1 or TC 9.1.12.1 or TC 9.1.18.1 or TC 9.1.20.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.6.1_1 | FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Rel-12 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1_1 or TC 9.1.12.1_1 or TC 9.1.18.1_1 or TC 9.1.20.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.6.2 | FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | Rel-10 and Rel-11 only | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2 or TC 9.1.12.2 or TC 9.1.18.2 or TC 9.1.20.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.6.2_1 | FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Rel-12 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2_1 or TC 9.1.12.2_1 or TC 9.1.18.2_1 or TC 9.1.20.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.7.1 | TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation | Rel-10 and Rel-11 only | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1 or TC 9.1.13.1 or TC 9.1.19.1 or TC 9.1.21.1 or TC 9.1.24.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.7.1_1 | TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Rel-12 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1_1 or TC 9.1.13.1_1 or TC 9.1.19.1_1 or TC 9.1.21.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.7.2 | TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation | Rel-10 and Rel-11 only | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2 or TC 9.1.13.2 or TC 9.1.19.2 or TC 9.1.21.2 or TC 9.1.24.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.7.2_1 | TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel-12 and forward) | Rel-12 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2_1 or TC 9.1.13.2_1 or TC 9.1.19.2_1 or TC 9.1.21.2_1 or TC 9.1.24.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.8.1 | FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 and Rel-11 only | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.8.1_1 | FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Rel-12 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.8.2 | FDD Relative RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.9.1 | TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 and Rel-11 only | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.9.1_1 | TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Rel-12 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.9.2 | TDD Relative RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.10.1 | FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 and Rel-11 only | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.10.1_1 | FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) | Rel-12 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.10.2 | FDD Relative RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.11.1 | TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 and Rel-11 only | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.11.1_1 | TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) | Rel-12 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.11.2 | TDD Relative RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.12.1 | FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Rel-10 and Rel-11 only | C18a |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1 or TC 9.1.12.1 or TC 9.1.18.1 or TC 9.1.20.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.12.1_1 | FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel-12 and forward) | Rel-12 | C18a |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1_1 or TC 9.1.12.1_1 or TC 9.1.18.1_1 or TC 9.1.20.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.12.2 | FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Rel-10 and Rel-11 only | C18a |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2 or TC 9.1.12.2 or TC 9.1.18.2 or TC 9.1.20.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.12.2_1 | FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel-12 and forward) | Rel-12 | C18a |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2_1 or TC 9.1.12.2_1 or TC 9.1.18.2_1 or TC 9.1.20.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.13.1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Rel-10 and Rel-11 only | C19a |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1 or TC 9.1.13.1 or TC 9.1.19.1 or TC 9.1.21.1 or TC 9.1.24.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.13.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel-12 and forward) | Rel-12 | C19a |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1_1 or TC 9.1.13.1_1 or TC 9.1.19.1_1 or TC 9.1.21.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.13.2 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz | Rel-10 and Rel-11 only | C19a |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2 or TC 9.1.13.2 or TC 9.1.19.2 or TC 9.1.21.2 or TC 9.1.24.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.13.2_1 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel-12 and forward) | Rel-12 | C19a |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2_1 or TC 9.1.13.2_1 or TC 9.1.19.2_1 or TC 9.1.21.2_1 or TC 9.1.24.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.14.1 | FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 only | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.14.1_1 | FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel-12 and forward) | Rel-12 | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.14.2 | FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.15.1 | TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 only | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.15.1_1 | TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel-12 and forward) | Rel-12 | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.15.2 | TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS | Rel-11 | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.1.16.1 | FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Rel-8 to Rel-11 | C50 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.16.1_1 | FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel-12 and forward) | Rel-12 | C50 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.16.2 | FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Rel-8 | C50 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.17.1 | FDD - FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Rel-8 to Rel-11 | C51 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.17.1_1 | FDD - FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel-12 and forward) | Rel-12 | C51 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.17.2 | FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Rel-8 to Rel-11 | C51 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.17.2_1 | FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth (Rel-12 and forward) | Rel-12 | C51 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.1.18.1 | FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Rel-11 only | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1 or TC 9.1.12.1 or TC 9.1.18.1 or TC 9.1.20.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.18.1_1 | FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Rel-12 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1_1 or TC 9.1.12.1_1 or TC 9.1.18.1_1 or TC 9.1.20.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.18.2 | FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Rel-11 only | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2 or TC 9.1.12.2 or TC 9.1.18.2 or TC 9.1.20.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.18.2_1 | FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Rel-12 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2_1 or TC 9.1.12.2_1 or TC 9.1.18.2_1 or TC 9.1.20.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.19.1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Rel-11 only | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1 or TC 9.1.13.1 or TC 9.1.19.1 or TC 9.1.21.1 or TC 9.1.24.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.19.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Rel-12 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1_1 or TC 9.1.13.1_1 or TC 9.1.19.1_1 or TC 9.1.21.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.19.2 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Rel-11 only | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2 or TC 9.1.13.2 or TC 9.1.19.2 or TC 9.1.21.2 or TC 9.1.24.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.19.2_1 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Rel-12 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2_1 or TC 9.1.13.2_1 or TC 9.1.19.2_1 or TC 9.1.21.2_1 or TC 9.1.24.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.20.1 | FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Rel-10 and Rel-11 only | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1 or TC 9.1.12.1 or TC 9.1.18.1 or TC 9.1.20.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.20.1_1 | FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth (Rel-12 and forward) | Rel-12 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.1_1 or TC 9.1.12.1_1 or TC 9.1.18.1_1 or TC 9.1.20.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.20.2 | FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Rel-10 and Rel-11 only | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2 or TC 9.1.12.2 or TC 9.1.18.2 or TC 9.1.20.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.20.2_1 | FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth (Rel-12 and forward) | Rel-12 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.1.6.2_1 or TC 9.1.12.2_1 or TC 9.1.18.2_1 or TC 9.1.20.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.21.1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz | Rel-10 and Rel-11 only | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1 or TC 9.1.13.1 or TC 9.1.19.1 or TC 9.1.21.1 or TC 9.1.24.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.21.1_1 | TDD Absolute RSRP Accuracy for E-UTRAN Carrier Aggregation for 5MHz + 5MHz (Rel-12 and forward) | Rel-12 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1_1 or TC 9.1.13.1_1 or TC 9.1.19.1_1 or TC 9.1.21.1_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.21.2 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz | Rel-10 and Rel-11 only | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2 or TC 9.1.13.2 or TC 9.1.19.2 or TC 9.1.21.2 or TC 9.1.24.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.21.2_1 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz (Rel-12 and forward) | Rel-12 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2_1 or TC 9.1.13.2_1 or TC 9.1.19.2_1 or TC 9.1.21.2_1 or TC 9.1.24.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.22 | FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD | Rel-12 | C141 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.23 | FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD | Rel-12 | C142 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.24.1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz | Rel-10 and Rel-11 only | C19b |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.1 or TC 9.1.13.1 or TC 9.1.19.1 or TC 9.1.21.1 or TC 9.1.24.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.24.1_1 | TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) | Rel-12 | C19b |  |  | UE supporting E-UTRA TDD and CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.24.2 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz | Rel-10 and Rel-11 only | C19b |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2 or TC 9.1.13.2 or TC 9.1.19.2 or TC 9.1.21.2 or TC 9.1.24.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.24.2_1 | TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) | Rel-12 | C19b |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.1.7.2_1 or TC 9.1.13.2_1 or TC 9.1.19.2_1 or TC 9.1.21.2_1 or TC 9.1.24.2_1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.1.25 | FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Rel-12 | C101 |  |  | UE supporting E-UTRA FDD and CRS based discovery signals measurement and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.26 | TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Rel-12 | C102 |  |  | UE supporting E-UTRA TDD and CRS based discovery signals measurement and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.27 | FDD-FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Rel-12 | C103 |  |  | UE supporting E-UTRA FDD and CRS based discovery signals measurement and Feature Group Indicator 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.28 | TDD-TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Rel-12 | C104 |  |  | UE supporting E-UTRA TDD and CRS based discovery signals measurement and Feature Group Indicator 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.29 | FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Rel-12 | C114 |  |  | UE supporting E-UTRA FDD and CSI-RS based discovery signal measurement and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.30 | TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Rel-12 | C115 |  |  | UE supporting E-UTRA TDD and CSI-RS based discovery signal measurement and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.31 | FDD-FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Rel-12 | C116 |  |  | UE supporting E-UTRA FDD and CSI-RS based discovery signal measurement and Feature Group Indicator 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.32 | TDD-TDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Rel-12 | C117 |  |  | UE supporting E-UTRA TDD and CSI-RS based discovery signal measurement and Feature Group Indicator 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.33 | FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Rel-12 | C128 |  |  | UE supporting E-UTRA FDD and CA and CRS based discovery signal measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.34 | TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Rel-12 | C129 |  |  | UE supporting E-UTRA TDD and CA and CRS based discovery signal measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.35 | FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Rel-12 | C118 |  |  | UE supporting E-UTRA FDD and CA and CSI-RS based discovery signal measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.36 | TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Rel-12 | C119 |  |  | UE supporting E-UTRA TDD and CA and CSI-RS based discovery signal measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.37 | 3DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C69 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.38 | 3DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C70 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.39 | 3DL FDD RSRP for E-UTRAN in Carrier Aggregation | Rel-10 and Rel-11 only | C71 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 only | C72 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.39_1 | 3DL FDD RSRP for E-UTRAN in Carrier Aggregation(Rel-12 and forward) | Rel-12 | C75 |  |  | UE supporting E-UTRA FDD and 3DL CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.40 | 3DL TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-10 and Rel-11 only | C73 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DLwith intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 only | C74 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.40_1 | 3DL TDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) | Rel-12 | C76 |  |  | UE supporting E-UTRA TDD and 3DL CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.41.1 | FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Rel-12 | C88 |  |  | UE supporting E-UTRA FD-FDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.41.2 | FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Rel-12 | C88 |  |  | UE supporting E-UTRA FD-FDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.42.1 | HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Rel-12 | C89 |  |  | UE supporting E-UTRA HD-FDD (UE category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.42.2 | HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Rel-12 | C89 |  |  | UE supporting E-UTRA HD-FDD (UE category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.43.1 | TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Rel-12 | C90 |  |  | UE supporting E-UTRA TDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.43.2 | TDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Rel-12 | C90 |  |  | UE supporting E-UTRA TDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.44 | 4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C69a |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.45 | 4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C70a |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.46 | 4DL FDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C75a |  |  | UE supporting E-UTRA FDD and 4DL CA |  |  |  |  |  |  | 2Rx, 4Rx |
| 9.1.47 | 4DL TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C76a |  |  | UE supporting E-UTRA TDD and 4DL CA |  |  |  |  |  |  | 2Rx, 4Rx |
| 9.1.48 | 5 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C69b |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.49 | 5 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C70b |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.50 | 5DL FDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C75b |  |  | UE supporting E-UTRA FDD and 5DL CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.51 | 5DL TDD RSRP for E-UTRAN in Carrier Aggregation | Rel-12 | C76b |  |  | UE supporting E-UTRA TDD and 5DL CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.52 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Rel-13 | C94c |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.53 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Rel-13 | C107d |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.54 | TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Rel-13 | C93b |  |  | UE supporting E-UTRA TDD and UE Category M1 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.55 | FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell | Rel-13 | C149 |  |  | UE supporting E-UTRA FDD and Downlink LAA with FDD as Pcell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.56 | FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell | Rel-13 | C152 |  |  | UE supporting E-UTRA TDD and Downlink LAA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.57 | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Rel-13 | C107f |  |  | UE supporting E-UTRA FD-FDD and UE Category M1 and CE Mode B and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.58 | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Rel-13 | C107e |  |  | UE supporting E-UTRA HD-FDD and UE Category M1 and CE Mode B and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.59 | TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Rel-13 | C93d |  |  | UE supporting E-UTRA TDD and UE Category M1 and CE Mode B and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.1.60 | FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell | Rel-13 | C150 |  |  | UE supporting E-UTRA FDD and Downlink LAA with FDD as Pcell and CSI-RS based discovery signals and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.1.61 | FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell | Rel-13 | C151 |  |  | UE supporting E-UTRA TDD and Downlink LAA and CSI-RS based discovery signals and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.1.1 | FDD Intra Frequency Absolute RSRQ Accuracy | Rel-8 | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.1.1_2 | FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Rel-13 | C01k |  |  | UE supporting E-UTRA FDD and UE Category 1bis and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.2.2.1 | TDD Intra Frequency Absolute RSRQ Accuracy | Rel-8 | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.2.1_2 | TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Rel-13 | C02k |  |  | UE supporting E-UTRA TDD and UE Category 1bis and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.2.3.1 | FDD - FDD Inter Frequency Absolute RSRQ Accuracy | Rel-8 | C01g |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.3.1_2 | FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Rel-13 | C01l |  |  | UE supporting E-UTRA FDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.3.2 | FDD - FDD Inter Frequency Relative Accuracy of RSRQ | Rel-8 | C01g |  |  | UE supporting E-UTRA FDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.3.2_2 | FDD - FDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Rel-13 | C01l |  |  | UE supporting E-UTRA FDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.4.1 | TDD - TDD Inter Frequency Absolute RSRQ Accuracy | Rel-8 to Rel-11 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.4.1_1 | TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) | Rel-12 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.4.1_2 | TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Rel-13 | C02l |  |  | UE supporting E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.4.2 | TDD -TDD Inter Frequency Relative Accuracy of RSRQ | Rel-8 to Rel-11 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.4.2_1 | TDD -TDD Inter Frequency Relative Accuracy of RSRQ (Rel-12 and forward) | Rel-12 | C02g |  |  | UE supporting E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.4.2_2 | TDD -TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Rel-13 | C02l |  |  | UE supporting E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.4A.1 | FDD - TDD Inter Frequency Absolute RSRQ Accuracy | Rel-9 | C42 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.4A.1_2 | FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Rel-13 | C42a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.4A.2 | FDD - TDD Inter Frequency Relative Accuracy of RSRQ | Rel-9 | C42 |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.4A.2_2 | FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Rel-13 | C42a |  |  | UE supporting E-UTRA FDD and E-UTRA TDD and UE category 1bis and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.5.1 | FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.1 or TC 9.2.11.1 or TC 9.2.21.1 or TC 9.2.23.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.5.2 | FDD Relative RSRQ Accuracy E-UTRA for Carrier Aggregation | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.2 or TC 9.2.11.2 or TC 9.2.21.2 or TC 9.2.23.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.6.1 | TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.1 or TC 9.2.12.1 or TC 9.2.22.1 or TC 9.2.24.1 or TC 9.2.27.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.6.2 | TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.2 or TC 9.2.12.2 or TC 9.2.22.2 or TC 9.2.24.2 or TC 9.2.27.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.7.1 | FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.2.8.1 | TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.2.9.1 | FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C45 |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.2.10.1 | TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Rel-10 | C46 |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.2.11.1 | FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.1 or TC 9.2.11.1 or TC 9.2.21.1 or TC 9.2.23.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.11.2 | FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.2 or TC 9.2.11.2 or TC 9.2.21.2 or TC 9.2.23.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.12.1 | TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.1 or TC 9.2.12.1 or TC 9.2.22.1 or TC 9.2.24.1 or TC 9.2.27.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.12.2 | TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.2 or TC 9.2.12.2 or TC 9.2.22.2 or TC 9.2.24.2 or TC 9.2.27.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.15.1 | FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 | C59 |  |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.2.16.1 | TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Rel-11 | C60 |  |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  |  |  |  |  |  |  |
| 9.2.17.1 | FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Rel-8 | C50 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.2.18.1 | FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Rel-8 | C51 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.18.2 | FDD - FDD Inter Frequency Relative Accuracy of RSRQ for 5MHz Bandwidth | Rel-8 | C51 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and Feature Group Indicators 16 and 25 |  |  |  |  |  |  |  |
| 9.2.19.1 | FDD-FDD Inter Frequency absolute WB-RSRQ | Rel-11 | C01h |  |  | UE supporting E-UTRA FDD and WB-RSRQ measurement and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.20.1 | TDD-TDD Inter Frequency absolute WB-RSRQ | Rel-11 | C02h |  |  | UE supporting E-UTRA TDD and WB-RSRQ measurement and Feature Group Indicators 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.21.1 | FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Rel-11 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.1 or TC 9.2.11.1 or TC 9.2.21.1 or TC 9.2.23.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.21.2 | FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Rel-11 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.2 or TC 9.2.11.2 or TC 9.2.21.2 or TC 9.2.23.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.22.1 | TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Rel-11 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.1 or TC 9.2.12.1 or TC 9.2.22.1 or TC 9.2.24.1 or TC 9.2.27.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.22.2 | TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Rel-11 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.2 or TC 9.2.12.2 or TC 9.2.22.2 or TC 9.2.24.2 or TC 9.2.27.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.23.1 | FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.1 or TC 9.2.11.1 or TC 9.2.21.1 or TC 9.2.23.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.23.2 | FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Rel-10 | C18 |  |  | UE supporting E-UTRA FDD and CA |  | Either TC 9.2.5.2 or TC 9.2.11.2 or TC 9.2.21.2 or TC 9.2.23.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.24.1 | TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.1 or TC 9.2.12.1 or TC 9.2.22.1 or TC 9.2.24.1 or TC 9.2.27.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.24.2 | TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Rel-10 | C19 |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.2 or TC 9.2.12.2 or TC 9.2.22.2 or TC 9.2.24.2 or TC 9.2.27.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.25.1 | Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Rel-12 | C67 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.25.2 | Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Rel-12 | C67 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.26.1 | Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Rel-12 | C142 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.26.2 | Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Rel-12 | C142 |  |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.27.1 | TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Rel-10 | C19b |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.1 or TC 9.2.12.1 or TC 9.2.22.1 or TC 9.2.24.1 or TC 9.2.27.1 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.27.2 | TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Rel-10 | C19b |  |  | UE supporting E-UTRA TDD and CA |  | Either TC 9.2.6.2 or TC 9.2.12.2 or TC 9.2.22.2 or TC 9.2.24.2 or TC 9.2.27.2 shall be executed. (Note 1) |  |  |  | 2Rx, 4Rx |  |
| 9.2.28 | FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Rel-12 | C101 |  |  | UE supporting E-UTRA FDD and CRS based discovery signals measurement and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.29 | TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Rel-12 | C102 |  |  | UE supporting E-UTRA TDD and CRS based discovery signals measurement and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.30 | FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Rel-12 | C103 |  |  | UE supporting E-UTRA FDD and CRS based discovery signals measurement and Feature Group Indicator 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.31 | TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Rel-12 | C104 |  |  | UE supporting E-UTRA TDD and CRS based discovery signals measurement and Feature Group Indicator 16 and 25 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.32 | FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Rel-12 | C128 |  |  | UE supporting E-UTRA FDD and CA and CRS based discovery signal measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.33 | TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Rel-12 | C129 |  |  | UE supporting E-UTRA TDD and CA and CRS based discovery signal measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.38 | 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-12 | C69 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.39 | 3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-12 | C70 |  |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.40 | 3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-10 | C71 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C72 |  |  | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.41 | 3DL TDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-10 | C73 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-11 | C74 |  |  | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.42.1 | FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Rel-12 | C88 |  |  | UE supporting E-UTRA FD-FDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.2.43.1 | HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Rel-12 | C89 |  |  | UE supporting E-UTRA HD-FDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.2.44.1 | TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Rel-12 | C90 |  |  | UE supporting E-UTRA TDD (UE Category 0) and Feature Group Indicator 16 |  |  |  |  |  |  |  |
| 9.2.45 | 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-12 | C69a |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell |  |  |  |  |  |  |  |
| 9.2.46 | 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-12 | C70a |  |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell |  |  |  |  |  |  |  |
| 9.2.47 | 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-12 | C69b |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell |  |  |  |  |  |  |  |
| 9.2.48 | 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Rel-12 | C70b |  |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell |  |  |  |  |  |  |  |
| 9.2.51 | FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell | Rel-13 | C149 |  |  | UE supporting E-UTRA FDD and Downlink LAA with FDD as Pcell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.2.52 | FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell | Rel-13 | C152 |  |  | UE supporting E-UTRA TDD and Downlink LAA with TDD as PCell |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.3.1 | E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy | Rel-9 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.3.2 | E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy | Rel-9 | C07 |  |  | UE supporting E-UTRA TDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.3.3 | E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | Rel-9 | C52 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and UTRA FDD |  |  |  |  |  |  |  |
| 9.4.1 | E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy | Rel-9 | C04 |  |  | UE supporting E-UTRA FDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.4.2 | E-UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy | Rel-9 | C07 |  |  | UE supporting E-UTRA TDD and UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.4.3 | E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | Rel-9 | C52 |  |  | UE supporting E-UTRA FDD and E-UTRA Band 31 and UTRA FDD |  |  |  |  |  |  |  |
| 9.5.1 | E-UTRAN FDD - UTRA TDD PCCPCH RSCP absolute accuracy | Rel-9 | C65 |  |  | UE supporting E-UTRA FDD and UTRA TDD and Feature Group Indicators 39 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C105 |  |  | UE supporting E-UTRA FDD and UTRA TDD and Feature Group Indicators 22 and not supporting UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.5.2 | E-UTRAN TDD - UTRA TDD PCCPCH RSCP absolute accuracy | Rel-9 | C66 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 39 |  |  |  |  |  | 2Rx, 4Rx |  |
|  |  | Rel-9 | C106 |  |  | UE supporting E-UTRA TDD and UTRA TDD and Feature Group Indicators 22 and not supporting UTRA FDD |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.6.1 | GSM RSSI accuracy for E-UTRAN FDD | Rel-9 | C08g |  |  | UE supporting E-UTRA FDD and GSM and Feature Group Indicator 16 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.6.2 | GSM RSSI accuracy for E-UTRAN TDD | Rel-9 | C09h |  |  | UE supporting E-UTRA TDD and GSM and Feature Group Indicator 16 and 23 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.9.1.1 | FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Rel-10 and Rel-11 only | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.9.1.1_1 | FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.9.1.2 | FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Rel-10 | C01f |  |  | UE supporting E-UTRA FDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.9.2.1 | TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Rel-10 and Rel-11 only | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.9.2.1_1 | TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel-12 and forward) | Rel-12 | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.9.2.2 | TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Rel-10 | C02f |  |  | UE supporting E-UTRA TDD and Feature Group Indicator 16 |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.11.1 | FS3 average RSSI accuracy case (PCell using FDD) | Rel-13 | C157 |  |  | UE supporting E-UTRA FDD and Downlink LAA with FDD as Pcell and RSSI measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.11.2 | FS3 average RSSI accuracy case (PCell using TDD) | Rel-13 | C158 |  |  | UE supporting E-UTRA TDD and Downlink LAA with TDD as Pcell and RSSI measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.12.1 | FS3 channel occupancy test (PCell using FDD) | Rel-13 | C157 |  |  | UE supporting E-UTRA FDD and Downlink LAA with FDD as Pcell and channel occupancy measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 9.12.2 | FS3 channel occupancy test (PCell using TDD) | Rel-13 | C158 |  |  | UE supporting E-UTRA TDD and Downlink LAA with TDD as Pcell and channel occupancy measurement |  |  |  |  |  | 2Rx, 4Rx |  |
| 11.1 | V2V UE Transmission Timing Accuracy Test | Rel-14 | C203 |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |
| 11.2 | Interruptions due to V2V sidelink communication | Rel-14 | C204 |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |

Table 4.2-1a: Applicability of RRM conformance test cases Conditions

C01 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 THEN R ELSE N/A

C01a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/13 AND A.4.4-1a/25) THEN R ELSE N/A

C01b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/25) THEN R ELSE N/A

C01c IF (A.4.1-1/1 AND A.4.4-1a/5) THEN R ELSE N/A

C01ch IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.4-1a/5) THEN R ELSE N/A

C01d IF (A.4.1-1/1 AND A.4.4-1a/5 AND A.4.4-1a/13 AND A.4.4-1a/25) THEN R ELSE N/A

C01e IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

C01eh IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

C01f IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/16) THEN R ELSE N/A

C01g IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/16 AND A.4.4-1a/25) THEN R ELSE N/A

C01h IF (A.4.1-1/1 AND A.4.4-1a/16 AND A.4.4-1a/25 AND A.4.5-1/7) THEN R ELSE N/A

C01i IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND NOT(A.4.5-1/40)) THEN R ELSE N/A

C01j IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/5 AND NOT( A.4.5-1/40)) THEN R ELSE N/A

C01k IF (A.4.1-1/1 AND A.4.4-1a/16 AND A.4.3-4a/1a) THEN R ELSE N/A

C01m IF A.4.3-4a/1a AND A.4.1-1/1 THEN R ELSE N/A

C01n IF A.4.3-4a/1a AND (A.4.1-1/1 AND A.4.4-1a/25) THEN R ELSE N/A

C01l IF (A.4.1-1/1 AND A.4.4-1a/16 AND A.4.4-1a/25 AND A.4.3-4a/1a) THEN R ELSE N/A

C02 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/2 THEN R ELSE N/A

C02a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-1b/13 AND A.4.4-1b/25 AND NOT(A.4.5-1/41)) THEN R ELSE N/A

C02b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-1b/25) THEN R ELSE N/A

C02c IF (A.4.1-1/2 AND A.4.4-1b/5) THEN R ELSE N/A

C02ch IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.4-1b/5) THEN R ELSE N/A

C02d IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-1b/5 AND A.4.4-1b/13 AND A.4.4-1b/25) THEN R ELSE N/A

C02e IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-1b/5 AND A.4.4-1b/25) THEN R ELSE N/A

C02eh IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.4-1b/5 AND A.4.4-1b/25) THEN R ELSE N/A

C02f IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-1b/16) THEN R ELSE N/A

C02g IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-1b/16 AND A.4.4-1b/25) THEN R ELSE N/A

C02h IF (A.4.1-1/2 AND A.4.4-1b/16 AND A.4.4-1b/25 AND A.4.5-1/7) THEN R ELSE N/A

C02i IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (NOT(A.4.1-1/2 AND A.4.5-1/41)) THEN R ELSE N/A

C02j IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/5 AND NOT(A.4.5-1/41)) THEN R ELSE N/A

C02k IF (A.4.1-1/2 AND A.4.4-1b/16 AND A.4.3-4a/1a) THEN R ELSE N/A

C02l IF (A.4.1-1/2 AND A.4.4-1b/16 AND A.4.4-1b/25 AND A.4.3-4a/1a) THEN R ELSE N/A

C02m IF A.4.3-4a/1a AND A.4.1-1/2 THEN R ELSE N/A

C02n IF A.4.3-4a/1a AND (A.4.1-1/2 AND A.4.4-1b/25) THEN R ELSE N/A

C03 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/2) THEN R ELSE N/A

C04 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3) THEN R ELSE N/A

C04a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/8 AND A.4.4-1a/22) THEN R ELSE N/A

C04b Void

C04c Void

C04d IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/5 AND A.4.4-1a/15 AND A.4.4-1a/22) THEN R ELSE N/A

C04e IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/22 AND A.4.4-1a/25) THEN R ELSE N/A

C04f IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/5 AND A.4.4-1a/19 AND A.4.4-1a/22) THEN R ELSE N/A

C04g IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/15 AND A.4.4-1a/22) THEN R ELSE N/A

C05 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4) THEN R ELSE N/A

C05a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/8 AND A.4.4-1b/22) THEN R ELSE N/A

C05b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/15 AND A.4.4-1b/25) THEN R ELSE N/A

C05c Void

C05d IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/5 AND A.4.4-1b/15 AND A.4.4-1b/25) THEN R ELSE N/A

C05e IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/22 AND A.4.4-1b/25) THEN R ELSE N/A

C06 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4) THEN R ELSE N/A

C06a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4 AND A.4.4-1a/8 AND A.4.4-1a/22) THEN R ELSE N/A

C06b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4 AND A.4.4-1a/15 AND A.4.4-1a/22) THEN R ELSE N/A

C07 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/3) THEN R ELSE N/A

C07a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/3 AND A.4.4-1b/8 AND A.4.4-1b/22) THEN R ELSE N/A

C07b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/3 AND A.4.4-1b/15 AND A.4.4-1b/22) THEN R ELSE N/A

C07c Void

C08 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/5) THEN R ELSE N/A

C08a IF (A.4.1-1/1 AND A.4.1-1/5 AND A.4.5-1/16 AND A.4.4-1a/9 AND A.4.4-1a/23) THEN R ELSE N/A

C08b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/5 AND A.4.4-1a/23 AND A.4.4-1a/25) THEN R ELSE N/A

C08c Void

C08d IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/5 AND A.4.4-1a/5 AND A.4.4-1a/15 AND A.4.4-1a/23) THEN R ELSE N/A

C08e IF (A.4.1-1/1 AND A.4.1-1/5 AND A.4.5-1/16 AND A.4.4-1a/9 AND A.4.4-1a/15 AND A.4.4-1a/23) THEN R ELSE N/A

C08f IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/5 AND A.4.4-1a/15 AND A.4.4-1a/23) THEN R ELSE N/A

C08g IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/5 AND A.4.4-1a/16 AND A.4.4-1a/23) THEN R ELSE N/A

C09 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5) THEN R ELSE N/A

C09a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5 AND A.4.4-1b/23 AND A.4.4-1b/25) THEN R ELSE N/A

C09b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5 AND A.4.5-1/16 AND A.4.4-1b/9 AND A.4.4-1b/23) THEN R ELSE N/A

C09c Void

C09d Void

C09e IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5 AND A.4.4-1b/5 AND A.4.4-1b/15 AND A.4.4-1b/23) THEN R ELSE N/A

C09f IF (A.4.1-1/2 AND A.4.1-1/5 AND A.4.5-1/16 AND A.4.4-1b/9 AND A.4.4-1b/15 AND A.4.4-1b/23) THEN R ELSE N/A

C09g IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5 AND A.4.4-1b/15 AND A.4.4-1b/23) THEN R ELSE N/A

C09h IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5 AND A.4.4-1b/16 AND A.4.4-1b/23) THEN R ELSE N/A

C10 IF (A.4.1-1/1 AND A.4.1-1/6) THEN R ELSE N/A

C10a IF (A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1a/12 AND A.4.4-1a/26) THEN R ELSE N/A

C11 IF (A.4.1-1/1 AND A.4.1-1/7) THEN R ELSE N/A

C11a IF (A.4.1-1/1 AND A.4.1-1/7 AND A.4.4-1a/11 AND A.4.4-1a/24) THEN R ELSE N/A

C12 Void

C13 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.5-1/2) THEN R ELSE N/A

C14 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.5-1/3) THEN R ELSE N/A

C15 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.5-1/44) THEN R ELSE N/A

C16 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.5-1/45) THEN R ELSE N/A

C17 Void

C18 IF (A.4.1-1/1 AND A.4.2-1/2) THEN R ELSE N/A

C18a IF (A.4.1-1/1 AND A.4.2-1/2) AND A.4.3-3a/7 THEN R ELSE N/A

C18b Void

C19 IF (A.4.1-1/2 AND A.4.2-1/2) THEN R ELSE N/A

C19a IF (A.4.1-1/2 AND A.4.2-1/2) AND A.4.3-3a/7 THEN R ELSE N/A

C19b IF (A.4.1-1/2 AND A.4.2-1/2) AND A.4.3-3a/8 THEN R ELSE N/A

C20 Void

C21 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND A.4.1-1/2 AND (A.4.4-1a/5 AND A.4.4-1b/5) AND (A.4.4-1a/25 AND A.4.4-1b/25) AND (A.4.4-1a/30 AND A.4.4-1b/30) THEN R ELSE N/A

C22 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/2 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C23 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND NOT A.4.4-1a/5) THEN R ELSE N/A

C24 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND NOT A.4.4-1b/5) THEN R ELSE N/A

C25 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4) THEN R ELSE N/A

C26 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4) THEN R ELSE N/A

C27 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/5) THEN R ELSE N/A

C28 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/5) THEN R ELSE N/A

C29 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/15) THEN R ELSE N/A

C30 Void

C31 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND (A.4.1-1/4 AND NOT A.4.1-1/3) AND A.4.4-1b/15 AND A.4.4-1b/22) THEN R ELSE N/A

C32 IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.4-3a/111) THEN R ELSE N/A

C32a IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.4-1a/25) AND A.4.3-3a/7 THEN R ELSE N/A

C32b IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.4-1a/25) THEN R ELSE N/A

C32c IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.4-3a/111) AND A.4.3-3a/7 THEN R ELSE N/A

C33 IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.4-3b/111) THEN R ELSE N/A

C33a IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.4-1b/25) AND A.4.3-3a/7 THEN R ELSE N/A

C33b IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.4-1b/25) THEN R ELSE N/A

C33c IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.4-3b/111 ) AND A.4.3-3a/7 THEN R ELSE N/A

C33d IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.4-3b/111) AND A.4.3-3a/8 THEN R ELSE N/A

C34 IF (A.4.1-1/2 AND A.4.1-1/6) THEN R ELSE N/A

C35 IF (A.4.1-1/2 AND A.4.1-1/7) THEN R ELSE N/A

C36 IF (A.4.1-1/2 AND A.4.1-1/6 AND A.4.4-1b/12 AND A.4.4-1b/26) THEN R ELSE N/A

C37 IF (A.4.1-1/2 AND A.4.1-1/7 AND A.4.4-1b/11 AND A.4.4-1b/24) THEN R ELSE N/A

C38 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/2 AND (A.4.4-1a/4 AND A.4.4-1b/4) AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C39 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1/ 3 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C39a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1/45 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C40 IF (A.4.1-1/2 AND A.4.1-1/6 AND A.4.4-1b/15) THEN R ELSE N/A

C41 IF (A.4.1-1/2 AND A.4.1-1/7 AND A.4.4-1b/15) THEN R ELSE N/A

C42 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/2 AND (A.4.4-1a/16 AND A.4.4-1b/16) AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C42a IF (A.4.1-1/1 AND A.4.1-1/2 AND (A.4.4-1a/16 AND A.4.4-1b/16) AND (A.4.4-1a/25 AND A.4.4-1b/25) AND A.4.3-4a/1a) THEN R ELSE N/A

C43 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.2-1/2 AND A.4.4-1a/15) THEN R ELSE N/A

C44 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.2-1/2 AND A.4.4-1b/15) THEN R ELSE N/A

C44a IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.2-1/2 AND A.4.4-1b/15) AND A.4.3-3a/7 THEN R ELSE N/A

C44b IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.2-1/2 AND A.4.4-1b/15) AND A.4.3-3a/8 THEN R ELSE N/A

C45 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-3a/115) THEN R ELSE N/A

C46 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.4-3b/115) THEN R ELSE N/A

C47 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.4-1a/25 AND NOT A.4.5-1/4) THEN R ELSE N/A

C48 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/3 AND A.4.4-1a/15 AND A.4.4-1a/22 AND NOT A.4.5-1/5) THEN R ELSE N/A

C49 IF (A.4.1-1/1 AND A.4.5-1/6) THEN R ELSE N/A

C50 IF (A.4.1-1/1 AND A.4.3-3/31 AND A.4.4-1a/16) THEN R ELSE N/A

C51 IF (A.4.1-1/1 AND A.4.3-3/31 AND A.4.4-1a/16 AND A.4.4-1a/25) THEN R ELSE N/A

C52 IF (A.4.1-1/1 AND A.4.3-3/31 AND A.4.1-1/3) THEN R ELSE N/A

C53 IF (A.4.1-1/1 AND A.4.5-1/6 AND A.4.1-1/3) THEN R ELSE N/A

C54 IF (A.4.1-1/1 AND A.4.5-1/6 AND A.4.1-1/3 AND A.4.4-1a/8 AND A.4.4-1a/22) THEN R ELSE N/A

C55 IF (A.4.1-1/1 AND A.4.5-1/6 AND A.4.1-1/3 AND A.4.4-1a/15 AND A.4.4-1a/22) THEN R ELSE N/A

C56 IF (A.4.1-1/1 AND A.4.5-1/6 AND A.4.4-1a/5) THEN R ELSE N/A

C57 IF (A.4.1-1/1 AND ((A.4.6.1-1/1 OR A.4.6.1-1/2) AND (A.4.6.1-2/1 OR A.4.6.1-2/2)) AND A.4.4-1a/5) THEN R ELSE N/A

C58 IF (A.4.1-1/2 AND ((A.4.6.1-1/1 OR A.4.6.1-1/2) AND (A.4.6.1-2/1 OR A.4.6.1-2/2)) AND A.4.4-1b/5) THEN R ELSE N/A

C58a IF (A.4.1-1/2 AND ((A.4.6.1-1/1 OR A.4.6.1-1/2) AND (A.4.6.1-2/1 OR A.4.6.1-2/2)) AND A.4.4-1b/5) AND A.4.3-3a/8 THEN R ELSE N/A

C59 IF (A.4.1-1/1 AND A.4.5-2/1 AND A.4.4-3a/115) THEN R ELSE N/A

C60 IF (A.4.1-1/2 AND A.4.5-2/1 AND A.4.5-2/2 AND A.4.4-3b/115) THEN R ELSE N/A

C61 IF (A.4.1-1/1 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3) THEN R ELSE N/A

C62 IF (A.4.1-1/2 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3) THEN R ELSE N/A

C62a IF (A.4.1-1/2 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3 AND A.4.3-3a/7 ) THEN R ELSE N/A

C62b IF (A.4.1-1/2 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3 AND A.4.3-3a/8) THEN R ELSE N/A

C63 IF (A.4.1-1/1 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3 AND A.4.4-1a/5) THEN R ELSE N/A

C64 IF (A.4.1-1/2 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3 AND A.4.4-1b/5) THEN R ELSE N/A

C64a IF (A.4.1-1/2 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3 AND A.4.4-1b/5 AND A.4.3-3a/7) THEN R ELSE N/A

C64b IF (A.4.1-1/2 AND (((A.4.6.1-1/1 AND A.4.6.1-2/1) OR (A.4.6.1-1/2 AND A.4.6.1-2/2)) OR (A.4.6.2-1/1 AND A.4.6.2-2/1) OR (A.4.6.3-1/1 AND A.4.6.3-2/1)) AND A.4.5-2/3 AND A.4.4-1b/5 AND A.4.3-3a/8) THEN R ELSE N/A

C65 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4 AND A.4.4-2a/39) THEN R ELSE N/A

C66 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-2b/39) THEN R ELSE N/A

C67 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15 AND (A.4.4-3a/111 AND A.4.4-3b/111)) THEN R ELSE N/A

C68 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14 AND (A.4.4-3a/111 AND A.4.4-3b/111)) THEN R ELSE N/A

C69 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15) THEN R ELSE N/A

C69a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15) THEN R ELSE N/A

C69b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15) THEN R ELSE N/A

C70 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14) THEN R ELSE N/A

C70a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14) THEN R ELSE N/A

C70b IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14) THEN R ELSE N/A

C71 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4)) THEN R ELSE N/A

C72 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2)) THEN R ELSE N/A

C73 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4)) THEN R ELSE N/A

C74 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2)) THEN R ELSE N/A

C75 IF (A.4.1-1/1 AND A.4.6-1/2) THEN R ELSE N/A

C75a IF (A.4.1-1/1 AND A.4.6-1/3) THEN R ELSE N/A

C75b IF (A.4.1-1/1 AND A.4.6-1/4) THEN R ELSE N/A

C75c IF (A.4.1-1/1 AND A.4.6-1/4 AND A.4.4-1a/25) THEN R ELSE N/A

C75d IF (A.4.1-1/1 AND A.4.6-1/4 AND A.4.4-1a/111) THEN R ELSE N/A

C76 IF (A.4.1-1/2 AND A.4.6-1/2) THEN R ELSE N/A

C76a IF (A.4.1-1/2 AND A.4.6-1/3) THEN R ELSE N/A

C76b IF (A.4.1-1/2 AND A.4.6-1/4) THEN R ELSE N/A

C76c IF (A.4.1-1/2 AND A.4.6-1/4 AND A.4.4-1b/25) THEN R ELSE N/A

C76d IF (A.4.1-1/2 AND A.4.6-1/4 AND A.4.4-1b/111) THEN R ELSE N/A

C77 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4 AND A.4.4-1a/15 AND A.4.4-2a/39) THEN R ELSE N/A

C78 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4 AND A.4.4-1a/15 AND A.4.4-1a/22) THEN R ELSE N/A

C79 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/15 AND A.4.4-2b/39) THEN R ELSE N/A

C80 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/5 AND A.4.4-1b/15 AND A.4.4-2b/39) THEN R ELSE N/A

C81 Void

C82 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/25 AND A.4.4-2b/39) THEN R ELSE N/A

C83 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND (A.4.1-1/4 AND NOT A.4.1-1/3) AND A.4.4-1b/15 AND A.4.4-1b/25) THEN R ELSE N/A

C84 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND (A.4.1-1/4 AND NOT A.4.1-1/3) AND A.4.4-1b/5 AND A.4.4-1b/15 AND A.4.4-1b/25) THEN R ELSE N/A

C85 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND (A.4.1-1/4 AND NOT A.4.1-1/3) AND A.4.4-1a/15 AND A.4.4-1a/22) THEN R ELSE N/A

C86 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND (A.4.1-1/4 AND NOT A.4.1-1/3) AND A.4.4-1b/22 AND A.4.4-1b/25) THEN R ELSE N/A

C87 Void

C88 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND NOT A.4.3-7/2 AND A.4.3-4a/1 AND A.4.4-1a/16) THEN R ELSE N/A

C89 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.3-4a/1 AND A.4.3-7/2 AND A.4.4-1a/16) THEN R ELSE N/A

C90 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.3-4a/1 AND A.4.4-1b/16) THEN R ELSE N/A

C91 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.4-1a/25) THEN R ELSE N/A

C92 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND A.4.4-1a/25) THEN R ELSE N/A

C93 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/2 AND A.4.3-4a/1 THEN R ELSE N/A

C93a IF A.4.1-1/2 AND A.4.3-4aa/1 THEN R ELSE N/A

C93b IF A.4.1-1/2 AND A.4.3-4aa/1 AND A.4.4-1a/16 THEN R ELSE N/A

C93c IF A.4.1-1/2 AND A.4.3-4aa/1 AND A.4.4-1a/5 THEN R ELSE N/A

C93d IF A.4.1-1/2 AND A.4.3-4aa/1 AND A.4.5-1/26 AND A.4.4-1a/16 THEN R ELSE N/A

C93e IF A.4.1-1/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) THEN R ELSE N/A

C93f IF A.4.1-1/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) AND A.4.5-1/2 THEN R ELSE N/A

C94 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND NOT A.4.3-7/2 AND A.4.3-4a/1 THEN R ELSE N/A

C94a IF A.4.1-1/1 AND NOT A.4.3-7/2AND A.4.3-4aa/1 THEN R ELSE N/A

C94b IF A.4.1-1/1 AND NOT A.4.3-7/2 AND A.4.3-4aa/1 AND A.4.4-1a/5 THEN R ELSE N/A

C94c IF A.4.1-1/1 AND NOT A.4.3-7/2 AND A.4.3-4aa/1 AND A.4.4-1a/16 THEN R ELSE N/A

C94d Void

C94e IF A.4.1-1/1 AND NOT A.4.3-7/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) THEN R ELSE N/A

C94f IF A.4.1-1/1 AND A.4.3-7/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) THEN R ELSE N/A

C94g IF A.4.1-1/1 AND NOT A.4.3-7/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) AND A.4.5-1/2 THEN R ELSE N/A

C94h IF A.4.1-1/1 AND NOT A.4.3-7/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) AND A.4.4-1a/5 THEN R ELSE N/A

C94i IF A.4.1-1/1 AND A.4.3-7/2 AND (A.4.3-4aa/1 AND A.4.5-1/26) AND A.4.4-1a/5 THEN R ELSE N/A

C94k Void

C94m Void

C95 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND NOT A.4.3-7/2 AND A.4.4-1a/5 AND A.4.3-4a/1 THEN R ELSE N/A

C96 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/2 AND A.4.4-1b/5 AND A.4.3-4a/1 THEN R ELSE N/A

C97 IF (A.4.1-1/1 AND A.4.5-1/20 AND A.4.4-1a/5) THEN R ELSE N/A

C98 IF (A.4.1-1/2 AND A.4.5-1/20 AND A.4.4-1b/5) THEN R ELSE N/A

C99 IF (A.4.1-1/1 AND A.4.5-1/20 AND A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

C100 IF (A.4.1-1/2 AND A.4.5-1/20 AND A.4.4-1b/5 AND A.4.4-1b/25) THEN R ELSE N/A

C101 IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.4-1a/16) THEN R ELSE N/A

C102 IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.4-1b/16) THEN R ELSE N/A

C103 IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.4-1a/16 AND A.4.4-1a/25) THEN R ELSE N/A

C104 IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.4-1b/16 AND A.4.4-1b/25) THEN R ELSE N/A

C105 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/1 AND A.4.1-1/4 AND (NOT A.4.1-1/3) AND A.4.4-1a/22) THEN R ELSE N/A

C106 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND (NOT A.4.1-1/3) AND A.4.4-1b/22) THEN R ELSE N/A

C107 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND A.4.3-4a/1 AND A.4.3-7/2 THEN R ELSE N/A

C107a IF A.4.1-1/1 AND A.4.3-7/2 AND A.4.3-4aa/1 THEN R ELSE N/A

C107b Void

C107c IF A.4.1-1/1 AND A.4.3-7/2 AND A.4.3-4aa/1 AND A.4.4-1a/5 THEN R ELSE N/A

C107d IF A.4.1-1/1 AND A.4.3-7/2 AND A.4.3-4aa/1 AND A.4.4-1a/16 THEN R ELSE N/A

C107e IF A.4.1-1/1 AND A.4.3-7/2 AND A.4.3-4aa/1 AND A.4.5-1/26 AND A.4.4-1a/16 THEN R ELSE N/A

C107f IF A.4.1-1/1 AND NOT A.4.3-7/2 AND A.4.3-4aa/1 AND A.4.5-1/26 AND A.4.4-1a/16 THEN R ELSE N/A

C108 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/2 AND NOT A.4.3-7/2 AND A.4.5-1/1 AND A.4.5-1/2 AND A.4.3-4a/1 THEN R ELSE N/A

C108a IF (A.4.3-4a/1a AND A.4.1-1/1) AND A.4.5-1/1 AND A.4.5-1/2 AND THEN R ELSE N/A

C108b IF (A.4.3-4a/1a AND A.4.1-1/1) AND A.4.5-1/1 AND A.4.5-1/2 AND A.4.4-1a/5 THEN R ELSE N/A

C109 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/2 AND A.4.5-1/1 AND A.4.5-1/2 AND A.4.3-4a/1 AND A.4.3-7/2 THEN R ELSE N/A

C110 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND A.4.3-7/2 AND A.4.3-4a/1 THEN R ELSE N/A

C111 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND A.4.3-7/2 AND A.4.4-1a/5 AND A.4.3-4a/1 THEN R ELSE N/A

C112 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/1 AND A.4.4-1a/5 AND A.4.3-4a/1 AND A.4.3-7/2 THEN R ELSE N/A

C113 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND A.4.1-1/2 AND A.4.5-1/1 AND A.4.5-1/44 AND A.4.4-1b/5 AND A.4.3-4a/1 THEN R ELSE N/A

C114 IF (A.4.1-1/1 AND A.4.5-1/20 AND A.4.4-1a/16) THEN R ELSE N/A

C115 IF (A.4.1-1/2 AND A.4.5-1/20 AND A.4.4-1b/16) THEN R ELSE N/A

C116 IF (A.4.1-1/1 AND A.4.5-1/20 AND A.4.4-1a/16 AND A.4.4-1a/25) THEN R ELSE N/A

C117 IF (A.4.1-1/2 AND A.4.5-1/20 AND A.4.4-1b/16 AND A.4.4-1b/25) THEN R ELSE N/A

C118 IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.5-1/20) THEN R ELSE N/A

C119 IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.5-1/20) THEN R ELSE N/A

C120 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-1b/19 AND A.4.4-1b/22) THEN R ELSE N/A

C121 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND (A.4.1-1/4 AND NOT A.4.1-1/3) AND A.4.4-1b/22 AND A.4.4-2b/37) THEN R ELSE N/A

C122 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1)) AND (A.4.1-1/2 AND A.4.1-1/4 AND A.4.4-2b/37 AND A.4.4-2b/39) THEN R ELSE N/A

C123 IF A.4.1-1/1 AND A.4.2-1/8 THEN R ELSE N/A

C123a IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.2-1/8 THEN R ELSE N/A

C123b IF A.4.1-1/1 AND A.4.2-1/8 AND A.4.5-1/40 THEN R ELSE N/A

C124 IF A.4.1-1/2 AND A.4.2-1/8 THEN R ELSE N/A

C125 IF A.4.1-1/1 AND A.4.5-1/27 THEN R ELSE N/A

C125a IF A.4.1-1/1 AND A.4.5-1/27 AND A.4.5-1/40 THEN R ELSE N/A

C126 IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.5-1/19 AND A.4.4-3a/111) THEN R ELSE N/A

C127 Void

C128 IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.5-1/19) THEN R ELSE N/A

C129 IF (A.4.1-1/2 AND A.4.2-1/2 AND A.4.5-1/19) THEN R ELSE N/A

C130 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND A.4.4-1a/25) THEN R ELSE N/A

C131 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND A.4.4-1a/25) THEN R ELSE N/A

C132 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.4-1b/25) THEN R ELSE N/A

C133 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND A.4.4-1a/25) THEN R ELSE N/A

C134 IF A.4.1-1/1 AND A.4.2-1/8 AND A.4.4-1a/5 THEN R ELSE N/A

C135 IF A.4.1-1/1 AND (A.4.2-1/8 AND A.4.5-1/27) AND A.4.4-1a/5 THEN R ELSE N/A

C136 IF A.4.1-1/2 AND A.4.2-1/8 AND A.4.4-1b/5 THEN R ELSE N/A

C137 IF A.4.1-1/1 AND A.4.2-1/8 AND (A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

C138 IF A.4.1-1/1 AND (A.4.2-1/8 AND A.4.5-1/27) AND (A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

C139 IF A.4.1-1/2 AND A.4.2-1/8 AND (A.4.4-1b/5 AND A.4.4-1b/25) THEN R ELSE N/A

C140 IF (A.4.1-1/1 AND A.4.5-1/40) THEN R ELSE N/A

C141 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/15) THEN R ELSE N/A

C142 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/1 AND A.4.5-1/14) THEN R ELSE N/A

C143 IF (A.4.1-1/2 AND A.4.5-1/41) THEN R ELSE N/A

C144 IF (A.4.1-1/1 AND A.4.5-1/32 AND A.4.4-1a/25) THEN R ELSE N/A

C145 IF (A.4.1-1/1 AND A.4.5-1/32 AND A.4.4-3a/111 AND A.4.6-1/1) THEN R ELSE N/A

C145a IF (A.4.1-1/1 AND A.4.5-1/15 AND A.4.5-1/32 AND A.4.4-1a/111 AND (NOT A.4.6.1-1/1)) THEN R ELSE N/A

C146 IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.4-3a/111 AND A.4.6-1/1) THEN R ELSE N/A

C146a IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.4-3a/111 AND (NOT A.4.6-1/1)) THEN R ELSE N/A

C147 IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.4-3 a/111) THEN R ELSE N/A

C148 IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.4-3 a/111) THEN R ELSE N/A

C149 IF (A.4.1-1/1 AND A.4.5-1/32) THEN R ELSE N/A

C150 IF (A.4.1-1/1 AND A.4.5-1/57 AND A.4.5-1/32 AND A.4.5-1/61) THEN R ELSE N/A

C151 IF (A.4.1-1/2 AND A.4.5-1/57 AND A.4.5-1/32 AND A.4.5-1/61) THEN R ELSE N/A

C152 IF (A.4.1-1/2 AND A.4.5-1/32) THEN R ELSE N/A

C153 IF (A.4.1-1/1 AND A.4.5-1/32 AND A.4.5-1/19 AND A.4.4-3a/111 AND A.4.6-1/1) THEN R ELSE N/A

C153a IF (A.4.1-1/1 AND A.4.5-1/32 AND A.4.5-1/19 AND A.4.4-3a/111 AND (NOT A.4.6-1/1)) THEN R ELSE N/A

C154 IF (A.4.1-1/8 AND A.4.3-4c/1) THEN R ELSE N/A

C155 IF (A.4.1-1/8 AND A.4.3-4c/1 AND A.4.4-1a/5) THEN R ELSE N/A

C156 IF A.4.1-1/1 AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.3-1/11 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND A.4.4-3a/111 THEN R ELSE N/A

C157 IF (A.4.1-1/1 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.5-1/33) THEN R ELSE N/A

C158 IF (A.4.1-1/2 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.5-1/33) THEN R ELSE N/A

C159 IF (A.4.1-1/2 AND A.4.5-1/32 AND A.4.4-1a/25) THEN R ELSE N/A

C160 IF (A.4.1-1/2 AND A.4.5-1/32 AND A.4.4-3a/111 AND A.4.6-1/1) THEN R ELSE N/A

C160a IF (A.4.1-1/2 AND A.4.5-1/32 AND A.4.4-1a/111 AND (NOT A.4.6.1-1/1)) THEN R ELSE N/A

C161 IF A.4.1-1/2 AND (A.4.6.1-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/12 OR A.4.6.3-1/11 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND A.4.4-3a/111 THEN R ELSE N/A

C162 IF (A.4.1-1/8 AND A.4.3-4c/1 AND A.4.5-1/34) THEN R ELSE N/A

C163 IF (A.4.1-1/1 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.4-3a/111) THEN R ELSE N/A

C164 IF (A.4.1-1/1 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND A.4.4-3a/111) THEN R ELSE N/A

C165 IF (A.4.1-1/2 AND (A.4.6.1-1/3 OR A.4.6.3-1/3 OR A.4.6.3-1/4) AND A.4.4-3b/111) THEN R ELSE N/A

C166 IF (A.4.1-1/2 AND (A.4.6.3-1/2 OR A.4.6.2-1/2) AND A.4.4-3b/111) THEN R ELSE N/A

C167 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/15 AND (A.4.4-3a/111 AND A.4.4-3b/111)) THEN R ELSE N/A

C168 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/2 AND A.4.5-1/14 AND (A.4.4-3a/111 AND A.4.4-3b/111)) THEN R ELSE N/A

C169 IF (A.4.1-1/1 AND A.4.1-1/2) AND (A.4.6.2-1/6 OR A.4.6.3-1/13 OR A.4.6.2-1/8 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17) AND A.4.4-3a/111 THEN R ELSE N/A

C170 IF (A.4.1-1/1 AND A.4.1-1/2) AND (A.4.6.3-1/14) AND A.4.4-3a/111 THEN R ELSE N/A

C171 IF A.4.1-1/1 AND (A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8) AND A.4.4-3a/111 THEN R ELSE N/A

C172 IF A.4.1-1/1 AND (A.4.6.3-1/14) AND A.4.4-3a/111 THEN R ELSE N/A

C173 IF A.4.1-1/2 AND (A.4.6.3-1/13 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.2-1/6 OR A.4.6.2-1/7 OR A.4.6.2-1/8) AND A.4.4-3a/111 THEN R ELSE N/A

C174 IF A.4.1-1/2 AND (A.4.6.3-1/14 OR A.4.6.1-1/5) AND A.4.4-3a/111 THEN R ELSE N/A

C175 IF A.4.1-1/1 AND A.4.2-1/8 AND A.4.4-1a/5 THEN R ELSE N/A

C176 IF A.4.1-1/1 AND A.4.2-1/8 THEN R ELSE N/A

C177 IF A.4.1-1/1 AND (A.4.2-1/8 AND A.4.5-1/27) THEN R ELSE N/A

C178 IF A.4.1-1/2 AND A.4.2-1/8 THEN R ELSE N/A

C179 IF (A.4.1-1/1 AND A.4.1-1/9) AND (A.4.2-1/9 OR A.4.2-1/10) THEN R ELSE N/A

C180 IF (A.4.1-1/2 AND A.4.1-1/9) AND (A.4.2-1/9 OR A.4.2-1/10) THEN R ELSE N/A

C181 IF (A.4.1-1/1 AND A.4.4-1a/5 AND A.4.5-1/40) THEN R ELSE N/A

C182 IF (A.4.1-1/2 AND A.4.4-1a/5 AND A.4.5-1/41) THEN R ELSE N/A

C183 Void

C184 Void

C185 IF (A.4.1-1/1 AND A.4.2-1/8 AND A.4.5-1/40) THEN R ELSE N/A

C186 IF (A.4.1-1/1 AND A.4.5-1/27 AND A.4.5-1/40) THEN R ELSE N/A

C187 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (A.4.4-3a/111 AND A.4.4-3b/111)) THEN R ELSE N/A

C188 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (A.4.4-3a/111 AND A.4.4-3b/111)) THEN R ELSE N/A

C189 IF (A.4.1-1/2 AND A.4.6-1/3 AND A.4.4-3b/111) THEN R ELSE N/A

C190 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/15 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C190a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/15 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C191 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/3 AND A.4.5-1/14 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C191a IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/4 AND A.4.5-1/14 AND (A.4.4-1a/25 AND A.4.4-1b/25)) THEN R ELSE N/A

C192 IF (A.4.1-1/2 AND (A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND A.4.4-1b/25) THEN R ELSE N/A

C193 IF (A.4.1-1/2 AND (A.4.6.1-1/4 OR A.4.6.3-1/6 OR A.4.6.3-1/7 OR A.4.6.3-1/9 OR A.4.6.3-1/10 OR A.4.6.3-1/11 OR A.4.6.3-1/12 OR A.4.6.2-1/4 OR A.4.6.2-1/5) AND A.4.4-1b/25) THEN R ELSE N/A

C194 IF (A.4.1-1/1 AND A.4.3-4a/1a) THEN R ELSE N/A

C195 IF (A.4.1-1/2 AND A.4.3-4a/1a) THEN R ELSE N/A

C196 IF (A.4.1-1/1 AND A.4.3-7/5) THEN R ELSE N/A

C197 IF (A.4.1-1/2 AND A.4.3-7/5) THEN R ELSE N/A

C198 IF (A.4.1-1/1 AND 4.4-1a/5 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.4-3a/111 AND A.4.6-1/1) THEN R ELSE N/A

C199 IF (A.4.1-1/2 AND 4.4-1a/5 AND A.4.5-1/19 AND A.4.5-1/32 AND A.4.4-3a/111 AND A.4.6-1/1) THEN R ELSE N/A

C200 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.2-1/8 AND A.4.5-1/59) THEN R ELSE N/A

C201 IF (A.4.1-1/2 AND A.4.2-1/8 AND A.4.5-1/59) THEN R ELSE N/A

C202 IF (A.4.1-1/8 AND A.4.3-4c/1 AND A.4.5-1/60) THEN R ELSE N/A

C203 IF (A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7/1) THEN R ELSE N/A

C204 IF ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/10 AND A.4.3-4d/2 AND A.4.5-7/1) THEN R ELSE N/A

C205 IF (A.4.1-1/1 AND A.4.4-1a/5 AND A.4.5-1/64) THEN R ELSE N/A

C206 IF (A.4.1-1/1 AND A.4.3-4a/1a AND A.4.4-1a/5) THEN R ELSE N/A

C207 IF (A.4.1-1/2 AND A.4.3-4a/1a AND A.4.4-1a/5) THEN R ELSE N/A

C208 IF (A.4.1-1/1 AND A.4.3-4a/1a AND A.4.4-1a/25) THEN R ELSE N/A

C209 IF (A.4.1-1/2 AND A.4.3-4a/1a AND A.4.4-1a/25) THEN R ELSE N/A

C210 IF (A.4.1-1/1 AND A.4.3-4a/1a AND A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

C211 IF (A.4.1-1/2 AND A.4.3-4a/1a AND A.4.4-1a/5 AND A.4.4-1a/25) THEN R ELSE N/A

<end of changes>