3GPP TSG-RAN WG5 Meeting #80	R5-185099

# Gothenburg, Sweden, 20-24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0068 | rev | -1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | L2 Preamble Parameter Update for Multi-PDN configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update L2 Default Pre-Test Conditions’ Preamble parameters for Multi-PDN configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated L2 Preamble parameters for Multi-PDN configuration for MAC 7.1.1.0-2, RLC 7.1.2.1.1-2 and PDCP 7.1.3.0-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L2 Default Pre-Test Conditions will not have updated Preamble parameters for Multi-PDN configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 7.1.1.0-2, Table 7.1.2.1.1-2 and Table 7.1.3.0-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact R5-184977r11.) corretced multiple SCG DRB setup condition in table 7.1.2.1.2-1. 2) Added default RLC AM mode condition for MAC, RLC and PDCP preamble Fixed formatting for "Primary DRB used for Data" Testing column. R5-185099--> Final version of R5-184977r1 |  |  |  |  |  |  |  |  |

<Start of first changes>

### 7.1.1	MAC

#### 7.1.1.0	Default Pre-Test Conditions for all MAC test cases

The following pre-test conditions shall be applied in all MAC test cases until the test case explicitly over writes these conditions

System Simulator:

-	The SS configures the test environment in accordance to the execution conditions in Table 7.1.1.0-1.

UE:

-	None

Preamble:

-	The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.1.0-2 and using the message condition UE TEST LOOP MODE A to return one PDCP SDU per DL PDCP SDU.

Table 7.1.1.0-1: Test environment

| Execution Condition | Cell configuration | System Information Combination ([4] clause FFS) |
| --- | --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | NR Cell 1 | FFS |
| ELSE IF [pc_EN_DC] | E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF [pc_NGEN_DC] | NG-RAN E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |

Table 7.1.1.0-2: Preamble parameters

| Execution Condition | Multi-PDN Condition | Generic Procedure Parameters | Primary DRB used for Data testing |
| --- | --- | --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD]s | FALSE | Connectivity(NR), Test loop function(On) One DRB | DRB on NR Cell |
|  | TRUE | Connectivity(NR), Test loop function(On) Two DRB |  |
| ELSE IF [pc_EN_DC] | FALSE | Connectivity(EN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF [pc_NGEN_DC] | FALSE | Connectivity(NGEN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |

# Table 7.1.1.0-3: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | FFS |
| ELSE IF [pc_EN_DC] | message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF [pc_NGEN_DC] | message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |

<END of first changes>

<Start of second changes>

### 7.1.2	RLC

Editor’s note: Intended to capture tests of RLC Layer behaviour defined in TS 38.322

#### 7.1.2.1	Default Pre-Test Conditions for all RLC test cases

The following pre-test conditions shall be applied in all RLC test cases until the test case explicitly over writes these conditions.

#### 7.1.2.1.1	Default Pre-Test Conditions for AM RLC test cases

System Simulator:

-	The SS configures the test environment in accordance to the execution conditions in Table 7.1.2.1.1-1.

UE:

-	None

Preamble:

-	The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.2.1.1-2 and the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU.

# Table 7.1.2.1.1-1: Test environment

| Execution Condition | Cell configuration | System Information Combination ([4] clause FFS) |
| --- | --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | NR Cell 1 | FFS |
| ELSE IF [pc_EN_DC] | E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF [pc_NGEN_DC] | NG-RAN E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |

# Table 7.1.2.1.1-2: Preamble parameters

| Execution Condition | Multi-PDN Condition | Generic Procedure Parameters | Primary DRB used for Data testing |
| --- | --- | --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD]s | FALSE | Connectivity(NR), Test loop function(On) One DRB | DRB on NR Cell |
|  | TRUE | Connectivity(NR), Test loop function(On) Two DRB |  |
| ELSE IF [pc_EN_DC] | FALSE | Connectivity(EN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF [pc_NGEN_DC] | FALSE | Connectivity(NGEN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |

# Table 7.1.2.1.1-3: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | FFS |
| ELSE IF [pc_EN_DC] | message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF [pc_NGEN_DC] | message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |

##### 7.1.2.1.2	Default Pre-Test Conditions for UM RLC test cases

Same Pre-test conditions as in clause 7.1.2.1.1 with the exceptions in Table 7.1.2.1.2-1.

# Table 7.1.2.1.2-1: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | FFS |
| ELSE IF [pc_EN_DC] | message condition MCG_and_SCG with condition UM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF [pc_NGEN_DC] | message condition MCG_and_SCG with condition UM is used for step 7 in 4.5.4.2 according to [4] |

<END of second changes>

<Start of third changes>

### 7.1.3	PDCP

Editor’s note: Intended to capture tests of PDCP Layer behaviour defined in TS 38.323. E.g. testing of PDCP lossless operation for single SCG bearer is handled here.

#### 7.1.3.0	Default Pre-Test Conditions for all PDCP test cases

The following pre-test conditions shall be applied in all PDCP test cases until the test case explicitly over writes these conditions

System Simulator:

-	The SS configures the test environment in accordance to the execution conditions in Table 7.1.3.0-1.

# UE:

-	None

Preamble:

-	The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.3.0-2 and using the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU.

# Table 7.1.3.0-1: Test environment

| Execution Condition | Cell configuration | System Information Combination ([4] clause FFS) |
| --- | --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | NR Cell 1 | FFS |
| ELSE IF [pc_EN_DC] | E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF [pc_NGEN_DC] | NG-RAN E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |

# Table 7.1.3.0-2: Preamble parameters

| Execution Condition | Multi-PDN Condition | Generic Procedure Parameters | Primary DRB used for Data testing |
| --- | --- | --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD]s | FALSE | Connectivity(NR), Test loop function(On) One DRB | DRB on NR Cell |
|  | TRUE | Connectivity(NR), Test loop function(On) Two DRB |  |
| ELSE IF [pc_EN_DC] | FALSE | Connectivity(EN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF [pc_NGEN_DC] | FALSE | Connectivity(NGEN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |

# Table 7.1.3.0-3: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF [pc_nrFDD] or [pc_nrTDD] | FFS |
| ELSE IF [pc_EN_DC] | message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF [pc_NGEN_DC] | message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |

<END of third changes>