3GPP TSG RAN WG5 Meeting #80

Gothenburg, Sweden, 20th – 24th August 2018

Source:		KDDI, Rohde & Schwarz,

Title:	Discussion on test point selection for Frequency Error test case in FR1

# Introduction

At RAN5#2-5G-NR Adhoc and RAN5#80, test point analyses for frequency error for FR1 have been endorsed [1, 2, 3]. This document is a merge of the three aforementioned documents.

The discussion on the default baseline for test point selection was endorsed in [4] and [5] at RAN5#1-NR adhoc meeting. Based on the above endorsement, the discussions for some test cases were presented and related CRs were agreed in RAN5 #78 meeting.

# Discussion

Firstly, the definition of Frequency Error for FR1 in the core spec [6] is as follows.

### 6.4.1	Frequency error

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of [TBD] compared to the carrier frequency received from the NR Node B.

In 36.521-1, relevant LTE test points are defined as follows.

Table 6.5.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | QPSK | 6 | 6 | QPSK | 6 | 6 |
| 3MHz | QPSK | 15 | 15 | QPSK | 15 | 15 |
| 5MHz | QPSK | 25 | 25 | QPSK | 25 | 25 |
| 5MHz | QPSK | 25 | N/A | QPSK | 20 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 15 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 53 | N/A |
| 10MHz | QPSK | 50 | 50 | QPSK | 50 | 50 |
| 10MHz | QPSK | 50 | N/A | QPSK | 25 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 20 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 15 | N/A |
| 15MHz | QPSK | 75 | 75 | QPSK | 75 | 75 |
| 15MHz | QPSK | 75 | N/A | QPSK | 50 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 25 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 20 | N/A |
| 20MHz | QPSK | 100 | 100 | QPSK | 100 | 100 |
| 20MHz | QPSK | 100 | N/A | QPSK | 75 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 50 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 25 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 20 | N/A |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. Note 3: Applicable only to E-UTRA FDD Band 31. The UL resource blocks shall be located at RBstart 10 (according to Table 7.3.3-2). |  |  |  |  |  |  |

In the following subclauses, we introduce the study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation.

## Test Environment

Based on the baseline agreed in [4], the same test environment as LTE can be reused for FR1. In LTE test case for Frequency Error, NC, TL/VL, TL/VH, TH/VL and TH/VH are defined.

Proposal 1: Define Test Environment as NC, TL/VL, TL/VH, TH/VL and TH/VH for Frequency Error in FR1.

## Test Frequencies

The main factor of Carrier Frequency Error is the frequency difference of the oscillating circuit or PLL in the transmitter. The frequency conversion to make the RF frequency from the reference frequency has small impact to the Carrier Frequency Error.

From the above, testing only 1 test frequency seems to be good to balance the test accuracy and time. From the perspective of test frequency, the most critical situation cannot be defined because the performance of the oscillating circuit has small impact to the conversion of RF frequency. So, it is preferable to test for Mid frequency range.

Proposal 2: Select Mid range as test frequency for NR Frequency Error for FR1.

## Test Channel Bandwidth

From the perspective of carrier frequency error, the test channel bandwidth seems to be no impact to it. The channel bandwidth is for the modulated signal, so it does not impact to the RF carrier frequency.

The above analysis indicates that testing only 1 test channel bandwidth seems to be enough. Considering the most critical situation, the highest channel bandwidth might be impact to Carrier Frequency Error because it treats the widest frequency range when the lowest subcarrier spacing is selected and full RBs are allocated. So, testing by the highest channel bandwidth is preferable.

Proposal 3: Select the highest channel bandwidth to measure Frequency Error.

## Test Subcarrier Spacing

The situation of the test subcarrier spacing is the same of that of the test channel bandwidth. From the perspective of Carrier Frequency Error, the test subcarrier spacing also seems to be no impact to it and then testing only 1 test subcarrier spacing seems to be enough. Considering the most critical situation, the lowest subcarrier spacing might be impact to Carrier Frequency Error because it treats the widest frequency range when the highest channel bandwidth is selected and the full RBs are allocated. So, testing by the highest subcarrier spacing is preferable.

Proposal 4: Select the lowest Subcarrier Spacing to test.

## DL and UL configuration

The frequency error shall be tested with the DL at REFSENS level [6]. In order to avoid tighter test requirements as in the reference sensitivity test case the same UL and DL RB allocation and waveform shall be applied. TS 38.101-1 requires that REFSENS is tested with DFT-s-OFDM as UL waveform.

Proposal 5: Apply DFT-s-OFDM as UL waveform for Frequency Error in FR1.

Proposal 6: Follow the UL and DL RB allocation of REFSENS TC for Frequency Error in FR1.

For the downlink full allocation is proposed as the UE’s ability to synchronize its frequency to the downlink is indirectly tested in the REFSENS test case.

Proposal 7: Apply full allocation for the downlink configuration.

## Number of test points for Frequency Error

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 1 | 1 | 1 | 1 | 5 |

# Conclusion

To sum up, the following updates for the selection of test points is proposed for frequency error in FR1:

Proposal 1: Define Test Environment as NC, TL/VL, TL/VH, TH/VL and TH/VH for Frequency Error in FR1.

Proposal 2: Select Mid range as test frequency for NR Frequency Error for FR1.

Proposal 3: Select the highest channel bandwidth to measure Frequency Error.

Proposal 4: Select the lowest Subcarrier Spacing to test.

Proposal 5: Apply DFT-s-OFDM as UL waveform for Frequency Error in FR1.

Proposal 6: Follow the UL and DL RB allocation of REFSENS TC for Frequency Error in FR1.

Proposal 7: Apply full allocation for the downlink configuration.

# References

- R5-181781, Discussion on test point selection for NR Frequency Error in FR1, KDDI, 3GPP TSG RAN5, RAN5 5G-NR Adhoc #2, Taoyuan, Taiwan, April 2018

- R5-182025, Discussion on test point selection for NR Frequency Error in FR1, Rohde & Schwarz, 3GPP TSG RAN5, RAN5 5G-NR Adhoc #2, Taoyuan, Taiwan, April 2018

- R5-184952, Discussion on test point selection for Frequency Error test case in FR1, Rohde & Schwarz, 3GPP TSG RAN5, RAN5 #80, Gothenburg, Sweden, August 2018

- R5-180065 , “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

- R5-180111, “Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases”, Ericsson, RAN 5 Ad hoc, Munich, Jan. 2018.

- TS 38.101-1 v15.0.0, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.