3GPP TSG-RAN5 Meeting #81	R5-186493

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 0064 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of TC 7.6.4 Narrow-band blocking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | TC 7.6.4 Narrow-band blocking for NR FR1 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of TC 7.6.4 Narrow-band blocking for NR FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR FR1 test case will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.4(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Test points selection for frequency, BW, SCS and UL configuration based on proposals described in R5-186749 |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.6.4	Narrow band blocking

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- UL power level configuration is TBD in RAN 4 38.101-1.

- Stand alone message contents in TS 38.508-1[5] subclause 4.6 is TBD

#### 7.6.4.1	Test Purpose

Verifies a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other NR Node B transmitters exist (except in the adjacent channels and spurious response).

#### 7.6.4.2		Test Applicability

This test applies to all types of NR UE release 15 and forward.

#### 7.6.4.3	Minimum Conformance Requirements

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.4.3-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.4.3-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100MHz |
| n1,n2, n3, n5, n7, n8, n12, n20, n25 n28, n34, n38, n39, n40, n41, n50, n51, n66, n70, n71, n74, n75, n76 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 |  | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 |  | 5.2125 | 7.7025 | 10.2075 | 13.0275 | 20.5575 | NA | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA |  | NA | NA | NA |  |  | TBD | TBD |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.6.4.

#### 7.6.4.4	Test Description

##### 7.6.4.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 7.6.4.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6.4.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid and Highest |  |  |
| Test SCS as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3.2.4.1-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.4.2 for TE diagram and section A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 , and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.6.4.4.3.

##### 7.6.4.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to Table 7.6.4.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 7.6.4.4.1-1. Since the UE has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6.4.5-1.

4.	Set the downlink signal level according to the table 7.6.4.5-1. Send uplink power control commands to the UE (less or equal to 1 dB step size should be used), to ensure that the UE output power is within 4dB below PCMAX_L,f,c of the target level in table 7.6.4.5-1, for at least the duration of the throughput measurement.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal at step 3.

##### 7.6.4.4.3	Message Contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

#### 7.6.4.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table 7.6.4.5-1.

Table 7.6.4.5-1: Narrow-band blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100MHz |
| n1,n2, n3, n5, n7, n8, n12, n20, n25 n28, n34, n38, n39, n40, n41, n50, n51, n66, n70, n71, n74, n75, n76 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 |  | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 |  | 5.2125 | 7.7025 | 10.2075 | 13.0275 | 20.5575 | NA | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA |  | NA | NA | NA |  |  | TBD | TBD |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>