3GPP TSG-RAN5 Meeting #81	R5-186571

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0038 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Clause 7.5B.3 in TS 38.521-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

7.5B.3	Adjacent Channel Selectivity for inter-band EN-DC within FR1

Editor’s note: this clause is incomplete. The following aspects are either missing or not yet determined:

-The test point selection analysis is incomplete:

-Test configuration needs further investigation

-Test tolerance analysis is incomplete

-Connection diagram is TBD: the interferer requirement is not defined in RAN4 -Bandwidth combination configuration for inter band EN-DC within FR1 is TBD in RAN4

7.5B.3.1	Test purpose

Same test purpose as in clause 7.5B.1.1.

7.5B.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1.

7.5B.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.5.3 in TS 38.521-1 [8] for the NR carrier.

The minimum conformance requirements for NR CC is specified in TS 38.101-1[2] clause 7.5

7.5B.3.4	Test description

7.5B.3.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.2B.4, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each inter-band EN-DC within FR1 configuration specified in clause [5.3B.X], and are shown in table 7.5B.3.4.1-1.

The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

# Table 7.5B.3.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  | TBD |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  | TBD |  |  |
| Test CC Combinations setting (NRB_agg) as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  | TBD |  |  |
| Test SCS for the NR cell as specified in 38.521-1 [8] Table 5.3.5-1 |  |  | TBD |  |  |
| Test Parameters for E-UTRA Configuration |  |  |  |  |  |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  | Uplink |  |
|  |  | N/A for Occupied bandwidth |  | Modulation | RB allocation |
| 5 MHz | MidRange |  |  | QPSK | 25 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] subclause 4.3.1 |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [TBD].

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.5B.3.4.3.

7.5B.3.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC on the NR CC according to Table 7.5B.3.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC on the NR CC according to Table 7.5B.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level on the NR CC to the value as defined in Table 7.5-3 in TS 38.101-1[2] (Case 1), Send Uplink power control commands to the UE (less or equal to TBD dB step size should be used) to ensure that the UE output power is within [TBD] dB of the target level in Table 7.5-3 in TS 38.101-1[2] (Case 1) for at least the duration of the Throughput measurement.

4.	Set the Interferer signal level to the value as defined in Table 7.5-3 in TS 38.101-1[2] (Case 1) and frequency below the wanted signal on the NR CC, using a modulated interferer bandwidth as defined in Annex [TBD].

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal in Case 1 at step 4.

7.	Set the Downlink signal level on the NR CC to the value as defined in Table 7.5-4 in TS 38.101-1[2] (Case 2) Send Uplink power control commands to the UE (less or equal to TBD dB step size should be used), to ensure that the UE output power is within [TBD] dB of the target level in Table 7.5-4 in TS 38.101-1[2] (Case 2) for at least the duration of the Throughput measurement.

8.	Set the Interferer signal level to the value as defined in Table 7.5-4 in TS 38.101-1[2] (Case 2) and frequency below the wanted signal on the NR CC, using a modulated interferer bandwidth as defined in Annex [TBD].

9.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H

10.	Repeat steps from 7 to 9, using an interfering signal above the wanted signal in Case 2 at step 8.

11.	Repeat for applicable channel bandwidths and operating band combinations in both Case 1 and Case 2.

7.5B.3.4.3	Message contents

# Same message contents with clause 7.5B.1.4.3

7.5B.3.5	Test requirement

For the NR CC, the throughput measurement of the carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 under the conditions specified in clause 7.5.5 in TS 38.521-1 [8]

<End of modified section>