3GPP TSG RAN WG5 Meeting #81	R5-186699

Spokane, US, 12th – 16th Nov 2018

Source:		Rohde & Schwarz

Title:	On RRM test case numbering

Agenda Item:		5.3.9.17

Document for:	Endorsement

# Introduction

LTE RRM test cases are normally executed with a defined cell bandwidth and duplex mode. NR is much more flexible than LTE, and there are other parameters like the Subcarrier Spacing and similar which can be varied for every test.

# Discussion

Opposite to LTE legacy RRM tests, where 1 Test Case  1 Test Configuration, currently defined RRM NR test cases follow the approach 1 Test Case  N Test Configurations

Observation 1: NR RRM test cases can be run under several different test configurations

Taking as example test case 4.7.1.1 SS-RSRP accuracy, the test can be run with the following configurations:

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

There are no longer separate tests for the duplex mode or the bandwidth, and also the SCS and the duplex mode of the LTE link are configurable.

From specification point of view, this way forward of keeping one test with many configurations is an advantage, as it allows us to keep the number of test low, improving readability and maintainability of the specification. However, from certification point of view, it would be beneficial to have the association 1 Test Case  1 Test Configuration, as in LTE, to keep the certification process as transparent as possible.

Observation 2: 1 Test Case  N Configuration is an advantage to keep the test specification short. However, for certification, 1 Test Case  1 Test Configuration would be preferable for transparency.

Hence, a possible compromise would be to differ between test case number (the test case as it will be defined in TS 38.533) and the test case ID, a unique identifier formed by test case number + test configuration index. The test case ID can be used for certification bodies, given a unique association between test case ID and the test configuration used, and providing the same transparency in LTE. The test case number is used in 3GPP, to keep the specification as short as possible.

Proposal 1: Introduce the concept of test case ID, a unique identifier formed by the test case number and the test configuration index.

In TS 38.533, the test case would look like this:

4.7.1.1	SS-RSRP accuracy

4.7.1.1.1 Test Purpose

FFS

4.7.1.1.2 Test applicability

FFS.

Test 4.7.1.1 can be run in the configurations defined in Table 4.7.1.1.2-1.

Table 4.7.1.1.2-1: Test Configurations for test case 4.7.1.1

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 4.7.1.1-1 | 1 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.1.1-2 | 2 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.1.1-3 | 3 | LTE FDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.7.1.1-4 | 4 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.1.1-5 | 5 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.1.1-6 | 6 | LTE TDD, NR: 30kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Proposal 2: Add a table to the Test Applicability section in TS 38.533 indicating the test case ID, config index and description for every RRM NR test.

In the Test Applicability specification, TS 38.522, the Test Case ID shall be used instead of the Test Case number.

| Test Case ID | Title | Release | Applicability |  | Additional Information |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comments | Number of TC Executions | Release on other RAT | Branch |
| E-UTRAN RRC_IDLE State Mobility |  |  |  |  |  |  |  |
| 4.7.1.1-1 | SS-RSRP accuracy | Rel-15 | Cxx1 | UE supporting NR FDD and LTE FDD | The UE is required to be tested only on one of the configurations |  |  |
| 4.7.1.1-2 |  |  | Cxx2 | UE supporting NR TDD and LTE FDD |  |  |  |
| 4.7.1.1-3 |  |  | Cxx3 | UE supporting NR TDD and 40 MHz BW and LTE FDD |  |  |  |
| 4.7.1.1-4 |  |  | Cxx4 | UE supporting NR FDD and LTE TDD |  |  |  |
| 4.7.1.1-5 |  |  | Cxx5 | UE supporting NR TDD and LTE TDD |  |  |  |
| 4.7.1.1-6 |  |  | Cxx6 | UE supporting NR TDD and 40 MHz BW and LTE TDD |  |  |  |

Proposal 3: Refer to the Test Case ID instead of the test case number in TS 38.522.

# Conclusion

This paper points the difference between legacy LTE and NR RRM test cases and proposes a way forward to minimize the number of test case entries in TS 38.533, improving the readability and maintainability of the test specification, and at the same time provide unique test identifiers for the certification bodies to allow a transparent certification process.

Observation 1: NR RRM test cases can be run under several different test configurations

Observation 2: 1 Test Case  N Configuration is an advantage to keep the test specification short. However, for certification, 1 Test Case  1 Test Configuration would be preferable for transparency.

Proposal 1: Introduce the concept of test case ID, a unique identifier formed by the test case number and the test configuration index.

Proposal 2: Add a table to the Test Applicability section in TS 38.533 indicating the test case ID, config index and description for every RRM NR test.

Proposal 3: Refer to the Test Case ID instead of the test case number in TS 38.522.

# References