3GPP TSG-RAN5 Meeting #81	R5-187199

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4697 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of LWA Test Case 8.2.5.4 and LWIP 8.2.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation (UK) Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_WLAN_radio-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Many updates to test case - Message contents in test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 8.2.5.4: Updated power level in Table 8.2.5.4.3.2-1. Corrected message contents in Steps 2, 5 and 6 TC 8.2.5.5: Updated power level in Table 8.2.5.5.3.2-1. Corrected message contents in Steps 2, 5 and 6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.4, 8.2.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |

**** Beginning of Revisions *****

#### 8.2.5.3

#### 8.2.5.4	LWA / WLAN Association Success / EUTRA RRC_Connected to WLAN (Event W1)

8.2.5.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W1 reporting }

ensure that {

when { UE  detects entering condition for Event W1 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {

when { UE  receives RRCConnectionReconfiguration including wlanToAddList }

then { UE adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after successfully associating with any WLAN identifier in the new WLAN mobility set, UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation }

}

8.2.5.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.11, 5.6.14.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.11]

The UE shall:

1>	consider the entering condition for this event to be satisfied when wlan-MobilitySet within VarWLAN-MobilityConfig does not contain any entries and condition W1-1, as specified below, is fulfilled;

…

Inequality W1-1 (Entering condition)

…

The variables in the formula are defined as follows:

Mn is the measurement result of WLAN(s) configured in the measurement object, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w1-Threshold as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Mn.

[TS 36.331, clause 5.6.14.2]

Upon reception of LWA configuration, the UE shall:

…

1>	else:

…

2>	if the received lwa-Config includes lwa-MobilityConfig:

…

3>	if the received lwa-MobilityConfig includes wlanToAddList:

4>	for each WLAN-Identifiers included in wlanToAddList:

5>	add WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwa-MobilityConfig includes associationTimer:

4>	start or restart timer T351 with the timer value set to the associationTimer;

3>	if the received lwa-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3>	start WLAN Status Monitoring as described in 5.6.15.4;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change (if success report is requested by the eNB) or its connection or connection attempt to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

8.2.5.4.3	Test description

8.2.5.4.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.4.3.2	Test procedure sequence

Table 8.2.5.4.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T0" and "T1" are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.4.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -60 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values for entering Event W1 |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -50 |  |

Table 8.2.5.4.3.2-2: Main behaviour

| Step | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row "T0" in table 8.2.5.4.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the row "T1" in table 8.2.5.4.3.2-1. | - | - | - | - |
| 5 | Check: UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | 1 | P |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry? | --> | WLANConnectionStatusReport | 3 | P |

8.2.5.4.3.3	Specific message contents

Table 8.2.5.4.3.3-1: RRCConnectionReconfiguration (Step 2 Table 8.2.5.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.4.3.3-2. |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-2: MeasConfig (Step 2 Table 8.2.5.4.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.4.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.4.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-3: MeasObjectWLAN-GENERIC (Step 2 Table 8.2.5.4.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-4: ReportConfigInterRAT-W1-WLAN (Step 5 Table 8.2.5.4.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.4.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-5: RRCConnectionReconfiguration (Step 6 Table 8.2.5.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 |  |
| Lwa-configuration-r13 | LWA-Configuration-GENERIC(SSID) Set as per Table 8.2.5.4.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.4.3.3-6: LWA-Configuration-GENERIC (Step 6 Table 8.2.5.4.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-34 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  |  |
| successReportRequested-r13 | true |  |  |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.5	LWIP / WLAN Association Success / EUTRA RRC_Connected to WLAN (Event W1)

8.2.5.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and configured for Event W1 reporting }

ensure that {
  when { UE  detects entering condition for Event W1 is met }

then { UE transmits a measurement report }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and configured with a WLAN mobility set }

ensure that {
  when { UE  receives RRCConnectionReconfiguration including wlanToAddList }

then { UE adds the new WLAN identifiers to the WLAN mobility set and sends RRCConnectionReconfigurationComplete }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { UE receives RRCConnectionReconfiguration that changes the WLAN mobility set and if successReportRequested-r13 is included in IE WLAN-MobilityConfig-r13 }

then { after successfully associating with any WLAN identifier in the new WLAN mobility set, UE transmits WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation }

}

8.2.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.4.11, 5.6.17.2, 5.6.15.2.2 and 5.6.15.2.3.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

…

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends

[TS 36.331, clause 5.5.4.11]

The UE shall:

1>	consider the entering condition for this event to be satisfied when wlan-MobilitySet within VarWLAN-MobilityConfig does not contain any entries and condition W1-1, as specified below, is fulfilled;

…

Inequality W1-1 (Entering condition)

…

The variables in the formula are defined as follows:

Mn is the measurement result of WLAN(s) configured in the measurement object, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w1-Threshold as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Mn.

[TS 36.331, clause 5.6.17.2]

The UE shall:

…

1>	else:

2>	if lwip-MobilityConfig is included:

…

3>	if the received lwip-MobilityConfig includes wlan-ToAddList:

4>	for each WLAN-Identifiers included in wlan-ToAddList:

5>	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3>	if the received lwip-MobilityConfig includes associationTimer:

4>	start timer T351 with the timer value set according to the value of associationTimer;

3>	if the received lwip-MobilityConfig includes successReportRequested:

4>	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

2>	if tunnelConfigLWIP is included:

3>	indicate to higher layers to configure the LWIP tunnel according to the received tunnelConfigLWIP [32];

3>	if lwip-Counter is included:

4>	determine the LWIP-PSK based on the KeNB key and received lwip-Counter value, as specified in TS 33.401 [32];

4>	forward the LWIP-PSK to upper layers for LWIP tunnel establishment;

2>	start WLAN Status Monitoring as described in 5.6.15.4;

[TS 36.331, clause 5.6.15.2.2]

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change (if success report is requested by the eNB) or its connection or connection attempt to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

[TS 36.331, clause 5.6.15.2.3]

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

8.2.5.5.3	Test description

8.2.5.5.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 and WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.5.5.3.2	Test procedure sequence

Table 8.2.5.5.3.2-1 illustrates the downlink power levels to be applied for Cell 1 and Cell 27 at various time instants of the test execution. Rows marked "T0" and "T1" are to be applied in the test procedure. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.5.5.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 27 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | WLAN Cell 27 is switched on |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -60 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values for entering Event W1 |
|  | RSRQ | dB | -4.15 | - |  |
|  | WLANRSSI |  | - | -50 |  |

Table 8.2.5.5.3.2-2: Main behaviour

| Step | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row “T0” in table 8.2.5.5.3.2-1 | - | - | - | - |
| 2 | The SS transmits an RRCConnectionReconfiguration message to configure WLAN measurement (Event W1) | <-- | RRCConnectionReconfiguration | - | - |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | - | - |
| 4 | The SS changes Cell parameters according to the r“w ”T1" in table 8.2.5.5.3.2-1. | - | - | - | - |
| 5 | Check: UE transmits a MeasurementReport message to report the Event W1 | --> | MeasurementReport | 1 | P |
| 6 | The SS transmits an RRCConnectionReconfiguration message including Cell 27 in IE wlanToAddList | <-- | RRCConnectionReconfiguration | - | - |
| 7 | Check: The UE transmits an RRCConnectionReconfigurationComplete | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 8 | Check: Does the UE transmit WLANConnectionStatusReport with WLAN-Status-r13 set to successfulAssociation before T351 expiry | --> | WLANConnectionStatusReport | 3 | P |

8.2.5.5.3.3	Specific message contents

Table 8.2.5.5.3.3-1: RRCConnectionReconfiguration (Step 2 Table 8.2.5.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { | 1 entry |  |  |
| measConfig | Set as per Table 8.2.5.5.3.3-2 |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-2: MeasConfig (Step 2 Table 8.2.5.5.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] | MeasObjectWLAN- GENERIC(Freq, SSID) | See Table 8.2.5.5.3.3-3 |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| reportConfig[1] | ReportConfigInterRAT-W1-WLAN(-55) | Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.5.3.2-1. |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | 1 |  |  |
| reportConfigId[1] | 1 | IdReportConfig-W1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-3: MeasObjectWLAN-GENERIC (Step 2 Table 8.2.5.5.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-3AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Same as used for Cell 27 | Band and Carrier information |  |
| wlan-ToAddModList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-4: ReportConfigInterRAT-W1-WLAN (Step 2 Table 8.2.5.5.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-9A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT-W1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | 46 | ThreshDB+101=46 where ThreshDB = -55dBm. Note: Set as per T1 setting for WLAN RSSI minus 5db in Table 8.2.5.5.3.2-1. |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-5: RRCConnectionReconfiguration (Step 6 Table 8.2.5.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition LWIP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not Present |  |  |
| mobilityControlInfo | Not Present |  |  |
| dedicatedInfoNASList | Not Present |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| securityConfigHO | Not Present |  |  |
| nonCriticalExtension{ |  | v890 |  |
| nonCriticalExtension{ |  | v920 |  |
| nonCriticalExtension{ |  | v1020 |  |
| nonCriticalExtension{ |  | v1130 |  |
| nonCriticalExtension{ |  | v1250 |  |
| nonCriticalExtension{ |  | v1310 | LWIP |
| Lwip-configuration-r13 | LWIP-Configuration-GENERIC(SSID) Set as per Table 8.2.5.5.3.3-6 | SSID set as per Table 4.4.8-1 of [18] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.5.5.3.3-6: LWIP-Configuration-GENERIC (Step 6 Table 8.2.5.5.3.2-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-33 with condition ADD, SUCCESS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 | Not Present |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  | ADD |
| ssid-r12[1] | Set as per Table 4.4.8-1 of [18] | Cell27 |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 |  | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.2.5.6	LWIP / WLAN Release / WLAN Association / EUTRA RRC_Connected to WLAN (Event W2)

**** End of Revisions *****