3GPP TSG-RAN5 Meeting #81	R5-187263

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.533 | CR | N/A | rev | N/A | Current version: | 0.5.0 |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for Addition of Test Case 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM CDMA TECHNOLOGIES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | N/A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Core spec TS 38.133 has added requirements for EN-DC FR1 Timing Advance Ajustment Accuracy |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds new test case content for 5G NR RRM TC for EN-DC FR1 Timing Advance Adjustment Accuracy |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 7.3.2.2 will be missing from test spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The Title has wrong Test Case number as 4.4.3.1, but actual test case is 7.3.2.2 |  |  |  |  |  |  |  |  |

### 7.3.2.2	Timing Advance adjustment accuracy

#### 7.3.2.2.1 	EN-DC FR1 timing advance adjustment accuracy

##### 7.3.2.2.1.1 	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3 of TS 38.133 [TBD].

### 7.3.2.2.1.2 Test Applicability

This test case is applicable to all types of E-UTRA UE Release-15 and forward, supporting EN-DC FR1

### 7.3.2.2.1.3 Minimum Conformance Requirement

The timing advance is initiated from gNB with MAC message that implies and adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [TBD].

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 7.3.2.2.1.3-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS38.213 [3].

Table 7.3.2.2.1.3-1: UE Timing Advance adjustment accuracy

| Sub Carrier Spacing, SCS kHz | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

#### 7.3.2.2.2	Test Description

##### 7.3.2.2.2.1 	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table [5.3.5-1] and Table [5.3.6-1] of 38.521-1.

Test Environment: Normal, as defined in TS 38.508 [TBD] clause TBD.

Frequencies to be tested: Mid Range, as defined in TS 38.508 [TBD] clause TBD.

For EN-DC within FR1 operation, setup the LTE link according to Annex TBD

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [TBD] Annex A, in Figure TBD for TE diagram and section TBD for UE diagram.

2.	The parameter settings for the cell are set up according to Table [TBD] and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex [C.0], [C.1], [C.2], [C.3.1] and uplink signals according to Annex [G.0], [G.1], [G.2], [G.3.1].

4.	Propagation conditions are set according to Annex [TBD].

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA or (EN-DC, DC bearer MCG and SCG) for NSA according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 7.3.2.2.2.3.

.

##### 7.3.2.2.2.2 Test Procedure

The test consists of a single cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and SRS are sent from the UE and received by the SS. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured. The UE Time Alignment Timer (timeAlignmentTimer IE), defined in TS 38.321 [TBD] clause TBD, shall be configured so that it does not expire in the duration of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA or (EN-DC, DC bearer MCG and SCG) for NSA according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 7.3.2.2.2.3.

2.	Set the parameters according to [TBD] in Tables [TBD] and [TBD]. Propagation conditions are set according to Annex [TBD].

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	SS shall transmit one message with a timing advance command, TA. The timing advance command, TA, shall be set to [31] which indicate adjustment of the current NTA value. The timing advance adjustment during T1 shall be NTA = [0].

6.	The UE shall transmit SRS and adjust its uplink timing at the beginning of sub-frame [n+6] for a timing advance command TA received in sub-frame [n]. Sub-frame [n] is the sub-frame where the UE received the time advance command.

7.	When T1 expires, the SS shall switch the timing advance command (TA) from T1 to T2 as specified in Table [TBD].

8.	SS shall transmit a sequence of messages with timing advance command, TA. The timing advance command, TA, shall be set to [39] which indicate adjustment of the current NTA value. The timing advance adjustment during T2 shall be NTA = [128].

9.	The UE shall transmit SRS and adjust its uplink timing at the beginning of sub-frame [n+6] for a timing advance command TA received in sub-frame [n]. Sub-frame [n] is the sub-frame where the UE received the time advance command.

10.	The result from the SRS and adjustment of the timing advance in step 9) is used to measure that the UE adjusts the timing of its transmission with a relative accuracy better than or equal to [± 4.5 × TS] to the signalled timing advance value compared to the timing of preceding uplink transmission.

11.	If the UE adjust the timing of its transmission within a relative accuracy greater than or equal to [± 4.5 × TS] to the signalled timing advance value compared to the timing of preceding uplink transmission then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

12.	The SS shall transmit RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	After the RRC connection release, the SS:

- [transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [7] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3)],

or

- [switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3].

14.	Repeat step 2-13 until the confidence level according to Tables [G.2.3-1] in Annex [G] clause [G.2] is achieved.

##### 7.3.2.2.2.3 Message Contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

Table xx: message exceptions for SA

FFS

Table yy: message exceptions for NSA

FFS

##### 7.3.2.2.2.3 Test Requirement

Table 7.3.2.2.1.3-1 defines the primary level settings.