3GPP TSG RAN WG5#81 	R5-187443

# Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0082 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TC 7.4B.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_QMC_Streaming-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 clarifies EN-DC FR1 tests applying the E-UTRA anchor-agnostic approach, it applicable to the inter-band EN-DC Maximumu input level test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Test configurations for NR carrier and E-UTRA carrier are specified in test description 2 Connection diagram, test procedure and test requirements are specified. 3 Add editor’s notes that the test for inter-band EN-DC configuration with more than one NR carrier is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.4B.3	Maximum Input Level for Inter-band EN-DC within FR1

Editor’s note: The following aspects are either missing or not yet determined:

-	This test is object to inter-band EN-DC configuration with one NR carrier, for more than one NR carrier, it needs FFS.

7.4B.3.1	Test purpose

Maximum input level for inter-band EN-DC tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB or a gNB.

7.4B.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC.

7.4B.3.3	Minimum conformance requirements

For inter-band EN-DC maximum input level is defined as the powers received at the UE antenna port over the Transmission bandwidth configuration of each CC in each CG, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel over each CC in each CG.

The minimum conformance requirements for NR CG is specified in TS 38.101-1 clause 7.4, and reference measurement channels are the same with the configurations in TS 36.101 and TS 38.101-1.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.4B.3

7.4B.3.4	Test Description

7.4B.3.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.2B.4, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each inter-band EN-DC configuration specified in clause 5.5B.4, and The configurations for NR carrier are shown in TS 38.521-1 [8] table 7.4.4.1-1, the configurations for E-UTRA carrier as anchor-agnostic are shown in table 7.4B.3.4.1-1.The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.4B.3.4.1-1: Test configuration table for E-UTRA carrier

| E-UTRA Test Parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  |  | Uplink |  |
|  |  | Modulation |  | RB allocation | Modulation | RB allocation |
| 5 MHz2 | MidRange1 | N/A |  | 0 | N/A | 0 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] subclause 4.3.1 |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2.1 for UE diagram..

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1,C.2,C3.1 and TS 38.521-1 [8] Annex C.0,C.1,C.2,C3.1for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H.0,H.1,H.2,H.3.1 and TS 38.521-1 [8] Annex G.0,G.1,G.2,G.3.1 for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.4B.3.4.3.

7.4B.3.4.2	Test Procedure

Same test procedure as specified in clause 7.4.4.2

7.4B.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1.

7.4B.3.5	Test Requirement

Same test requirement as specified in TS 38.521-1 [8] table 7.4.5