# 3GPP TSG-RAN5 Meeting #81			 				       R5-187891

# Spokane, USA, 12th - 16th Nov 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0011 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 UE RSRPB Function 38.509 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Last Table in Section 6.5.2 with X1 to X24 detail was sticking out of Page Border |  |  |  |  |  |  |  |  |
| Summary of change: |  | Formatted this Last Table in Section 6.5.2 within Page Border and made the undefined bit 7 and 8 of Octet 1 as “Reserved”, Spec Ref to 38.215 was [21], changed it to [XX], added skip indicator reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table will not look good in Spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 and 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Editorial CR, the Doc was earlier Endorsed in Jaipur Adhoc with Final Tdoc of R5-186176 Corrected few items on Cover Page (Editorial Changes) , and resubmitting with above new Tdoc r1 has first row of table font changed to TAH r2 has CR number changed from 0010 to 0011 on Title Page 187803r2 Final Tdoc R5-187891 |  |  |  |  |  |  |  |  |

<< Start of changes >>

# 2	References

<< Unchanged sections skipped >>

[XX]	3GPP TS 38.215: "NR; Physical Layer Measurements".

<< Unchanged sections skipped >>

## 5.5	UE SS-RSRPB per receiver branch reporting

### 5.5.1	General

In 38.215 [XX], section 5.1.18 defines SS-RSRPB as below, with its applicability only to FR2 and in RRC_CONNECTED Mode.

| Definition | SS reference signal received power per branch (SS-RSRPB) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals (SS). The measurement time resource(s) for SS-RSRPB are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. For SS-RSRPB determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRPB using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in 3GPP TS 38.213 [5]. SS-RSRPB shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If higher-layers indicate certain SS/PBCH blocks for performing SS-RSRPB measurements, then SS-RSRPB is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, SS-RSRPB is not defined. For frequency range 2, SS-RSRPB shall be measured for each receiver branch based on the combined signal from antenna elements corresponding to the receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency |

The SS uses the SS-RSRPB reporting procedure to command the UE to report SS-RSRP per UE receiver branch. The report from the UE shall be a vector of values, where the number of the reported values equals the number of receiver branches on the UE.

Figure 5.5.1-1: UE SS-RSRPB reporting procedure

### 5.5.2	Initiation

The SS requests the UE to start reporting SS-RSRP per receiver branch by transmitting a SS-RSRPB REPORT REQUEST message. And the UE responds back reporting SS-RSRP per receiver branch via SS-RSRPB REPORT RESPONSE.

### 5.5.3	Reception of SS-RSRPB REPORT REQUEST message by UE

When the UE receives SS-RSRPB REPORT REQUEST message then the UE shall:

- if the UE is operating in FR2 AND in RRC_CONNECTED state:

2>  if the MeasObjectId signalled by the IE SS-RSRPB Measurement Config is configured to the UE by RRC Signalling and PeriodicalReportConfig is defined for this MeasObject

3>  Reply with the SS-RSRPB REPORT RESPONSE using the Measurement configuration as in the Measurement Object identified in the SS-RSRPB Measurement Config with the same reportInterval, reportAmount,  Filter Coefficient

4> Report the SS-RSRPB measurement for the serving PCI and best SSB-ID configured in the MeasObject

5> FFS to have multiple cells and/or SSB-ID

2> else:

3> use default filtercoefficient of fc4, default reportInterval of 640ms, and default reportAmount of r16

1> else:

2>	the UE behaviour is unspecified.

## The SS-RSRPB Report Request & SS-RSRPB Report Response Message Octets are defined below in Section 6.5

<< Unchanged sections skipped >>

## 6.5	UE SS-RSRP per receiver branch reporting messages

## 6.5.1	SS-RSRPB REPORT REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| SS-RSRPB Measurement Config |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | octet 1 |

where SS-RSRPB Measurement Config is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X8 | X7 | X6 | X5 | X4 | X3 | X2 | X1 | octet 1 |

This maps to the MeasObjectId as configured by the RRC Reconfig Message

### 6.5.2	SS-RSRPB REPORT RESPONSE

This message is only sent in the direction UE to SS representing the measured SS-RSRPB. To report SS-RSRPB (0 to 127).  per measured SSB, UE will send 3 Octets, the First Octet reporting the Measured SSB ID (values 0 to 63 consuming 6 bits X1 to X6 of Octet 1), Second Octet reporting SS-RSRPB for Branch#0 and third Octet reporting SS-RSRPB for Branch#1

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| SS-RSRPB report response param |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | octet 1 |

where SS-RSRPB report response param is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  | X6 | X5 | X4 | X3 | X2 | X1 | octet 1 |
| 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | bit no. |
| X16 | X15 | X14 | X13 | X12 | X11 | X10 | X9 | octet 2 |
| 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | bit no. |
| X24 | X23 | X22 | X21 | X20 | X19 | X18 | X17 | octet 3 |

where X1 to X24 spanning over three octets will have the following definition:

| Definition |  | 8 |  | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 |  | bit no. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB ID |  |  | Reserved |  |  | X6 | X5 | X4 | X3 | X2 |  | X1 |  | octet 1 |
| RSRPB Branch #0 | X16 |  |  | X15 | X14 |  | X13 | X12 | X11 | X10 | X9 |  | octet 2 |  |
| RSRPB Branch #1 |  | X24 |  | X23 | X22 |  | X21 | X20 | X19 | X18 | X17 |  | octet 3 |  |

<< End of changes >>