3GPP TSG-RAN5 Meeting #81 	R5-187950

# Spokane, USA, 12 -16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4511 | rev | 1 | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.5.2A.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from: R5-186440r1. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.5.2A.2.2	Carrier leakage for CA (inter-band DL CA and UL CA)

6.5.2A.2.2.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE Release 11 and forward that support inter-band DL CA and UL CA.

6.5.2A.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0.

6.5.2A.2.2.4	Test description

6.5.2A.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | NA | QPSK | 5 | P_5@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 3 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 4 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 6 | 15 | 50 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 7 | 15 | 50 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 8 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 10 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 12 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 13 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 14 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 15 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 16 | 50 | 15 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 17 | 50 | 15 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 18 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 20 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 22 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 23 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 26 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 28 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 30 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 31 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| X+1 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| X+2 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMCs are set according to in Table 6.5.2A.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.2.2.4.3.

6.5.2A.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.2.2.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2A.2.2.5-1.

Table 6.5.2A.2.2.5-1: Test requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

< End of changes >