3GPP TSG-RAN5 Meeting #81 	R5-187951

# Spokane, USA, 12 -16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4512 | rev | 1 | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.5.2A.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from: R5-186441r1. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.5.2A.3.2	In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA)

6.5.2A.3.2.1	Test Purpose

Same test purpose as in clause 6.5.2A.3.1.1.

6.5.2A.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

This test case also applies to all types of E-UTRA UE release 14 and forward that supports uplink LAA.

6.5.2A.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.2.4	Test description

6.5.2A.3.2.4.1	Initial conditions

Same as in clause 6.5.2A.3.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.5.2A.3.1.4.1-1  use Table 6.5.2A.3.2.4.1-1.

-	Instead of clause 6.5.2A.3.1.4.3  use clause 6.5.2A.3.2.4.3.

Table 6.5.2A.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. Note 3: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |

Table 6.5.2A.3.2.4.1-2: Test Configuration Table for Uplink LAA CA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 1) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC RB Allocation (LCRB @ RBstart) SCC RB Allocation(L@ RBstart) Note2 |  |  |  |
| 1 | 100 | N/A for this test | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 2 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 3 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 4 | 100 |  | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 5 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 6 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 7 | 100 |  | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 8 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 9 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 10 | 100 |  | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 11 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 12 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 13 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 14 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 15 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| 16 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 17 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 18 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| 19 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 20 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 21 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| 22 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 23 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 24 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| Note 1: CA Configuration, Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths for uplink LAA are specified in Table 5.4.2A.1-2. Note 2: For eLAA SCC, the RBstart and L are defined in TS 36.213[10] clause 8.1.4. |  |  |  |  |  |  |  |  |

6.5.2A.3.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 (or DCI format 0A for SCC in the case of uplink LAA) for C_RNTI to schedule the UL RMC according to Table 6.5.2A.3.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.3.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508 [7] subclause4.6 with the exceptions specified in Tables 6.5.2A.1.2.4.3-1 and 6.5.2A.1.2.4.3-2.

6.5.2A.3.2.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2A.3.2.5-1.

Table 6.5.2A.3.2.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

< End of changes >