3GPP TSG-RAN5 Meeting #81	R5-188219

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0062 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of receiver spurious emission tests for FR1 inter-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test case 7.9B 3 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sections under 7.9 is added. New test case 7.9B.3 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 7.9B.3 remains missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is outcome of 1 revision of R5-187277: R1 : - CR title aligned with 3GU. Cover page format changed to v11.2. - Setence to indicate that LTE anchor agnostic apply is added to minimum conformance requirement section according to R5-187815 - Initial condition step 6 is updated according to R5-187821. - Descriptions about E-UTRA configuration in initial conditions are aligned with the other test cases. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

##### 7.5B.3.5	Test requirement

For the NR CC, the throughput measurement of the carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 under the conditions specified in clause 7.5.5 in TS 38.521-1 [8]

## 7.9	Spurious emissions

## 7.9B	Spurious emissions for EN-DC in FR1

### 7.9B.1	Spurious Emissions for intra-band contiguous EN-DC in FR1

TBD.

### 7.9B.2	Spurious Emissions for intra-band non-contiguous EN-DC in FR1

TBD.

### 7.9B.3	Spurious Emissions for inter-band EN-DC within FR1

##### 7.9B.3.1	Test purpose

Same test purpose as in clause 7.9.1 in TS 38.521-1 [8] for the NR carrier.

##### 7.9B.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1.

##### 7.9B.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.9.3 in TS 38.521-1 [8] for the NR carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.9B.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

##### 7.9B.3.4	Test description

Same test description as in clause 7.9.4  in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8], the following steps are added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3. The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS36.521-1[10].

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Step 6 of Initial conditions as in clause 7.9.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

##### 7.9B.3.5	Test requirement

Same test requirement as in clause 7.9.5  in TS 38.521-1 [8].

<<< END OF CHANGES >>>