3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 4DL CA Rx tests to support LAA band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_LAA-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LAA Band support for 4DL CA Rx test cases are required as 4CA band combinations including band 46 are being added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 4CA Rx test cases : 1. Test applicability is updated to add Rel-13 UEs supporting band46 2. Test requirements for Band 46 is added 3. Test system uncertainity and tolerances in Annex F.1.3 and F.3.3 are updated to support LAA by refering back the 2CA test cases which already support LAA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA band combinations including operating band 46 can’t be tested in 4DL CA Rx test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.6.2A.7, Annex F.1.3, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

Table 7.1-2a: 3DL CA Name/Release mapping

| Number of Bands | 3CA Band Combinations | Release for test applicability | Name |
| --- | --- | --- | --- |
| 1 | CA_XD | Rel-10 | 3DL CA with FDD CA_XD |
|  | CA_XD | Rel-10 | 3DL CA with TDD CA_XD |
| 1 | CA_XA-XB | Rel-11 | 3DL CA with FDD CA_XA-XB |
|  | CA_XA-XB | Rel-11 | 3DL CA with TDD CA_XA-XB |
| 1 | CA_XA-XC | Rel-11 | 3DL CA with FDD CA_XA-XC |
|  | CA_XA-XC | Rel-11 | 3DL CA with TDD CA_XA-XC |
| 2 | CA_XA-YB | Rel-11 | 3DL CA with FDD CA_XA-YB |
|  | CA_XA-YB | Rel-11 | 3DL CA with TDD CA_XA-YB |
|  | CA_XA-YB | Rel-12 | 3DL CA with FDD-TDD CA_XA-YB |
| 2 | CA_XA-YC | Rel-11 | 3DL CA with FDD CA_XA-YC |
|  | CA_XA-YC | Rel-11 | 3DL CA with TDD CA_XA-YC |
|  | CA_XA-YC | Rel-12 | 3DL CA with FDD-TDD CA_XA-YC |
| 3 | CA_XA-YA-ZA | Rel-10 | 3DL CA with FDD CA_XA-YA-ZA |
|  |  | Rel-10 | 3DL CA with TDD CA_XA-YA-ZA |
|  |  | Rel-12 | 3DL CA with FDD-TDD CA_XA-YA-ZA |
| 2 | CA_XA-XA-YA | Rel-11 | 3DL CA with FDD CA_XA-XA-YA |
| 1 | CA_XA-XA-XA | Rel-11 | 3DL CA with FDD CA_XA-XA-XA |
| Note1: CA configurations involving downlink only operation in Band 46 are release independent from Rel-13 onwards (LAA was introduced in Rel-13). The 10 MHz channel bandwidth for Band 46 was introduced in TS 36.101 Rel-14 [2] and can be implemented in a release independent way from Rel-13. |  |  |  |

Table 7.1-2b: 4DL CA Name/Release mapping

| Number of Bands | 4CA Band Combinations | Release for test applicability | Name |
| --- | --- | --- | --- |
| 1 | CA_XE | Rel-11 | 4DL CA with TDD CA_XE |
| 2 | CA_XA-XD | Rel-11 | 4DL CA with FDD CA_XA-XD |
|  |  | Rel-11 | 4DL CA with TDD CA_XA-XD |
|  |  | Rel-12 | 4DL CA with FDD-TDD CA_XA-XD |
| 2 | CA_XB-YB | Rel-10 | 4DL CA with FDD CA_XB-YB |
|  | CA_XC-YB | Rel-10 | 4DL CA with FDD CA_XC-YB |
|  |  | Rel-12 | 4DL CA with FDD-TDD CA_XC-YB |
| 2 | CA_XC-YC | Rel-10 | 4DL CA with FDD CA_XC-YC |
|  |  | Rel-10 | 4DL CA with TDD CA_XC-YC |
| 1 | CA_XC-XC | Rel-11 | 4DL CA with TDD CA_XC-XC |
| 2 | CA_XA-XA-YB | Rel-11 | 4DL CA with FDD CA_XA-XA-YB |
|  | CA_XA-YA-YB | Rel-11 | 4DL CA with FDD CA_XA-YA-YB |
| 3 | CA_XA-YA-ZB | Rel-11 | 4DL CA with FDD CA_XA-YA-ZB |
| 3 | CA_XA-YA-ZC | Rel-11 | 4DL CA with FDD CA_XA-YA-ZC |
|  |  | Rel-12 | 4DL CA with FDD-TDD CA_XA-YA-ZC |
| 2 | CA_XA-YA-YC | Rel-11 | 4DL CA with FDD CA_XA-YA-YC |
|  |  | Rel-12 | 4DL CA with FDD-TDD CA_XA-YA-YC |
| 2 | CA_XA-XC-YA | Rel-12 | 4DL CA with FDD-TDD CA_XA-XC-YA |
| 3 | CA_XA-YA-ZC | Rel-12 | 4DL CA with FDD-TDD_XA-YA-ZC |
| 2 | CA_XA-XA-YA-YA | Rel-11 | 4DL CA with FDD CA_XA-XA-YA-YA |
| 3 | CA_XA-YA-YA-ZA | Rel-11 | 4DL CA with FDD CA_XA-YA-YA-ZA |
|  |  | Rel-12 | 4DL CA with FDD-TDD CA_XA-YA-YA-ZA |
| 4 | CA_XA-YA-ZA-RA | Rel-11 | 4DL CA with FDD CA_XA-YA-ZA-RA |
|  |  | Rel-12 | 4DL CA with FDD-TDD CA_XA-YA-ZA-RA |
| Note1: CA configurations involving downlink only operation in Band 46 are release independent from Rel-13 onwards (LAA was introduced in Rel-13). The 10 MHz channel bandwidth for Band 46 was introduced in TS 36.101 Rel-14 [2] and can be implemented in a release independent way from Rel-13. |  |  |  |

Table 7.1-2c: 5DL CA Name/Release mapping

| Number of Bands | 5CA Band Combinations | Release for test applicability | Name |
| --- | --- | --- | --- |
| 1 | CA_XF | Rel-12 | 5DL CA with TDD CA_XF |
| 1 | CA_XA-XE | Rel-11 | 5DL CA with TDD CA_XA-XE |
| 1 | CA_XC-XD | Rel-11 | 5DL CA with FDD CA_XC-XD |
|  |  | Rel-11 | 5DL CA with TDD CA_XC-XD |
| 1 | CA_XA-XA-XD | FFS | 5DL CA with FDD CA_XA-XA-XD |
|  |  | FFS | 5DL CA with TDD CA_XA-XA-XD |
| 1 | CA_XA-XC-XC | FFS | 5DL CA with FDD CA_XA-XC-XC |
|  |  | FFS | 5DL CA with TDD CA_XA-XC-XC |
| 2 | CA_XA-YE | Rel-11 | 5DL CA with TDD CA_XA-YE |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YE |
| 2 | CA_XC-YD | Rel-11 | 5DL CA with FDD CA_XC-YD |
|  |  | Rel-11 | 5DL CA with TDD CA_XC-YD |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XC-YD |
| 2 | CA_XA-YA-YD | Rel-11 | 5DL CA with TDD CA_XA-YA-YD |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-YD |
| 2 | CA_XA-YC-YC | Rel-11 | 5DL CA with FDD CA_XA-YC-YC |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YC-YC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YC-YC |
| 2 | CA_XA-YA-YA-YC | FFS | 5DL CA with FDD CA_XA-XA-XA-YC |
|  |  | FFS | 5DL CA with TDD CA_XA-XA-XA-YC |
|  |  | FFS | 5DL CA with FDD-TDD CA_XA-XA-XA-YC |
| 2 | CA_XA-XA-YD | Rel-11 | 5DL CA with TDD CA_XA-XA-YD |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-XA-YD |
| 2 | CA_XA-XC-YC | Rel-10 | 5DL CA with FDD CA_XA-XC-YC |
|  |  | Rel-10 | 5DL CA with TDD CA_XA-XC-YC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-XC-YC |
| 2 | CA_XA-XA-YA-YC | Rel-11 | 5DL CA with FDD CA_XA-XA-YA-YC |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-XA-YA-YC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-XA-YA-YC |
| 2 | CA_XA-XA-YA-YA-YA | FFS | 5DL CA with FDD CA_XA-XA-YA-YA-YA |
|  |  | FFS | 5DL CA with TDD CA_XA-XA-YA-YA-YA |
|  |  | FFS | 5DL CA with FDD-TDD CA_XA-XA-YA-YA-YA |
| 2 | CA_XA-XA-XA-YC | FFS | 5DL CA with FDD CA_XA-XA-XA-YC |
|  |  | FFS | 5DL CA with TDD CA_XA-XA-XA-YC |
|  |  | FFS | 5DL CA with FDD-TDD CA_XA-XA-XA-YC |
| 3 | CA_XA-YA-ZD | Rel-11 | 5DL CA with TDD CA_XA-YA-ZD |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-ZD |
| 3 | CA_XA-YC-ZC | Rel-11 | 5DL CA with FDD CA_XA-YC-ZC |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YC-ZC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YC-ZC |
| 3 | CA_XA-YA-ZA-ZC | Rel-11 | 5DL CA with FDD CA_XA-YA-ZA-ZC |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YA-ZA-ZC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-ZA-ZC |
| 3 | CA_XA-YA-ZA-ZA-ZA | FFS | 5DL CA with FDD CA_XA-YA-ZA-ZA-ZA |
|  |  | FFS | 5DL CA with TDD CA_XA-YA-ZA-ZA-ZA |
|  |  | FFS | 5DL CA with FDD-TDD CA_XA-YA-ZA-ZA-ZA |
| 3 | CA_XA-YA-YA-ZC | Rel-11 | 5DL CA with FDD CA_XA-YA-YA-ZC |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YA-YA-ZC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-YA-ZC |
| 3 | CA_XA-YA-YA-ZA-ZA | Rel-11 | 5DL CA with FDD CA_XA-YA-YA-ZA-ZA |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YA-YA-ZA-ZA |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-YA-ZA-ZA |
| 4 | CA_XA-YA-ZA-RC | Rel-11 | 5DL CA with FDD CA_XA-YA-ZA-RC |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YA-ZA-RC |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-ZA-RC |
| 4 | CA_XA-YA-ZA-RA-RA | Rel-11 | 5DL CA with FDD CA_XA-YA-ZA-RA-RA |
|  |  | Rel-11 | 5DL CA with TDD CA_XA-YA-ZA-RA-RA |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-ZA-RA-RA |
| 5 | CA_XA-YA-ZA-RA-SA | Rel-12 | 5DL CA with FDD CA_XA-YA-ZA-RA-SA |
|  |  | Rel-12 | 5DL CA with TDD CA_XA-YA-ZA-RA-SA |
|  |  | Rel-12 | 5DL CA with FDD-TDD CA_XA-YA-ZA-RA-SA |
| Note 1: X, Y, Z, R and S in this table correspond to different bands i.e. X != Y != Z != R != S. Note 2: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. CA_XA-YA-YD represents the set of CA_XA-YD-YA, YD-YA-XA, YA-XA-YD and YA-YD-XA Note 3: CA configurations involving downlink only operation in Band 46 are release independent from Rel-13 onwards (LAA was introduced in Rel-13). The 10 MHz channel bandwidth for Band 46 was introduced in TS 36.101 Rel-14 [2] and can be implemented in a release independent way from Rel-13. |  |  |  |

## << Unchanged sections omitted >>

##### 7.6.2A.7.5	Test Requirement

Except for the spurious response frequencies recorded at the final step of test procedure, the throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table s below, according to the type of CA.

For Table 7.6.2A.7.5-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where NRB is the allocated number of resource blocks in the frequency channel. For these exceptions the requirements of subclause 7.7A.5 Spurious Response for CA are applicable.

Table 7.6.2A.7.5-1: Out-of-band blocking parameters (intra-band contiguous)

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 | 9 |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |

Table 7.6.2A.7.5-2: Out of band blocking (intra-band contiguous)

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | range 1 | range 2 | range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_1C, CA_2C, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_40C, CA_41C, CA_40D, CA_40E, CA_42C1, CA_42D1, CA_42E1, CA_66B, CA_66C, CA_66D, CA_70C | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| Note 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

For Table 7.6.2A.7.5-4 frequency range 1, 2, and 3, the number of spurious response frequencies recorded in the step 9 of the test procedure shall not exceed exceptions per downlink in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7A.5 Spurious Response for CA are applicable.

Table 7.6.2A.7.5-3: Out-of-band blocking parameters (inter-band, intra-band non-contiguous)

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Pw in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2.5. Note 2: The reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. |  |  |  |  |  |  |  |

Table 7.6.2A.7.5-4: Out-of-band blocking for inter-band carrier aggregation with one uplink carrier

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pwanted | dBm | Table 7.6.2A.7.5-3 for all component carriers |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 or 15 < f – FDL_High(j) < 60 | -85 < f – FDL_Low(j ) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(j ) – 85 or FDL_High(j ) + 85 ≤ f ≤ FDL_Low(j +1) – 85 with j < X or FDL_High(X) + 85 ≤ f ≤ 12750 |
| Note 1: FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination (X ≤ 5 for the present version of this specification). Note 2: For FDL_Low(j+1) – FDL_High(j) < 145 MHz and FInterferer in FDL_High(j) < f < FDL_Low(j+1), FInterferer can be in both Range 1 and Range 2. Then the lower of the PInterferer applies. Note 3: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5A.5.5 and 7.6.1A.5.5 shall be applied for carrier j. Note 4: RIB,c according to Table 7.3.3-1A applies when serving cell c is measured. Note 5: For inter-band CA combinations containing Bands 42 or 43, the interferer with respect to Band 42 or Band 43 shall have power level (PInterferer) for Range 3 modified to -20 + RIB,c dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. Note 6: For inter-band CA combinations containing Bands 7 and 38 simultaneously, for FInterferer Bands 7 and 38 are considered as one single band as follows: FDL_Low = 2570 MHz and FDL_High = 2690 MHz. For Range 2, the following applies for FDL_Low: -95 < f – FDL_Low ≤ -60 or 60 ≤ f – FDL_High < 85. For Range 3 the following applies 1 ≤ f ≤ FDL_Low -95 or FDL_High + 85 ≤ f ≤ 12750. |  |  |  |  |

Table 7.6.2A.7.5-4a: Out-of-band blocking for inter-band carrier aggregation with band 46 and with one active uplink

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pwanted | dBm | Table 7.6.2A.5.5-3 for all component carriers |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c (NOTE 5) |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 with j ≤ K or 15 < f – FDL_High(j) < 60 with j ≤ K | -85 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(j) – 85 or FDL_High(j) + 85 ≤ f ≤ 12750 |
| NOTE 1: FDL_Low(j) and FDL_High(j), j = 1,…,K,…N, denote the respective lower and upper frequency limits of the (non-overlapping) operating bands of the CA configuration numbered in increasing order of frequency, with N the number of bands in the band combination and K the number of bands with FDL_High ≤ 3800 MHz. NOTE 2: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5A.5.5 and 7.6.1A.5.5 shall be applied for carrier j ≤ K. NOTE 3: For FDL_Low(j) – 60 MHz ≤ f ≤ FDL_High(j) + 60 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5A.5.5 and 7.6.1A.5.5 shall be applied for carrier K < j ≤ N. NOTE 4: RIB,c according to Table 7.3.3-1A applies when serving cell c is measured. NOTE 5: The power level (PInterferer) for Range 3 is modified to -20 dBm for FInterferer > 4400 MHz except for band combinations with Band 42 or Band 43 for which PInterferer for Range 3 is modified to -20 dBm for FInterferer > 2800 MHz. |  |  |  |  |

For Table 7.6.2A.7.5-5 in frequency range 1, 2 and 3, the number of spurious response frequencies recorded in the step 9 of the test procedure shall not exceed exceptions per assigned E-UTRA channel per sub-block gap of the E-UTRA CA configuration are allowed for spurious response frequencies when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7A.5 spurious response for CA are applicable.

For Table 7.6.2A.7.5-5 in frequency range 4 the number of spurious response frequencies recorded in the step 9 of the test procedure shall not exceed exceptions per assigned E-UTRA channel per sub-block gap of the E-UTRA CA configuration are allowed for spurious response frequencies when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7A.5 spurious response for CA are applicable.

Table 7.6.2A.7.5-5: Out of band blocking (intra-band non-contiguous)

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | range 1 | range 2 | range 3 | range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42 (NOTE 2), 43 (NOTE 2), 44, 45, 65, 66, 68, 70 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17 | FInterferer | MHz | - | - | - | FUL_low - FUL_high |
| NOTE 1: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 3: For the UE that supports both Band 4 and Band 66, the out-of-blocking frequency range for Band 4 is defined relative to FDL_low and FDL_high of Band 66. NOTE 4: For a UE supporting CA_20A-28A the requirements for Band 20 and Band 28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high by Band 20 (Table 5.5A-2). |  |  |  |  |  |  |

For a UE, which supports CA configuration in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C, PInterferer power defined in Tables 7.6.2A.7.5 2, 7.6.2A.7.5-4 and 7.6.2A.7.5-5 is increased by the amount given by ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C accordingly.

## << Unchanged sections omitted >>

## << Unchanged sections omitted >>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3 Reference sensitivity power level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.3_1 Reference sensitivity level with 4 Rx antenna ports |  | Same as 7.3 |  |  |  |
| 7.3A.1 Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) |  | Same as 7.3 for each CC |  |  |  |
| 7.3A.2 Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.3 Reference sensitivity level for CA (inter-band DL CA without UL CA) |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.3A.4 Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.5 Reference sensitivity level for 3DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.6 Reference sensitivity level for CA (inter-band DL CA and UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.7 Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.9 Reference sensitivity level for 4DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.10 Reference sensitivity level for 5DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3B Reference Sensitivity Level for UL-MIMO |  | Same as 7.3 |  |  |  |
| 7.3D.1 Reference sensitivity level for ProSe Direct Discovery |  | Sidelink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3D.2 Reference sensitivity level for ProSe Direct Communication |  | Same as 7.3D.1 |  |  |  |
| 7.3E Reference Sensitivity Level for UE category 0 |  | Same as 7.3 |  |  |  |
| 7.3EA Reference Sensitivity Level for UE category M1 |  | Same as 7.3 |  |  |  |
| 7.3EB Reference sensitivity level for UE category 1bis |  | Same as 7.3 |  |  |  |
| 7.3EC Reference Sensitivity Level for UE category M2 |  | Same as 7.3 |  |  |  |
| 7.3F.1 Reference sensitivity level without repetitions for UE category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3F.2 Reference sensitivity level with repetitions for category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3G.1 Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.3G.2 Reference sensitivity level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA |  |  |  |
| 7.3G.3 Reference sensitivity level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.3G.1 |  |  |  |
| 7.4 Maximum input level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.4_1 Maximum input level with 4 Rx antenna ports |  | Same as 7.4 |  |  |  |
| 7.4A.1 Maximum input level for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.4 for each CC |  | Uncertainties apply for each CC |  |
| 7.4A.1 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.2 Maximum input level for CA (intra band contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.2 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.5 Maximum input level for 3DL CA |  | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.5_H Maximum input level for SCE-L1 for 3DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.7 Maximum input level for 4DL CA |  | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.7_H Maximum input level for 4DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.8_H Maximum input level for 5DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4B Maximum Input Level for UL-MIMO |  | Downlink power same as 7.4 Uplink power measurement same as 7.4, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.4D.1 Maximum input level for ProSe Direct Discovery |  | Sidelink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.4D.2 Maximum input level for ProSe Direct Communication |  | Same as 7.4D.1 |  |  |  |
| 7.4E Maximum input level for UE category 0 |  | Same as 7.4 |  |  |  |
| 7.4EA Maximum input level for UE category M1 |  | Same as 7.4 |  |  |  |
| 7.4EB Maximum input level for UE Category 1bis |  | Same as 7.4 |  |  |  |
| 7.4EC Maximum input level for UE category M2 |  | Same as 7.4 |  |  |  |
| 7.4F Maximum input level for UE category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.4G.1 Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.4G.2 Maximum input level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA |  |  |  |
| 7.4G.3 Maximum input level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.4G.1 |  |  |  |
| 7.4_H Maximum input level for SCE-L1 |  | Same as 7.4 |  |  |  |
| 7.5 Adjacent Channel Selectivity (ACS) |  | ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB |  |
| 7.5_1 Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports |  | Same as 7.5 |  |  |  |
| 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.5 for each CC |  | Same as 7.5 The wanted signal level uncertainty applies for each CC. Overall ACS uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA without UL CA) |  | Downlink power same as 7.5 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter band DL CA without UL CA) |  | ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | The wanted signal level uncertainty applies for each CC Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6GHz Impact of interferer ACLR 0.1dB |  |
| 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA) |  | Downlink power same as 7.5 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.5 Adjacent Channel Selectivity (ACS) for 3DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.7 Adjacent Channel Selectivity (ACS) for 4DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.8 Adjacent Channel Selectivity (ACS) for 5DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO |  | ACS value same as 7.5 Uplink power measurement same as 7.5, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.5D.1 Adjacent channel selectivity (ACS) for ProSe Direct Discovery |  | ACS value ±1.1 dB, f ≤ 3.0GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB |  |
| 7.5D.2 Adjacent channel selectivity (ACS) for ProSe Direct Communication |  | Same as 7.5D.1 |  | Same as 7.5D.1 |  |
| 7.5E Adjacent Channel Selectivity (ACS) for UE category 0 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EA Adjacent Channel Selectivity (ACS) for UE category M1 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EB Adjacent Channel Selectivity (ACS) for UE Category 1bis |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EC Adjacent Channel Selectivity (ACS) for UE category M2 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5F Adjacent Channel Selectivity (ACS) for UE category NB1 |  | ACS value ±1.1 dB, f ≤ 3.0GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.1 Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | ACS ±2.2 dB, 4.2GHz < f ≤ 6GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.2 Adjacent channel selectivity (ACS) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | ACS ±2.2 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.3 Adjacent channel selectivity (ACS) for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.5G.1 |  | Same as 7.5G.1 |  |
| 7.6.1 In-band blocking |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.1 for each CC |  | Same as 7.6.1 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.5 dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.5 In-band blocking for 3DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.7 In-band blocking for 4DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.8 In-band blocking for 5DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1B In-band blocking for UL-MIMO |  | Blocking same as 7.6.1 Uplink power measurement same as 7.6.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication |  | Same as 7.6.1D.1 |  | Same as 7.6.1D.1 |  |
| 7.6.1E In-band blocking for UE category 0 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1EB In-band blocking for UE Category 1bis |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1F In-band blocking for category NB1 |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB f ≤ 4.2GHz, 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1EA In-band blocking for UE category M1 |  | Blocking ±1.4 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1EC In-band blocking for UE category M2 |  | Same as 7.6.1EA |  | Same as 7.6.1EA |  |
| 7.6.2 Out of-band blocking |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2 for each CC |  | Same as 7.6.2 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.5 Out-of-band blocking for 3DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.7 Out-of-band blocking for 4DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.8 Out-of-band blocking for 5DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2B Out-of-band blocking for UL-MIMO |  | Blocking same as 7.6.2 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.6.2E Out of-band blocking for UE category 0 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2F Out of-band blocking for category NB1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EA Out-of-band blocking for UE category M1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EB Out-of-band blocking for UE category 1bis |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.3 Narrow band blocking |  | Blocking ±1. 1 dB, f ≤ 3.0GHz ±1. 5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.3 for each CC |  | Same as 7.6.3 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.5 Narrow band blocking for 3DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.7 Narrow band blocking for 4DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.8 Narrow band blocking for 5DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3B Narrow band blocking for UL-MIMO |  | Blocking same as 7.6.3 Uplink power measurement same as 7.6.3, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery |  | Blocking ±1.1 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication |  | Same as 7.6.3D.1 |  | Same as 7.6.3D.1 |  |
| 7.6.3E Narrow band blocking for UE category 0 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EA Narrow band blocking for UE category M1 |  | Blocking ±1. 1 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3EB Narrow band blocking for UE category 1bis |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EC Narrow band blocking for UE category M2 |  | Same as 7.6.3EA |  | Same as 7.6.3EA |  |
| 7.7 Spurious response |  | Same as 7.6.2 |  | Same as 7.6.2. |  |
| 7.7_1 Spurious response with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.5 Spurious response for 3DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.7 Spurious response for 4DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.8 Spurious response for 5DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7B Spurious response for UL-MIMO |  | Same as 7.7 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.7D.1 Spurious response for ProSe Direct Discovery |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.7D.2 Spurious response for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.7E Spurious response for UE category 0 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EA Spurious response for UE category M1 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EB Spurious response for UE Category 1bis |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7F Spurious response for category NB1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.7G.1 Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.7G.2 Spurious response for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB Same as 7.6.2 for E-UTRA |  | Sams as 7.7G.1 Same as 7.6.2 |  |
| 7.7G.3 Spurious response for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.7G.1 |  | Same as 7.7G.1 |  |
| 7.8.1 Wide band intermodulation |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB CW Interferer level ± 0.8dB Mod Interferer level ± 1.0dB |  |
| 7.8.1_1 Wide band Intermodulation with 4 Rx antenna ports |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1A.1 Wideband intermodulation for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.8.1 for each CC |  | Same as 7.8.1 The wanted signal level uncertainty applies for each CC. Overall intermodulation uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.8.1A.2 Wideband intermodulation for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.3 Wideband intermodulation for CA (inter band DL CA without UL CA) |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz ±3.8 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB CW Interferer level ± 0.8dB Mod Interferer level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.8.1A.4 Wideband intermodulation for CA (intra band non-contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.5 Wideband intermodulation for 3DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.7 Wideband intermodulation for 4DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.8 Wideband intermodulation for 5DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1B Wide band intermodulation for UL-MIMO |  | Intermodulation same as 7.8.1 Uplink power measurement same as 7.8.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.8.1D.1 Wide band Intermodulation for ProSe Direct Discovery |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2 |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB |  |
| 7.8.1D.2 Wide band Intermodulation for ProSe Direct Communication |  | Same as 7.8.1D.2 |  | Same as 7.8.1D.2 |  |
| 7.8.1E Wide band intermodulation for UE category 0 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EA Wide band Intermodulation for UE category M1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EB Wide band Intermodulation for UE category 1bis |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EC Wide band Intermodulation for UE category M2 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1F Wide band intermodulation for category NB1 |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB |  |
| 7.8.1G.1 Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Intermodulation ±3.8 dB, 4.2GHz < f ≤ 6GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.9 Spurious emissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |  |  |
| 7.9_1 Spurious emissions with 4 Rx antenna ports |  | Same as 7.9 |  |  |  |
| 7.9A Spurious emissions for CA |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| 7.9E Spurious emissions for UE category 0 |  | Same as 7.9 |  |  |  |
| 7.9EB Spurious emissions for UE Category 1bis |  | Same as 7.9 |  |  |  |
| 7.9F Spurious emissions for category NB1 |  | Same as 7.9 |  |  |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |  |  |  |

## << Unchanged sections omitted >>

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 7.3 Reference sensitivity power level | Reference sensitivity power level: For 1.4MHz -102.2dBm -103.2dBm -105.2dBm -106.2dBm For 3MHz -99.2dBm -100.2dBm -102.2dBm For 5MHz -97dBm -98dBm -98.5dBm -99dBm -100dBm -96.5dBm Band 9 with Multi band For 10MHz -94dBm -95dBm -95.5dBm -96dBm -97dBm -93.5dBm Band 9 with Multi band For 15MHz -92.2dBm -93.2dBm -93.7dBm -94.2dBm -95.2dBm -91.7dBm Band 9 with Multi band For 20MHz -91dBm -92dBm -93dBm -94dBm -90.5dBm Band 9 with Multi band T-put limit = 95% of maximum for the Ref Meas channel | All cases: f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3_1 Reference sensitivity level with 4 Rx antenna ports | Same as 7.3 | Same as 7.3 | Same as 7.3 |
| 7.3A.1 Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) | PCC and SCC Value in Table 7.3.1-1 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.3 | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |
| 7.3A.2 Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) | Same as 7.3A.1 | Same as 7.3 | Same as 7.3A.1 |
| 7.3A.3 Reference sensitivity level for CA (inter-band DL CA without UL CA) | PCC and SCC Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bA, 7.3.1A-0d, 7.3.1A-0eA PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel | All cases: f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bA, 7.3.1A-0d, 7.3.1A-0eA +TT PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |
| 7.3A.4 Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA) | PCC Value in Table 7.3.1-1 SCC Value in Table 7.3.1-1 +ΔRIBNC in Table 7.3.1A-3 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.3 | PCC Value in TS 36.101 Table 7.3.1-1+TT SCC Value in TS 36.101 Table 7.3.1-1+TT +ΔRIBNC in TS 36.101 Table 7.3.1A-3 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |
| 7.3A.5 Reference sensitivity level for 3DL CA | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.3A.3 | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |
| 7.3A.6 Reference sensitivity level for CA (inter-band DL CA and UL CA) | TBD | TBD | TBD |
| 7.3A.7 Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) | PCC Value in Table 7.3.1-1+ ΔR2UL_PCC in Table 7.3.1A-4 SCC Value in Table 7.3.1-1 + ΔR2UL_PCC in Table 7.3.1A-4 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A or Table 7.3.1-1B where applicable T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.3 | PCC Value in TS 36.101 Table 7.3.1-1+TT+ ΔR2UL_PCC in Table 7.3.1A-4 SCC Value in TS 36.101 Table 7.3.1-1+TT + ΔR2UL_SCC in Table 7.3.1A-4 PCC and SCC values are each increased by ΔRIB,c in Table 7.3.3-1A where applicable T-put limit unchanged |
| 7.3A.9 Reference sensitivity level for 4DL CA | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.3A.3 | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |
| 7.3A.10 Reference sensitivity level for 5DL CA | PCC and SCCs Value in Table 7.3.1-1 Exceptions in Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.1A-3, 7.3.1-1B or/and ΔRIBNC in Table 7.3.1A-3 where applicable T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.3A.3 | PCC and SCC Value in TS 36.101 Table 7.3.1-1+TT Exceptions in TS 36.101 Tables 7.3.1A-0a, 7.3.1A-0bC, 7.3.1A-0d +TT PCC and SCCs values are each increased by ΔRIB,c in Table 7.3.3-1A, 7.3.3-1B or/and ΔRIBNC in TS 36.101 Table 7.3.1A-3 where applicable T-put limit unchanged |
| 7.3B Reference Sensitivity Level for UL-MIMO | Same as 7.3 | Same as 7.3 | Same as 7.3 |
| 7.3D.1 Reference sensitivity level for ProSe Direct Discovery | Values as defined in table 7.3D.0.1 | All cases: f ≤ 3.0GHz 0.7dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3D.2 Reference sensitivity level for ProSe Direct Communication | Values as defined in table 7.3D.0.2 | Same as 7.3D.1 | Same as 7.3D.1 |
| 7.3E Reference Sensitivity Level for UE category 0 | Same as 7.3 | Same as 7.3 | Same as 7.3 |
| 7.3EA Reference Sensitivity Level for UE category M1 | Reference sensitivity power level as specified T-put limit = 95% of maximum for the Ref Meas channel | All cases: 0.7dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3EB Reference sensitivity level for UE category 1bis | Same as 7.3 | Same as 7.3 | Same as 7.3 |
| 7.3EC Reference Sensitivity Level for UE category M2 | Same as 7.3EA | Same as 7.3EA | Same as 7.3EA |
| 7.3F.1 Reference sensitivity level without repetitions for UE category NB1 | Reference sensitivity power level: -108.2dBm T-put limit = 95% of maximum for the Ref Meas channel | f ≤ 3.0GHz 0.7dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3F.2 Reference sensitivity level with repetitions for category NB1 | Reference sensitivity power level: TBD T-put limit = TBD% of maximum for the Ref Meas channel | f ≤ 3.0GHz 0.7dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3G.1 Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Reference sensitivity power level: For 10MHz -90.4dBm For 20MHz -87.5dBm T-put limit = 95% of maximum for the Ref Meas channel | All cases: 4.2GHz < f ≤ 6GHz 1.5 dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3G.2 Reference sensitivity level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Reference sensitivity power level for V2X sidelink: For 10MHz -90.4dBm For 20MHz -87.5dBm Reference sensitivity power level for E-UTRA: For 1.4MHz -101.7dBm -102.2dBm For 3MHz -98.7dBm -99.2dBm -100.2dBm For 5MHz -97dBm -97.2dBm -98dBm -98.5dBm -100dBm For 10MHz -90.4dBm -94dBm -94.2dBm -95dBm -95.5dBm -97dBm For 15MHz -91.2dBm -92.2dBm -93.2dBm -93.7dBm -95.2dBm For 20MHz -87.5dBm -90dBm -91dBm -92dBm -94dBm T-put limit = 95% of maximum for the Ref Meas channel | All cases: 4.2GHz < f ≤ 6GHz 1.5 dB f ≤ 3.0GHz 0.7dB 3.0GHz < f ≤ 4.2GHz 1.0 dB | Formula: Reference sensitivity power level + TT T-put limit unchanged |
| 7.3G.3 Reference sensitivity level for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.3G.1 | Same as 7.3G.1 | Same as 7.3G.1 |
| 7.4 Maximum input level | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7 dBm 3.0GHz < f ≤ 4.2GHz: -26.0 dBm T-put limit unchanged Uplink power measurement window comprises four quantities: 1. UE power step size 1dB 2. UE Power step tolerance ±1dB 3. Test system power measurement at top of window: f ≤ 3.0GHz ±0.7 dB 3.0GHz < f ≤ 4.2GHz ±1.0 dB. 4. Test system power measurement at bottom of window: f ≤ 3.0GHz ±0.7 dB 3.0GHz < f ≤ 4.2GHz ±1.0 dB. Items 1 to 4 are added arithmetically: Overall UL power window size: f ≤ 3.0GHz: (1dB+1dB+0.7dB+0.7dB) =3.4dB 3.0GHz < f ≤ 4.2GHz: (1dB+1dB+1dB+1dB) =4dB Top of window is aligned to UL power requirement, hence +0dB, -3.4dB or +0dB, -4.0dB according to frequency |
| 7.4_1 Maximum input level with 4 Rx antenna ports | Same as 7.4 | Same as 7.4 | Same as 7.4 |
| 7.4A.1 Maximum input level for CA (intra band contiguous DL CA and UL CA) | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4, but the target power for each CC is scaled according to the allocation: For PCC: 10log(P_LCRB/NRB_alloc) For SCC: 10log(S_LCRB/NRB_alloc) |
| 7.4A.1 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC – TT Signal level: f ≤ 3.0GHz: -27.7 dBm 3.0GHz < f ≤ 4.2GHz: -28.0 dBm T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4, but the target power for each CC is scaled according to the allocation: For PCC: 10log(P_LCRB/NRB_alloc) For SCC: 10log(S_LCRB/NRB_alloc) |
| 7.4A.2 Maximum input level for CA (intra band contiguous DL CA without UL CA) | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.4A.2 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA without UL CA) | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7 dBm 3.0GHz < f ≤ 4.2GHz: -26.0 dBm4.2GHz < f ≤ 6.0GHz: -26.5 dBm T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7 dBm 3.0GHz < f ≤ 4.2GHz: -26.0 dBm T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.4A.5 Maximum input level for 3DL CA | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.4A.5_H Maximum input level for SCE-L1 for 3DL CA | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |
| 7.4A.7 Maximum input level for for 4DL CA | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB 4.2GHz < f ≤ 6.0GHz 1.5 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |
| 7.4A.7_H Maximum input level for 4DL CA for 256QAM in DL | Same as 7.4A.7 | Same as 7.4A.7 | Same as 7.4A.7 |
| 7.4A.8_H Maximum input level for 5DL CA for 256QAM in DL | Signal level for each CC as specified T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Signal level for each CC - TT T-put limit unchanged Uplink power measurement window is calculated in the same way as 7.4 |
| 7.4B Maximum Input Level for UL-MIMO | Same as 7.4 | Same as 7.4 | Same as 7.4 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.4D.1 Maximum input level for ProSe Direct Discovery | Signal level -22dBm T-put limit = 95% of maximum for the Ref Meas channel | f ≤ 3.0GHz 0.7 dB | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -22.7 dBm T-put limit unchanged |
| 7.4D.2 Maximum input level for ProSe Direct Communication | Same as 7.4D.1 | Same as 7.4D.1 | Same as 7.4D.1 |
| 7.4E Maximum input level for UE category 0 | Same as 7.4 | Same as 7.4 | Same as 7.4 |
| 7.4EA Maximum input level for UE category M1 | Same as 7.4 | Same as 7.4 | Same as 7.4 |
| 7.4EB Maximum input level for UE category 1bis | Same as 7.4 | Same as 7.4 | Same as 7.4 |
| 7.4EC Maximum input level for UE category M2 | Same as 7.4 | Same as 7.4 | Same as 7.4 |
| 7.4_H Maximum input level for SCE-L1 | Signal level -27dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -27.7 dBm 3.0GHz < f ≤ 4.2GHz: -28.0 dBm T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.4F Maximum imput level for UE category NB1 | Signal level -25dBm T-put limit = 95% of maximum for the Ref Meas channel | f ≤ 3.0GHz 0.7 dB | Formula: Maximum input level - TT Signal level: f ≤ 3.0GHz: -25.7dBm T-put limit unchanged |
| 7.4G.1 Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Signal level -22dBm T-put limit = 95% of maximum for the Ref Meas channel | 4.2GHz < f ≤ 6GHz 1.5 dB | Formula: Maximum input level - TT Signal level: 4.2GHz < f ≤ 6GHz : -23.5dBm T-put limit unchanged |
| 7.4G.2 Maximum input level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Signal level -22dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 4.2GHz < f ≤ 6GHz 1.5 dB for V2X sidelink f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB | Same as 7.4G.1 for V2X sidelink Same as 7.4 for E-UTRA |
| 7.4G.3 Maximum input level for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.4G.1 | Same as 7.4G.1 | Same as 7.4G.1 |
| 7.5 Adjacent Channel Selectivity (ACS) | Case 1: Wanted signal power, all BWs: (REFSENS + 14 dB) Interferer signal power For 1.4 MHz, 3 MHz, 5 MHz, 10 MHz BW: (REFSENS + 45.5 dB) For 15 MHz BW: (REFSENS + 42.5 dB) For 20 MHz BW: (REFSENS + 39.5 dB) Case 2: Wanted signal power For 1.4 MHz, 3 MHz, 5 MHz, 10 MHz BW: -56.5 dBm For 15 MHz BW: -53.5 dBm For 20 MHz BW: -50.5 dBm Interferer signal power, all BWs: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.5_1 Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports | Same as 7.5 | Same as 7.5 | Same as 7.5 |
| 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra-band contiguous DL CA and UL CA) | Case 1: Wanted signal power: (REFSENS + 14 dB) Interferer signal power For CA BW Class C: (Aggregated power + 22.5 dB) Case 2: Wanted signal power: -47.5 +10log(NRB,c/NRB agg) dBm Interferer signal power For CA BW Class C: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA without UL CA) | Same as 7.5A.1 | Same as 7.5A.1 | Same as 7.5A.1 |
| 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter band DL CA without UL CA) | Same as 7.5 for each CC | Same as 7.5 | Same as 7.5 for each CC |
| 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.5 for each CC | Same as 7.5 | Same as 7.5 for each CC |
| 7.5A.5 Adjacent Channel Selectivity (ACS) for 3DL CA | Intra-band contiguous DL CA Case 1: Wanted signal power: (REFSENS + 14 dB) Interferer signal power For CA BW Class B, Class C: (Aggregated power + 22.5 dB) For CA BW Class D: (Aggregated power + 20.7 dB) Case 2: Wanted signal power: For CA BW Class B: -50.5 +10log(NRB,c/NRB agg) dBm For CA BW Class C: -47.5 +10log(NRB,c/NRB agg) dBm For CA BW Class D: -43.9 +10log(NRB,c/NRB agg) dBm Interferer signal power -25 dBm Inter-band CA Same as 7.5 for each CC T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.5A.7 Adjacent Channel Selectivity (ACS) for 4DL CA | Same as 7.5A.5 for each CC | Same as 7.5A.5 | Same as 7.5A.5 for each CC |
| 7.5A.8 Adjacent Channel Selectivity (ACS) for 5DL CA | Same as 7.5A.5 for each CC except for following additional requirements Intra-band contiguous DL CA Case 1: For CA BW Class F: (Aggregated power + 18.5 dB) Case 2: Wanted signal power: For CA BW Class F: -43.5 +10log(NRB,c/NRB agg) dBm | Same as 7.5A.5 | Same as 7.5A.5 for each CC |
| 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO | Same as 7.5 | Same as 7.5 | Same as 7.5 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.5D.1 Adjacent channel selectivity (ACS) for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.5D.2 Adjacent channel selectivity (ACS) for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.5D.1 | Same as 7.5D.1 |
| 7.5E Adjacent Channel Selectivity (ACS) for UE category 0 | Same as 7.5 | Same as 7.5 | Same as 7.5 |
| 7.5EA Adjacent Channel Selectivity (ACS) for UE category M1 | Same as 7.5 | Same as 7.5 | Same as 7.5 |
| 7.5EB Adjacent Channel Selectivity (ACS) for UE category 1bis | Same as 7.5 | Same as 7.5 | Same as 7.5 |
| 7.5EC Adjacent Channel Selectivity (ACS) for UE category M2 | Same as 7.5 | Same as 7.5 | Same as 7.5 |
| 7.5F Adjacent Channel Selectivity (ACS) for UE category NB1 | ACS 1: Wanted signal power: (REFSENS + 14 dB) Interferer signal power For GSM interferer: (REFSENS + 42 dB) For E-UTRA interferer: (REFSENS + 47 dB) ACS 2: Wanted signal power For GSM interferer: -53 dBm For E-UTRA interferer: -58 dBm Interferer signal power: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.5G.1 Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Case 1: Wanted signal power, all BWs: (REFSENS + 14 dB) Interferer signal power For 10 MHz BW: (REFSENS + 45.5 dB) For 20 MHz BW: (REFSENS + 39.5 dB) Case 2: Wanted signal power For 10 MHz BW: -56.5 dBm For 20 MHz BW: -50.5 dBm Interferer signal power, all BWs: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.5G.2 Adjacent channel selectivity (ACS) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Case 1: Wanted signal power, all BWs: (REFSENS + 14 dB) Interferer signal power For 10 MHz BW: (REFSENS + 45.5 dB) For 20 MHz BW: (REFSENS + 39.5 dB) Case 2: Wanted signal power For 10 MHz BW: -56.5 dBm For 20 MHz BW: -50.5 dBm Interferer signal power, all BWs: -25 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.5G.3 Adjacent channel selectivity (ACS) for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.5G.1 | Same as 7.5G.1 | Same as 7.5G.1 |
| 7.6.1 In-band blocking | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.6.1 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.5 In-band blocking for 3DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.7 In-band blocking for 4DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1A.8 In-band blocking for 5DL CA | Same as 7.6.1A.1 | Same as 7.6.1A.1 | Same as 7.6.1A.1 |
| 7.6.1B In-band blocking for UL-MIMO | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.6.1D.1 | Same as 7.6.1D.1 |
| 7.6.1E In-band blocking for UE category 0 | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 |
| 7.6.1F In-band blocking for category NB1 | Wanted signal power: (REFSENS + 6dB) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1EA In-band blocking for UE category M1 | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB | Same as 7.6.1 |
| 7.6.1EB In-band blocking for UE category 1bis | Same as 7.6.1 | Same as 7.6.1 | Same as 7.6.1 |
| 7.6.1EC In-band blocking for UE category M2 | Same as 7.6.1EA | Same as 7.6.1EA | Same as 7.6.1 |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -56dBm or -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB 4.2GHz < f ≤ 6GHz 0dB, -4.3dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.6.1G.1 | Same as 7.6.1G.1 | Same as 7.6.1G.1 |
| 7.6.2 Out of-band blocking | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.6.2 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.5 Out-of-band blocking for 3DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.7 Out-of-band blocking for 4DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2A.8 Out-of-band blocking for 5DL CA | Same as 7.6.2A.1 | Same as 7.6.2A.1 | Same as 7.6.2A.1 |
| 7.6.2B Out-of-band blocking for UL-MIMO | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.6.2D.1 | Same as 7.6.1D.1 |
| 7.6.2E Out of-band blocking for UE category 0 | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 |
| 7.6.2F Out of-band blocking for category NB1 | Wanted signal power: (REFSENS + 6dB) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.2EA Out-of-band blocking for UE category M1 | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm, -30dBm or -15dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB | Same as 7.6.2 |
| 7.6.2EB Out-of-band blocking for UE category 1bis | Same as 7.6.2 | Same as 7.6.2 | Same as 7.6.2 |
| 7.6.3 Narrow band blocking | Wanted signal power,: (REFSENS + BW dependent value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.6.3 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.5 Narrow band blocking for 3DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.7 Narrow band blocking for 4DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3A.8 Narrow band blocking for 5DL CA | Same as 7.6.3A.1 | Same as 7.6.3A.1 | Same as 7.6.3A.1 |
| 7.6.3B Narrow band blocking for UL-MIMO | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.6.3D.1 | Same as 7.6.3D.1 |
| 7.6.3E Narrow band blocking for UE category 0 | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 |
| 7.6.3EA Narrow band blocking for UE category M1 | Wanted signal power,: (REFSENS + BW dependent value) Interferer signal power: -55dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB | Same as 7.6.3 |
| 7.6.3EB Narrow band blocking for UE category 1bis | Same as 7.6.3 | Same as 7.6.3 | Same as 7.6.3 |
| 7.6.3EC Narrow band blocking for UE category M2 | Same as 7.6.3EA | Same as 7.6.3EA | Same as 7.6.3 |
| 7.7 Spurious response | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.7_1 Spurious response with 4 Rx antenna ports | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | Same as 7.7 | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4A.1 |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.5 Spurious response for 3DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.7 Spurious response for 4DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7A.8 Spurious response for 5DL CA | Same as 7.7A.1 | Same as 7.7A.1 | Same as 7.7A.1 |
| 7.7B Spurious response for UL-MIMO | Same as 7.7 | Same as 7.7 | Same as 7.7 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.7D.1 Spurious response for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7D.2 Spurious response for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.7D.1 | Same as 7.7D.1 |
| 7.7E Spurious response for UE category 0 | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7EA Spurious response for UE category M1 | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7EB Spurious response for UE category 1bis | Same as 7.7 | Same as 7.7 | Same as 7.7 |
| 7.7F Spurious response for category NB1 | Wanted signal power: (REFSENS + 6 dB) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7G.1 Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7G.2 Spurious response for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Wanted signal power: (REFSENS + BW dependent value) Interferer signal power: -44dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |
| 7.7G.3 Spurious response for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 7.7G.1 | Same as 7.7G.1 | Same as 7.7G.1 |
| 7.8.1 Wide band intermodulation | Wanted signal power: For 1.4 MHz BW: (REFSENS + 12 dB) For 3 MHz BW: (REFSENS + 8 dB) For 5 MHz and 10MHz BW: (REFSENS + 6 dB) For 15 MHz BW: (REFSENS + 7 dB) For 20 MHz BW: (REFSENS + 9 dB) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 |
| 7.8.1_1 Wide band Intermodulation with 4 Rx antenna ports | Same as 7.8.1 | Same as 7.8.1 | Same as 7.8.1 |
| 7.8.1A.1 Wideband intermodulation for CA (intra band contiguous DL CA and UL CA) | Wanted signal power: (REFSENS + CA BW Class specific value) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB f ≤ 3.0GHz 0dB, -3.4dB 3.0GHz < f ≤ 4.2GHz 0dB, -4.0dB | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged Uplink power measurement window same as 7.4 A.1 |
| 7.8.1A.2 Wideband intermodulation for CA (intra band contiguous DL CA without UL CA) | Same as 7.8.1A.1 | Same as 7.8.1A.1 | Same as 7.8.1A.1 |
| 7.8.1A.3 Wideband intermodulation for CA (inter band DL CA without UL CA) | Same as 7.8.1A.1 | Same as 7.8.1A.1 | Same as 7.8.1A.1 |
| 7.8.1A.4 Wideband intermodulation for CA (intra band non-contiguous DL CA without UL CA) | Same as 7.8.1A.1 | Same as 7.8.1A.1 | Same as 7.8.1A.1 |
| 7.8.1A.5 Wideband intermodulation for 3DL CA | Same as 7.8.1A.1 | Same as 7.8.1A.1 | Same as 7.8.1A.1 |
| 7.8.1A.7 Wideband intermodulation for 4 DL CA | Same as 7.8.1A.1 | Same as 7.8.1A.1 | Same as 7.8.1A.1 |
| 7.8.1A.8 Wideband intermodulation for 5 DL CA | Same as 7.8.1A.1 | Same as 7.8.1A.1 | Same as 7.8.1A.1 |
| 7.8.1B Wide band intermodulation for UL-MIMO | Same as 7.8.1 | Same as 7.8.1 | Same as 7.8.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 7.8.1D.1 Wide band Intermodulation for ProSe Direct Discovery | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power + TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |
| 7.8.1D.2 Wide band Intermodulation for ProSe Direct Communication | Signal levels as defined in minimum requirements T-put limit = 95% of maximum for the Ref Meas channel | Same as 7.8.1D.1 | Same as 7.8.1D.1 |
| 7.8.1E Wide band intermodulation for UE category 0 | Same as 7.8.1 | Same as 7.8.1 | Same as 7.8.1 |
| 7.8.1EA Wide band Intermodulation for UE category M1 | Same as 7.8.1 | Same as 7.8.1 | Same as 7.8.1 |
| 7.8.1EB Wide band Intermodulation for UE category 1bis | Same as 7.8.1 | Same as 7.8.1 | Same as 7.8.1 |
| 7.8.1EC Wide band Intermodulation for UE category M2 | Same as 7.8.1 | Same as 7.8.1 | Same as 7.8.1 |
| 7.8.1F Wide band intermodulation for category NB1 | Wanted signal power: (REFSENS + 6 dB) CW Interferer power: -46 dBm E-UTRA Interferer power: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel | 0 dB | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |
| 7.8.1G.1 Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Wanted signal power: For 10MHz BW: (REFSENS + 6 dB) For 20 MHz BW: (REFSENS + 9 dB) CW Interferer power, all BWs: -46 dBm Modulated Interferer power:, all BWs: -46 dBm T-put limit = 95% of maximum for the Ref Meas channel Uplink power | 0 dB | Formula: Wanted signal power +TT CW Interferer signal power unchanged Modulated Interferer signal power unchanged T-put limit unchanged |
| 7.9 Spurious emissions | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz 12.75GHz f 19 GHz: -47dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |
| 7.9_1 Spurious emissions with 4 Rx antenna ports | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9A Spurious emissions for CA | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |
| 7.9E Spurious emissions for UE category 0 | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9EA Spurious emissions for UE category M1 | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9EB Spurious emissions for UE category 1bis | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9F Spurious emissions for category NB1 | Same as 7.9 | Same as 7.9 | Same as 7.9 |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 30MHz f < 1GHz: -57dBm / 100kHz 1GHz f 12.75 GHz: -47dBm / 1MHz 12.75GHz f 26 GHz: -47dBm / 1MHz | 0 dB | Formula: Minimum Requirement + TT |

## << End of changes >>