3GPP TSG-RAN5 Meeting#82	R5-191794

# Athens, Greece, 25th Feb. – 1st Mar. 2019

Title:							SNR estimation update for FR2 priority 1 and 2 TRx TCs

Source:			Anritsu

Agenda Item:			5.3.8.17

Document for:			Endorsement

# 1.	Introduction

# SNR estimation has been discussed for each test case in [1]-[6] for this 1 year, however it is not finalized yet. In RAN5 NR#4 AdHoc some companies provided SNR estimation but the it shows the different SNR level as shown ever in [1]-[3]. The way forward for SNR estimation assumptions is agreed in [7] to align the assumptions used for SNR estimation among each company. These documents provide the updated SNR based on the agreement in [7], and also give some views to reduce the issues came from maximum EIRP requirement +43dBm.

# 2.	Discussion

# 2.1 Estimated SNR

Table 1 shows the estimated SNR and the proposed ΔSNR and relaxation amount. SNR for MOP is updated compared to the previous document by re-considering the some system design HW/SW optimization, based on the some views that MOP needs to be measured without relaxation. Also, the SNR is estimiated depends on the maximum EIRP level. The realized SNR level is trade off of many aspects(HW/SW complexity, cost, test time, etc…) . The idea of mitigating such issues while keeping the SNR better is discussed in 2.2.

Table 1: SNR for Priority 1 and 2 Tx test cases

| Test case | Applicable Frequency Range [GHz] | UL signal level [dBm] | Estimated SNR for total component [dB] | Proposed ΔSNR ΔSNR=10log10(1+2/SNR), SNR in linear and defined for total component power | Proposed relaxation of test requirement |
| --- | --- | --- | --- | --- | --- |
| MOP (Min Peak EIRP, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 20.7dBm/ChBW (22.4 – 1.7) | 20.6+ ChBW | 0.1 | N/A |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 18.9dBm/ChBW (20.6 – 1.7) | 14.8+ ChBW | 0.28 | N/A |
| MOP (TRP, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 23dBm/ChBW | 10.6 + ChBW (EIRPMAX> 33dBm) 25.0+ ChBW (EIRPMAX≤ 33dBm) | 0.7 0.1 | N/A N/A |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 23dBm/ChBW | -1.2 + ChBW (EIRPMAX > 33dBm) 15.0+ ChBW (EIRPMAX ≤ 33dBm) | 0.41-0.98 0.27 | 10.2-14.2 - ChBW N/A |
| MOP (Spherical Coverage, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 11.5dBm/ChBW | -0.9 + ChBW (EIRPMAX > 33dBm) 13.5 + ChBW (EIRPMAX ≤ 33dBm) | 0.41-0.98 0.37 | 9.9-13.9 - ChBW N/A |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 8.0dBm/ChBW | -16.2 + ChBW (EIRPMAX > 33dBm) 9.3 + ChBW (EIRPMAX ≤ 33dBm) | 0.41-0.98 0.92 | 25.2-29.2 - ChBW N/A |
| Off power | 23.45 GHz ≤ f ≤ 32.125 GHz | -35dBm/ChBW | -15.0 + ChBW | 0.41 | 28.0 - ChBW |
|  | 32.125 GHz < f ≤ 40.8 GHz |  | -23.1 + ChBW | 0.41 | 36.1 - ChBW |
| Rx spurious | 6 GHz ≤ f ≤ 20 GHz | -47dBm/1MHz | 6.3 | 0.41 | 6.7 |
|  | 20 GHz < f ≤ 40 GHz |  | -3.2 | 0.41 | 16.2 |
|  | 40 GHz < f ≤ 80 GHz |  | -16.6 | 0.41 | 29.6 |
| Tx spurious | 6 GHz ≤ f ≤ 20 GHz | -13dBm/1MHz | ≥ 20 | 0.1 | N/A |
|  | 20GHz ≤ f ≤ 80 GHz |  | ≥ 13 | 0.41 | N/A |
| SEM | 23.45 GHz ≤ f ≤ 32.125 GHz | -13dBm/1MHz | 11.1(EIRPMAX > 33dBm) 20.8(EIRPMAX ≤ 33dBm) | 0.41 0.1 | 1.9 N/A |
|  | 32.125 GHz < f ≤ 40.8 GHz |  | 4.2(EIRPMAX > 33dBm) 22.9 (EIRPMAX ≤ 33dBm) | 0.41 0.1 | 8.8 N/A |
| ACLR | 23.45 GHz ≤ f ≤ 32.125 GHz | 23dBm(TRP)-17dBc=6dBm/ChBW | 4.0 + ChBW(EIRPMAX > 33dBm) 13.8 + ChBW (EIRPMAX ≤ 33dBm) | 0.41 0.35 | Depends on assumed TRP N/A |
|  | 32.125 GHz < f ≤ 40.8 GHz | 23dBm(TRP)-16dBc=7dBm/ChBW | -1.8 + ChBW(EIRPMAX > 33dBm) 16.8+ ChBW (EIRPMAX ≤ 33dBm) | 0.41 0.18 | Depends on assumed TRP N/A |
| NOTE ChBW = 10log10(400MHz/ChBW), 9.03dB for ChBW=50MHz. NOTE 2 : According to the agreement in [7], the SNR estimation is only valid between 23.45 and 30.3 GHz. SNR needs to be revisited once the new band and its core requirement is introduced between 30.3GHz and 32.125GHz. |  |  |  |  |  |

Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43 dBm – Pathloss tolerable (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

# All the SNR is provided based on the “single” test setup.

# Proposal 1 : Use the values in Table 1 to determination of MU and relaxation amount

# 2.2. Impact from Maximum Peak EIRP level

The challenging aspect for the mmWave test system for conformance testing is that TE needs to measure the wide range of UL power. Especially, the maximum EIRP level of +43dBm, which originated from regulatory requirement, cause the many negative consequences from conformance test system PoV as below :

- Complicated switch box (result in increased HW complexity and cost of the system)

- Increased MU(reduce the SNR) of other test case than maximum EIRP

- Increased measurement time(trade off relation with HW complexity)

The issue is that +43dBm EIRP is not expected in the real UEs. If the TRP is +23 dBm(this will not also likely though..), to realize 43dBm EIRP, 20dB gain is required, and such gain will require many number of array antenna e.g. 100 elements, and will not likely to be used at least for smart phone type UE . In fact, in the some discussion in RAN4 about performance test link budget, the antenna gain of 7~8 dB  is assumed as a typical UE antenna gain.  Hence, it is expected actual UE will not reach +43dBm, and at most +30dBm or so is expected.

Observation 1 : +43dBm maximum peak EIRP cause negative consequence though +43dBm EIRP is not likely to be achieved with the real UE

Hence, we propose to reduce the negative effects from unrealistic EIRP requirement, we propose followings.

Option 1 : Reduce the maximum EIRP +43 dBm test requirement to X dBm

Option 2 : UE declaration of upper limit of maximum EIRP

Option 1 will solve more issues than Option 2 among a), b) and c) above. Option 2 can still solve some issues and is beneficial.

Proposal 2 : Introduce the Option 1 or Option 2 to solve the issues originated from “+43dBm maximum Peak EIRP”.

# 4.	Conclusion

Proposal 1 : Use the values in Table 1 to determination of MU and relaxation amount

Observation 1 : +43dBm maximum peak EIRP cause negative consequence though +43dBm EIRP is not likely to be achieved with the real UE

- Complicated switch box (result in increased HW complexity and cost of the system)

- Increased MU(reduce the SNR) of other test case than maximum EIRP

- Increased measurement time(trade off relation with HW complexity)

Proposal 2 : Introduce the Option 1 or Option 2 to solve the issues originated from “+43dBm maximum Peak EIRP”.

Option 1 : Reduce the maximum EIRP +43 dBm test requirement to X dBm

Option 2 : UE declaration of upper limit of maximum EIRP

# 5.	References

[1] R5-185806, “Testability issue of low PSD test cases in FR2 (2)”, Anritsu, RAN5 NR#3 AdHoc

[2] R5-187273, “Consideration on MU and TT for FR2 low PSD tests”, Anritsu, RAN5#81,

[3] R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#82

[4] R5-190446, “On the power level to derive SNR for FR2 TRx tests”, Anritsu, RAN5 NR#4 AdHoc

[5] R5-190460, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5 NR#4 AdHoc

[6] R5-190601, “On relaxation of test requirements for Low and High PSD test cases”, Keysight, RAN5 NR#4 AdHoc

[7] R5-190980, “Way forward for SNR estimation assumptions”, Anritsu/Keysight/R&S, RAN5 NR#4 AdHoc