3GPP TSG-RAN WG5 Meeting # 82	R5-191882

# Athens, Greece,  25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0210 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Error Vector Magnitude for intra-band contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The test case for Error Vector Magnitude for intra-band contiguous EN-DC has been added in TS 38.101-3 and the 5GS_NR_LTE-UEConTest WP indicates that a new test case needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of TC 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case 6.4B.2.1.1 is incomplete and the WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4B.2.1(new), 6.4B.2.1.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a resubmission of the approved CR of RAN5#81 R5-187828 CR0070 with additional changes marked in yellow. - Test procedure is updated for 256 QAM. - Test tolerance is added in Test requirements |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.4B.2.1	Transmit Modulation Quality for intra-band contiguous EN-DC

6.4B.2.1.1	Error Vector Magnitude for intra-band contiguous EN-DC

Editor’s note: The following aspects are either missing or not yet determined:

- TP analysis is TBD.

- Message contents are incomplete.

6.4B.2.1.1.1	Test purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in sub-clause 6.4B.2.1.4.3. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and the duration of PUCCH/PUSCH channel, or one hop, if frequency hopping is enabled for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient as defined in subclause 6.3.3.3 of [8].

6.4B.2.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band EN-DC.

6.4B.2.1.1.3	Minimum conformance requirements

For the intra-band contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2 of [5] for the MCG and 6.4.2 of [2] for the SCG with EN-DC configured.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.4B.2.1.1.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

6.4B.2.1.1.4	Test description

6.4B.2.1.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 6.4B.2.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.4B.2.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  | Lowest, Highest (NOTE 4) |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  | Lowest, mid and highest SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation (NOTE 3) | RB allocation (NOTE 1) |
| 1 | N/A for EVM testing | QPSK | 0 RB | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | QPSK | 0 RB | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 |  | QPSK | 0 RB | DFT-s-OFDM QPSK | Inner Full |
| 4 |  | QPSK | 0 RB | DFT-s-OFDM QPSK | Outer Full |
| 5 |  | QPSK | 0 RB | DFT-s-OFDM 16 QAM | Inner Full |
| 6 |  | QPSK | 0 RB | DFT-s-OFDM 16 QAM | Outer Full |
| 7 |  | QPSK | 0 RB | DFT-s-OFDM 64 QAM | Outer Full |
| 8 |  | QPSK | 0 RB | DFT-s-OFDM 256 QAM | Outer Full |
| 9 |  | QPSK | 0 RB | CP-OFDM PI/2 BPSK | Inner Full |
| 10 |  | QPSK | 0 RB | CP-OFDM PI/2 BPSK | Outer Full |
| 11 |  | QPSK | 0 RB | CP-OFDM QPSK | Inner Full |
| 12 |  | QPSK | 0 RB | CP-OFDM QPSK | Outer Full |
| 13 |  | QPSK | 0 RB | CP-OFDM 16 QAM | Inner Full |
| 14 |  | QPSK | 0 RB | CP-OFDM 16 QAM | Outer Full |
| 15 |  | QPSK | 0 RB | CP-OFDM 64 QAM | Outer Full |
| 16 |  | QPSK | 0 RB | CP-OFDM 256 QAM | Outer Full |
| 17 |  | QPSK | Outer Full | DFT-s-OFDM PI/2 BPSK | 0 RB |
| 18 |  | QPSK | Outer_1RB_Left | DFT-s-OFDM PI/2 BPSK | 0 RB |
| 19 |  | QPSK | Outer_1RB_Right | DFT-s-OFDM PI/2 BPSK | 0 RB |
| 20 |  | 16QAM | Outer Full | DFT-s-OFDM PI/2 BPSK | 0 RB |
| 21 |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM PI/2 BPSK | 0 RB |
| 22 |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM PI/2 BPSK | 0 RB |
| NOTE 1: The specific configuration of each RB allocation is defined in TS 38.521-1 [8] Table 6.1-1 Common UL configuration NOTE 2: Outer Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested - Highest ENBW: NR component with highest NRB is tested. NOTE 5: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  |  |  |  |

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.4B.2.1.1.4.3.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4B.2.1.1.4.2		Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4B.2.1.1.4.1-1 on E-UTRA CC and NR CC respectively. Since the UE has no payload and no loopback data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE for NR and E-UTRA carrier until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

4.	For modulations except 256 QAM, send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz on E-UTRA CC and Pmin + PW ± PW, where Pmin is the minimum output power according to TS 38.521-1 [8] Table 6.3.1.3-1 and PW is the power window according to Table 6.4B.2.1.1.4.2-1 for the carrier frequency f and the channel bandwidth BW on NR CC. 
For 256 QAM, send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz on E-UTRA CC and Pmin + 10dB + PW ± PW, where Pmin is the minimum output power according to TS 38.521-1 [8] Table 6.3.1.3-1 and PW is the power window according to Table 6.4B.2.1.1.4.2-1 for the carrier frequency f and the channel bandwidth BW on NR CC.

5.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

NOTE1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.4B.2.1.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-89 PUSCH-Config without CP-OFDM condition. When switching to CP-OFDM waveform, send an NR RRCReconfiguration message with CP-OFDM condition.

Table 6.4B.2.1.1.4.2-1: Power Window (dB) for EVM PUSCH

|  | f ≤ 3GHz | 3GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | [1.7] | [2.0] | [2.2] |
| 40MHz < BW ≤ 100MHz | [2.1] | [2.3] | [2.5] |

6.4B.2.1.1.4.3		Message contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.

6.4B.2.1.1.5	Test requirements

The PUSCH EVM, derived in TS 38.521-1 [8] Annex E.4.2, shall not exceed the values in Table 6.4B.2.1.1.5-1.

The PUSCH, derived in TS 38.521-1 [8] Annex E.4.6.2, shall not exceed the values in Table 6.4B.2.1.1.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4B.2.1.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 + TT |
| QPSK | % | 17.5 + TT |
| 16 QAM | % | 12.5 + TT |
| 64 QAM | % | 8 + TT |
| 256 QAM | % | 3.5 + TT |
| Note 1: TT is defined in Table 6.4B.2.1.1.5-2. |  |  |

Table 6.4B.2.1.1.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 0 |
| QPSK | % | 0 |
| 16 QAM | % | 0 |
| 64 QAM | % | 0 |
| 256 QAM | % | 0.3 for 15 dBm < PUL 0.8 for -25 dBm < PUL≤ 15 dBm 1.1 for -40dBm ≤ PUL ≤ -25dBm |

<< End of changes >>