3GPP TSG-RAN WG4 Meeting #82

Athens, GR, 25 Feb - 1 Mar, 2019

Source:        Samsung

Title:           TP analysis on FR1 6.5A.2.4.1.1 NR ACLR for CA (2UL CA)

# Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC NR ACLR, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for NR ACLR of 2UL inter-band CA case.

# Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined for QPSK and 16QAM as table below.

Table 6.6.2.3A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 6 | 25 | N/A for this test | QPSK | QPSK | 13 | 5 | 8 |
| 2 | 6 | 25 |  | QPSK | QPSK | 31 | 6 | 25 |
| 3 | 6 | 25 |  | 16QAM | 16QAM | 13 | 5 | 8 |
| 4 | 6 | 25 |  | 16QAM | 16QAM | 31 | 6 | 25 |
| 5 | 6 | 25 |  | QPSK | 16QAM | 30 | 5 | 25 |
| 6 | 6 | 50 |  | QPSK | QPSK | 17 | 5 | 12 |
| 7 | 6 | 50 |  | QPSK | QPSK | 56 | 6 | 50 |
| 8 | 6 | 50 |  | 16QAM | 16QAM | 17 | 5 | 12 |
| 9 | 6 | 50 |  | 16QAM | 16QAM | 56 | 6 | 50 |
| 10 | 6 | 50 |  | QPSK | 16QAM | 55 | 5 | 50 |
| 11 | 25 | 15 |  | QPSK | QPSK | 12 | 8 | 4 |
| 12 | 25 | 15 |  | QPSK | QPSK | 40 | 25 | 15 |
| 13 | 25 | 15 |  | 16QAM | 16QAM | 12 | 8 | 4 |
| 14 | 25 | 15 |  | 16QAM | 16QAM | 40 | 25 | 15 |
| 15 | 25 | 15 |  | QPSK | 16QAM | 23 | 8 | 15 |
| 16 | 25 | 25 |  | QPSK | QPSK | 16 | 8 | 8 |
| 17 | 25 | 25 |  | QPSK | QPSK | 50 | 25 | 25 |
| 18 | 25 | 25 |  | 16QAM | 16QAM | 16 | 8 | 8 |
| 19 | 25 | 25 |  | 16QAM | 16QAM | 50 | 25 | 25 |
| 20 | 25 | 25 |  | QPSK | 16QAM | 33 | 8 | 25 |
| 21 | 25 | 50 |  | QPSK | QPSK | 16 | 4 | 12 |
| 22 | 25 | 50 |  | QPSK | QPSK | 65 | 15 | 50 |
| 23 | 25 | 50 |  | 16QAM | 16QAM | 16 | 4 | 12 |
| 24 | 25 | 50 |  | 16QAM | 16QAM | 65 | 15 | 50 |
| 25 | 25 | 50 |  | QPSK | 16QAM | 54 | 4 | 50 |
| 26 | 50 | 25 |  | QPSK | QPSK | 20 | 12 | 8 |
| 27 | 50 | 25 |  | QPSK | QPSK | 75 | 50 | 25 |
| 28 | 50 | 25 |  | 16QAM | 16QAM | 20 | 12 | 8 |
| 29 | 50 | 25 |  | 16QAM | 16QAM | 75 | 50 | 25 |
| 30 | 50 | 25 |  | QPSK | 16QAM | 37 | 12 | 25 |
| 31 | 50 | 50 |  | QPSK | QPSK | 24 | 12 | 12 |
| 32 | 50 | 50 |  | QPSK | QPSK | 100 | 50 | 50 |
| 33 | 50 | 50 |  | 16QAM | 16QAM | 24 | 12 | 12 |
| 34 | 50 | 50 |  | 16QAM | 16QAM | 100 | 50 | 50 |
| 35 | 50 | 50 |  | QPSK | 16QAM | 62 | 12 | 50 |
| 36 | 50 | 100 |  | QPSK | QPSK | 30 | 12 | 18 |
| 37 | 50 | 100 |  | QPSK | QPSK | 150 | 50 | 100 |
| 38 | 50 | 100 |  | 16QAM | 16QAM | 30 | 12 | 18 |
| 39 | 50 | 100 |  | 16QAM | 16QAM | 150 | 50 | 100 |
| 40 | 50 | 100 |  | QPSK | 16QAM | 112 | 12 | 100 |
| 41 | 75 | 75 |  | QPSK | QPSK | 32 | 16 | 16 |
| 42 | 75 | 75 |  | QPSK | QPSK | 150 | 75 | 75 |
| 43 | 75 | 75 |  | 16QAM | 16QAM | 32 | 16 | 16 |
| 44 | 75 | 75 |  | 16QAM | 16QAM | 150 | 75 | 75 |
| 45 | 75 | 75 |  | QPSK | 16QAM | 91 | 16 | 75 |
| 46 | 100 | 50 |  | QPSK | QPSK | 30 | 18 | 12 |
| 47 | 100 | 50 |  | QPSK | QPSK | 150 | 100 | 50 |
| 48 | 100 | 50 |  | 16QAM | 16QAM | 30 | 18 | 12 |
| 49 | 100 | 50 |  | 16QAM | 16QAM | 150 | 100 | 50 |
| 50 | 100 | 50 |  | QPSK | 16QAM | 68 | 18 | 50 |
| 51 | 100 | 75 |  | QPSK | QPSK | 34 | 18 | 16 |
| 52 | 100 | 75 |  | QPSK | QPSK | 175 | 100 | 75 |
| 53 | 100 | 75 |  | 16QAM | 16QAM | 34 | 18 | 16 |
| 54 | 100 | 75 |  | 16QAM | 16QAM | 175 | 100 | 75 |
| 55 | 100 | 75 |  | QPSK | 16QAM | 93 | 18 | 75 |
| 56 | 100 | 100 |  | QPSK | QPSK | 36 | 18 | 18 |
| 57 | 100 | 100 |  | QPSK | QPSK | 200 | 100 | 100 |
| 58 | 100 | 100 |  | 16QAM | 16QAM | 36 | 18 | 18 |
| 59 | 100 | 100 |  | 16QAM | 16QAM | 200 | 100 | 100 |
| 60 | 100 | 100 |  | QPSK | 16QAM | 118 | 18 | 100 |
| 61 | 15 | 25 |  | QPSK | QPSK | 12 | 4 | 8 |
| 62 | 15 | 25 |  | QPSK | QPSK | 40 | 15 | 25 |
| 63 | 15 | 25 |  | 16QAM | 16QAM | 12 | 4 | 8 |
| 64 | 15 | 25 |  | 16QAM | 16QAM | 40 | 15 | 25 |
| 65 | 15 | 25 |  | QPSK | 16QAM | 40 | 15 | 25 |
| 66 | 25 | 6 |  | QPSK | QPSK | 13 | 8 | 5 |
| 67 | 25 | 6 |  | QPSK | QPSK | 31 | 25 | 6 |
| 68 | 25 | 6 |  | 16QAM | 16QAM | 13 | 8 | 5 |
| 69 | 25 | 6 |  | 16QAM | 16QAM | 31 | 25 | 6 |
| 70 | 25 | 6 |  | QPSK | 16QAM | 30 | 25 | 5 |
| 71 | 25 | 75 |  | QPSK | QPSK | 24 | 8 | 16 |
| 72 | 25 | 75 |  | QPSK | QPSK | 100 | 25 | 75 |
| 73 | 25 | 75 |  | 16QAM | 16QAM | 24 | 8 | 16 |
| 74 | 25 | 75 |  | 16QAM | 16QAM | 100 | 25 | 75 |
| 75 | 25 | 75 |  | QPSK | 16QAM | 100 | 25 | 75 |
| X+1 | 75 | 25 |  | QPSK | QPSK | 24 | 16 | 8 |
| X+2 | 75 | 25 |  | QPSK | QPSK | 100 | 75 | 25 |
| X+3 | 75 | 25 |  | 16QAM | 16QAM | 24 | 16 | 8 |
| X+4 | 75 | 25 |  | 16QAM | 16QAM | 100 | 75 | 25 |
| X+5 | 75 | 25 |  | QPSK | 16QAM | 41 | 16 | 25 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for ACLR for single carrier mode is defined as table below.

Table 6.5.2.4.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 23 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 33 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 43 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 5 | Default |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 19 | Default |  |  |  | CP-OFDM QPSK | Inner_Full |
| 20 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 23 | Default |  |  |  | CP-OFDM 16 QAM | Inner_Full |
| 24 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 27 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 30 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For Power Class 3 testing, include two steps for UE operating in bands n40, n77, n78 and n79, with IE [P-Boost-BPSK] set to 1 and 0 separately. |  |  |  |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

## Test Environment

In LTE, E-UTRA ACLR for CA is tested under NC, TL/VL, TL/VH, TH/VL, TH/VH. According to baseline endorsed in [1], it’s proposed to reuse NC, TL/VL, TL/VH, TH/VL, TH/VH for NR ACLR of inter-band UL CA.

Proposal 1: Define test environment as NC, TL/VL, TL/VH, TH/VL, TH/VH for NR ACLR in FR1 for UL inter-band CA.

## Test Subcarrier Spacing

In [3] the smallest and highest SCS are selected for Single Carrier ACLR verification. It is proposed to reuse the same way for inter-band UL CA.

Proposal 2: Select smallest and highest SCS for ACLR in FR1 for inter-band UL CA.

## Test Frequencies

In LTE, NR ACLR for Inter-band CA is tested for both Low range and High range. The same test coverage as for LTE would be enough to ensure that the ACLR obeys the test requirement at low range and high range.

Proposal 3: Test ACLR for Low range and High range frequencies in FR1 for inter-band UL CA.

## Test Channel Bandwidth

In LTE, ACLR for inter-band CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for ACLR test for inter-band UL CA.

Proposal 4: Test ACLR for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

## Test Modulations and waveforms

For single carrier TP analysis presented in [3] both DFT-s-OFDM and CP-OFDM with all modulation permutation are selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test ACLR for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

## RB allocation

Considering RB allocation defined in LTE CA and NR SC mode, it is proposed to consider RB allocation as below:

Proposal 6: Test ACLR for CA as RB allocation as table 1 .

Table 1: PRB allocation

| Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
| --- | --- | --- |
|  | PCC | SCC |
| DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM PI/2 BPSK | Outer_Full | Outer_Full |
| DFT-s-OFDM PI/2 BPSK | Inner_Full | Inner_Full |
| DFT-s-OFDM QPSK | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM QPSK | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM QPSK | Outer_Full | Outer_Full |
| DFT-s-OFDM QPSK | Inner_Full | Inner_Full |
| DFT-s-OFDM 16 QAM | Inner_Full | Inner_Full |
| DFT-s-OFDM 16 QAM | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM 16 QAM | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM 16 QAM | Outer_Full | Outer_Full |
| DFT-s-OFDM 64 QAM | Outer_1RB_Left | Outer_1RB_Left |
| DFT-s-OFDM 64 QAM | Outer_1RB_Right | Outer_1RB_Right |
| DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| CP-OFDM QPSK | Inner_Full | Inner_Full |
| CP-OFDM QPSK | Outer_1RB_Left | Outer_1RB_Left |
| CP-OFDM QPSK | Outer_1RB_Right | Outer_1RB_Right |
| CP-OFDM QPSK | Outer_Full | Outer_Full |
| CP-OFDM 16 QAM | Inner_Full | Inner_Full |
| CP-OFDM 16 QAM | Outer_1RB_Left | Outer_1RB_Left |
| CP-OFDM 16 QAM | Outer_1RB_Right | Outer_1RB_Right |
| CP-OFDM 16 QAM | Outer_Full | Outer_Full |
| CP-OFDM 64 QAM | Outer_1RB_Left | Outer_1RB_Left |
| CP-OFDM 64 QAM | Outer_1RB_Right | Outer_1RB_Right |
| CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| CP-OFDM 256 QAM | Outer_Full | Outer_Full |

## Number of test points for CA ACLR

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 for Outer/Inner allocation 1 for Edge allocation | 14 for Outer/Inner allocation 14 for Edge allocation | 840 |

# Conclusion

For inter-band UL CA NR ACLR requirement, the test condition is proposed as below:

Proposal 1: Define test environment as NC, TL/VL, TL/VH, TH/VL, TH/VH for NR ACLR in FR1 for UL inter-band CA.

Proposal 2: Select smallest and highest SCS for ACLR in FR1 for inter-band UL CA.

Proposal 3: Test ACLR for Low range and High range frequencies in FR1 for inter-band UL CA.

Proposal 4: Test ACLR for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

Proposal 5: Test ACLR for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

Proposal 6: Test ACLR for CA as RB allocation as table 1 .

# Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-180887, "Discussion on test point selection for NR ACLR in FR1"

[4] 3GPP TS 36.521-1 v15.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).