3GPP TSG RAN WG5 #82	R5-192194

# Athens, Greece, 25 Feb 2019 - 1 Mar, 2019

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.521-4 | CR | N/A | rev |  | Current version: | 0.3.0 |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for addition of 2Rx TDD FR1 TypeA and CSI-RS overlapped TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM CDMA TECHNOLOGIES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | N/A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 38.521-4 WP requires addition of 2Rx TDD FR1 TypeA PDSCH with CSI-RS overlapped performance |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Test Case for addition of 2Rx TDD FR1 TypeA PDSCH with CSI-RS overlapped performance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 38.521-4 WP cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.2.2.1	2Rx TDD FR1 PDSCH mapping Type A performance

<< Unchanged Section >>

<< Start of changes >>

##### 5.2.2.2.2 	2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance

###### 5.2.2.2.2_1		2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA

###### 5.2.2.2.2_1.1	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.2_1.1-3, with the addition of test parameters in table 5.2.2.2.2_1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.2_1.1-1.

Table 5.2.2.2.2_1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| [Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH] | [1-1] |

Table 5.2.2.2.2_1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 40 |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
|  | Subcarrier spacing | kHz | 30 |
| PDCCH configuration | Number of PRBs in CORESET | PRBs | 102 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 |
| NZP CSI-RS for CSI acquisition | OFDM symbols in the PRB used for CSI-RS |  | l0 = 13 |
|  | CSI-RS periodicity |  | 5 |
| ZP CSI-RS for CSI acquisition | Subcarrier index in the PRB used for CSI-RS |  | (k0, k1, k2, k3)=(2, 4, 6, 8) |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CSI-RS periodicity |  | 5 |
| Number of HARQ Processes |  |  | 8 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | Specific to each UL-DL pattern |

Table 5.2.2.2.2_1.1-3: Minimum performance for Rank 2

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-7.1 TDD | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 | TBD |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.2.2.1.2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

-SNR requirements is TBD

-Annex for measurement uncertainty and test tolerance is TBD

-Connection diagram is TBD

-Propagation condition description in Annex is TBD

-Minimum test time is TBD

-Message contents are TBD

5.2.2.2.2_1.2	Test Purpose

[Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and CSI-RS overlapped with PDSCH]

5.2.2.2.2_1.3	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.2.2_1.4	Test Description

5.2.2.2.2_1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [5] clause 5.1.2.1 and 5.1.2.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [5] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure TBD for TE diagram and section TBD for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 5.2-1 and Table 5.2.2.2.2_1.1-2 and as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions for the NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA or (EN-DC, DC bearer MCG and SCG, Connected without release On) for NSA according to TS 38.508-1 [5] clause 4.5. Message content are defined in clause 5.2.2.2.2_1.4.3.

5.2.2.2.2_1.4.2	Test Procedure

1.	SS transmits PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to Table 5.2.2.2.2_1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to 5.2.2.2.2_1.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.2.2_1.1-2, 5.2.2.2.2_1.1-3 and 5.2-1 as appropriate.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause TBD. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Tables TBD and TBD in Annex TBD clause TBD.

5.2.2.2.2_1.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1

5.2.2.2.2_1.4.3_1	Message exceptions for SA

Table xx: message exceptions for SA

FFS

5.2.2.2.2_1.4.3_2	Message exceptions for NSA

Table yy: message exceptions for NSA

FFS

5.2.2.2.2_1.5	Test Requirements

Table 5.2.2.2.2_1.1-2 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A TBD for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.2_1.1-3 for the specified SNR including test tolerances for all throughput tests

Table 5.2.2.2.2_1.5-1: Test requirement for Rank 2

| Test num. | Reference channel | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-7.1 TDD | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 | TBD + TT |