3GPP TSG-RAN5 Meeting #84	R5-195533

# Ljubljana, Slovenia,  26th Aug – 30th Aug

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4872 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Test Parameter on Table 7.3A.2.4.1-1 for 5B BW SET 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dekra Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2019-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test parameters 15 25 QPSK 15+25 QPSK 15 P_15@0 & 25 15 QPSK 25+15 QPSK 25 P_25@0 are missed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Test Parameters 15 25 QPSK 15+25 QPSK 15 P_15@0 & 25 15 QPSK 25+15 QPSK 25 P_25@0 on Table 7.3A.2.4.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 5B BW SET 1 wont be available |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

### 7.3A.2	Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA)

#### 7.3A.2.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA. This test case applies only for CA Configurations that do not support UL CA.

#### 7.3A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.2.4	Test description

##### 7.3A.2.4.1	Initial conditions

Same as in clause 7.3A.1.4.1 with the following exceptions:

-	Instead of Table 7.3A.1.4.1-1 use Table 7.3A.2.4.1-1.

Table 7.3A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 7.1-2 |  |  |  | B: Low range, High range C: Low range, High range (Note 4) |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg, Highest NRB_agg (Notes 3, 4) |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | CC MOD | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | QPSK | 6+25 | QPSK | 6 | P_6@0 | - | - | - |
| 15 | 25 | QPSK | 15+25 | QPSK | 15 | P_15@0 | - | - | - |
| 15 | 50 | QPSK | 15+50 | QPSK | 15 | P_15@0 | - | - | - |
| 25 | 15 | QPSK | 25+15 | QPSK | 25 | P_25@0 | - | - | - |
| 25 | 25 | QPSK | 25+25 | QPSK | 20 | P_20@0 | - | - | - |
| 25 | 25 | QPSK | 25+25 | QPSK | 25 | P_25@0 | - | - | - |
| 25 | 50 | QPSK | 25+50 | QPSK | 20 | P_20@0 | - | - | - |
| 25 | 100 | QPSK | 25+100 | QPSK | 25 | P_25@0 | - | - | - |
| 50 | 25 | QPSK | 50+25 | QPSK | 25 | P_25@25 | - | - | - |
| 50 | 50 | QPSK | 50+50 | QPSK | 25 | P_25@25 | - | - | - |
| 50 | 50 | QPSK | 50+50 | QPSK | 50 | P_50@0 | - | - | - |
| 50 | 75 | QPSK | 50+75 | QPSK | 50 | P_50@0 | - | - | - |
| 75 | 25 | QPSK | 75+25 | QPSK | 75 | P_75@0 | - | - | - |
| 75 | 50 | QPSK | 75+50 | QPSK | 75 | P_75@0 | - | - | - |
| 75 | 75 | QPSK | 75+75 | QPSK | 75 | P_75@0 | - | - | - |
| 100 | 25 | QPSK | 100+25 | QPSK | 50 | P_50@50 | - | - | - |
| 100 | 25 | QPSK | 100+25 | QPSK | 100 | P_100@0 | - | - | - |
| 100 | 50 | QPSK | 100+50 | QPSK | 75 | P_75@25 | - | - | - |
| 100 | 50 | QPSK | 100+50 | QPSK | 100 | P_100@0 | - | - | - |
| 100 | 75 | QPSK | 100+75 | QPSK | 100 | P_100@0 | - | - | - |
| 100 | 100 | QPSK | 100+100 | QPSK | 50 | P_50@50 | - | - | - |
| 100 | 100 | QPSK | 100+100 | QPSK | 75 | P_75@25 | - | - | - |
| 100 | 100 | QPSK | 100+100 | QPSK | 100 | P_100@0 | - | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3.3-2 for UE supporting one uplink carrier is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For CA_70C all test frequency IDs shall be tested according to table 4.3.1.1.70A-1, in TS 36.508 [7]. |  |  |  |  |  |  |  |  |  |

<< End of changes>>