3GPP TSG-RAN5 Meeting #84	R5-195539

# Ljubljana, Slovenia,  26th Aug – 30th Aug

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4878 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | adding points 25 25 & 75 25 that are missing on Table 6.5.2A.3.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dekra Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | adding points 25 25 & 75 25 that are missing on Table 6.5.2A.3.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |
| Summary of change: |  | adding points 25 25 & 75 25 that are missing on Table 6.5.2A.3.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2DL/2UL: DL-CA_66B / UL-CA_66B would not be available in clause 6.5.2A.3.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

##### 6.5.2A.3.1	In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA)

6.5.2A.3.1.1	Test Purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks.

For an allocated component carrier, the in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one SC-FDMA symbol, accordingly.

For a non allocated component carrier a spectral measurement is specified.

6.5.2A.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.5.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.1.4	Test description

6.5.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2, CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB | SCC NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 Note 2 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 Note 3 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 3 Note 2 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 4 Note 3 | 75 | 25 |  | QPSK | 8 | P_8@67 | S_0@0 | - | - |
| 5 Note 2 | 75 | 75 | N.A. | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 6 Note 3 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 7 Note 2 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 8 Note 3 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 9 Note 2 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 10 Note 3 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 11 Note 2 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 12 Note 3 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 13 Note 2 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 14 Note 3 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 15 Note 2 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 16 Note 3 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 17 Note 2 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 18 Note 3 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 19 Note 2 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 20 Note 3 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 21 Note 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 22 Note 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 23 Note 2 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 24 Note 3 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 Note 2 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 26 Note 3 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Applicable only for PCC-SCC: CC1-CC2. Note 3: Applicable only for PCC-SCC: CC2-CC1. Note 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

<< End of changes>>