3GPP TSG-RAN5 Meeting #84	R5-195763

# Ljubljana, Slovenia,  26th Aug – 30th Aug

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4920 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding points 25 25 & 75 25 on Table 6.3.5A.2.1.4.1-1: Test Configuration Table for UL-CA_66B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dekra Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding points 25 25 & 75 25 on Table 6.3.5A.2.1.4.1-1: Test Configuration Table for UL-CA_66B |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding points 25 25 & 75 25 on Table 6.3.5A.2.1.4.1-1: Test Configuration Table for UL-CA_66B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2DL/2UL: DL-CA_66B / UL-CA_66B would not be available in clause 6.3.5A.2.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

##### 6.3.5A.2.1	Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA)

6.3.5A.2.1.1	Test purpose

To verify the ability of the UE transmitter to change the output power in both assigned component carrier in the uplink with a defined power step sizes between sub-frames on the two respective component carrier.

6.3.5A.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.3.5A.2.1.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 6.3.5A.2.0.

6.3.5A.2.1.4	Test description

6.3.5A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.5A.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] clause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] clause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (Note 3) (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 6 13 2 16 | P_5@20 P_5@20 P_1@24 P_8@17 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 25 | 50 | N/A for this test | QPSK | 6 13 2 16 | P_5@20 P_5@20 P_1@24 P_8@17 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 50 | 50 |  | QPSK | 6 13 2 16 | P_5@45 P_5@45 P_1@49 P_8@42 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 75 | 25 |  | QPSK | 6 13 2 16 | P_5@70 P_5@70 P_1@74 P_8@67 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 75 | 75 |  | QPSK | 6 13 2 16 | P_5@70 P_5@70 P_1@74 P_8@67 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 25 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 50 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 75 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 100 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. Note 3: The UL allocation is changed as part of the test procedure. The Test Configuration Table entries list the combinations used, with the sequence of usage as determined by the test procedure for each sub-test. |  |  |  |  |  |  |  |  |

<< End of changes>>