3GPP TSG-RAN5 Meeting #84	R5-195771

# Ljubljana, Slovenia,  26th Aug – 30th Aug

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4928 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding points 25 25, & 75 25 that are missing on Table 7.4A.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dekra Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2019-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding points 25 25, & 75 25 that are missing on Table 7.4A.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding points 25 25, & 75 25 that are missing on Table 7.4A.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2DL/2UL: DL-CA_66B / UL-CA_66B would not be available in clause 7.4A.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

#### 7.4A.1.4	Test description

##### 7.4A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 7.4A.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS36.508 [7] clause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 7.1-2 |  |  |  | Mid range |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 3) |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  | UL Allocation |  |  |  |
| PCC NRB | SCC NRB | CC MOD | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| 25 | 25 | 64-QAM | 25+25 | QPSK | 50 | P_25@0 | S_25@0 |
| 50 | 25 | 64-QAM | 50+25 | QPSK | 25 | P_25@25 | S_0@0 |
| 50 | 50 | 64-QAM | 50+50 | QPSK | 25 | P_25@25 | S_0@0 |
| 50 | 100 | 64-QAM | 50+100 | QPSK | 150 | P_50@0 | S_100@0 |
| 75 | 25 | 64-QAM | 75+25 | QPSK | 100 | P_75@0 | S_25@0 |
| 75 | 50 | 64-QAM | 75+50 | QPSK | 75 | P_75@0 | S_0@0 |
| 75 | 75 | 64-QAM | 75+75 | QPSK | 75 | P_75@0 | S_0@0 |
| 75 | 75 | 64-QAM | 75+75 | QPSK | 129 | P_75@0 | S_54@0 |
| 75 | 75 | 64-QAM | 75+75 | QPSK | 150 | P_75@0 | S_75@0 |
| 100 | 25 | 64QAM | 100+25 | QPSK | 50 | P_50@0 | S_0@0 |
| 100 | 25 | 64-QAM | 100+25 | QPSK | 125 | P_100@0 | S_25@0 |
| 100 | 50 | 64-QAM | 100+50 | QPSK | 75 | P_75@25 | S_0@0 |
| 100 | 50 | 64-QAM | 100+50 | QPSK | 150 | P_100@0 | S_50@0 |
| 100 | 75 | 64-QAM | 100+75 | QPSK | 175 | P_100@0 | S_75@0 |
| 100 | 100 | 64-QAM | 100+100 | QPSK | 50 | P_50@50 | S_0@0 |
| 100 | 100 | 64-QAM | 100+100 | QPSK | 75 | P_75@25 | S_0@0 |
| 100 | 100 | 64-QAM | 100+100 | QPSK | 130 | P_100@0 | S_30@0 |
| 100 | 100 | 64-QAM | 100+100 | QPSK | 200 | P_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA Configuration only the appropriate Uplink RB allocation value according to table 7.3A.0-1 is tested per CA Configuration Test CC combination. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate if the UE supports 2 Rx antennas or Figure A.80 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1

4.	The UL and DL Reference Measurement channels are set according to Table 7.4A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.4A.1.4.3.

<< End of changes>>