3GPP TSG-RAN5 Meeting #84	R5-195823

# Ljubljana, Slovenija,  26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0180 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Introduction of TC 6.2A.1.2.1 UE Maximum output power Spherical coverage (2UL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TC6.2A.1.2.1 UE Maximum output power – Spherical coverage (2UL CA_ is intrduced in WP, so need to add in test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of UE Maximum output power Spherical Coverage (2UL CA). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC6.2A.1.2 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.2 (new), F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.2A.1.2	UE maximum output power - Spherical coverage

6.2A.1.2.1	Spherical coverage for CA (2UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Test Procedure is incompleted due to duration for UE to find the best beam to use is TBD.

6.2A.1.2.1.1	Test purpose

To verify that the spatial coverage of the UE for CA in expected directions is acceptable.

6.2A.1.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.2A.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.0.

6.2A.1.2.1.4	Test description

6.2A.1.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.2A.1.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.1.2.1.4.1-1: Intra-band Contiguous CA Test Configuration Table (single CC requirement)

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH (NOTE 2) |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCC are mapped onto physical frequencies according to Table 6.1-2 |  |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC | BAND | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | nX | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC | nX | 50 |  |  | - | - |
| 2 | PCC | nX | 50 |  |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC | nX | 100 |  |  | - | - |
| 3 | PCC | nX | 50 |  |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC | nX | 200 |  |  | - | - |
| 4 | PCC | nX | 100 |  |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC | nX | 100 |  |  | - | - |
| 5 | PCC | nX | 200 |  |  | DFT-s-OFDM QPSK | Inner Full |
|  | SCC | nX | 200 |  |  | - | - |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. |  |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2A.1.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.1.2.1.4.3

6.2A.1.2.1.4.2	Test procedure

1.	Configure PCC and SCC according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [10] clause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in clause 6.2A.1.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2A.1.2.1.4.3.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec to ensure that the UE transmits at its maximum output power.

6.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (Note 1) for the UE Tx beam selection to complete.

7.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration. Repeat EIRP measurements for all directions in the sphere according to EIRP measurement procedure defined in Annex K.1.3. After a rotation, allow TBD ms for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

8.	Calculate a cumulative distribution function for the measured EIRP values.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2A.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.2.1.5	Test requirement

The defined %-tile EIRP in measurement distribution derived in step 8 shall exceed the values specified in Table 6.2A.1.2.1.5-1 to Table 6.2A.1.2.1.5-4.

Table 6.2A.1.2.1.5-1: Intra-band Contiguous CA UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| CA_n257D | 32.0-TT |
| CA_n257G | 32.0-TT |
| CA_n260D | 30.0-TT |
| CA_n260G | 30.0-TT |
| CA_n260O | 30.0-TT |
| CA_n261D | 32.0-TT |
| CA_n261G | 32.0-TT |
| CA_n261O | 32.0-TT |

Table 6.2A.1.2.1.5-2: Intra-band Contiguous CA UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| CA_n257D | 18.0-TT |
| CA_n257G | 18.0-TT |
| CA_n261D | 18.0-TT |
| CA_n261G | 18.0-TT |
| CA_n261O | 18.0-TT |

Table 6.2A.1.2.1.5-3: Intra-band Contiguous CA UE spherical coverage for power class 3 for single band UE or multiband UE declaring MBs = 0 in all FR2 bands

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| CA_n257D | 11.5-TT |
| CA_n257G | 11.5-TT |
| CA_n260D | 8-TT |
| CA_n260G | 8-TT |
| CA_n260O | 8-TT |
| CA_n261D | 11.5-TT |
| CA_n261G | 11.5-TT |
| CA_n261O | 11.5-TT |

Table 6.2A.1.2.1.5-3a: UE spherical coverage for power class 3 for multi band UE declaring MBs>0 in any FR2 band

| ID | Supported FR2 bands set | Test requirement (dB) (Note 1) |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 3) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CA_n257D/G | CA_n258 | CA_n260D/G/O | CA_n261D/G/O |  |  |
| 1 | n257, n258 | 11.5-TT-MBs |  |  |  | 1.25 |  |
| 2 | n257, n260 | 11.5-TT-MBs |  | 8-TT-MBs |  | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 3 | n258, n260 |  |  | 8-TT-MBs |  | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 4 | n258, n261 |  |  |  | 11.5-TT-MBs | 1.25 |  |
| 5 | n260, n261 |  |  | 8-TT-MBs | 11.5-TT-MBs | 0.75 | No relaxation allowed for n260 |
| 6 | n257, n258, n260 | 11.5-TT-MBs |  | 8-TT-MBs |  | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 7 | n257, n258, n261 | 11.5-TT-MBs |  |  | 11.5-TT-MBs | 1.75 |  |
| 8 | n257, n260, n261 | 11.5-TT-MBs |  | 8-TT-MBs | 11.5-TT-MBs | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 9 | n258, n260, n261 |  |  | 8-TT-MBs | 11.5-TT-MBs | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 10 | n257, n258, n260, n261 | 11.5-TT-MBs |  | 8-TT-MBs | 11.5-TT-MBs | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| Note 1: MBs is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-3 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant Note 3: Max allowed sum of MBs over all supported FR2 bands as defined in clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |

Table 6.2A.1.2.1.5-4: Intra-band Contiguous CA UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| CA_n257D | 25-TT |
| CA_n257G | 25-TT |
| CA_n260D | 19-TT |
| CA_n260G | 19-TT |
| CA_n260O | 19-TT |
| CA_n261D | 25-TT |
| CA_n261G | 25-TT |
| CA_n261O | 25-TT |

## << Unchanged sections omitted >>

F.1.2	Measurement of transmitter

Editor’s note: The measurement uncertainties for 6.5.2.1 Spectrum Emission Mask and 6.5.2.3 Adjacent Channel Leakage Ratio are based on a preliminary MU assessment and require an approval of the uncertainty contributors to be included in the uncertainty assessment as well as the contributors need further technical analysis.

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP DUT ≤ 15 cm ±4.89 dB (FR2a) ±5.09 dB (FR2b) DUT ≤ 30 cm ±4.79 dB (FR2a) ±4.99 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2, B.3-2-3 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP DUT ≤ 15 cm ±4.42 dB (FR2a) ±4.62 dB (FR2b) DUT ≤ 30 cm ±4.32 dB (FR2a) ±4.52 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2, B.3-2-3 in TR 38.903) |
| 6.2.1.2 UE maximum output power (Spherical coverage) | TBD |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | TBD |  |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3.1 Minimum output power | TBD |  |

## << Unchanged sections omitted >>

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The TT values defined in this section are valid only until RAN5#85 (November 2019).

- RAN5 expects to have the following actions performed by RAN5#85 in efforts to improve the TT values

- For updated TT evaluation only, MU analysis taking into the account the data sheet from commercially available test systems or any additional experimental data to justify the values

- UE vendors’ have supplied further input on TT based on the performance of the existing mmWave UE against the RF test cases.

- At least one statistical analysis has been done on possibilities to improve test confidence level.

- Measurement results & evaluation of network performance / commercially available devices has been provided.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (DUT ≤ 15 cm) 3.18 dB (FR2a) 3.31 dB (FR2b) IFF (DUT ≤ 30 cm) 3.11 dB (FR2a) 3.24 dB (FR2b) Max EIRP 0 dB | Minimum peak EIRP TT = 0.65 x MTSUIFF |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (DUT ≤ 15 cm) 2.87 dB (FR2a) 3.00 dB (FR2b) IFF (DUT ≤ 30 cm) 2.81 dB (FR2a) 2.94 dB (FR2b) | Max TRP TT = 0.65 x MTSUIFF |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 TBD PC4 TBD |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | TBD |  |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3.1 Minimum output power | TBD |  |

## << End of changes >>