3GPP TSG-WG5 Meeting #84	R5-196529

# Ljubljana, Slovenia,  26th Aug 2019 - 30th Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4863 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SCell in dormant state. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-UEConTest |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding test case for euCA work item as planned in euCA work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case for adding SCell in dormant state through RRCConnectionReconfiguration with and without mobilityControlInfo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dormant SCell functionality will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-2 CR 1289 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.11.1	CA / RRC connection reconfiguration / SCell addition in dormant mode / Success

8.2.2.11.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing sCellToAddModList with a SCell addition and sCellState indicates dormant }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in dormant state and sends an RRCConnectionReconfigurationComplete message and report CQI/PMI/RI/PTI/CRI for the SCell according to the periodicity indicated by cqi-ReportPeriodicSCell-r15 }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionReconfiguration message containing mobilityControlInfo and sCellToAddModList with SCell addition and sCellState indicates dormant }

then { UE adds the new SCell, configures lower layers to consider the SCell to be in dormant state and sends an RRCConnectionReconfigurationComplete message and report CQI/PMI/RI/PTI/CRI for the SCell according to the periodicity indicated by cqi-ReportPeriodicSCell-r15 }

}

8.2.2.11.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331, clauses 5.3.5.3, 5.3.5.4, 5.3.10.3b and 11.2.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	stop timer T370, if running;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

…

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

…

NOTE 1a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for connection to the target cell, as specified in TS 36.133 [16], if makeBeforeBreak is configured.

1>	reset MCG MAC and SCG MAC, if configured;

1>	release uplinkDataCompression, if configured;

1>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 2a:	At handover the pdcp-reestablish flag will be set for all RBs configured with NR PDCP in nr-RadioBearerConfig TS 38.331 [82] which will cause the PDCP entity to be re-established also for these RBs.

1>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

…

1>	apply the value of the newUE-Identity as the C-RNTI;

…

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

…

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

…

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the securityConfigHO (without suffix) is included in the RRCConnectionReconfiguration:

…

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

NOTE 2b:	If the UE needs to update the S-KeNB key as specified in 5.3.10.10, the UE updates the S-KeNB after updating the KeNB key.

2>	store the nextHopChainingCount value;

…

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

…

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

…

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…

1>	perform the measurement related actions as specified in 5.5.6.1;

…

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

…

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

…

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE when sameSFN-Indication is not present in mobilityControlInfo.

[TS 36.331, clause 5.3.10.3b]

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

…

2>	else if sCellState is configured for the SCell and indicates dormant:

3>	configure lower layers to consider the SCell to be in dormant state;

…

[TS 36.331, clause 11.2]

The UE performance requirements for RRC procedures are specified in the following table, by means of a value N:

N = the number of 1ms subframes from the end of reception of the E-UTRAN -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> E-UTRAN response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation) where N=20 (Ref. TS 36.331 Fig. 11.2-1).

8.2.2.11.1.3	Test Description

8.2.2.11.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is the PCell, Cell3 is the SCell to be added.

-	Cell 3 is active SCell according to [18] cl. 6.3.4.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.2.11.1.3.2	Test procedure sequence

Table 8.2.2.11.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.2.11.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that camping on Cell 1 is guarantee. |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | Power levels are such that entry condition for event A3 (measId 1) is satisfied: |

Table 8.2.2.11.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing an sCellToAddModList with SCell Cell 3 addition and sCellState indicates dormant. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message to confirm SCell addition of Cell 3 in dormant mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does UE report CQI/PMI/RI/PTI/CRI on Cell 3 for the next 400 ms? | - | (CSI) | 1 | P |
| 4 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup event A3 reporting configuration. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 6 | The SS changes Cell 3 parameters according to the row "T1" in table 8.2.4.17.1.3.2-1. | - | - | - | - |
| 7 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport | - | - |
| 8 | The SS transmits an RRCConnectionReconfiguration message with mobilityControlInfo on Cell 1 to order the UE to perform handover to Cell 3 and sCellToAddModList with SCell Cell 1 addition and sCellState indicates dormant. | <-- | RRCConnectionReconfiguration | - | - |
| 9 | UE transmits an RRCConnectionReconfigurationComplete message on Cell 3 to confirm the successful completion of the handover and SCell addition of Cell 1 in dormant mode. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10 | Check: Does UE report CQI/PMI/RI/PTI/CRI on Cell 1 for next 400 ms? | - | (CSI) | 2 | P |

8.2.2.11.1.3.3	Specific message contents

Table 8.2.2.11.1.3.3-1: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| Uplink_CA | The UE supports carrier aggregation in UL under the test band. |
| Band > 64 | If band > 64 is selected |

Table 8.2.2.11.1.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.2.2.11.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-DormantSCell |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sCellToAddModList-r10 |  | SCellToAddMod-r10-f2-Add | SCell addition for Cell 3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sCellToAddModListExt-v1430 |  | SCellToAddModListExt-v1430-DormantSCell |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.2.11.1.3.3-3: RadioResourceConfigDedicated-DormantSCell (Table 8.2.2.11.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AAA RadioResourceConfigDedicated-SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SCell_AddMod ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated- DormantSCell |  |  |
| } |  |  |  |

Table 8.2.2.11.1.3.3-4: PhysicalConfigDedicated-DormantSCell (Table 8.2.2.11.1.3.3-3)

| Derivation Path: 36.508 clause 4.8.2 table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  |  | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |
| cqi-ReportConfigSCell-r15 | CQI-ReportConfigSCell-r15-DormantSCell |  |  |  |  |
| } |  |  |  |  |  |

Table 8.2.2.11.1.3.3-5: CQI-ReportConfigSCell-r15-DormantSCell (Table 8.2.2.11.1.3.3-4)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  |  | Condition |
| CQI-ReportConfigSCell-r15-DormantSCell ::= SEQUENCE { |  |  |  |  |  |  |
| cqi-ReportPeriodicSCell-r15 CHOICE { |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |
| cqi-pmi-ConfigIndexDormant-r15 |  |  |  |  |  |
| ri-ConfigIndexDormant-r15 |  |  |  |  |  |
| csi-SubframePatternDormant-r15 CHOICE { |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |
| csi-MeasSubframeSet1-r15 |  |  |  |  |  |
| csi-MeasSubframeSet2-r15 |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| cqi-FormatIndicatorDormant-r15 CHOICE { |  |  |  |  |  |
| widebandCQI-r15 SEQUENCE { |  |  |  |  |  |
| csi-ReportMode-r15 |  |  |  |  |  |
| } |  |  |  |  |  |
| subbandCQI-r15 SEQUENCE { |  |  |  |  |  |
| k-r15 |  |  |  |  |  |
| periodicityFactor-r15 |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 8.2.2.11.1.3.3-6: SCellToAddMod-r10-f2-Add (Table 8.2.2.11.1.3.3-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |  |
| sCellIndex-r10 | 1 |  |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  |  | Band > 64 |
| } |  |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-f2 |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  |  | Band > 64 |
| } |  |  |  |  |  |

Table 8.2.2.11.1.3.3-7: RadioResourceConfigCommonSCell-r10-f2 (Table 8.2.2.11.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| Ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-Bandwidth-r10 | Same uplink system bandwidth as used for Cell 3 | optional | FDD |
|  | Not present |  | TDD |
| additionalSpectrumEmissionSCell-r10 | Same additionalSpectrumEmission as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.11.1.3.3-8: RadioResourceConfigDedicatedSCell-r10-f2 (Table 8.2.2.11.1.3.3-6)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | Uplink_CA |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.11.1.3.3-6: SCellToAddModListExt-v1430-Dormant (Table 8.2.2.11.1.3.3-2)

| Derivation Path: 36.331 clause |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| SCellToAddModListExt-v1430 ::= SEQUENCE (SIZE (1..maxSCell-r13)) OF SEQUENCE { | 1 entry |  |  |  |
| sCellState-r15 | dormant |  |  |  |
| } |  |  |  |  |  |

Table 8.2.2.11.1.3.3-7: RRCConnectionReconfiguration (step 4, Table 8.2.2.11.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.2.11.1.3.3-8: MeasConfig (Table 8.2.2.11.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.6-1 condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) | Cell 1 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.11.1.3.3-9: MeasurementReport (step 7, Table 8.2.2.11.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |  |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |  |  |  |  |
| measId |  | 1 |  |  |  |  |  |
| measResultPCell ::= SEQUENCE { |  |  |  | Report Cell 1 |  |  |  |
| rsrpResult |  | (0..97) |  |  |  |  |  |
| rsrqResult |  | (0..34) |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |  |  |  |
| measResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  |  |  |  |  |
| physCellId[1] |  | physicalCellId of Cell 3 |  |  |  |  |  |
| cgi-Info[1] |  | Not present |  |  |  |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |  |  |  |
| rsrpResult |  | (0..97) |  |  |  |  |  |
| rsrqResult |  | (0..34) |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measResultForECID-r9 |  | Not present |  |  |  |  |  |
| locationInfo-r10 |  | Not present |  |  |  |  |  |
| measResultServFreqList-r10 SEQUENCE (SIZE (1..maxServCell-r10)) OF SEQUENCE { |  |  |  |  |  |  |  |
| servFreqId-r10 |  | 1 |  |  |  |  |  |
| measResultSCell-r10 SEQUENCE { |  |  |  | Cell 3 |  |  |  |
| rsrpResultSCell-r10 |  | (0..97) |  |  |  |  |  |
| rsrqResultSCell-r10 |  | (0..34) |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.2.2.11.1.3.3-10: RRCConnectionReconfiguration (step 8, Table 8.2.2.11.1.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-DormantSCell |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { |  |  |  |  |
| SCellIndex-r10[1] |  | 1 |  |  |
| } |  |  |  |  |
| sCellToAddModList-r10 |  | SCellToAddMod-r10-f1-Add | SCell addition for Cell 1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sCellToAddModListExt-v1430 |  | SCellToAddModListExt-v1430-DormantSCell |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.2.11.1.3.3-11: RadioResourceConfigDedicated-DormantSCell (Table 8.2.2.11.1.3.3-10)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-19AAA RadioResourceConfigDedicated-SCell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SCell_AddMod ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated- DormantSCell |  |  |
| } |  |  |  |

Table 8.2.2.11.1.3.3-12: PhysicalConfigDedicated-DormantSCell (Table 8.2.2.11.1.3.3-11)

| Derivation Path: 36.508 clause 4.8.2 table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  |  | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |
| cqi-ReportConfigSCell-r15 | CQI-ReportConfigSCell-r15-DormantSCell |  |  |  |  |
| } |  |  |  |  |  |

Table 8.2.2.11.1.3.3-13: CQI-ReportConfigSCell-r15-DormantSCell (Table 8.2.2.11.1.3.3-12)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| CQI-ReportConfigSCell-r15-DormantSCell ::= SEQUENCE { |  |  |  |  |  |
| cqi-ReportPeriodicSCell-r15 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cqi-pmi-ConfigIndexDormant-r15 |  |  |  |  |
| ri-ConfigIndexDormant-r15 |  |  |  |  |
| csi-SubframePatternDormant-r15 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| csi-MeasSubframeSet1-r15 |  |  |  |  |
| csi-MeasSubframeSet2-r15 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cqi-FormatIndicatorDormant-r15 CHOICE { |  |  |  |  |
| widebandCQI-r15 SEQUENCE { |  |  |  |  |
| csi-ReportMode-r15 |  |  |  |  |
| } |  |  |  |  |
| subbandCQI-r15 SEQUENCE { |  |  |  |  |
| k-r15 |  |  |  |  |
| periodicityFactor-r15 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.2.11.1.3.3-14: MobilityControlInfo (Table 8.2.2.11.1.3.3-10)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq SEQUENCE {} | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.11.1.3.3-15: SCellToAddMod-r10-f1 (Table 8.2.2.11.1.3.3-10)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |  |
| sCellIndex-r10[1] | 1 |  |  |  |
| cellIdentification-r10[1] SEQUENCE { |  |  |  |  |
| physCellId-r10[1] | Physical Cell Identity of Cell 1 |  |  |  |
| dl-CarrierFreq-r10[1] | Same downlink EARFCN as used for Cell 1 |  |  |  |
| dl-CarrierFreq-r10[1] | maxEARFCN |  |  | Band > 64 |
| } |  |  |  |  |
| radioResourceConfigCommonSCell-r10[1] | RadioResourceConfigCommonSCell-r10-f1 |  |  |  |
| dl-CarrierFreq-v1090[1] | Same downlink EARFCN as used for Cell 1 |  |  | Band > 64 |
| } |  |  |  |  |

Table 8.2.2.11.1.3.3-16: RadioResourceConfigCommonSCell-r10-f3 (Table 8.2.2.11.1.3.3-10)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |