3GPP TSG-RAN WG5 #84	R5-197506
Ljubljana, SI, 26 - 30 Aug 2019

Agenda item:	5.3.5.17

Source:	Keysight Technologies

Title:	Wireless Cable Mode Procedure for Demodulation Tests

Document for:	Discussion and Endorsement

# Introduction

This contribution introduces a procedure to verify that the DUT is in the wireless cable mode with sufficient isolation.

# Discussion

In [1], the “virtually cabled” scenario was defined which is the underlying requirement to perform the demodulation test cases in a wireless cable mode.

The test method shall achieve isolation between two nominally orthogonal paths from the dual-polarised TRxP to the DUT, enabling independent control of the signals reaching each baseband receiver

- Test method supports up to DL MIMO rank 2 transmissions

- DUTs shall support power measurement per receiver port (SS-RSRPB). Measurement equipment may use SS-RSRPB reporting from the DUT to achieve isolation and to enable independent control of the signals reaching each baseband receiver.

- SS-RSRPB measurements and reporting are done under noise-free conditions and use static channel conditions.

- Once established, the setup is expected to be fixed and to be used with UE Tx/Rx beamlock to allow testing of DUT baseband features under a “virtually cabled” scenario.

- Baseline setup includes a capability to select the best UE RX beam during initial call setup.

This wireless cable mode will ensure that the radiated tests with DL MIMO rank 2 corresponds to a conducted test at the antenna ports.

Unfortunately, few details about this wireless cable mode have been captured in [1] or [3]. Many similarities to the Radiated Two-Stage methodology for LTE MIMO OTA [4] can be observed and therefore applied to achieving the NR FR2 wireless cable mode.

In [5], an initial isolation requirement of [15] dB is proposed for the NR FR1 demodulation wireless cable mode which matches the isolation requirement in [4] for LTE with bandwidths of up to 20 MHz and somewhat difficult dynamic fading conditions.

Observation: Very few details about the wireless cable mode have been captured so far

# Wireless Cable Mode Verification Procedure

The following procedure is suggested to validate the wireless cable mode has been achieved. As defined in [2], the demodulation tests have to be performed in the RX beam peak direction:

4.5.4 Angle of arrival

Unless otherwise stated, the downlink signal and noise are aligned to arrive in the UE Rx beam peak direction as defined in TS 38.101-2 [7].

which is considered as the test point for this procedure.

The following isolation improvement and isolation verification procedure is proposed to validate the wireless cable mode for demodulation test cases.

- Select any of the three Alignment Options (1, 2, or 3) to mount the DUT inside the QZ.

- If the re-positioning concept is applied to demodulation test cases, position the DUT in DUT Orientation 1 if the RX beam peak is within 0o≤≤90o. Otherwise, position the DUT in DUT Orientation 2 (Option 1 or 2). If the re-positioning concept is not applied to demodulation test cases, position the DUT in DUT Orientation 1

- Connect the SS (System Simulator) using static propagation conditions with the DUT through the measurement antenna with PolLink= polarization to form the RX beam towards the RX beam peak direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE RX beam selection to complete.

- Adjust the DL power of the SS to obtain PDL = TBD at the center of QZ

- Perform the isolation of the branches to achieve the wireless cable mode. The inverse channel matrix approach in [4] is one suitable approach. Alternate approaches are not precluded.

- To verify the wireless cable mode and thus the min. isolation between branches

  - Query SS-RSRPB(PolMeas=PolLink=) from the DUT for the -polarization and convert the two measurements in dBm, i.e., SS-RSRPBB1 and SS-RSRPBB2

  - Calculate the isolation from -polarization into Branch 1, i.e., ISO,B1 = SS-RSRPBB1 - SS-RSRPBB2 and the isolation into Branch 2, i.e., ISO,B2 = SS-RSRPBB2 - SS-RSRPBB1

  - Connect the SS (System Simulator) using static propagation conditions with the DUT through the measurement antenna with PolLink= polarization to form the RX beam towards the RX beam peak direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE RX beam selection to complete.

  - Adjust the DL power of the SS to obtain PDL = TBD at the center of QZ

  - Query SS-RSRPB(PolMeas=PolLink=) from the DUT for -polarization and convert the two measurements in dBm, i.e., SS-RSRPBB1 and SS-RSRPBB2

  - Calculate the isolation from -polarization into Branch 2, i.e., ISO,B2 = SS-RSRPBB2 - SS-RSRPBB1 and the isolation into Branch 1, i.e., ISO,B1 = SS-RSRPBB1 - SS-RSRPBB2

- If either of the isolations pairs, ISO,B1 and ISO,B2 or ISO,B2 and ISO,B1 exceed TBD, the wireless cable mode has been achieved.

Proposal: Use the procedure outlined above as the baseline to verify the wireless cable mode.

# Conclusion

The following observations and proposals were made in this contribution

Observation: Very few details about the wireless cable mode have been captured so far

Proposal: Use the procedure outlined above as the baseline to verify the wireless cable mode.

# References

- TR 38.810, Study on test methods, V16.3.0 (2019-06)

- R4-157267, Measurement uncertainty analysis for RTS, Keysight Technologies, 3GPP TSG RAN WG4 Meeting #77, November 2015

- TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V15.2.0 (2019-06)

- TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE), V15.0.0 (2018-09)

- R5-196818, Min. Isolation Requirements for Wireless Cable Mode, Keysight Technologies, 3GPP TSG-RAN WG5 #84, August 2019