3GPP TSG-RAN WG5 #85		R5-198265
Reno, NV, USA, 18 – 22 November 2019

Agenda item:	5.3.5.17

Source:	Keysight Technologies

Title:	Proposals for Beam Peak and TRP Measurement Grids for PC1 UEs

Document for:	Discussion and Endorsement

# Introduction

This contribution is presenting the Beam Peak and TRP measurement grid simulation results for PC1 UEs.

# PC1 Antenna Pattern and Beam Steering Assumptions

The simulations in this contribution are based on the following proposals for PC1 antenna pattern and beam steering assumptions [1], i.e.,

Proposal 1: Number of antenna arrays – 1 Proposal 2: Number of elements in array – 12 x 12 Proposal 3: Max DUT sizes – 30 cm Proposal 4: Max DUT radiating aperture – 10.6 cm Proposal 5: Power supply type – AC Power Supply Proposal 6: Max weight of UE – 10 lbs Proposal 7: Beam steering range and granularity in xz plane – 4 (from 30o to 150o) Proposal 8: Beam steering range and granularity in xy plane – 4 (from -60o to 60o)

The 12x12 reference antenna pattern based on the assumptions in [1] and [2] is illustrated in Figure 1.

Figure 1: Illustration of the 12x12 Antenna Pattern

# TRP Measurement Grid

The simulation assumptions including the antenna patterns for the TRP measurement grids are the same as Clause G.1 [2] except a 12x12 antenna array assumption instead of 8x2.

The results tabulated in this section outline the results of a statistical analyses with the positioning concept taken into account, i.e., the analyses were performed with and without the assumption that the beam peak direction is oriented away from the hemisphere towards the pole at = 180o. Additionally, the standard deviations are presented when ranges of pattern values are disregarded (zeroed out). For the constant-step size measurement grids, three cases were investigated, i.e., no pattern values are disregarded, values only at one latitude at =180o, and the values at the bottom two latitudes are disregarded. The results with the re-positioning concept applied are summarized in Table 1 for the sin(theta) and the Clenshaw-Curtis quadratures while the results without the re-positioning concept applied are summarized in Table 2.

For the constant density measurement grids, a similar investigation was performed using the Charged Particle implementation. Since there are no latitudes defined, two cases investigated were: no pattern values are disregarded and values between165o ≤ ≤ 180o are disregarded. The results with the re-positioning concept applied are summarized in Table 3 for the Charged Particle implementation while the results without the re-positioning concept applied are summarized in Table 4.

In order to make a reasonable trade-off between measurement uncertainties, at least the following number of points shall be included in the measurement grid for TRP measurements PC1 UEs based on the assumption that the standard deviation does not exceed 0.25dB. If the re-positioning concept is not applied to TRP test cases:

-	480 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.23 dB

-	25 latitudes and 48 longitudes (1106 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.07dB with the allowance to skip and interpolate measurements at the pole at =180o, see Annex M.4.4 [3]

-	21 latitudes and 40 longitudes (762 unique grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.24 dB with the allowance to skip and interpolate measurements at the pole at =180o, see Annex M.4.4 [3]

If the re-positioning concept is applied to TRP test cases:

-	500 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.24 dB with the allowance to skip and interpolate measurements beyond 165o in , see Annex M.4.4 [3]

-	25 latitudes and 48 longitudes (1106 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.14dB with the allowance to skip and interpolate measurements beyond 165o in , see Annex M.4.4 [3]

-	21 latitudes and 40 longitudes (762 unique grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.25 dB with the allowance to skip and interpolate measurements beyond 162o in , see Annex M.4.4 [3]

Table 1: Statistics of quadrature approaches for constant step size measurement grids for the 12x12 reference antenna array with the re-positioning concept applied.

| Number of |  | Step Size = | Number of Latitudes disregarded | Mean Error [dB] | Std. Dev [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |
| 37 | 72 | 5 | 0 | 0.00 | 0.03 | Sin(theta) | yes |
| 37 | 72 | 5 | 0 | 0.00 | 0.00 | Clenshaw-Curtis | yes |
| 37 | 72 | 5 | 1 | 0.00 | 0.03 | Sin(theta) | yes |
| 37 | 72 | 5 | 1 | 0.00 | 0.01 | Clenshaw-Curtis | yes |
| 37 | 72 | 5 | 2 | -0.01 | 0.07 | Sin(theta) | yes |
| 37 | 72 | 5 | 2 | -0.01 | 0.06 | Clenshaw-Curtis | yes |
| 25 | 48 | 7.5 | 0 | -0.01 | 0.09 | Sin(theta) | yes |
| 25 | 48 | 7.5 | 0 | 0.00 | 0.02 | Clenshaw-Curtis | yes |
| 25 | 48 | 7.5 | 1 | -0.01 | 0.08 | Sin(theta) | yes |
| 25 | 48 | 7.5 | 1 | 0.00 | 0.03 | Clenshaw-Curtis | yes |
| 25 | 48 | 7.5 | 2 | -0.02 | 0.14 | Sin(theta) | yes |
| 25 | 48 | 7.5 | 2 | -0.02 | 0.11 | Clenshaw-Curtis | yes |
| 21 | 40 | 9 | 0 | -0.02 | 0.28 | Sin(theta) | yes |
| 21 | 40 | 9 | 0 | 0.00 | 0.22 | Clenshaw-Curtis | yes |
| 21 | 40 | 9 | 1 | -0.02 | 0.28 | Sin(theta) | yes |
| 21 | 40 | 9 | 1 | -0.01 | 0.23 | Clenshaw-Curtis | yes |
| 21 | 40 | 9 | 2 | -0.04 | 0.31 | Sin(theta) | yes |
| 21 | 40 | 9 | 2 | -0.03 | 0.25 | Clenshaw-Curtis | yes |
| 19 | 36 | 10 | 0 | -0.05 | 0.62 | Sin(theta) | yes |
| 19 | 36 | 10 | 0 | -0.03 | 0.51 | Clenshaw-Curtis | yes |
| 19 | 36 | 10 | 1 | -0.04 | 0.58 | Sin(theta) | yes |
| 19 | 36 | 10 | 1 | -0.03 | 0.52 | Clenshaw-Curtis | yes |
| 19 | 36 | 10 | 2 | -0.08 | 0.61 | Sin(theta) | yes |
| 19 | 36 | 10 | 2 | -0.07 | 0.54 | Clenshaw-Curtis | yes |
| 16 | 30 | 12 | 0 | -0.26 | 1.48 | Sin(theta) | yes |
| 16 | 30 | 12 | 0 | -0.23 | 1.44 | Clenshaw-Curtis | yes |
| 16 | 30 | 12 | 1 | -0.23 | 1.48 | Sin(theta) | yes |
| 16 | 30 | 12 | 1 | -0.23 | 1.46 | Clenshaw-Curtis | yes |
| 16 | 30 | 12 | 2 | -0.32 | 1.52 | Sin(theta) | yes |
| 16 | 30 | 12 | 2 | -0.26 | 1.46 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 0 | -0.78 | 2.72 | Sin(theta) | yes |
| 13 | 24 | 15 | 0 | -0.72 | 2.68 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 1 | -0.77 | 2.71 | Sin(theta) | yes |
| 13 | 24 | 15 | 1 | -0.76 | 2.68 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 2 | -0.83 | 2.74 | Sin(theta) | yes |
| 13 | 24 | 15 | 2 | -0.85 | 2.69 | Clenshaw-Curtis | yes |

Table 2: Statistics of quadrature approaches for constant step size measurement grids for the 12x12 reference antenna array without the re-positioning concept applied.

| Number of |  | Step Size = | Number of Latitudes disregarded | Mean Error [dB] | Std. Dev [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |
| 37 | 72 | 5 | 0 | 0.00 | 0.03 | Sin(theta) | No |
| 37 | 72 | 5 | 0 | 0.00 | 0.00 | Clenshaw-Curtis | No |
| 37 | 72 | 5 | 1 | 0.00 | 0.03 | Sin(theta) | No |
| 37 | 72 | 5 | 1 | 0.00 | 0.02 | Clenshaw-Curtis | No |
| 37 | 72 | 5 | 2 | -0.03 | 0.21 | Sin(theta) | No |
| 37 | 72 | 5 | 2 | -0.02 | 0.18 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 0 | -0.01 | 0.09 | Sin(theta) | No |
| 25 | 48 | 7.5 | 0 | 0.00 | 0.02 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 1 | -0.01 | 0.07 | Sin(theta) | No |
| 25 | 48 | 7.5 | 1 | 0.00 | 0.06 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 2 | -0.06 | 0.38 | Sin(theta) | No |
| 25 | 48 | 7.5 | 2 | -0.05 | 0.35 | Clenshaw-Curtis | No |
| 21 | 40 | 9 | 0 | -0.02 | 0.29 | Sin(theta) | no |
| 21 | 40 | 9 | 0 | -0.01 | 0.22 | Clenshaw-Curtis | no |
| 21 | 40 | 9 | 1 | -0.02 | 0.27 | Sin(theta) | no |
| 21 | 40 | 9 | 1 | -0.01 | 0.24 | Clenshaw-Curtis | no |
| 21 | 40 | 9 | 2 | -0.09 | 0.57 | Sin(theta) | no |
| 21 | 40 | 9 | 2 | -0.08 | 0.53 | Clenshaw-Curtis | no |
| 19 | 36 | 10 | 0 | -0.04 | 0.59 | Sin(theta) | No |
| 19 | 36 | 10 | 0 | -0.03 | 0.51 | Clenshaw-Curtis | No |
| 19 | 36 | 10 | 1 | -0.04 | 0.60 | Sin(theta) | No |
| 19 | 36 | 10 | 1 | -0.04 | 0.52 | Clenshaw-Curtis | No |
| 19 | 36 | 10 | 2 | -0.15 | 0.83 | Sin(theta) | No |
| 19 | 36 | 10 | 2 | -0.14 | 0.77 | Clenshaw-Curtis | No |
| 16 | 30 | 12 | 0 | -0.24 | 1.50 | Sin(theta) | No |
| 16 | 30 | 12 | 0 | -0.22 | 1.44 | Clenshaw-Curtis | No |
| 16 | 30 | 12 | 1 | -0.28 | 1.50 | Sin(theta) | No |
| 16 | 30 | 12 | 1 | -0.23 | 1.46 | Clenshaw-Curtis | No |
| 16 | 30 | 12 | 2 | -0.42 | 1.64 | Sin(theta) | No |
| 16 | 30 | 12 | 2 | -0.37 | 1.59 | Clenshaw-Curtis | No |
| 13 | 24 | 15 | 0 | -0.79 | 2.73 | Sin(theta) | No |
| 13 | 24 | 15 | 0 | -0.75 | 2.67 | Clenshaw-Curtis | No |
| 13 | 24 | 15 | 1 | -0.75 | 2.71 | Sin(theta) | No |
| 13 | 24 | 15 | 1 | -0.72 | 2.67 | Clenshaw-Curtis | No |
| 13 | 24 | 15 | 2 | -0.97 | 2.83 | Sin(theta) | No |
| 13 | 24 | 15 | 2 | -0.93 | 2.76 | Clenshaw-Curtis | No |

Table 3: Statistics for constant density measurement grid types for the 12x12 reference antenna array with the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | Mean Error [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 700 | none | 0.00 | 0.05 | yes |
| 680 | none | 0.00 | 0.06 | yes |
| 660 | none | 0.00 | 0.07 | yes |
| 640 | none | 0.00 | 0.07 | yes |
| 620 | none | 0.00 | 0.08 | yes |
| 600 | none | 0.00 | 0.09 | yes |
| 580 | none | 0.00 | 0.11 | yes |
| 560 | none | 0.00 | 0.11 | yes |
| 540 | none | 0.00 | 0.14 | yes |
| 520 | none | -0.01 | 0.16 | yes |
| 500 | none | -0.01 | 0.19 | yes |
| 480 | none | -0.01 | 0.23 | yes |
| 460 | none | 0.00 | 0.28 | yes |
| 440 | none | -0.02 | 0.32 | yes |
| 420 | none | -0.02 | 0.41 | yes |
| 400 | none | -0.02 | 0.45 | yes |
| 380 | none | -0.05 | 0.56 | yes |
| 360 | none | -0.04 | 0.65 | yes |
| 340 | none | -0.08 | 0.82 | yes |
| 320 | none | -0.10 | 0.93 | yes |
| 300 | none | -0.16 | 1.16 | yes |
| 700 | 165o-180o | -0.04 | 0.15 | yes |
| 680 | 165o-180o | -0.04 | 0.16 | yes |
| 660 | 165o-180o | -0.04 | 0.15 | yes |
| 640 | 165o-180o | -0.03 | 0.15 | yes |
| 620 | 165o-180o | -0.04 | 0.16 | yes |
| 600 | 165o-180o | -0.03 | 0.16 | yes |
| 580 | 165o-180o | -0.04 | 0.18 | yes |
| 560 | 165o-180o | -0.04 | 0.18 | yes |
| 540 | 165o-180o | -0.04 | 0.20 | yes |
| 520 | 165o-180o | -0.04 | 0.21 | yes |
| 500 | 165o-180o | -0.04 | 0.24 | yes |
| 480 | 165o-180o | -0.04 | 0.27 | yes |
| 460 | 165o-180o | -0.05 | 0.33 | yes |
| 440 | 165o-180o | -0.04 | 0.35 | yes |
| 420 | 165o-180o | -0.05 | 0.43 | yes |
| 400 | 165o-180o | -0.05 | 0.47 | yes |
| 380 | 165o-180o | -0.07 | 0.59 | yes |
| 360 | 165o-180o | -0.08 | 0.67 | yes |
| 340 | 165o-180o | -0.10 | 0.84 | yes |
| 320 | 165o-180o | -0.15 | 0.95 | yes |
| 300 | 165o-180o | -0.18 | 1.17 | yes |

Table 4: Statistics for constant density measurement grid types for the 12x12 reference antenna array without the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | Mean Error [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 700 | none | 0.00 | 0.05 | no |
| 680 | none | 0.00 | 0.06 | no |
| 660 | none | 0.00 | 0.07 | no |
| 640 | none | 0.00 | 0.07 | no |
| 620 | none | 0.00 | 0.08 | no |
| 600 | none | 0.00 | 0.09 | no |
| 580 | none | 0.00 | 0.11 | no |
| 560 | none | 0.00 | 0.11 | no |
| 540 | none | 0.00 | 0.14 | no |
| 520 | none | 0.00 | 0.16 | no |
| 500 | none | 0.00 | 0.19 | no |
| 480 | none | -0.01 | 0.23 | no |
| 460 | none | -0.01 | 0.28 | no |
| 440 | none | 0.00 | 0.32 | no |
| 420 | none | -0.02 | 0.41 | no |
| 400 | none | -0.02 | 0.45 | no |
| 380 | none | -0.04 | 0.56 | no |
| 360 | none | -0.05 | 0.65 | no |
| 340 | none | -0.07 | 0.82 | no |
| 320 | none | -0.10 | 0.93 | no |
| 300 | none | -0.16 | 1.15 | no |
| 700 | 165o-180o | -0.12 | 0.56 | no |
| 680 | 165o-180o | -0.10 | 0.52 | no |
| 660 | 165o-180o | -0.10 | 0.50 | no |
| 640 | 165o-180o | -0.08 | 0.45 | no |
| 620 | 165o-180o | -0.10 | 0.54 | no |
| 600 | 165o-180o | -0.09 | 0.47 | no |
| 580 | 165o-180o | -0.11 | 0.57 | no |
| 560 | 165o-180o | -0.10 | 0.53 | no |
| 540 | 165o-180o | -0.12 | 0.58 | no |
| 520 | 165o-180o | -0.10 | 0.54 | no |
| 500 | 165o-180o | -0.10 | 0.55 | no |
| 480 | 165o-180o | -0.08 | 0.49 | no |
| 460 | 165o-180o | -0.11 | 0.61 | no |
| 440 | 165o-180o | -0.12 | 0.64 | no |
| 420 | 165o-180o | -0.11 | 0.67 | no |
| 400 | 165o-180o | -0.11 | 0.69 | no |
| 380 | 165o-180o | -0.16 | 0.81 | no |
| 360 | 165o-180o | -0.14 | 0.85 | no |
| 340 | 165o-180o | -0.18 | 0.96 | no |
| 320 | 165o-180o | -0.23 | 1.15 | no |
| 300 | 165o-180o | -0.24 | 1.24 | no |

# Beam Peak Measurement Grid

The simulation assumptions including the antenna patterns for the TRP measurement grids are the same as Clause G.2 [2] except a 12x12 antenna array assumption instead of 8x2.

For the simulations, the relative orientation of the simulated antenna array and the measurement grid was altered randomly. The statistical results from simulations using 50,000 random orientations are then used to determine mean error, standard deviation and percentile analysis on CDF curve of all maximum EIRPs for each measurement grid. The EIRPs are normalized by the known 12x12 antenna peak antenna gain.

Sample histograms and CDF distributions for the beam peak error for constant step-size measurement grids are shown in Figure 2 and for the constant density measurement grid (based on the charged particle implementation) in Figure 3. The histograms show a half-normal distribution.

Given the half-normal distribution, the MU term should be based on the determination of the offset from the beam peak that contains 95% of the distribution (alternatively, the value at which the CDF is 5%).  This offset shall be considered a systematic error in the MU budget. The various statistical metrics are illustrated in Figure 4.

Figure 2: Histogram of maximum beam peak errors for sample constant-step size meausurement grids (left: 2.5o, right: 7.5o step size)

Figure 3: Histogram of maximum beam peak errors for sample constant density measurement grids (left: 5000, right: 1000 grid points)

Figure 4: Statistical metrics for a sample half-normal distribution

The mean error and the standard deviation, and the offset at which the CDF is 5% are tabulated in Table 5 for the constant step size grids Table 6 for the constant density grids.

Table 5: Statistical Analyses of the 50k simulations for the constant step size grids

| Angular Step Size [o] | Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| --- | --- | --- | --- | --- |
| 2.5 | 10226 | 0.14 | 0.09 | 0.31 |
| 3.0 | 7082 | 0.20 | 0.14 | 0.45 |
| 3.6 | 4902 | 0.28 | 0.20 | 0.65 |
| 4.0 | 3962 | 0.35 | 0.24 | 0.80 |
| 4.5 | 3122 | 0.44 | 0.31 | 1.02 |
| 5.0 | 2522 | 0.55 | 0.38 | 1.26 |
| 6.0 | 1742 | 0.80 | 0.55 | 1.83 |
| 7.5 | 1106 | 1.25 | 0.88 | 2.94 |
| 9.0 | 762 | 1.85 | 1.31 | 4.31 |
| 10.0 | 614 | 2.30 | 1.66 | 5.45 |
| 12.0 | 422 | 3.42 | 2.52 | 8.25 |
| 15.0 | 266 | 5.68 | 4.31 | 14.27 |

Table 6: Statistical Analyses of the 50k simulations for the constant-density grids

| Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| --- | --- | --- | --- |
| 8000 | 0.13 | 0.08 | 0.26 |
| 7000 | 0.15 | 0.09 | 0.30 |
| 6000 | 0.17 | 0.10 | 0.35 |
| 6500 | 0.16 | 0.09 | 0.32 |
| 5000 | 0.21 | 0.12 | 0.41 |
| 4500 | 0.23 | 0.14 | 0.46 |
| 4000 | 0.26 | 0.15 | 0.52 |
| 3500 | 0.30 | 0.18 | 0.60 |
| 3000 | 0.35 | 0.21 | 0.69 |
| 2000 | 0.53 | 0.31 | 1.05 |
| 1500 | 0.70 | 0.42 | 1.41 |
| 1000 | 1.06 | 0.64 | 2.15 |

In Table 7, the minimum number of unique grid points are listed for each grid type investigated for sample systematic errors of ‘Beam Peak Search’ of 0.3 to 0.7dB. The option with the 0.7dB seems to be best compromise in terms of MU and test points/test time. It should be noted that for PC3 devices, a systematic error of 0.5dB was selected.

Table 7: Minimum number of unique grid points for sample systematic errors

| Systematic Error of ‘Beam Peak Search’: Offset from Beam Peak at which CDF is 5% | Minimum Number of Unique Grid Points for Constant Step Size Grid | Minimum Number of Unique Grid Points for Constant Density Grid |
| --- | --- | --- |
| 0.3dB | 10226 (2.5o step size) | 7000 |
| 0.4dB | N/A | 5000 |
| 0.5dB | 7082 (3ostep size) | 4500 |
| 0.6dB | N/A | 3500 |
| 0.7dB | 4902 (3.6o step size) | 3000 |

Taking into account simulation results above and in order to make a reasonable trade-off with measurement uncertainties, it is recommended to use for beam peak search the following measurement grids leading to a systematic error of “Beam Peak Search” of 0.7 dB:

-	Constant density grid (using the charged particle implementation) with at least 3000 grid points.

-	Constant step size grid with at least 4902 grid points, corresponding to an angular step size of 3.6º.

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Adopt the proposed measurement grids and assumptions for PC1 UEs

# References

- R5-198203, MU/MTSU Assumptions for PC1/Fixed Wireless Access (FWA) UE, Qualcomm, 3GPP TSG-RAN WG5 #85, November 2019

- TR38.810, Study on test methods, V16.4.0 (2019-09)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.10 (2019-09)