3GPP TSG-RAN5 Meeting #86-e		R5-200318
Electronic Meeting, 17th – 28th February 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	CDF/PDF Scaling Factor

Document for:	Approval

# Introduction

This contribution suggests augmenting the PDF scaling factor in the test specification for constant step-size measurement grids to properly calculate the CDF/CCDF for spherical coverage test cases.

# Discussion

Currently, the test specification [1] states that the CDF calculations for constant step size grids require a sin() scaling of the PDF

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using maximum(EIRP(PolLink=), EIRP(PolLink=)) for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

For the TRP, it has already be shown that Clenshaw-Curtis weights yield more accurate results when compared to the classical sin() weights, e.g., by not applying a zero weight at the poles [2]. It therefore seems more appropriate and accurate to apply a scaling factor that is related to the Clenshaw-Curtis weights, W(),which has been introduced in Section M.4.2.1 of [1] already

There is no simple closed-form expression for the Clenshaw-Curtis weights; however, a numerical straightforward approach is available in, i.e., W(θi)=ciN1-j=1int(N2)bj4j2-1cos⁡(2jθi) with bj= 1, j=n/22, j<n/2 and ci= 1, i=0 mod N2, otherwise

As the measurement uncertainty simulations were performed with the sin() weights, it is suggested to augment the CDF/PDF scaling with the Clenshaw-Curtis weights for constant step size measurement grids, i.e., allow scaling using either the sin() or Clenshaw-Curtis weights.

Proposal 1: Augment the CDF/PDF scaling using sin() weights with Clenshaw-Curtis weights for constant step size measurement grids.

While the quadrature weight for the classical sin() quadrature is Wsin()=sin()*, one approach to scale the PDFs could be using the WCC()/ term. As outlined in Table 1, this approach would yield a scaling value at the equator of slightly greater than 1. In order to keep the scaling factor at =90o to 1, it is instead suggested to scale the PDFs with the WCC()/WCC(90) term which yields a value of 1 at =90o.

Table 1: Comparison of different scaling terms (for a constant step size grid with =15o step size

|  | sin() | WCC()/ | WCC()/WCC( =90o) |
| --- | --- | --- | --- |
| 0 | 0 | 0.0267 | 0.0267 |
| 15 | 0.2588 | 0.2523 | 0.2523 |
| 30 | 0.5 | 0.5025 | 0.5019 |
| 45 | 0.7071 | 0.7057 | 0.7053 |
| 60 | 0.866 | 0.8670 | 0.8664 |
| 75 | 0.9659 | 0.9652 | 0.9645 |
| 90 | 1 | 1.0007 | 1.0000 |
| 105 | 0.9659 | 0.9652 | 0.9645 |
| 120 | 0.866 | 0.8670 | 0.8664 |
| 135 | 0.7071 | 0.7057 | 0.7053 |
| 150 | 0.5 | 0.5025 | 0.5019 |
| 165 | 0.2588 | 0.2523 | 0.2523 |
| 180 | 0 | 0.0267 | 0.0267 |

Proposal 2: Augment the sin() weights the PDFs need to be scaled by with the WCC()/WCC(90o) weights for constant step size grids.

A CR implementing this proposed approach is in [3]

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Augment the CDF/PDF scaling using sin(q) weights with Clenshaw-Curtis weights for constant step size measurement grids.

Proposal 2: Augment the sin() weights the PDFs need to be scaled by with the WCC()/WCC(90o) weights for constant step size grids.

# References

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.20 (2019-12)

- R4-1811091, On TRP Measurement Grids for mm-wave using Clenshaw-Curtis Weights, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #88, August 2018

- R5-200319, CR to 38.521-2 on CDF/PDF Scaling Factor, Keysight Technologies, 3GPP TSG-RAN5 Meeting #86-e, February 2020