3GPP TSG-RAN5 Meeting #86-e		R5-200321
Electronic Meeting, 17th – 28th February 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	NR FR2 Demodulation MU Table

Document for:	Discussion for Endorsement

# Introduction

This contribution provides a consolidated MU Table for Mode 1 (conditions with external noise) and for Mode 2 (noise free conditions) demodulation test cases.

# Discussion

In RAN5#84, three different contributions discussed MU elements for demodulation test cases [1][2][3]. This contribution proposes a consolidated MU table for Mode 1 demodulation test cases based on previous discussions and endorsements in Table 1.

Table 1: Uncertainty Contributions for Mode 1 Demodulation Test Cases

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment |  | Normal | 2.00 |  |
| 2 | Measure distance uncertainty |  | Rectangular | 1.73 |  |
| 3 | Quality of Quiet Zone (NOTE 7) |  | Actual | 1.00 |  |
| 4 | Mismatch |  | Actual | 1.00 |  |
| 5 | Standing wave between the DUT and measurement antenna |  | U-shaped | 1.41 |  |
| 6 | gNB emulator SNR uncertainty |  | Normal | 2.00 |  |
| 7 | gNB emulator DL EVM |  | Actual | 1.00 |  |
| 8 | gNB emulator fading model impairments |  | Normal | 2.00 |  |
| 9 | AWGN absolute power uncertainty |  | Actual | 1.00 |  |
| 10 | Test System Frequency Flatness Uncertainty |  | Actual | 1.00 |  |
| 11 | Phase curvature |  | U-shaped | 1.41 |  |
| 12 | Amplifier uncertainties |  | Normal | 2.00 |  |
| 13 | Random uncertainty |  | Normal | 2.00 |  |
| 14 | Influence of the XPD |  | U-shaped | 1.41 |  |
| 15 | Insertion Loss Variation |  | Rectangular | 1.73 |  |
| 16 | RF leakage (from measurement antenna to the receiver/transmitter) |  | Actual | 1.00 |  |
| 17 | DUT repositioning |  | Rectangular | 1.73 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 18 | Mismatch |  | U-shaped | 1.41 |  |
| 19 | Amplifier Uncertainties |  | Normal | 2.00 |  |
| 20 | Misalignment of positioning System |  | Normal | 2.00 |  |
| 21 | Uncertainty of the Network Analyzer |  | Normal | 2.00 |  |
| 22 | Uncertainty of the absolute gain of the calibration antenna |  | Normal | 2.00 |  |
| 23 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  | Rectangular | 1.73 |  |
| 24 | Phase centre offset of calibration antenna |  | Rectangular | 1.73 |  |
| 25 | Quality of quiet zone for calibration process (NOTE 7) |  | Actual | 1.00 |  |
| 26 | Standing wave between reference calibration antenna and measurement antenna |  | U-shaped | 1.41 |  |
| 27 | Influence of the calibration antenna feed cable |  | Normal | 2.00 |  |
| 28 | Insertion Loss Variation |  | Rectangular | 1.73 |  |
|  | Systematic uncertainties (NOTE 3) |  |  |  | Value |
| 29 | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  |  |

The consolidated MU table for Mode 2 demodulation test cases based on the EIS MU table in [4], specifically Table B.19.2-2, in Table 2.

This contribution is merely intended to define the baseline tables and does not address specific MU elements which needs to be addressed separately.

Table 2: Uncertainty Contributions for Mode 2 Demodulation Test Cases

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment |  | Normal | 2.00 |  |
| 2 | Measure distance uncertainty |  | Rectangular | 1.73 |  |
| 3 | Quality of Quiet Zone |  | Actual | 1.00 |  |
| 4 | Mismatch |  | Actual | 1.00 |  |
| 5 | Standing wave between the DUT and measurement antenna |  | U-shaped | 1.41 |  |
| 6 | gNB uncertainty on absolute level |  | Normal | 2.00 |  |
| 7 | Phase curvature |  | U-shaped | 1.41 |  |
| 8 | Amplifier uncertainties |  | Normal | 2.00 |  |
| 9 | Random uncertainty |  | Normal | 2.00 |  |
| 10 | Influence of the XPD |  | U-shaped | 1.41 |  |
| 11 | Insertion Loss Variation |  | Rectangular | 1.73 |  |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) |  | Actual | 1.00 |  |
| 13 | Multiple measurement antenna uncertainty |  | Actual | 1.00 |  |
| 14 | DUT repositioning |  | Rectangular | 1.73 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch |  | U-shaped | 1.41 |  |
| 16 | Amplifier Uncertainties |  | Normal | 2.00 |  |
| 17 | Misalignment of positioning System |  | Normal | 2.00 |  |
| 18 | Uncertainty of the Network Analyzer |  | Normal | 2.00 |  |
| 19 | Uncertainty of the absolute gain of the calibration antenna |  | Normal | 2.00 |  |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  | Rectangular | 1.73 |  |
| 21 | Phase centre offset of calibration antenna |  | Rectangular | 1.73 |  |
| 22 | Quality of quiet zone for calibration process |  | Actual | 1.00 |  |
| 23 | Standing wave between reference calibration antenna and measurement antenna |  | U-shaped | 1.41 |  |
| 24 | Influence of the calibration antenna feed cable |  | Normal | 2.00 |  |
| 25 | Insertion Loss Variation |  | Rectangular | 1.73 |  |
|  | Systematic uncertainties |  |  |  | Value |
| 26 | Systematic error related to beam peak search |  |  |  |  |

It is proposed to use Table 1 as a baseline for Mode 1 NR FR2 demodulation test cases.

Proposal 1: Use Table 1 as a baseline for Mode 1 NR FR2 demodulation test cases

It is proposed to use Table 2 as a baseline for Mode 2 NR FR2 demodulation test cases.

Proposal 2: Use Table 2 as a baseline for Mode 2 NR FR2 demodulation test cases

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Use Table 1 as a baseline for Mode 1 NR FR2 demodulation test cases

Proposal 2: Use Table 2 as a baseline for Mode 2 NR FR2 demodulation test cases

# References

- R5-196582, On FR2 Demodulation MU, Anritsu, 3GPP TSG-RAN5 #84 Meeting, August 2019

- R5-196820, On MU Elements for Demodulation test cases, Keysight Technologies, 3GPP TSG-RAN5 #84 Meeting, August 2019

- R5-196851, Discussion on FR2 Demod MU contributors, Rohde & Schwarz, 3GPP TSG-RAN5 #84 Meeting, August 2019

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.2.0 (2019-12)