3GPP TSG-RAN5 Meeting #87-e	R5-202316

# Electronic Meeting, 18th - 29th May 2020

Title:							Noise impact for FR2 TRx test cases

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

This paper provides the SNR estimations and proposals to cater for low PSD testability issue for P1/P2 TRx test cases.

# 2.	Discussion

Table 1 summarizes the SNR estimation for remaining P1/P2 test cases.

Table 1 SNR estimations for P1 and P2 test cases

| Test case | Applicable Frequency Range [GHz] | UL signal level [dBm] | Estimated SNR for total component [dB] | ΔSNR=10log10(1+1/SNR), SNR in linear and defined for total component power | Proposed relaxation of test requirement |
| --- | --- | --- | --- | --- | --- |
| ACLR | See separated paper [1] |  |  |  |  |
| Spurious Co-existence | n260 (Aggressor band : n257, n261) | -2dBm/100MHz (-22dBm/MHz) | 1.0 | 1.0 or 0.41 | 5.0 or 9.0 |
|  | n257, n261 (Aggressor band : n260) | -5dBm/100MHz (-25dBm/MHz) | 2.7 | 1.0 or 0.41 | 3.3 or 7.3 |
|  | 57 GHz ≤ f ≤ 66GHz | 2dBm/100MHz (-18dBm/MHz) | 10 | 0.41 | 0 |
| NOTE ChBW = 10log10(400MHz/ChBW), e.g. 9.03dB for ChBW=50MHz. NOTE 2 : According to the agreement in [7], the SNR estimation is only valid between 23.45 and 30.3 GHz for other than spurious emission. SNR needs to be revisited once the new band and its core requirement is introduced between 30.3GHz and 32.125GHz. NOTE 4 : 14dB PAPR is assumed in this estimation to judge the LNA availability. |  |  |  |  |  |

# 3.	Proposals

Proposal 1 : Use Table 1 for determination of test requirement relaxation value and SNR for P1 and P2 FR2 TRx Test Case

# 4.	References

[1] R5-20xxxx, “On the FR2 ACLR Testability”,  Anritsu, RAN5#87-e

Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43 dBm (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |