3GPP TSG-RAN5 Meeting #87-e		R5-202460
Electronic Meeting, 18 May – 29 May 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On FR2 OBW MU

Document for:	Discussion for Endorsement

# Introduction

This contribution is discussing FR2 OBW MU. This topic has been discussed for long ([1]-[13]) without agreement. This document provides additional simulations in order to achieve agreement on FR2 OBW MU.

# Simulation results

In last RAN5 meeting (RAN5#86e), main TE vendors provided different simulations ([11], [12], [13]) from the information provided to close AP#84.22 ([9]) but no agreement was possible due to a misalignment on the simulation assumptions.

|  | MU definition | OBW calculation |
| --- | --- | --- |
| R5-200739 [11] (Anritsu) | (OBWsignal+noise+flatness – NRB*SCS*12*0.99) / CBW *100 | OBW = Width of frequency span centring on carrier centre frequency which contains 99% power of 2*CBW(or 1.5*CBW) |
| R5-200869 [12] (R&S) | (OBWsignal+noise - OBWsignal) / CBW *100 | OBW = (Higher frequency edge above which contains 0.5% of total power in 2.0*CBW(or 1.5*CBW) ) – (Lower frequency edge below which contains 0.5% of total power in 2*CBW(or 1.5*CBW) ) |
| R5-200874 [13] (Keysight) | (OBWsignal+noise - OBWsignal) / OBWsignal *100 | OBW = Width of frequency span centring on carrier centre frequency which contains 99% power of 2*CBW |

Keysight has updated their simulations with the following updates:

- Update 1: Defining OBW MUs as (OBWsignal+noise - OBWsignal) / CBW *100 in order to align its definition with FR1.

- Update 2: Providing simulations results for 1.5*CBW and 2*CBW for the spectrum considered to run OBW measurement

- Update 3: Considering a spectrum flatness of 2.19 dB.

The spectra of each polarization and of the total power are plotted for 100 MHz CHBW and 4x100 MHz in Figures 1 and 2, respectively. During RAN5#85, it has been confirmed that the 4x100 MHz CA dataset can be applied to derive the MU for OBW for 400 MHz single carrier.

The MBW assumed for the information provided in [9] has been 1 MHz.

According to the dataset provided for 100 MHz signal:

| - Reference V and H signal spectrum for CBW=100 MHz | - Reference V and H signal spectrum for CBW=100 MHz |
| --- | --- |

Figure 1. Reference signal for CBW=100 MHz ([9])

| Centre frequency | 27.9997 GHz |
| --- | --- |
| OBW (span=2.0⋅CHBW) | 94.4 MHz |
| OBW (span=1.5⋅CHBW) | MHz |
| CP (dBm) | 20.28 dBm |
| ACLR below channel | 34.09 dB |
| ACLR above channel | 36.44 dB |

Table 1: Results for the 100 MHz data set

Considering SNR referred to channel power, the following simulation results have been obtained:

| - No spectral flatness, 2*CBW | - 2.19 flat spectral flatness, 2*CBW |
| --- | --- |
| - No spectral flatness, 1.5*CBW | - 2.19 flat spectral flatness, 1.5*CBW |

Figure 2. OBW error for CBW=100 MHz ([9])

According to the dataset provided for 4*100 MHz signal:

| - Reference V and H signal spectrum for CBW=4*100 MHz | - Reference V and H signal spectrum for CBW=4*100 MHz |
| --- | --- |

Figure 3. Reference signal for CBW=4*100 MHz ([9])

| Centre frequency | 28 GHz |
| --- | --- |
| OBW (span=2.0⋅CHBW) | 398 MHz |
| OBW (span=1.5⋅CHBW) | MHz |
| CP (dBm) | 18.29 dBm |
| ACLR below channel | 25.91 dB |
| ACLR above channel | 24.11 dB |

Table 2: Results for the 4*100 MHz data set

Considering SNR referred to channel power, the following simulation results have been obtained:

| - No spectral flatness, 2*CBW | - 2.19 flat spectral flatness, 2*CBW |
| --- | --- |
| - No spectral flatness, 1.5*CBW | - 2.19 flat spectral flatness, 1.5*CBW |

Figure 4. OBW error for CBW=4*100 MHz ([9])

Observation 1: In order to run similar analysis with frequency band n260, similar information to the one provided to close AP#84.22 is required.

Proposal 1: OEM and chipset vendors to provide the information on the Tx spectrum shape (PSD, slope), to help determine MU of FR2 OBW test for frequency band n260.

# Available SNR in different TE for UL maximum output power

Different TE vendors provided SNR for maximum output power test case as:

| R5-191794 [14] (Anritsu) |  |
| --- | --- |
| R5-192125 [15] (R&S) |  |
| R5-193187 [16] (Keysight) |  |

Table 3: Available SNR for MOP test case

# Reducing the analysed span from 2*CBW to 1.5*CBW

As seen in the simulation results, one approach to reduce the noise impact is to reduce the span analysed for the 99% OBW test procedure is to reduce the span from 2*CBW to 1.5*CBW.

OBW is a regulatory test case and any change to the measurement procedure should be regulatory compliant as much as possible. Checking FCC regulations [17], OBW test procedure refers to ANSI C63.26-2015 [18].

According to ANSI C63.26-2015 [17], for the Occupied bandwidth—Power bandwidth (99%) measurement procedure:

“The frequency span for the spectrum analyser shall be set wide enough to capture all modulation products including the emission skirts (typically a span of 1.5 × OBW is sufficient).”

If span is reduced to 1.5*CBW, taking into account available SNR for different TE vendors, measurements at 28 GHz frequency band could be performed for either 100 MHz and 400 MHz without relaxing the requirements if the measurement uncertainty for this FR2 test case is defined as CBW-0.99*Tx BW configCBW*100% in order to accommodate variations on UE ACLR as well as on SS spectral flatness.

Proposal 2: Reduce FR2 OBW span from 2*CBW to 1.5 *CBW.

Proposal 3: Define FR2 OBW MU as CBW-0.99*Tx BW configCBW*100% for FR2a frequency range in order to accommodate variations on UE ACLR as well as on SS spectral flatness.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: In order to run similar analysis with frequency band n260, similar information to the one provided to close AP#84.22 is required.

Proposal 1: OEM and chipset vendors to provide the information on the Tx spectrum shape (PSD, slope), to help determine MU of FR2 OBW test for frequency band n260.

Proposal 2: Reduce FR2 OBW span from 2*CBW to 1.5 *CBW.

Proposal 3: Define FR2 OBW MU as CBW-0.99*Tx BW configCBW*100% for FR2a frequency range in order to accommodate variations on UE ACLR as well as on SS spectral flatness.

# References

- R5-188189, “On the FR2 MU for occupied BW and ACLR”, Anritsu, RAN5#81

- R5-190390, “On QoQZ MU Impact on OBW”, Keysight, RAN5 NR#4 AdHoc

- R5-192660, “On the FR2 OBW MU”, Anritsu, RAN5#82

- R5-191617, “On OBW measurement uncertainty”, Keysight, RAN5#82

- R5-194353, “On the noise impact to FR2 OBW”, Anritsu, RAN5#83

- R5-192984, “On the SNR for FR2 TRx test cases”, R&S, Rohde & Schwarz, RAN5 NR#5 AdHoc

- R5-193187, “On noise impact”, Keysight, RAN5 NR#5 AdHoc

- R5-195506, Action Points at RAN5#84 Ljubljana

- R5-198175, “Discussion on AP#84.22 Tx Spectrum Shape for FR2 OBW MU”, Qualcomm, RAN5#85

- R5-197626, “Impact of noise for FR2 OBW”, Anritsu, RAN5#84

- R5-200739, “On FR2 OBW MU”, Anritsu, RAN5#86e

- R5-200869, “On the MU of FR2 OBW”, Rohde & Schwarz, RAN5#86e

- R5-200874, “On FR2 OBW MU”, Keysight Technologies, RAN5#86e

- R5-191794, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5#82

- R5-192125, “On the SNR for FR2 TRx test cases”, Rohde & Schwarz, RAN5#82

- R5-193187, “On noise impact”, Keysight Technologies, RAN5 NR#5 AdHoc

- 842590 D01 Upper Microwave Flexible Use Service v01, “Basic Certification Requirements and Measurement Procedures for Upper Microwave Flexible Use Service Devices”, 2019

- ANSI C63.26-2015, “Accredited Standards Committee C63®—Electromagnetic Compatibility “, American National Standard for Compliance Testing of Transmitters Used in Licensed Radio Services C63, 2015