3GPP TSG-RAN WG5 #87-e		R5-201605r1
Electronic Meeting, 18 - 29 May 2020

Agenda item:	5.3.2.17

Source:	Rohde & Schwarz

Title:	RRM FR2 MU: Representative test case 7.7.1.1

Document for:	Endorsement

# Introduction

In RAN5 #86-e, the list of representative RRM test cases was agreed in [1]. This contribution will analyze one of the test cases of the list, f) Test case 7.7.1.1 Intra-frequency SS-RSRP Accuracy.

7.7.1.1 is based on TS 38.133 Test Case A.7.7.1.1 [2], copied below for reference

#### A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.3.1.1 and 10.1.3.1.2 for intra-frequency measurements.

A.7.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in Table A.7.7.1.1.2-2 and A.7.7.1.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table  A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1. The test consists of two time phases T1 and T2.

Table A.7.7.1.1.2-1: SS-RSRP Intra frequency SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.1.1.2-2: SS-RSRP  Intra frequency general test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 24 |  | 100: NRB,c = 24 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |

Table A.7.7.1.1.2-3: SS-RSRP Intra frequency OTA related test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | According to clause A.3.15.1 |  | According to clause A.3.15.1 |  |
| Note1 | dBm/15kHzNote4 | -91.6 |  | N/A |  |
| Note1 | dBm/SCSNote4 | -82.6 |  | N/A |  |
|  | dB | 6.0 | 1.0 | N/A | N/A |
| Es | dBm/SCSNote4 |  |  | (Table B.2.2-2 Rx Beam Peak +3.1dB) | (Table B.2.2-2 Rx Beam Peak +3.1dB) |
| SSB_RPNote2 | dBm/SCS | -76.6 | -81.6 | (Table B.2.2-2 Rx Beam Peak +3.1dB) | (Table B.2.2-2 Rx Beam Peak +3.1dB) |
| BB Note6 | dB | 2.44 | -5.98 | -5.98 | -5.98 |
| IoNote2 | dBm/95.04 MHz Note4 | -50.05 |  | (Table B.2.2-2 Rx Beam Peak +30.70dB) |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

##### A.7.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.3.1.1 and relative accuracy requirements in clause 10.1.3.1.2. The following requirements are to be verified:

During T1:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table A.7.7.1.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1.

During T2:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table A.7.7.1.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1.

During T1 and T2:

Relative accuracy of Cell 1 during T2 compared with Cell 1 during T1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1

Relative accuracy of Cell 2 during T2 compared with Cell 2 during T1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1.

Table A.7.7.1.1.3-1: SS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| Cell 1 | SSB_RP1 -δ +Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP1 +δ +Gmax |
| Cell 2 | SSB_RP2 -δ +Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP2 +δ +Gmax |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.3.1.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

The core requirements, in Section 10.1.3.1 of [2], are also copied here:

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.3.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR2.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.2-1: SS-RSRP Intra frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6 | 9 | ≥-6 | Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. |  |  |  |  |  |

The side conditions are captured in Annex B.2.2 of [2]:

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Please note that this analysis also applies to the NSA test case 5.7.1.1, based on A.5.7.1.1 in [2].

# Discussion

From an initial analysis of the RAN4 requirements, a few initial observations can be done:

- Metric is single-directional EIS

- It is a multi-cell scenario with 2 intra-frequency NR cells

- The test is 1 AoA.

- The test is performed in the RX Beam Peak of the DUT.

- The test has two sub-tests.

  - Subtest 1 has average DL signal powers and AWGN.

  - Subtest 2 has DL signal powers close to the minimum SSB_RP limit and no AWGN.

[1] identifies the following MU quantities, that are relevant for 7.7.1.1:

•	DL AWGN absolute power and wanted DL signal absolute power

- DL applied SNR

This contribution will focus on the first of the MU quantities DL AWGN absolute power and wanted DL signal absolute power.

The RRM baseline setup is defined in TS 38.508-1 [3] Section 7.1.3.2, as follows:

#### 7.1.3.2	RRM baseline setup

The RRM baseline setup shall fulfil the capabilities detailed in this section.

The following permitted test setups are considered for OTA RRM testing:

-	DFF test setup as described in Clause B.2.2.

-	Simplified DFF test setup as described in Clause B.2.3.

-	IFF test setup as described in Clause B.2.4.

-	Enhanced IFF test setup based in the IFF test setup described in Clause B.2.4, with the enhancements described in this clause.

-	IFF+DFF Hybrid teset setup as described in Clause B.2.2 for DFF TRxP(s) and B2.4 for IFF TRxP(s), with the enhancements described in this clause.

Annex B.2 captures the permitted test methods for RF Tx and Rx, Performance, RRM and Signalling Conformance test.

Observation 1: The RRM baseline test setup is based in the RF test setups as defined in B.2 of TS 38.508-1 [3].

Observation 2: The RRM baseline test setups based on DFF, Enhanced IFF and IFF+DFF Hybrid are an extension of the RF test setup in B.2 of TS 38.508-1 [3].

Several MU analyses have already been conducted for RF test cases in TS 38.521-2 [4]. RRM MU analysis can take advantage of some of the parameters. Given the observations above, the test case 7.3 Reference Sensitivity [4], is identified as the closest RF test case match for RRM 7.7.1.1, due to the following reasons:

- The test case has EIS metric and is performed in the RX Beam Peak

- The power levels are close to the required power levels for the RRM test.

Proposal 1: use the MU analysis of RF test case “7.3 Reference Sensitivity” as starting point for the MU analysis for DL AWGN absolute power and wanted DL signal absolute power for RRM TC 7.7.1.1.

There are still some important differences that will need to be addressed for the RRM test case:

- Additional MU as result of the multi-cell scenario (e.g. effects of combining both intra-frequency cells and transmitting them from the same antenna).

- Impact of AWGN

- Lower DL powers for Subtest 2.

Nonetheless, the RF test case can be used as basis to make some initial progress. The results of the MU analysis for 7.3. is captured in Annex B.19 in [5]. This analysis already includes the definition of several measurement uncertainty contributors. Given that both tests use the same metric and test directions, and that the RRM test setup is based on the RF test setup (see Observation 1), it is reasonable that (at least) the same MU contributors also apply for the RRM test case.

Proposal 2: the uncertainty contribution definitions for the EIS measurement for the RF reference sensitivity test in B.19 in [5] shall be reused for RRM test case 7.7.1.1 for DL AWGN absolute power and wanted DL signal absolute power. Additional MU contributors are not precluded.

Additionally to the RF MU contributors, there are already three RRM MU contributors defined in Annex E.2 of [5]: “gNB emulator SNR uncertainty”, “gNB emulator Downlink EVM” and “gNB emulator fading model impairments”. 7.7.1.1 does not require fading, so the third MU contributor has no influence in 7.7.1.1. As pointed out during the discussion, the gNB emulator SNR uncertainty is a separate contributor to the DL applied SNR and will not be handled in this paper. The gNB emulator Downlink EVM is a side condition for the RRM test cases, not a MU contributor.

Proposal 2 has been implemented in [6].

The RF MU analysis for reference sensitivity has been done based on IFF setup. The RRM setup based on IFF (i.e. the 1-AoA test setup) is the same as the RF IFF, so all individual values can be reused. For the RRM setup based on Enhanced IFF, considering Observations 1 and 2, some of these values are likely to change. On the other hand, some of the values, such as the gNB emulator uncertainty, are not affected by the number of antenna in the chamber and will remain the same. This needs to be analyzed case by case.

Proposal 4: The RRM setup based on IFF can reuse all available values from the RF IFF MU analysis for the MU quantity DL AWGN absolute power and wanted DL signal absolute power.

Observation 3: The RRM setup based on Enhanced IFF can reuse some the uncertainty values defined for RF IFF. The analysis needs to be done in a case-by-case manner.

The following values have been identified as reusable from IFF to Enhanced IFF setup.

Proposal 5: The RRM setup based on enhanced IFF can reuse the following based from IFF for the MU quantity DL AWGN absolute power and wanted DL signal absolute power:

- Positioning misalignment

- Measure distance uncertainty

- Standing wave between the DUT and measurement antenna

- gNB uncertainty on absolute level

- Phase curvature

- Amplifier uncertainties (Stage 1 and 2)

- Random uncertainty

- Insertion loss variation (Stage 1 and 2)

- RF leakage

- Misalignment of positioning system

- Uncertainty of the network analyzer

- Uncertainty of the absolute gain of the calibration antenna

- Positioning and pointing misalignment between the reference antenna and the measurement antenna

- Phase centre offset of the calibration antenna

- Standing wave between reference calibration antenna and measurement antenna.

- Influence of the calibration antenna feed cable

- Systematic error related to beam peak search.

Proposals 4 and 5 have been implemented in [6].

As pointed out during the discussion, the MU quantities for RRM test cases shall apply for all test cases and not for a single test case individually. Given that 7.7.1.1 is a representative test case for DL AWGN absolute power and wanted DL signal absolute power, the analysis done here holds true for all RRM tests.

Proposal 6: This analysis applies for the MU quantity “DL AWGN absolute power and wanted DL signal absolute power” for all RRM test cases.

# Conclusion

Based on the discussion on this paper, we propose the following:

Observation 1: The RRM baseline test setup is based in the RF test setups as defined in B.2 of TS 38.508-1 [3].

Observation 2: The RRM baseline test setups based on DFF, Enhanced IFF and IFF+DFF Hybrid are an extension of the RF test setup in B.2 of TS 38.508-1 [3].

Proposal 1: use the MU analysis of RF test case “7.3 Reference Sensitivity” as starting point for the MU analysis for DL AWGN absolute power and wanted DL signal absolute power for RRM TC 7.7.1.1.

Proposal 2: the uncertainty contribution definitions for the EIS measurement for the RF reference sensitivity test in B.19 in [5] shall be reused for RRM test case 7.7.1.1 for DL AWGN absolute power and wanted DL signal absolute power. Additional MU contributors are not precluded.

Proposal 4: The RRM setup based on IFF can reuse all available values from the RF IFF MU analysis for the MU quantity DL AWGN absolute power and wanted DL signal absolute power.

Observation 3: The RRM setup based on Enhanced IFF can reuse some the uncertainty values defined for RF IFF. The analysis needs to be done in a case-by-case manner.

Proposal 5: The RRM setup based on enhanced IFF can reuse the following based from IFF for the MU quantity DL AWGN absolute power and wanted DL signal absolute power:

- Positioning misalignment

- Measure distance uncertainty

- Standing wave between the DUT and measurement antenna

- gNB uncertainty on absolute level

- Phase curvature

- Amplifier uncertainties (Stage 1 and 2)

- Random uncertainty

- Insertion loss variation (Stage 1 and 2)

- RF leakage

- Misalignment of positioning system

- Uncertainty of the network analyzer

- Uncertainty of the absolute gain of the calibration antenna

- Positioning and pointing misalignment between the reference antenna and the measurement antenna

- Phase centre offset of the calibration antenna

- Standing wave between reference calibration antenna and measurement antenna.

- Influence of the calibration antenna feed cable

- Systematic error related to beam peak search.

Proposal 6: This analysis applies for the MU quantity “DL AWGN absolute power and wanted DL signal absolute power” for all RRM test cases.

# References

- R5-201155, “FR2 RRM MU and choice of test cases to evaluate”, Anritsu, RAN5 #86-e.

- TS 38.133, “NR; Requirements for support of radio resource management”.

- TS 38.508-1, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- TS 38.521-2, “NR; UE conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone“.

- TR 38.903, “NR; Derivation of test tolerances and measurement uncertainty for UE conformance test cases”.

- R5-201606, “CR to 38.903 for RRM TC 7.7.1.1”.