3GPP TSG-RAN5 Meeting 87-e	R5-202921

# Electric Meeting, 18th – 29th May. 2020

Title:							FR2 RRM timing measurement uncertainty

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

This paper proposes measurement uncertainties for FR2 RRM timing test cases.

# 2.	Discussion

# In FR1, following values are proposed in [1] and are captured in Annex F of 38.533.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc | dB | ±1.5 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc | dB | ±0.3 | Same as in LTE |
| Fading profile uncertainty* | dB | ±0.7Note 3 | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.101-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.101-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Considering 2 Tx Antenna |  |  |  |

# In [1], FR1 timing measurement uncertainty is determined as 25% of core requirement, which is almost the same ratio of MU to core requirement in LTE. In FR2, we can take the same approach upon determining the measurement uncertainty of timing tests.

# Following tables are the timing error limit and timing adjustment size requirement from latest TS 38.133.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1.2-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

# Table 1 summarizes the smallest timing requirements for timing error limit and timing adjustment rate limit.

Table 1 Smallest timing requirement

|  | # LTE | # NR |
| --- | --- | --- |
| # Timing error limit | # 12 * Ts = 391ns | # FR1: 7*64*Tc = 228ns FR2 : 3*64*Tc = 98ns |
| # Adjustment rate limit | # 3.5 * Ts = 114ns | # FR1 : 5.5*64*Tc= 179ns FR2 : 2.5*64*Tc=81ns |

# “Timing error limit” is associated with MU term "Uplink signal transmit timing relative to downlink”. By calculating 25% of FR2 requirement, following value is obtained.

0.25*3*64*Tc  = 48*Tc (= 24.5ns)

The “Uplink signal transmit timing relative to downlink” depends on the total path length of DL and UL of the test system if timing error is measured by DL transmit time minus UL reception time at the SS. However as the test system path length is fixed and can be premeasured, it can be cancelled out by subtracting it to the measured time difference. The remaining main MU factor is the timing measurement error originated from baseband process, which is typically smaller than the 24.5ns(sub-ns to several ns). Hence the MU of 48Tc will be achievable in the actual test equipment and hence is proposed as measurement uncertainty for FR2.

# Proposal 1: Adopt ±[48] Tc  as measurement uncertainty for FR2 “Uplink signal transmit timing relative to downlink“

“Adjustment rate” limit is associated with MU terms “Relative transmits timing accuracy during UE timing adjustment” and “Timing Advance Adjustment accuracy”. By calculating 25% of FR2 requirement, following value is obtained.

# 0.25*2.5*64*Tc  = 40*Tc (= 20.25ns)

# Different from “Uplink signal transmit timing relative to downlink” ,” Relative transmit timing accuracy during UE timing adjustment” and “Timing Advance Adjustment accuracy” are associated with relative timing difference of 2 UL transmissions and hence path length of the test system is not a matter. Then main MU factor of the relative timing error measurement is originated from baseband process and is typical smaller than 40Tc(sub-ns to several ns). Hence +/- 40Tc is proposed for measurement uncertainty in FR2.

# Proposal 2: Adopt ±[40] Tc  as measurement uncertainty for FR2 “Relative transmit timing accuracy during UE timing adjustment” and “Timing Advance Adjustment accuracy”

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

# Proposal 1: Adopt ±[48] Tc  as measurement uncertainty for FR2 “Uplink signal transmit timing relative to downlink“

# Proposal 2: Adopt ±[40] Tc  as measurement uncertainty for FR2 “Relative transmit timing accuracy during UE timing adjustment” and “Timing Advance Adjustment accuracy”

# 4.	References

[1] R5-190058, “FR1 MU for RRM timing measurements”, Anritsu, RAN5 NR#4 AdHoc, Jan 2019