3GPP TSG-RAN5 Meeting #88-e	R5-203474

# Electronic Meeting, 17th – 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4938 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case to test Paging with eDRX and WUS in enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tech Mahindra Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of new test case 8.1.1.9 for paging with eDRX and WUS in enhacend coverage as per eMTC4 Work Plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.1.1.9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eMTC4 WP will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-2 CR 1313 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.1.9	RRC / Paging / eDRX Operation / Short cycle configured  / Enhanced Coverage / WUS

8.1.1.9.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state in enhanced cell coverage and configured for short eDRX and WUS }

ensure that {

when { when network sends WUS followed by Paging message with a matched identity }

then { UE establishes RRC connection }

}

8.1.1.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304 clause 7.4 [TS 36.304, clause 7.4]

When the UE supports WUS and WUS configuration is provided in system information, the UE shall monitor WUS using the WUS parameters provided in System Information. When the UE supports GWUS and GWUS configuration is provided in system information, UE shall monitor WUS using the GWUS parameters provided in System Information as defined in clause 7.5. When DRX is used and the UE detects WUS the UE shall monitor the following PO. When extended DRX is used and the UE detects WUS the UE shall monitor the following numPOs POs or until a paging message including the UE's NAS identity is received, whichever is earlier. If the UE does not detect WUS the UE is not required to monitor the following PO(s). If the UE missed a WUS occasion (e.g. due to cell reselection), it monitors every PO until the start of next WUS or until the PTW ends, whichever is earlier.

-	numPOs = Number of consecutive Paging Occasions (PO) mapped to one WUS provided in system information where (numPOs≥1).

The WUS configuration, provided in system information, includes time-offset between end of WUS and start of the first PO of the numPOs POs UE is required to monitor. The timeoffset in subframes, used to calculate the start of a subframe g0 (see TS 36.213 [6]), is defined as follows:

-	for UE using DRX, it is the signalled timeoffsetDRX;

-	for UE using eDRX, it is the signalled timeoffset-eDRX-Short if timeoffset-eDRX-Long is not broadcasted;

-	for UE using eDRX, it is the value determined according to Table 7.4-1 if timeoffset-eDRX-Long is broadcasted

Table 7.4-1: Determination of GAP between end of WUS and associated PO

|  |  | timeoffset-eDRX-Long |  |
| --- | --- | --- | --- |
|  |  | 1000ms | 2000ms |
| UE Reported wakeUpSignalMinGap-eDRX | 40ms or not reported | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 240ms | timeoffset-eDRX-Short | timeoffset-eDRX-Short |
|  | 1000ms | timeoffset-eDRX-Long | timeoffset-eDRX-Long |
|  | 2000ms | timeoffset-eDRX-Short | timeoffset-eDRX-Long |

The timeoffset is used to determine the actual subframe g0 as follows (taking into consideration resultant SFN and/or H-SFN wrap-around of this computation):

g0 = PO – timeoffset, where PO is the Paging Occasion subframe as defined in clause 7.1

For UE using eDRX, the same timeoffset applies between the end of WUS and associated first PO of the numPOs POs for all the WUS occurrences for a PTW.

The timeoffset, g0, is used to calculate the start of the WUS as defined in TS 36.213 [6].

8.1.1.9.3	Test description

8.1.1.9.3.1	Pre-test conditions

System Simulator:

-	Cell 1 supports CE.

UE:

-	The UE is configured to request the use of eDRX  (in ATTACH REQUEST message ).

-     The UE is configured for WUS operation

Preamble:

-	The UE is in state Registered, Idle mode (state 2-CE) according to [18]. UE sent “extended DRX parameters IE” in ATTACH REQUEST message and received “extended DRX parameters IE” in ATTACH ACCEPT  message

8.1.1.9.3.2	Test procedure sequence

Table 8.1.1.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits WUS followed by Paging message with a matched identity | <-- | Paging | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest message? | --> | RRCConnectionRequest | 1 | P |
| 3 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment? | --> | RRCConnectionSetupComplete | 1 | P |
| 5-8 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | P |
| 10 | SS initiates Detach procedure with the Detach Type IE “re-attach not required” and no EMM cause IE included, as defined in Table 8.1.1.7.3.3-1. | <-- | DETACH REQUEST | - | - |
| 11 | The UE transmits a DETACH ACCEPT message on Cell 1. | --> | DETACH ACCEPT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |

8.1.1.9.3.3	Specific message contents

Table 8.1.1.9.3.3-1: SystemInformationBlockType1-BR-r13 (all steps, table 8.1.1.9.3.2-1)

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.9.3.3-2: SystemInformationBlockType2 (all steps, table 8.1.1.9.3.2-1)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| wus-Config-r15 ::= SEQUENCE { |  |  |  |
| maxDurationFactor-r15 | one16th |  |  |
| numPOs-r15 | n2 |  |  |
| freqLocation-r15 | n4 |  |  |
| timeOffsetDRX-r15 | ms80 |  |  |
| timeOffset-eDRX-Short-r15 | ms160 |  |  |
| timeOffset-eDRX-Long-r15 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.9.3.3-3 ATTACH ACCEPT (Preamble)

| Derivation Path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2,56 seconds |  |
| eDRX value | ‘0010’B | 20,48 seconds |  |