3GPP TSG RAN WG5 Meeting #88-e						R5-204027

Electronic Meeting, 17th – 28th August 2020

Source: 		Samsung

Title:	Discussion on remaining parameters for FR2 EIRP Tx OFF power MU

Agenda item:		5.3.2.17

Document for:		Discussion and Endorsement

# 1.	Introduction

It was decided at previous meetings to include an editor’s note for “testability of OFF power needs further study” [1] in all test cases that are dependent on EIRP OFF power that consist of the following test cases:

- 6.3.3.2: General ON/OFF time mask

- 6.3A.3.1: Transmit ON/OFF time mask for CA (2UL CA)

- 6.3D.3.1: General ON/OFF time mask for UL MIMO

- 6.3D.3.4: SRS time mask for UL MIMO

The test cases listed above minimum conformance requirements are as such:

The maximum allowed EIRP OFF power level is -30dBm at beam peak direction. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

At RAN5#87-e meeting EIRP measurements for Transmit OFF power was included to Annex B.8 [2]. However, there are still some remaining parameters for FR2 EIRP Tx OFF power MU that are still open. In this contribution, we briefly discuss and provide our views and recommendations for some of the remaining parameters concerning FR2 EIRP Tx OFF power.

# 2.	Discussion

For the test cases that are dependent on FR2 EIRP Tx OFF power, the test requirements of EIRP transmit OFF power [1] is shown in Table 1.

Table 1:  Test Requirements of EIRP Transmit OFF power

|  | Channel bandwidth/Opera Output Power [dBm] / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Transmit OFF power | ≤ -30 dBm |  |  |  |
| Transmission OFF measurement bandwidth | 47.52 MHz | 95.04 MHz | 190.08 MHz | 380.16 MHz |

At RAN5#87-e meeting two contributions were provided on FR2 EIRP Tx OFF power in contributions [2] and [3]. Now, in regards to contribution [2], it was endorsed. In contribution [3], Annex B.8 was updated to introduce Table B.8-2: MU threshold for EIRP measurement for Transmit OFF power and Annex B.8.2 was updated to include Table B.8.2-4: Uncertainty assessment for EIRP measurement. The remaining parameters that are still open are highlighted in yellow as shown below taken from TR38.903 [4].

Table B.8-2: MU threshold for EIRP measurement for Transmit OFF power

| Frequency | MBW | Power | Threshold MU value (NOTE1) |
| --- | --- | --- | --- |
| 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Off Power | TBD |
| 32.125GHz < f <= 40.8GHz |  |  | TBD |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.8.2-3 |  |  |  |

Table B.8.2-4: Uncertainty assessment for EIRP measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 8) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch (NOTE 1) | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | Uncertainty of the RF power measurement equipment (NOTE 2) | 2.50 | Normal | 2.00 | 1.25 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.10 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Influence of beam peak search grid (NOTE 3) | 0.00 | Actual | 1 | 0.00 |
| 14 | Multiple measurement antenna uncertainty (NOTE 9) | 0.15 | Actual | 1 | 0.15 |
| 15 | DUT repositioning (NOTE 3) | 0.08 | Rectangular | 1.73 | 0.08 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Uncertainty of the Network Analyzer | [1.5] | Normal | 2.00 | [0.75] |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 23 | Quality of quiet zone for calibration process (NOTE 8) | 0.4 | Actual | 1.00 | 0.4 |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 5) |  |  |  | Value |
| 27 | Systematic error related to beam peak search |  |  |  | 0.5 |
| 28 | Influence of noise (23.45GHz <= f <= 32.125GHz) |  |  |  | TBD |
| 29 | Influence of noise (32.125GHz < f <= 40.8GHz) |  |  |  | TBD |
| Total measurement uncertainty |  |  |  |  | Value |
| EIRP Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | TBD |
| EIRP Expanded uncertainty (32.125GHz < f <= 40.8GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | TBD |
| NOTE 1: The analysis was done only for the case of operating at TX OFF power, in-band, non-CA. NOTE 2: The assessment assumes DUT Off power. NOTE 3: This contributor shall only be considered for EIRP measurements. NOTE 4: Void NOTE 5: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 6: Void. NOTE 7: Void NOTE 8: Value based on procedure defined in Annex D.2 of TR 38.810 for Quiet Zone size less or equal to 30 cm. NOTE 9: Applies to the system which has a structure of mechanical feed antenna positioning. |  |  |  |  |  |

Based on the Low PSD contributions in [5] and [6] we selection to define relaxation values for EIRP Transmit OFF Power to be in alignment with ACLR MU values with brackets to provide one meeting cycle for further review. It is also proposed to update the Table B.8-2 to include each CBW.

Table B.8-2: MU threshold for EIRP measurement for Transmit OFF power

| Frequency | CBW | Power | Threshold MU value (NOTE1) |
| --- | --- | --- | --- |
| 23.45GHz <= f <= 32.125GHz | 50MHz | P = Off Power | [5.39] |
|  | 100MHz |  | [5.59] |
|  | 200MHz |  | [5.99] |
|  | 400MHz |  | [6.09] (NOTE2) |
| 32.125GHz < f <= 40.8GHz | 50MHz | P = Off Power | 5.59 |
|  | 100MHz |  | 5.99 |
|  | 200MHz |  | [6.09] (NOTE3) |
|  | 400MHz |  | [6.09] (NOTE4) |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.8.2-3 NOTE 2: This value is based on the relaxation of (MPR – 5.0) dB for MPR > 5.0dB. NOTE 3: Not applicable for MPR > 5.0dB NOTE 4: Not applicable for MPR > 2.0dB |  |  |  |

Proposal 1: Agree to the MU threshold for EIRP measurement for Transmit OFF power values with brackets.

At RAN5#87-e meeting, Table B.8.2-4: Uncertainty assessment for EIRP measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) was introduced, but there are still some parameters that have brackets or TBD. The following remaining parameters concerning FR2 EIRP Tx OFF power are shown in Table below.

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 8) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch (NOTE 1) | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | Uncertainty of the RF power measurement equipment (NOTE 2) | 2.50 | Normal | 2.00 | 1.25 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.10 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Influence of beam peak search grid (NOTE 3) | 0.00 | Actual | 1 | 0.00 |
| 14 | Multiple measurement antenna uncertainty (NOTE 9) | 0.15 | Actual | 1 | 0.15 |
| 15 | DUT repositioning (NOTE 3) | 0.08 | Rectangular | 1.73 | 0.08 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Uncertainty of the Network Analyzer | 1.5 | Normal | 2.00 | 0.75 |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 23 | Quality of quiet zone for calibration process (NOTE 8) | 0.4 | Actual | 1.00 | 0.4 |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 5) |  |  |  | Value |
| 27 | Systematic error related to beam peak search |  |  |  | 0.5 |
| 28 | Influence of noise (23.45GHz <= f <= 32.125GHz) |  |  |  | Table B.17.2-3 |
| 29 | Influence of noise (32.125GHz < f <= 40.8GHz) |  |  |  | Table B.17.2-3 |
| Total measurement uncertainty |  |  |  |  | Value |
| EIRP Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [5.09] + Influence of Noise |
| EIRP Expanded uncertainty (32.125GHz < f <= 40.8GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [5.09] + Influence of Noise |
| NOTE 1: The analysis was done only for the case of operating at TX OFF power – MPR – MBR (Multi-band relaxation), in-band, non-CA. NOTE 2: The assessment assumes DUT Off power – MPR – MBR (Multi-band relaxation). NOTE 3: This contributor shall only be considered for EIRP measurements. NOTE 4: Void NOTE 5: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 6: Void. NOTE 7: Void NOTE 8: Value based on procedure defined in Annex D.2 of TR 38.810 for Quiet Zone size less or equal to 30 cm. NOTE 9: Applies to the system which has a structure of mechanical feed antenna positioning. |  |  |  |  |  |

We propose the brackets are removed for the Uncertainty of the Network Analyzer parameters that were proposed by Rohde and Schwarz.

Proposal 2: Set Uncertainty of the Network Analyzer as 1.5dB for FR2 EIRP OFF Power MU and Standard uncertainty as 0.75 dB and remove the brackets.

We propose to use the ACLR parameters as baseline for FR2 EIRP OFF Power MU and set the values in brackets for one meeting cycle.

Proposal 3: Set Influence of noise to reference ACLR Table B.17.2-3 for FR2 EIRP OFF Power MU.

Proposal 4: Set EIRP Expanded uncertainty to [5.09] with brackets + influence of noise to be in alignment with ACLR.

Based on the FR2 NR TT Way Forward update in [7] we propose FR2 EIRP OFF Power TT = 0.65xMU since this value is applied for ACLR test cases.

Proposal 5: Set FR2 EIRP OFF Power TT = 0.65xMU.

Proposal 6: Test proposals 1-5 are agreed to be included in TR38.903.

# 3.	Conclusion

In this contribution, we further discussed our views on some of the remaining parameters for FR2 EIRP Tx OFF power MU that are still open. Based on the discussion, our observation and proposals are summarized as follows:

Proposal 1: Agree to the MU threshold for EIRP measurement for Transmit OFF power values with brackets.

Proposal 2: Set Uncertainty of the Network Analyzer as 1.5dB for FR2 EIRP OFF Power MU and Standard uncertainty as 0.75 dB and remove the brackets.

Proposal 3: Set Influence of noise to reference ACLR Table B.17.2-3 for FR2 EIRP OFF Power MU.

Proposal 4: Set EIRP Expanded uncertainty to [5.09] with brackets + influence of noise to be in alignment with ACLR.

Proposal 5: Set FR2 EIRP OFF Power TT = 0.65xMU.

Proposal 6: Test proposals 1-5 are agreed to be included in TR38.903.

# References

[1]	3GPP TS 38.101-2: "NR: User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[2]	R5-202872, "Discussion on FR2 EIRP Tx OFF power MU for test cases usage", Samsung.

[3]	R5-202938, "Addition of EIRP to Transmit OFF power MU analysis", Samsung.

[4] 3GPP TR 38.903: "NR: Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases".

[5]	R5-193706, "Handling on Low PSD items", NTT DOCOMO.

[6] R5-193383, "WF on Low PSD issue", NTT DOCOMO.

[7]	R5-192989 "FR2 NR TT Way Forward update", Telecom Italia.