3GPP TSG-RAN5 Meeting #88-e	R5-204219

# Online, 17th - 21st Aug 2020

Title:	Proposals for new GNSS scenarios for TS 37.571-X

Source:	Spirent Communications. (richard.catmur@spirent.com)

Agenda item:	5.5.16 and 6.6.7.5

Document for:	Information

Abstract of document:

Following the discussion of issues with the current GNSS scenarios used for testing BeiDou (BDS), described in R5-202023, and following RAN 5 endorsement of a basic way-forward, some companies have held off-line discussions to agree a more detailed way-forward. The proposals are detailed below for information. Any other company wishing to comment or take part in further off-line discussions is invited to contact the author.

Detail:

1. It is clear that if the BDS scenarios are to be updated, then all the other GNSS scenarios also should be updated at the same time to bring them up to a similar level of realism. Note that this will also automatically add the new BDS B1C signal.

2. In order to provide the most realistic simulations possible, we will use “live orbital data” from a web site or websites that lists the actual orbital data used at some (recent) point in time. The websites to be used is currently under discussion off-line.

3. In order to simplify our work we will only use one scenario/set of orbital data per GNSS.

4. In TS 37.571-5 we will record the orbital data in a format(s) to be agreed (TLE or Yuma or Rinex). This may require more than one format as Yuma is only almanac, Rinex is ephemeris and it is agreed that both will be specified (see below).

5. We will ensure the RAN 4 / RAN 5 requirements for the number of satellites per GNSS is achieved. Currently this does not seem possible for BDS but this is still under investigation.

6. To avoid too many changes, we will use/specify the same locations as used currently (note therefore we will not take into account the requirements of NavIC).

7. In order to avoid any issues with UEs when using the current, very old, dates and times, we will specify much more recent (fixed) date(s) and time(s). The date(s) will be at least this year (2020), after the latest GPS roll-over.

8. In order to reduce the amount of unnecessary work in making the changes above, we will no longer specify in TS 37.571-5 all the assistance data as is done today. Much of this is highly repetitive and prone to errors etc. As well as time(s) and locations we will just specify the almanac (as Yuma?) and the ephemeris (as Rinex?). The atmospheric conditions will be specified in the Rinex files. The specification of the Acquisition assistance is under discussion and it is possible we will just specify search windows etc. It will be for Test Vendors to create and correctly format the appropriate Assistance Data.

9. We will propose a “grace period” until the end of 2021 for the final cut-over to the new data discussed above to avoid too much re-validation etc.

10 It is currently proposed to start introducing the new data etc. into the specifications at the November RAN 5 meeting.