3GPP TSG RAN Meeting #88-e	 	R5-204270

Electronic Meeting, 17th  – 28th August 2020

Source:		Rohde & Schwarz

Title:	On the MU of FR2 OBW

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

The measurement uncertainty (MU) of occupied bandwidth (OBW) has been discussed during the last meetings [1] to [15]. However, no final conclusion has been reached. This contribution is an updated version of the paper submitted to RAN5#87-e [13] with respect to the agreement under TE vendors to apply 28.0 GHz as center frequency for the data provided in [9].

# Discussion

The spectra of each polarization and of the total power are plotted for 100 MHz CHBW and 4x100 MHz in Figures 1 and 2, respectively. During RAN5#85, it has been confirmed that the 4x100 MHz CA dataset can be applied to derive the MU for OBW for 400 MHz single carrier.

Figure 1: Spectrum for 100 MHz CHBW from [9]

Figure 2: Spectrum for 400 MHz CHBW from [9]

Table 1 summarizes some parameters of the dataset. It has been agreed to reanalyze the OBW MU based on a channel center frequency of 28.0 GHz rather than deriving the channel center frequency from the data set. Interpolation of the provided data has been applied in order to improve the resolution of the OBW results. The OBW has been calculated applying an evaluation window whose center is aligned on the center of the channel.

Table 1: Results of the two data sets

|  | 100 MHz CHBW dataset | 4x100 MHz CA dataset |
| --- | --- | --- |
| center frequency (GHz) | 28.0 | 28.0 |
| OBW (span=1.5⋅CHBW) | 96.344 MHz | 397.92 MHz |
| CP (dBm) | 20.292 | 18.274 |

Comparing the OBW obtained for a center frequency based on the spectrum data to that retrieved for a center frequency of 28.0 GHz results in a deviation of 6.56 MHz. In Figure 3, the limits detected by the centralized OBW approach are displayed as red lines.

Figure 3: Limits obtained with the centralized approach for the 400 MHz CHBW dataset from Ref [9]

Observation 1: The centralized OBW calculation approach can overestimate the OBW when the spectrum is not aligned to the center frequency.

For the channel power CP, it is assumed that the RAW data has been measured with a resolution bandwidth of 1 MHz. If the RBW has been different, there will be a constant shift in the absolute power level which should not be relevant for the OBW MU derivation.

In the following, we refer the SNR to the channel power. The SNR value is a total SNR accounting for both polarizations. In the diagrams the influence of noise on the OBW is visible. In the last meeting it has been agreed to limit the span to 1.5 times the CHBW [13].

For this span, the following results are obtained:

Figure 4: Dependence of the relative deviation on the SNR. Left panel: 100 MHz CHBW. Right panel: 400 MHz CHBW

Since there has not been a conclusion on the metric which is applied to specify the MU in TS 38.521-2 the absolute deviation is depicted in Figure 5.

Figure 5: Dependence of the OBW on the SNR. Left panel: 100 MHz CHBW. Right panel: 400 MHz. 
Red line: Test requirement.

Based on measurements of the frequency characteristics of our test systems, we obtain a measurement uncertainty contribution of 1.0% for 100 MHz CHBW  and 2.1% for 400 MHz CHBW.

Proposal 1: For the MU due to frequency response apply 1.0% for 50 MHz and 100 MHz CHBWs and 2.1% for 200 MHz and 400 MHz CHBWs.

It has been agreed to apply DFT-s-OFDM as waveform for OBW test in the last meeting in order to optimize the SNR ratio.

In order to combine the MU based due to SNR and frequency response, we propose the following equations:

ΔSNR=OBWsignal+noise-OBWsignalCHBW*100%							(1)

Δflatness=OBWsignal+ripples-OBWsignalCHBW*100%							(2)

Δtotal=ΔSNR+Δflatness									(3)

In the last equation a linear sum is used since the impact of noise is systematic and acts always in the same direction

Proposal 2: Calculate the total MU based on equations (1) to (3).

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, the impact of SNR and frequency flatness on the OBW MU has been analyzed.

Observation 1: The centralized OBW calculation approach can overestimate the OBW when the spectrum is not aligned to the center frequency.

Proposal 1: For the MU due to frequency response apply 1.0% for 50 MHz and 100 MHz CHBWs and 2.1% for 200 MHz and 400 MHz CHBWs.

ΔSNR=OBWsignal+noise-OBWsignalCHBW*100%							(1)

Δflatness=OBWsignal+ripples-OBWsignalCHBW*100%							(2)

Δtotal=ΔSNR+Δflatness									(3)

Proposal 2: Calculate the total MU based on equations (1) to (3).

# References

- R5-188189, On the FR2 MU for occupied BW and ACLR, Anritsu, 3GPP TSG RAN WG 5 Meeting #81, November 2018

- R5-190390, On QoQZ MU Impact on OBW, Keysight, 3GPP TSG RAN WG 5 Meeting #4-5G-NR Adhoc, January 2019

- R5-192660, On the FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #82, February 2019

- R5-191617, On OBW measurement uncertainty, Keysight, 3GPP TSG RAN WG 5 Meeting #82, February 2019

- R5-194353, On the noise impact to FR2 OBW, Anritsu, 3GPP TSG RAN WG 5 Meeting #83, May 2019

- R5-192984, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #5-5G-NR Adhoc, April 2019

- R5-193187, On noise impact, Keysight, 3GPP TSG RAN WG 5 Meeting #5-5G-NR Adhoc, April 2019

- R5-197626, Impact of noise for FR2 OBW, Anritsu, 3GPP TSG RAN WG 5 Meeting #83, May 2019

- R5-198175, Discussion on AP#84.22 Tx Spectrum Shape for FR2 OBW MU, Qualcomm, 3GPP TSG RAN WG 5 Meeting #85, November 2019

- R5-200869, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #86-e, February 2020

- R5-200739, On FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #86-e, February 2020

- R5-200874, On FR2 OBW MU, Keysight Technologies, 3GPP TSG RAN WG 5 Meeting #86-e, February 2020

- R5-202458, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020

- R5-202318, On FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020

- R5-202460; On FR2 OBW MU, Keysight Technologies, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020