3GPP TSG-RAN5 Meeting #88-e	R5-204085r1

# Online, , 17th Aug 2020 - 28th Aug 2020

Title:							On FR2 OBW MU

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

OBW MUs have been discussed in [1-16] but no conclusion was reached as of now. In [9], action item for UE/Chip vendors for the spectrum assumption for deriving the OBW MUs are made, and the reference spectrum is provided in [10]. In RAN5#86e, OBW MU analyses based on the reference spectrum [10] are provided [11]-[16]. This paper provide the simulation result and proposes MU for FR2 OBW considering the discussions in RAN5#87-e.

# 2.	Discussion

# 2.1	MU Definition

# As already mentioned in [14], the several possibilities of metrics are proposed. As the OBW MU is defined as a ratio to channel BW in FR1 NR, our preference is to adopt the MU definition based on Option A1 for consistency between FR1 and FR2.

Proposal 1 : Define OBW Measurement Error based with (OBWmeas - OBWsignal) / CBW *100 [%CBW]

where OBWsignal is the OBW without any distortion (noise/ripple) from TE, and OBWmeas is the measured OBW with TE having some noise and ripple.

2.2 Simulation

In the RAN5#86e, Qualcomm indicated that the centre frequency of the spectrum is exactly 28GHz, which is different from the assumption in previous simulation [14]. Other conditions are same as previous simulation [14]. As setting the OBW measurement sapn to x1.5 of CBW is already agreed, we only provide simulation for x1.5 BW.

Fig.1, 2, 3 and 4 show the simulation result when centre of OBW measurement span is aligned with 28GHz. Fig 1 and 2 show the measurement error [%CBW], while Fig 3 and 4 show the absolute measurement error in [MHz]. Due to the asymmetricity of reference spectrum against 28GHz, the OBW measurement result becomes worse than the previous simulation [14].

| Figure 1 BW=400MHz Error [%CBW] | Figure 2 BW=100MHz Error [%CBW] |
| --- | --- |
| Figure 3 BW=400MHz, Absolute Error[MHz] | Figure 4 BW=100MHz, Absolute Error[MHz] |

Table 1 shows the OBWsignal (w/o noise/ripple) for the reference spectrum and is used as a basis for the measurement error.

Table 1 OBWsiginal of the reference spectrum

|  | OBW meas span = 1.5 x CBW | OBW meas span = 2.0 x CBW |
| --- | --- | --- |
| 100MHz | 96.3MHz | 96.3MHz |
| 400MHz | 397.8MHz | 398.6MHz |

2.1.2 Updated simulation during RAN5#88-e with updated simulation assumption

During the RAN5#88-e meeting, it is shown that the center frequency can be 27998.4 MHz rather than 28000.0MHz. This section provides the updated simulation result for center frequency of 27998.4MHz.

| Figure 5 BW=400MHz Error [%CBW] | Figure 6 BW=100MHz Error [%CBW] |
| --- | --- |
| Figure 7 BW=400MHz, Absolute Error[MHz] | Figure 8 BW=100MHz, Absolute Error[MHz] |

Table 2 shows the OBWsignal (w/o noise/ripple) for the reference spectrum and is used as a basis for the measurement error.

Table 2 OBWsiginal of the reference spectrum with center frequency = 27998.4 MHz

|  | OBW meas span = 1.5 x CBW |
| --- | --- |
| 100MHz | 94.1MHz |
| 400MHz | 394.6MHz |

Also, the result for other control points than 3 is simulated. Following figures shows the result for the maximum value(97.5%-tile) among various # of control points assumptions : flat, 2, 3, 4, …. 10, 20, 30, 40, 50. Full set of raw data is available in the Annex A. We observe that for low SNR region, the number of 4 or 3 control points gives the worst case value, while for better SNR region some more control points(e.g. 7) gives the worst case value.

| Figure 9 BW=400MHz Error [%CBW] | Figure 10 BW=100MHz Error [%CBW] |
| --- | --- |

2.1.3 Observations

Based on the analysis in 2.1.2, the reasonable MU limit can be 2.0%[CBW] for 100MHz and 1.35%[CBW](=error limit) for 400MHz.

Achievable SNR based on R5-200868 and required SNR for 2.0%[CBW] for 100Mhz and 1.35%[CBW] for 400Mhz and is given in the below table.

Table 3 Achievable SNR by TEV

|  | SNR with R5-200868 (w/o MBR,MPR,T(MPR)) | MBR | MPR | T(MPR) | SNR | Required SNR for 2.0%[CBW] for 100MHz BW | Required SNR for 1.35%[CBW] for 400MHz BW |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR2a | 30 | 1.7 | 3 | 2 | 23.3 | 21 | 24 |
|  | 30 | 0.75 | 3 | 2 | 24.25 | 21 | 24 |
| FR2b | 26.1 | 1.7 | 3 | 2 | 19.4 | 21 | 24 |
|  | 26.1 | 0.75 | 3 | 2 | 20.35 | 21 | 24 |

Observation 1 : For FR2a 100MHz, required SNR for measurement error of 2.0% [CBW] can be achieved with the test equipment

Observation 2 : For FR2a 400MHz, required SNR for measurement error of 1.35% [CBW] can be achieved with the test equipment if MBR=0.75 applies

Observation 3 : For FR2b 100MHz, required SNR for measurement error of 2.0% [CBW] cannot be achieved with the test equipment

Observation 4 : For FR2a 400MHz, required SNR for measurement error of 1.35% [CBW] cannot be achieved with the test equipment

2.2 Proposals

Summarizing the observations in section 2.1 we propose following

Table 4 Proposal of MTSU and OBW measurement span for FR2(CBW<=400MHz)

| # CBW | # Item | # FR2a | # FR2b |
| --- | --- | --- | --- |
| # 50Mhz, 100MHz | # MTSU | # 2% of CBW | # FFS |
|  | # OBW measurement span | # >= 1.5 x CBW | # FFS |
| # 200MHz, 400MHz | # MTSU | # 1.35% of CBW (If only MBR=0.75dB applies) | # FFS. # x |
|  | # OBW measurement span | # >= 1.5 x CBW | # FFS |

# Proposal 2 : Agree on the Table 4 for FR2 OBW MTSU/measurement span

# Note that for FR2a 100MHz case, MU = 1.35% [CBW] can be also feasible. Hence, applying 1.35%[CBW] for FR2a 100MHz will also possible acceptable.

# 3.	Conclusion

In this paper FR2 OBW MU and testability limit is studied based on the previous discussions.

# RAN5 is asked to endorse following proposals.

# Proposal 1 : Define OBW Measurement Error based with (OBWmeas - OBWsignal) / CBW *100 [%CBW]

# Proposal 2 : Agree on the Table 4 for FR2 OBW MTSU/measurement span

# 4.	References

[1] R5-188189, “On the FR2 MU for occupied BW and ACLR”, Anritsu, RAN5#81

[2] R5-190390, “On QoQZ MU Impact on OBW”, Keysight, RAN5 NR#4 AdHoc

[3] R5-192660, “On the FR2 OBW MU”, Anritsu, RAN5#82

[4] R5-191617, “On OBW measurement uncertainty”, Keysight, RAN5#82

[5] R5-194353, “On the noise impact to FR2 OBW”, Anritsu, RAN5#83

[6] R5-192984, “On the SNR for FR2 TRx test cases”, R&S, Rohde & Schwarz, RAN5 NR#5 AdHoc

[7] R5-193187, “On noise impact”, Keysight, RAN5 NR#5 AdHoc

[8] ITU-R SM 443-4, “SM.443 : Bandwidth measurement at monitoring stations”, https://www.itu.int/rec/R-REC-SM.443-4-200702-I/en

[9] R5-197626, “Impact of noise for FR2 OBW”, Anritsu, RAN5#83

[10] R5-198175, “Discussion on AP#84.22 Tx Spectrum Shape for FR2 OBW MU”, Qualcomm, RAN5#85

[11] R5-200874, “On FR2 OBW MU”, Kesight, RAN5#86e

[12] R5-200869, “On the MU of FR2 OBW”, Rohde&Shwarltz, RAN5#86e

[13] R5-200739, “On FR2 OBW MU “, Anritsu, RAN5#86e

[14] R5-202318, “On FR2 OBW MU”, Anritsu, RAN5#87e

[15] R5-202458, “On the MU of FR2 OBW”, ROHDE & SCHWARZ, RAN5#87e

[16] R5-202460, “On FR2 OBW MU”, Keysight Technologies UK Ltd, RAN5#87e

Table 5 100MHz measurement error

| SNR[dB] | Error limit | 100MHz, 1.5, flat | 100MHz, 1.5, 2p | 100MHz, 1.5, 3p | 100MHz, 1.5, 4p | 100MHz, 1.5, 5p | 100MHz, 1.5, 6p | 100MHz, 1.5, 7p | 100MHz, 1.5, 8p | 100MHz, 1.5, 9p | 100MHz, 1.5, 10p | 100MHz, 1.5, 20p | 100MHz, 1.5, 30p | 100MHz, 1.5, 40p | 100MHz, 1.5, 50p | Max | Arg Max p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15.00 | 5.88 | 24.43 | 30.11 | 41.45 | 42.21 | 41.60 | 41.00 | 40.68 | 40.64 | 40.29 | 39.74 | 37.82 | 36.40 | 35.36 | 34.50 | 42.21 | 4 |
| 16.00 | 5.88 | 16.85 | 23.67 | 37.73 | 38.67 | 38.15 | 36.90 | 37.17 | 36.32 | 35.69 | 35.13 | 33.61 | 30.92 | 29.77 | 28.69 | 38.67 | 4 |
| 17.00 | 5.88 | 7.47 | 15.31 | 32.79 | 33.61 | 33.24 | 32.22 | 31.22 | 31.01 | 30.71 | 30.26 | 26.94 | 24.36 | 22.51 | 21.05 | 33.61 | 4 |
| 18.00 | 5.88 | 1.57 | 5.28 | 26.73 | 27.10 | 26.64 | 25.36 | 25.09 | 24.34 | 23.46 | 23.73 | 18.86 | 15.53 | 13.49 | 11.77 | 27.10 | 4 |
| 19.00 | 5.88 | 0.98 | 1.39 | 18.52 | 19.41 | 17.82 | 16.02 | 14.64 | 15.36 | 15.69 | 15.45 | 9.06 | 5.04 | 2.67 | 2.22 | 19.41 | 4 |
| 20.00 | 5.88 | 0.70 | 0.95 | 8.09 | 7.86 | 5.85 | 5.38 | 5.10 | 5.46 | 4.98 | 4.30 | 1.81 | 1.50 | 1.38 | 1.26 | 8.09 | 3 |
| 21.00 | 5.88 | 0.53 | 0.70 | 1.71 | 1.75 | 1.54 | 1.51 | 1.62 | 1.63 | 1.52 | 1.49 | 1.17 | 1.09 | 1.11 | 1.01 | 1.75 | 4 |
| 22.00 | 5.88 | 0.40 | 0.56 | 1.17 | 1.16 | 1.09 | 1.12 | 1.22 | 1.18 | 1.12 | 1.06 | 0.94 | 0.92 | 0.92 | 0.88 | 1.22 | 7 |
| 23.00 | 5.88 | 0.31 | 0.45 | 0.96 | 0.90 | 0.88 | 0.92 | 1.01 | 0.98 | 0.93 | 0.89 | 0.80 | 0.79 | 0.82 | 0.80 | 1.01 | 7 |
| 24.00 | 5.88 | 0.24 | 0.37 | 0.79 | 0.77 | 0.75 | 0.82 | 0.89 | 0.85 | 0.80 | 0.76 | 0.70 | 0.70 | 0.75 | 0.74 | 0.89 | 7 |
| 25.00 | 5.88 | 0.19 | 0.31 | 0.69 | 0.66 | 0.66 | 0.74 | 0.80 | 0.75 | 0.70 | 0.67 | 0.62 | 0.64 | 0.70 | 0.69 | 0.80 | 7 |
| 26.00 | 5.88 | 0.15 | 0.26 | 0.62 | 0.58 | 0.58 | 0.68 | 0.74 | 0.70 | 0.64 | 0.60 | 0.57 | 0.60 | 0.65 | 0.66 | 0.74 | 7 |
| 27.00 | 5.88 | 0.12 | 0.22 | 0.56 | 0.53 | 0.55 | 0.63 | 0.69 | 0.65 | 0.57 | 0.56 | 0.54 | 0.58 | 0.62 | 0.63 | 0.69 | 7 |
| 28.00 | 5.88 | 0.09 | 0.19 | 0.51 | 0.48 | 0.50 | 0.60 | 0.66 | 0.59 | 0.54 | 0.52 | 0.51 | 0.54 | 0.60 | 0.61 | 0.66 | 7 |
| 29.00 | 5.88 | 0.07 | 0.17 | 0.48 | 0.44 | 0.46 | 0.57 | 0.63 | 0.57 | 0.52 | 0.51 | 0.49 | 0.53 | 0.60 | 0.59 | 0.63 | 7 |
| 30.00 | 5.88 | 0.06 | 0.15 | 0.46 | 0.43 | 0.44 | 0.56 | 0.61 | 0.55 | 0.49 | 0.47 | 0.47 | 0.52 | 0.58 | 0.58 | 0.61 | 7 |
| 31.00 | 5.88 | 0.05 | 0.14 | 0.44 | 0.40 | 0.43 | 0.53 | 0.60 | 0.53 | 0.48 | 0.46 | 0.45 | 0.50 | 0.57 | 0.58 | 0.60 | 7 |
| 32.00 | 5.88 | 0.04 | 0.13 | 0.43 | 0.39 | 0.42 | 0.53 | 0.58 | 0.52 | 0.45 | 0.44 | 0.44 | 0.47 | 0.56 | 0.56 | 0.58 | 7 |
| 33.00 | 5.88 | 0.03 | 0.12 | 0.41 | 0.37 | 0.40 | 0.53 | 0.57 | 0.50 | 0.45 | 0.43 | 0.44 | 0.47 | 0.54 | 0.56 | 0.57 | 7 |
| 34.00 | 5.88 | 0.02 | 0.11 | 0.40 | 0.36 | 0.39 | 0.51 | 0.57 | 0.49 | 0.43 | 0.43 | 0.43 | 0.47 | 0.53 | 0.56 | 0.57 | 7 |
| 35.00 | 5.88 | 0.02 | 0.11 | 0.39 | 0.36 | 0.39 | 0.51 | 0.55 | 0.48 | 0.43 | 0.42 | 0.43 | 0.46 | 0.53 | 0.55 | 0.55 | 7 |
| 36.00 | 5.88 | 0.01 | 0.11 | 0.38 | 0.35 | 0.39 | 0.50 | 0.55 | 0.48 | 0.44 | 0.42 | 0.42 | 0.46 | 0.54 | 0.56 | 0.56 | 50 |
| 37.00 | 5.88 | 0.01 | 0.10 | 0.38 | 0.35 | 0.38 | 0.50 | 0.55 | 0.48 | 0.42 | 0.42 | 0.43 | 0.47 | 0.53 | 0.55 | 0.55 | 7 |
| 38.00 | 5.88 | 0.01 | 0.10 | 0.37 | 0.34 | 0.37 | 0.49 | 0.56 | 0.48 | 0.43 | 0.42 | 0.42 | 0.46 | 0.54 | 0.56 | 0.56 | 7 |
| 39.00 | 5.88 | 0.01 | 0.10 | 0.37 | 0.34 | 0.37 | 0.50 | 0.55 | 0.48 | 0.42 | 0.41 | 0.42 | 0.46 | 0.54 | 0.54 | 0.55 | 7 |
| 40.00 | 5.88 | 0.01 | 0.09 | 0.37 | 0.33 | 0.37 | 0.48 | 0.53 | 0.47 | 0.42 | 0.40 | 0.41 | 0.45 | 0.52 | 0.54 | 0.54 | 50 |

Table 6 400MHz measurement error

| SNR[dB] | Error limit | 400Mhz, 1.5, flat | 400Mhz, 1.5, 2p | 400Mhz, 1.5, 3p | 400Mhz, 1.5, 4p | 400Mhz, 1.5, 5p | 400Mhz, 1.5, 6p | 400Mhz, 1.5, 7p | 400Mhz, 1.5, 8p | 400Mhz, 1.5, 9p | 400Mhz, 1.5, 10p | 400Mhz, 1.5, 20p | 400Mhz, 1.5, 30p | 400Mhz, 1.5, 40p | 400Mhz, 1.5, 50p | Max | Arg Max p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15.00 | 1.35 | 20.58 | 25.99 | 36.80 | 37.57 | 37.05 | 36.47 | 36.17 | 36.04 | 35.84 | 35.28 | 33.39 | 32.09 | 31.10 | 30.30 | 37.57 | 4 |
| 16.00 | 1.35 | 14.59 | 20.94 | 33.70 | 34.62 | 34.29 | 33.24 | 33.30 | 32.47 | 31.99 | 31.47 | 30.20 | 27.80 | 26.80 | 25.80 | 34.62 | 4 |
| 17.00 | 1.35 | 7.83 | 14.84 | 30.02 | 30.90 | 30.56 | 29.86 | 28.89 | 28.67 | 28.42 | 27.96 | 25.40 | 23.11 | 21.52 | 20.22 | 30.90 | 4 |
| 18.00 | 1.35 | 1.64 | 8.19 | 26.19 | 26.69 | 26.41 | 25.62 | 24.94 | 24.37 | 23.56 | 23.57 | 20.02 | 17.26 | 15.87 | 14.14 | 26.69 | 4 |
| 19.00 | 1.35 | 1.11 | 1.71 | 21.62 | 22.43 | 21.67 | 20.04 | 18.55 | 18.40 | 18.59 | 18.41 | 14.58 | 10.91 | 8.73 | 7.48 | 22.43 | 4 |
| 20.00 | 1.35 | 0.83 | 1.27 | 16.71 | 17.05 | 15.54 | 14.81 | 13.59 | 13.32 | 13.29 | 13.55 | 8.13 | 4.60 | 2.04 | 1.66 | 17.05 | 4 |
| 21.00 | 1.35 | 0.65 | 1.07 | 10.57 | 11.70 | 9.50 | 8.02 | 7.90 | 7.64 | 8.38 | 8.41 | 1.75 | 1.45 | 1.35 | 1.31 | 11.70 | 4 |
| 22.00 | 1.35 | 0.51 | 0.92 | 4.57 | 4.92 | 2.67 | 1.99 | 1.93 | 2.13 | 2.00 | 1.82 | 1.38 | 1.23 | 1.12 | 1.15 | 4.92 | 4 |
| 23.00 | 1.35 | 0.40 | 0.84 | 1.60 | 1.60 | 1.48 | 1.41 | 1.48 | 1.52 | 1.54 | 1.48 | 1.22 | 1.08 | 0.99 | 1.04 | 1.60 | 4 |
| 24.00 | 1.35 | 0.32 | 0.75 | 1.34 | 1.33 | 1.27 | 1.25 | 1.32 | 1.31 | 1.33 | 1.26 | 1.11 | 0.99 | 0.92 | 0.96 | 1.34 | 3 |
| 25.00 | 1.35 | 0.25 | 0.70 | 1.18 | 1.19 | 1.17 | 1.14 | 1.22 | 1.21 | 1.17 | 1.12 | 1.02 | 0.92 | 0.85 | 0.91 | 1.22 | 7 |
| 26.00 | 1.35 | 0.20 | 0.65 | 1.10 | 1.12 | 1.06 | 1.03 | 1.13 | 1.11 | 1.11 | 1.05 | 0.96 | 0.89 | 0.82 | 0.86 | 1.13 | 7 |
| 27.00 | 1.35 | 0.16 | 0.62 | 1.04 | 1.04 | 1.01 | 1.00 | 1.09 | 1.10 | 1.01 | 0.97 | 0.91 | 0.86 | 0.76 | 0.83 | 1.10 | 8 |
| 28.00 | 1.35 | 0.13 | 0.59 | 1.00 | 0.98 | 0.92 | 0.95 | 1.03 | 1.02 | 0.97 | 0.92 | 0.88 | 0.83 | 0.74 | 0.80 | 1.03 | 7 |
| 29.00 | 1.35 | 0.10 | 0.56 | 0.93 | 0.94 | 0.89 | 0.92 | 1.01 | 0.98 | 0.96 | 0.89 | 0.86 | 0.81 | 0.72 | 0.79 | 1.01 | 7 |
| 30.00 | 1.35 | 0.08 | 0.56 | 0.93 | 0.91 | 0.87 | 0.88 | 1.00 | 0.97 | 0.91 | 0.86 | 0.85 | 0.80 | 0.69 | 0.77 | 1.00 | 7 |
| 31.00 | 1.35 | 0.06 | 0.54 | 0.90 | 0.87 | 0.86 | 0.88 | 0.99 | 0.95 | 0.89 | 0.84 | 0.83 | 0.79 | 0.69 | 0.78 | 0.99 | 7 |
| 32.00 | 1.35 | 0.05 | 0.53 | 0.88 | 0.86 | 0.83 | 0.85 | 0.95 | 0.92 | 0.88 | 0.83 | 0.81 | 0.77 | 0.69 | 0.76 | 0.95 | 7 |
| 33.00 | 1.35 | 0.04 | 0.51 | 0.85 | 0.83 | 0.80 | 0.85 | 0.96 | 0.93 | 0.87 | 0.82 | 0.80 | 0.76 | 0.66 | 0.75 | 0.96 | 7 |
| 34.00 | 1.35 | 0.03 | 0.53 | 0.87 | 0.84 | 0.80 | 0.84 | 0.93 | 0.90 | 0.86 | 0.82 | 0.80 | 0.76 | 0.67 | 0.74 | 0.93 | 7 |
| 35.00 | 1.35 | 0.03 | 0.51 | 0.84 | 0.81 | 0.80 | 0.84 | 0.94 | 0.91 | 0.84 | 0.80 | 0.78 | 0.75 | 0.66 | 0.73 | 0.94 | 7 |
| 36.00 | 1.35 | 0.02 | 0.50 | 0.83 | 0.82 | 0.79 | 0.82 | 0.95 | 0.90 | 0.84 | 0.79 | 0.80 | 0.75 | 0.66 | 0.72 | 0.95 | 7 |
| 37.00 | 1.35 | 0.02 | 0.51 | 0.84 | 0.82 | 0.78 | 0.82 | 0.93 | 0.90 | 0.83 | 0.80 | 0.81 | 0.76 | 0.65 | 0.73 | 0.93 | 7 |
| 38.00 | 1.35 | 0.01 | 0.51 | 0.82 | 0.81 | 0.77 | 0.80 | 0.92 | 0.89 | 0.84 | 0.78 | 0.78 | 0.75 | 0.65 | 0.72 | 0.92 | 7 |
| 39.00 | 1.35 | 0.01 | 0.51 | 0.81 | 0.80 | 0.77 | 0.82 | 0.92 | 0.90 | 0.83 | 0.79 | 0.79 | 0.76 | 0.64 | 0.72 | 0.92 | 7 |
| 40.00 | 1.35 | 0.01 | 0.50 | 0.82 | 0.80 | 0.77 | 0.80 | 0.91 | 0.88 | 0.85 | 0.79 | 0.79 | 0.74 | 0.65 | 0.73 | 0.91 | 7 |