3GPP TSG-RAN5 Meeting #88-e	R5-204086r1

# Online, , 17th Aug 2020 - 28th Aug 2020

Title:							Noise impact for FR2 test case where MPR applies

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

This paper provides the SNR estimations for FR2 test cases for MPR and ACLR, and proposes the testability limit and MU value for “influence of noise”.

# 2.	Discussion

# MPR test is to test allowed maximum power reduction due to higher modulation, etc, and is represented as a reduction in dB from maximum output power.

# When MPR applies, the UE is allowed to have tolerance(+/- T ) for the power reduction and is shown in following table in 38.101-2.

Table 6.2.4-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3.1 |  |  |

# The measured power PUMAX,f,c is bounded by following formula in configured output power section in 38.101-2.

PPowerclass – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

# In conformance test, P-MPRf,c is zero,

# For UE conformance testing P-MPRf,c shall be 0 dB.

# Then MPR and A-MPR test cases, the minimum power level that TE needs to measure is

# P = Minimum Peak EIRP -  MBR – MAX(MPR,A-MPR) – T(MAX(MPR, A-MPR)) ….. (1)

# In the ACLR testability paper[1], the power level assumption missed to consider the term of T(MPR), hence it is proposed to re-visit the SNR and testability limit for FR2 ACLR.

# MPR

# The estimated SNR is based on the latest estimates from TEVs which are summarized in ACLR testability paper [1].  The SNR is inversely calculated based on the R5-200868[2] which indicates 30.0dB for 22.4dBm, 26.1dB for 20.6dBm.

# UL Power, SNR and DSNR are summarized in the below table for the worst case assumption is BW=400MHz, MPR=9.0dB and  T(9.0) = 5.0dB.

Table 1 Delta SNR for MPR test

|  | UL Power [dBm/400MHz] | SNR [dB] | SNR [dB] |
| --- | --- | --- | --- |
| FR2a | 22.4-0.75-9-5=7.65 | 15.25 | 0.13 |
| FR2b | 20.6-0.75-9-5=5.85 | 11.35 | 0.31 |

# Proposal 1 : For FR2 MPR test case, apply SNR=0.13dB for FR2a and SNR=0.31dB for FR2b for MPR test case up to 400MHz aggregated BW for PC3

# ACLR

# The updated upper limit of MPR value for which not relaxation applies is shown in Table 2. The red character means the value is less than maximum MPR defined in current test points, i.e. not all the test points are measured as are.

Table 2 Upper limit of MPR for which no relaxation applies for FR2 ACLR test
 (metric = Peak EIRP based ACLR)

|  | FR2a | FR2b |
| --- | --- | --- |
| ChBW (50MHz) | 9.5 (was 14.0) | 7.5 (was 11.0) |
| ChBW (100MHz) | 7.0 (was 11.0) | 5.0 (was 8.0) |
| ChBW (200MHz) | 5.0 (was 8.0) | 3.3.5 (was 5.0) |
| ChBW (400MHz) | 3.0 (was 5.0) | 2.0 (was 2.0) |

Proposal 2 : For FR2 ACLR test case, test FR2 ACLR without relaxation for the configuration with MPR not greater than the values in Table 2.

Relaxation / Test policy for FR2 ACLR is proposed as below. (Same policy as proposed/agreed in [1]).

Proposal 3 : For FR2 ACLR test case for FR2a, apply the relaxation of (MPR for test points – value in Table 2) > 0 for dBc value.

For FR2b, it is proposed not to test the test points with MPR values bigger than those in Table 2.

Proposal 4 : For FR2 ACLR test case for FR2b, do not test the test points with MPR values bigger than Table 2

# The delta SNR value needs to be adjusted according to Table 2. The SNR value after considering the T(MPR) is shown in Table 3, which are derived based on worst case MPR value 7.5dB(CBW<=200Mhz) and 9.0dB(CBW=400MHz).

Table 3 SNR [dB] value for FR2 ACLR MU

|  | FR2a | FR2b |
| --- | --- | --- |
| ChBW (50MHz) | 0.54 (was 0.3) | 1.0 (was 0.5) |
| ChBW (100MHz) | 1.0 (was 0.5) | 1.0 (was 0.9) |
| ChBW (200MHz) | 1.0 (was 0.9) | 1.0 |
| ChBW (400MHz) | 1.0 | 1.0 |

Proposal 5 : Apply SNR in Table 3 for FR2 ACLR for up to 400MHz aggregated BW for PC3.

Test Tolerance :

As the delta SNR works as a bias to increase the measurement result, it was agreed in [3] to not to consider noise impact in TT for minimum peak EIRP and spherical coverage test. Same approach can be applied for MPR test as it is lower limit requirements.

Proposal 6 : Define TT for FR2 MPR test with the following equation for PC3 single carrier test up to 400MHz

Lower limit : TT = 0.65 x (MTSUIFF -  ) for FR2a, 0.65 x (MTSUIFF -  ) for FR2b,

# 4.	Proposals

RAN5 is asked to endorse following proposals.

# Proposal 1 : For FR2 MPR test case, apply SNR=0.13dB for FR2a and SNR=0.31dB for FR2b for MPR test case up to 400MHz aggregated BW for PC3

Proposal 2 : For FR2 ACLR test case, test FR2 ACLR without relaxation for the configuration with MPR not greater than the values in Table 2.

Proposal 3 : For FR2 ACLR test case for FR2a, apply the relaxation of (MPR for test points – value in Table 2) > 0 for dBc value.

Proposal 4 : For FR2 ACLR test case for FR2b, do not test the test points with MPR values bigger than Table 2

Proposal 5 : Apply SNR in Table 3 for FR2 ACLR for up to 400MHz aggregated BW for PC3.

Proposal 6 : Define TT for FR2 MPR test with the following equation for PC3 single carrier test up to 400MHz

Lower limit : TT = 0.65 x (MTSUIFF - 13 ) for FR2a, 0.65 x (MTSUIFF - 31 ) for FR2b,

# 5.	References

[1] R5-202321, “On the FR2 ACLR Testability”, Anritsu, RAN5#87e

[2] R5-200868, “On the SNR for FR2 TRx test cases”, R&S, RAN5#86e

[3]  R5-199562, “FR2 NR TT Way Forward update”, Telecom Italia et. al, RAN5#84

Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43 dBm (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |