3GPP TSG-RAN5 Meeting #90-e	R5-210031

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1919 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MDT test case 8.1.6.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, SRTC, Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to work plan of NR_SON_MDT-UEConTest, test case 8.1.6.1.2.3 should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MDT test case 8.1.6.1.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS38.523-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.6.1.2.3	Logged MDT / Intra-frequency measurement, logging and reporting

8.1.6.1.2.3.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a NR cell}

ensure that {

when { UE camps on a NR cell in the cellGlobalIdList}

then { UE performs logged measurements}

}

(2)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a NR cell}

ensure that {
  when { UE camps on a NR cell not in the cellGlobalIdList}

then { UE does not perform logged measurements}

}

(3)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a NR cell}

ensure that {
  when { UE camps on a NR cell in the trackingAreaCodeList}

then { UE performs logged measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a NR cell}

ensure that {
  when { UE camps on a NR cell not in the trackingAreaCodeList}

then { UE does not perform logged measurements }

}

8.1.6.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, and 5.7.10.3.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

- set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and 8.1.6.1.2.3.3	Test description

8.1.6.1.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1[4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on Cell 1.

8.1.6.1.2.3.3.2	Test procedure sequence

Table 8.1.6.1.2.3.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.3.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | -85 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 | -78 | NR Cell 11 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | -85 | NR Cell 1 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | -78 | NR Cell 2 becomes the highest ranked cell. |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | -85 | NR Cell 1 becomes the highest ranked cell. |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |  |

Table 8.1.6.1.2.3.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 11 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 1 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 2 becomes the highest ranked cell. |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 1 becomes the highest ranked cell. |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |  |

Table 8.1.6.1.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message with a cellGlobalIdList on Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T1" in Table 8.1.6.1.2.3.3.2-1. | - | - | - | - |
| 4-9 | Steps 1 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11 | - | - | - | - |
| 10 | Wait 30 seconds for UE logging interval timer to expire at least once | - | - | - | - |
| 11-18 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 19 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest |  |  |
| 20 | Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for Cell 11? | --> | NR RRC: UEInformationResponse | 1 | P |
| 21 | SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 22 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T2" in Table 8.1.6.1.2.3.3.2-1. | - | - | - | - |
| 23-28 | Steps 1 to 6 of generic test procedure in TS 38.508-1[4] subclause 4.9.5.2.2-1 are performed on Cell 2. | - | - | - | - |
| 29 | Wait 30 seconds for UE logging interval timer to expire at least once | - | - | - | - |
| 30 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 31 | The UE transmits an RRCRRCSetupRequest message. | --> | NR RRC: RRCRRCSetupRequest | - | - |
| 32 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to Cell 2 then the steps 33a1-33a7 are executed. |  |  |  |  |
| 33a1 | The UE transmit an RRCSetupComplete message including logMeasAvailable IE set it to true. | --> | NR RRC: RRCSetupComplete | - | - |
| 33a2-33a5 | Steps 6 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 33a6 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest |  |  |
| 33a7 | Check: Does the UE send an UEInformationResponse message with logged serving cell measurements of Cell 2? | --> | NR RRC: UEInformationResponse | 2 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to Cell 2 then the step 33b1 is executed. | - | - | - | - |
| 33b1 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 2 | P |
| 34 | The SS transmits a LoggedMeasurementConfiguration message with a trackingAreaCodeList on Cell 2. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 35 | The SS transmits a RRCRelease message to release RRC connection and move to RRC_IDLE on Cell 2. | <-- | NR RRC: RRCRelease | - | - |
| 36 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T3" in Table 8.1.6.1.2.3.3.2-1. | - | - | - | - |
| 37-42 | Steps 1 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11 | - | - | - | - |
| 43 | The SS transmits a Paging message on Cell 11. | <-- | NR RRC: Paging | - | - |
| 44 | The UE transmits an RRCRRCSetupRequest message on Cell 11. | --> | NR RRC: RRCSetupRequest | - | - |
| 45 | The SS transmit an RRCSetup message on Cell 11. | <-- | NR RRC: RRCSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to Cell 11 then the steps 46a1-46a7 are executed. |  |  |  |  |
| 46a1 | The UE transmit an RRCSetupComplete message including logMeasAvailable IE set it to true. | --> | NR RRC: RRCSetupComplete | - | - |
| 46a2-46a5 | Steps 6 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 46a6 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest |  |  |
| 46a7 | Check: Does the UE send an UEInformationResponse message with logged serving cell measurements of Cell 11? | --> | NR RRC: UEInformationResponse | 4 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to Cell 11 then the step 46b1 is executed. | - | - | - | - |
| 46b1 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 4 | P |
| 47 | The SS transmits an RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 48 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T4" in Table 8.1.6.1.2.3.3.2-1. | - | - | - | - |
| 49-54 | Steps 1 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 1. | - | - | - | - |
| 55 | Wait 30 seconds for UE logging interval timer to expire at least once | - | - | - | - |
| 56-63 | Steps 2 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 64 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest |  |  |
| 65 | Check: Does the UE send an UEInformationResponse include logMeasReport with logged serving cell measurements of Cell 1? | --> | NR RRC: UEInformationResponse | 3 | P |

8.1.6.1.2.3.3.3	Specific message contents

FFS