3GPP TSG-RAN5 Meeting #90-e	R5-210046

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4991 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial TC BT measurement collection in CEF logging to the correct subclause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_MDT_BT_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The LTE_MDT_BT_WLAN test cases have been added in the wrong clauses of TS 36.523-1. These test cases should be of Inter-RAT type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the LTE_MDT_BT_WLAN test case 8.6.8.7 to the correct subclause 8.6.9.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTE_MDT_BT_WLAN test case is still in the wrong clause of TS 36.523-1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.8.7, 8.6.9.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

8.6.8.7	Void

<< End of changes >>

< Unchanged sections omitted >

<<< START OF CHANGES >>>

8.6.9.5	Connection Establishment Failure logging / Logging and reporting / Bluetooth measurement collection

8.6.9.5.1	Test Purpose (TP)

(1)

with { UE detects connection establishment failure }

ensure that {

when { UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCConnectionSetupComplete message with logMeasAvailableBT included }

}

(2)

with { UE has connection establishment failure information available in the VarConnEstFailReport including logMeasResultListBT }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends the UEInformationResponse message containing connEstFailReport with Bluetooth measurement collection }

}

(3)

with { UE has connection establishment failure information available in the VarConnEstFailReport including logMeasResultListBT }

ensure that {

when { UE receives the UEInformationRequest message containing connEstFailReportReq and a Bluetooth beacon is not included in bt-NameListConfig }

then { UE sends the UEInformationResponse message not including measurement result of this Bluetooth beacon which is not included in bt-NameListConfig }

}

8.6.9.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.4, 5.3.3.6, 5.3.10.9, and 5.6.5.3.

[TS 36.331, clause 5.3.10.9 Other configuration]

The UE shall:

…

1>	if the received otherConfig includes the bt-NameListConfig:

2>	if bt-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation.

[TS 36.331, clause 5.3.3.6 T300 expiry]

The UE shall:

1> if timer T300 expires:

…

2> except for NB-IoT, store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

[TS 36.331, clause 5.3.3.4 Reception of the RRCConnectionSetup by the UE]

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

…

1> set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

…

4>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

…

4>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

5>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.6.9.5.3	Test description

8.6.9.5.3.1	Pre-test conditions

System Simulator:

- Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- Cell 40 and Cell 41 are configured as per 36.508 cl 4.4.9

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.9.5.3.2	Test procedure sequence

Table 8.6.9.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2). | <-- | RRCConnectionRelease | - | - |
| 4 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 4A | The SS is configured to not transmit contention resolution in step 5 RACH procedure. | - | - | - | - |
| 5 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 6 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 7 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 8 | The SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 9 | Check : Does the UE transmits an RRCConnectionSetupComplete message including connEstFailInfoAvailable with logMeasAvailableBT. UE initiates the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC : RRCConnectionSetupComplete NAS : SERVICE REQUEST | 1 | P |
| 10-13 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport with logMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? Check: Does the UE send a UEInformationResponse message with connEstFailReport without measurement result of Bluetooth beacon not including in bt-NameListConfig (Bluetooth beacon 2)? | --> | UEInformationResponse | 2,3 | P |

8.6.9.5.3.3	Specific message contents

Table 8.6.9.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| OtherConfig-r9 ::= SEQUENCE { |  |  |  |
| powerPrefIndicationConfig-r11 CHOICE {} | Not present |  |  |
| bw-PreferenceIndicationTimer-r14 | Not present | ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} |  |
| rlm-ReportConfig-r14 CHOICE {} | Not present |  |  |
| overheatingAssistanceConfig-r14 CHOICE {} | Not present |  |  |
| ailc-BitConfig-r15 | Not present | BOOLEAN |  |
| bt-NameListConfig-r15 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..maxBT-Name-r15)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r15[1] | Set as per Table 4.4.9-1 of [18] | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| wlan-NameListConfig-r15 CHOICE {} | Not present |  |  |
| } |  |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.5.3.3-2: RRCConnectionSetupComplete (step 9, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r11 | true |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| logMeasAvailableBT-r15 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.9.5.3.3-3: UEInformationRequest (step 14, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23A with condition ConnEstFail |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |

Table 8.6.9.5.3.3-4: UEInformationResponse (step 15, Table 8.6.9.5.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReport-r11 SEQUENCE { |  |  |  |
| failedCellId-r11 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| locationInfo-r11 | Not present or any allowed value |  |  |
| measResultFailedCell-r11 SEQUENCE { |  |  |  |
| rsrpResult-r11 | (0..97) |  |  |
| rsrqResult-r11 | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r11 | Not present |  |  |
| numberOfPreamblesSent-r11 | Any allowed value | INTEGER (1..200) |  |
| contentionDetected-r11 | Any allowed value | BOOLEAN |  |
| maxTxPowerReached-r11 | Any allowed value | BOOLEAN |  |
| timeSinceFailure-r11 | Any allowed value | INTEGER (0..172800) |  |
| measResultListEUTRA-v1130 | Not present |  |  |
| measResultFailedCell-v1250 | Not present |  |  |
| failedCellRSRQ-Type-r12 | Not present |  |  |
| measResultListEUTRA-v1250 | Not present |  |  |
| measResultFailedCell-v1360 | Not present |  |  |
| logMeasResultListBT-r15 SEQUENCE { | 1 entry | Report Bluetooth beacon 1 |  |
| LogMeasResultBT-r15 SEQUENCE { |  |  |  |
| bt-Addr-r15 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r15 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<< End of changes >>