3GPP TSG-RAN5 Meeting #90-e	R5-210049

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4994 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TC WLAN measurement collection in Logged MDT for specific sys infos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_MDT_BT_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The LTE_MDT_BT_WLAN test cases have been added in the wrong clauses of TS 36.523-1. These test cases should be of Inter-RAT type. And the specific SystemInformation is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Move the LTE_MDT_BT_WLAN test case 8.6.2.15 to the correct subclause 8.6.3.6. 2. Add the specific message content for the SystemInformationBlockType17 message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTE_MDT_BT_WLAN test case is still in the wrong clause of TS 36.523-1. The specific SystemInformation is still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.15, 8.6.3.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

8.6.2.15	Void

<< End of changes >>

< Unchanged sections omitted >

<<< START OF CHANGES >>>

8.6.3.6	Logged MDT / Logging and reporting / WLAN measurement collection

8.6.3.6.1	Test Purpose (TP)

(1)

with { UE received RRCConnectionSetup message }

ensure that {

when { UE has WLAN logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the logMeasAvailableWLAN in the RRCConnectionSetupComplete message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r10 including logMeasResultListWLAN }

}

(3)

with { UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a WLAN node is not included in wlan-NameList }

ensure that {

when { UE in E-UTRA RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r10 not including the measurement result of the WLAN node not included in wlan-NameList }

}

8.6.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331 clauses 5.3.3.4, 5.6.5.3, 5.6.6.3, 5.6.8.2.

[TS 36.304, clause 8]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.6.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	store the received targetMBSFN-AreaList, if included, in VarLogMeasConfig;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2]

While T330 is running, the UE shall:

…

1>	if not suspended, perform the logging in accordance with the following:

…

2>	else if:

3>	if the UE is in any cell selection state (as specified in TS 36.304 [4]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

3>	else if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

…

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	if wlan-NameList is included in VarLogMeasConfig:

4>	if detailed WLAN measurements are available:

5>	include logMeasResultListWLAN, in order of decreasing RSSI for WLAN APs;

3>	if bt-NameList is included in VarLogMeasConfig:

4>	if detailed Bluetooth measurements are available:

5>	include logMeasResultListBT, in order of decreasing RSSI for Bluetooth beacons;

…

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 5.3.3.4]

The UE shall:

…

1>	set the content of RRCConnectionSetupComplete message as follows:

…

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

…

4>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

4>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableWLAN;

…

1>	submit the RRCConnectionSetupComplete message to lower layers for transmission;

1>	the procedure ends

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.3.6.3	Test description

8.6.3.6.3.1	Pre-test conditions

System Simulator:

- Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell27b are configured as per 36.508 cl 4.4.8 with the OFFLOAD condition

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.3.6.3.2	Test procedure sequence

Table 8.6.3.6.3.2-1 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.6.3.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 27 (WLAN AP1) | Cell 27b (WLAN AP2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |

Table 8.6.3.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCConnectionRelease message. | <-- | RRCConnectionRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging | - | - | - | - |
| 4 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 6 | SS transmit an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 7 | Check: Does the UE transmit an RRCConnectionSetupComplete message with logMeasAvailableWLAN on Cell 1? | --> | RRCConnectionSetupComplete | 1 | P |
| 8-11 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r10 with one entry (WLAN AP 1) measurement result in the IE logMeasResultListWLAN. | --> | UEInformationResponse | 2,3 | P |

8.6.3.6.3.3	Specific message contents

Table 8.6.3.6.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.6.3.6.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-4.0A LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension-r10 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| plmn-IdentityList-r11 |  | Not present |  |  |
| areaConfiguration-v1130 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| targetMBSFN-AreaList-r12 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| bt-NameList-r15 |  | Not present |  |  |
| wlan-NameList-r15 SEQUENCE { |  | 1 entry | WLAN AP 1 |  |
| WLAN-Name-r15[1] |  | Set as per Table 4.4.8-1 of [18] | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

-	The specific message content for the SystemInformationBlockType17 message in Table 8.6.3.6.3.3-1A.

Table 8.6.3.6.3.3-1A: SystemInformationBlockType17 (Preamble and all steps of Table 8.6.3.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 2 Entry |  |
| wlan-Identifiers[1] SEQUENCE{ |  |  |  |
| ssid-r12[1] | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[1] | Not present |  |  |
| hessid-r12[1] | Not present |  |  |
| } |  |  |  |
| wlan-Identifiers[2] SEQUENCE{ |  |  |  |
| ssid-r12[2] | Set as per Table 4.4.8-2 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12[2] | Not present |  |  |
| hessid-r12[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 8.6.3.6.3.3-2: RRCConnectionSetupComplete (step 7, Table 8.6.3.6.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-18 RRCConnectionSetupComplete |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not checked |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailableBT-r15 |  | Not checked |  |  |  |  |  |
| logMeasAvailableWLAN-r15 |  | TRUE |  |  |  |  |  |
| idleMeasAvailable-r15 |  | Not checked |  |  |  |  |  |
| flightPathInfoAvailable-r15 |  | Not checked |  |  |  |  |  |
| connectTo5GC-r15 |  | Not checked |  |  |  |  |  |
| registeredAMF-r15 |  | Not checked |  |  |  |  |  |
| s-NSSAI-list-r15 SEQUENCE {} |  | Not checked |  |  |  |  |  |
| ng-5G-S-TMSI-Bits-r15 CHOICE {} |  | Not checked |  |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.3.6.3.3-3: UEInformationRequest (step 12, Table 8.6.3.6.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23A UEInformationRequest, condition “Logged MDT”

Table 8.6.3.6.3.3-4: UEInformationResponse (step 13, Table 8.6.3.6.3.2-2)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-23B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Not checked |  |  |
| traceReference-r10 |  | Not checked |  |  |
| traceRecordingSessionRef-r10 |  | Not checked |  |  |
| tce-Id-r10 |  | Not checked |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF SEQUENCE { |  |  |  |  |
| locationInfo-r10 |  | Not checked |  |  |
| relativeTimeStamp-r10 |  | Not checked |  |  |
| servCellIdentity-r10 |  | Not checked |  |  |
| measResultServCell-r10 SEQUENCE {} |  | Not checked |  |  |
| measResultNeighCells-r10 SEQUENCE {} |  | Not checked |  |  |
| measResultListEUTRA-v1090 |  | Not checked |  |  |
| measResultListMBSFN-r12 |  | Not checked |  |  |
| measResultServCell-v1250 |  | Not checked |  |  |
| servCellRSRQ-Type-r12 |  | Not checked |  |  |
| measResultListEUTRA-v1250 |  | Not checked |  |  |
| inDeviceCoexDetected-r13 |  | Not checked |  |  |
| measResultServCell-v1360 |  | Not checked |  |  |
| logMeasResultListBT-r15 |  | Not checked |  |  |
| logMeasResultListWLAN-r15 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF SEQUENCE { |  | 1 entry | Report WLAN AP1 |  |
| wlan-Identifiers-r15[1] SEQUENCE { |  |  |  |  |
| ssid-r12 |  | ssid for WLAN AP 1 |  |  |
| bssid-r12 |  | Not present |  |  |
| hessid-r12 |  | Not present |  |  |
| } |  |  |  |  |
| rssiWLAN-r15[1] |  | Any allowed value |  |  |
| rtt-WLAN-r15[1] |  | Any allowed value |  |  |
| } |  |  |  |  |
| anyCellSelectionDetected-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not checked |  |  |
| logMeasAvailableBT-r15 |  | Not checked |  |  |
| logMeasAvailableWLAN-r15 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<< End of changes >>