3GPP TSG-RAN5 Meeting #90-e						R5-210197

Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1950 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to RLC test case 7.1.2.3.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per R5-205671 for 7.1.2.3.6 it is not required for 7.1.2.3.8 to include MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig in the configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the IEs MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig to not present in the NR RRCReconfiguration message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The prose will not be aligned to the TTCN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact ! |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of Modified Section >

##### 7.1.2.3.8	AM RLC / Reconfiguration of RLC parameters by upper layers

7.1.2.3.8.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { t-PollRetransmit value is changed during reconfiguration of RLC parameters by upper layers}

then { UE starts using new t-PollRetransmit value }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { t-Reassembly value is changed during reconfiguration of RLC parameters by upper layers }

then { UE starts using new t-Reassembly value }

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { t-StatusProhibit value is changed during reconfiguration of RLC parameters by upper layers }

then { UE starts using new t-StatusProhibit value }

}

7.1.2.3.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.3.3.1, 5.3.3.2, 5.3.3.3, 5.3.4 and 7.3. TS 38.331 clause 5.3.5.5.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.3.1]

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

[TS 38.322, clause 5.3.3.2]

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

After submitting an AMD PDU including a poll to lower layer and after incrementing of TX_Next if necessary, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to TX_Next – 1;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

[TS 38.322, clause 5.3.3.3]

Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall:

-	if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN:

-	if t-PollRetransmit is running:

-	stop and reset t-PollRetransmit.

[TS 38.322, clause 5.3.4]

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with SN = TX_Next – 1 for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in section 5.3.3.2.

[TS 38.322, clause 7.3]

The following timers are configured by 3GPP TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see sub clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see sub clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see sub clause 5.3.4).

[TS 38.331, clause 5.3.5.5.4]

For each RLC-Bearer-Config received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE’s current configuration contains a RLC bearer with the received logicalChannelIdentity:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

NOTE:	The network does not re-associate an already configured logical channel with another radio bearer. Hence servedRadioBearer is not present in this case.

7.1.2.3.8.3	Test description

7.1.2.3.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.8.3.1-1.

Table 7.1.2.3.8.3.1-1: RLC parameters

| Parameter | Value |
| --- | --- |
| t-Reassembly | ms150 |
| t-StatusProhibit | ms300 |
| t-PollRetransmit | ms400 |
| pollPDU | infinity |
| pollByte | infinity |

7.1.2.3.8.3.2	Test procedure sequence

Table 7.1.2.3.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1-29 | Same expected sequence as in Table 7.1.2.3.8.3.2-2 with (X=0, t-Reassembly = ms150, t-StatusProhibit = ms300, t-PollRetransmit=ms400) Note 1. | - | - | 1,2,3 | - |
| 30 | The SS transmits NR RRCReconfiguratioin message to reconfigure RLC in the UE and set: - t-Reassembly to ms200, - t-StatusProhibit to ms400, - t-PollRetransmit to ms500. (Note 1) | <-- | RRCReconfiguration | - | - |
| 30A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 31-59 | Same expected sequence as in Table 7.1.2.3.8.3.2-2 with (X=11, t-Reassembly = ms200, t-StatusProhibit = ms400, t-PollRetransmit=ms500). | - | - | 1,2,3 | - |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

Table 7.1.2.3.8.3.2-2: Behaviour Sequence (X, t-Reassembly, t-StatusProhibit,t-PollRetransmit)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 2 | The SS transmits 4 AMD PDUs with P=0 and SN=X, X+1, X+2 and X+4. The SS record time TA when AMD PDU#5 (with SN=X+4) is sent. A time spacing of 20 ms is applied. | <-- | AMD PDU#1 (SN=X, P=0) AMD PDU#2 (SN=X+1, P=0) AMD PDU#3 (SN=X+2, P=0) AMD PDU#5 (SN=X+4, P=0) | - | - |
| 3 | The SS waits for 70 ms after the transmission of the first AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 3 UL grants of size 848 bits with a time spacing of 20 ms. (Note 1) | <-- | (UL grants, 848 bits) | - | - |
| 4 | The UE transmits RLC SDU#1+X. | --> | (RLC SDU#1+X) | - | - |
| 5 | The UE transmits RLC SDU#2+X. | --> | (RLC SDU#2+X) | - | - |
| 6 | The UE transmits RLC SDU#3+X. | --> | (RLC SDU#3+X) | - | - |
| 7 | 60 ms after step 3 the SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 8 | The SS starts the UL default grant transmission. | - | - | - | - |
| 9 | Check 1: Does the UE transmit a STATUS PDU with NACK_SN=X+3 and ACK_SN=X+5? Record time TB (Note 5). Check 2: Is (TB – TA ) = t-Reassembly? | --> | STATUS PDU | 2 | P |
| 10 | 100 ms after the Status Report received at Step 9, the SS sends 4 AMD PDUs with P=0 and SN=X+5, X+6, X+8 and X+9. A time spacing of 20 ms is applied. | <-- | AMD PDU#6 (SN=X+5, P=0) AMD PDU#7 (SN=X+6, P=0) AMD PDU#9 (SN=X+8, P=0) AMD PDU#10 (SN=X+9, P=0) | - | - |
| 11 | Check 1: Does the UE transmit a Status Report with NACK_SN=X+3 and ACK_SN=X+7? Record time TC Check 2: (TC – TB) = t-StatusProhibit? | --> | STATUS PDU | 3 | P |
| 12 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 13 | After 250 ms the SS transmits 3 AMD PDUs with SN=X+3, X+7 and X+9. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=X+9. A time spacing of 20 ms is applied. | <-- | AMD PDU#4 (SN=X+3, P=0) AMD PDU#8 (SN=X+7, P=0) AMD PDU#10 (SN=X+9, P=1) | - | - |
| 14 | The SS waits for 150 ms after AMD PDU#4 transmission to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant of size 88 bits (UL Grant Allocation type 3). (Note 2)(Note 4) | <-- | (UL grant, 88 bits) | - | - |
| 15 | The UE transmits a Status Report with no NACK_SN and ACK_SN=X+10. | --> | STATUS PDU | - | - |
| 16 | In the subframe following the one scheduled in step 14 the SS assigns 7 UL grants of size 848 bits (UL Grant Allocation type 2) with a time spacing of 20 ms. (Note 1) | <-- | (UL grants, 848 bits) | - | - |
| 17 | The UE transmits RLC SDU#4+X. | --> | (RLC SDU#4+X) | - | - |
| 18 | The UE transmits RLC SDU#5+X. | --> | (RLC SDU#5+X) | - | - |
| 19 | The UE transmits RLC SDU#6+X. | --> | (RLC SDU#6+X) | - | - |
| 20 | The UE transmits RLC SDU#7+X. | --> | (RLC SDU#7+X) | - | - |
| 21 | The UE transmits RLC SDU#8+X. | --> | (RLC SDU#8+X) | - | - |
| 22 | The UE transmits RLC SDU#9+X. | --> | (RLC SDU#9+X) | - | - |
| 23 | The UE transmits RLC SDU#10+X. | --> | (RLC SDU#10+X) | - | - |
| 24 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 25 | The SS transmits an AMD PDU to the UE. | <-- | AMD PDU#11 (SN=X+10, P=0) | - | - |
| 26 | The SS starts the UL default grant transmission. | - | - | - | - |
| 27 | The UE transmits an AMD PDU with the same data as received in the corresponding DL AMD PDU. Record time TD. | --> | AMD PDU#11 (SN=X+10, P=1) | - | - |
| 28 | Check 1: Does the UE set the poll bit as both the transmission and retransmission buffers become empty? Record time TE. Check 2: Is (TE – TD ) = t-PollRetransmit? | --> | AMD PDU#11 (SN=X+10, P=1) | 1 | P |
| 29 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 848 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time. Note 2: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit a Status Report with ACK_SN(3 byte) + 2 byte MAC PDU subheader and (2 byte short BSR). 4 Bytes additional space provided to confirm UE does not include NACK_SN and conformant UE instead will include MAC Padding. Note 3: Every DL AMD PDU contains 1 RLC SDU size of 99 bytes. Note 4: Timing difference between step 11 to step 15 is equal to t-statusProhibit timer. UE starts SR from step 13 with AMD-PDU#4. 150 ms is chosen so that UE does not reach sr-transMax(n16) otherwise UE RACHes. Note 5: Time TB should be recorded by factoring in the SR-Periodicity as configured in the SchedulingRequestResourceConfig->periodicityAndOffset. |  |  |  |  |  |

7.1.2.3.8.3.3	Specific message contents

Table 7.1.2.3.8.3.3-1: RRCReconfiguration (step 30, Table 7.1.2.3.8.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.8.3.3-2: CellGroupConfig (Table 7.1.2.3.8.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.2.2.8.3.3-3: RLC-BearerConfig (Table 7.1.2.3.8.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-148 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | Set to LCID of the DRB under test |  |  |
| rlc-Config | RLC-Config |  |  |
| } |  |  |  |

Table 7.1.2.2.8.3.3-4: RLC-Config (Table 7.1.2.3.8.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-149 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| t-PollRetransmit | ms500 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| t-Reassembly | ms200 |  |  |
| t-StatusProhibit | ms400 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < End of Modified Section >