3GPP TSG-RAN5 Meeting #90-e	R5-210200

# Electronic Meeting, 22nd February – 5th March

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1952 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to test case 8.1.4.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The RRCConnectionReconfiguration message defined in 36.508 Table 4.6.1-8 with condition HO-TO-EN-DC includes the field nr-SecondaryCellGroupConfig-r15 which points to 38.508-1, table 4.6.1-13 with condition EN-DC. This in turn points to RLC-BearerConfig with conditions AM and DRB2, however due to multiple PDU sessions possibly configured in the pre-test conditions, DRB2 may not longer be available to use for the SCG. The nr-RadioBearerConfig1 field points to 38.508-1 Table 4.6.3-132 with condition EN-DC_DRB. This table hard codes the DRB-ToAddMod to use a DRB-Identity of 2 and an eps-BearerIdentity of 6. There is no indication if this is a default bearer or a dedicated bearer, and if it is a dedicated bearer, which default bearer it should be linked to. 2. 38.508-1 allows multiple PDU sessions to be configured in the pre-test conditions. There is no indication in this test case of which PDU session should be used for the handover, and/or what should happen to the other PDU sessions. 3. System information combination NR-6 should be used for NR Cell 1, combination 31 should be used for EUTRA Cell 1 and only the MIB should be broadcast on the NR PSCell 3. 4. TP(1) talks about an EN-DC cell but there is no such cell type defined. 5. Conformance requirements clause contains unnecessary text duplication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Condition DRBn is used in the RRCConnectionReconfiguration message defined in 36.508 Table 4.6.1-8. 2. Modified the PDU session and configured the DRB in NR5GC at the beginning of the test case, before the handover to provide the missing NAS information for the DRB that will become the SCG in EN-DC. 3. Correct the system information combinations defined in the pre-test conditions. 4. TP(1) text is updated to clarify what particular configuration is being conformance tested. 5. Duplicated text replaced by a pointer, one new core specification reference has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1, 2. The UE will not have the correct RRC and NAS information about the bearers to be used for handover 3. The system information combination sent in each cell will be incorrect 4. Unclear Test Purpose will remain. 5. Unnecessary duplication will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.4.2.1.2	Inter-RAT handover / From NR to EN-DC / Success

8.1.4.2.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromNRCommand message with the embedded RRCConnectionReconfiguration message including nr-SecondaryCellGroupConfig }

then { UE transmits an RRCConnectionReconfigurationComplete message embedding an RRCReconfigurationComplete message on the E-UTRA cell }

}

8.1.4.2.1.2.2	Conformance requirements

Same as TC 8.1.4.2.1.1 with the addition of: Rel-16 TS 36.331, clause 5.3.5.4 and Rel-16 TS 37.340, clause 10.16.1.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

…

1>	if the received RRCConnectionReconfiguration includes the sk-Counter:

2>	perform key update procedure as specified in in TS 38.331 [82], clause 5.3.5.7;

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3.

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

[TS 37.340, clause 10.16.1]

Inter-system HO from NR to E-UTRA with EN-DC configuration is supported in this version of the specification. N26 based inter-system HO will be executed between source RAN and gNB.

8.1.4.2.1.2.3	Test description

8.1.4.2.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell. System information combination NR-6 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1.

-	E-UTRA Cell 1 (PCell) and NR Cell 3 (PSCell) are the target cells. System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1 is used in the E-UTRA cell. Only NR MIB is broadcasted on NR Cell 3.

UE:

-	None

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.4.2.1.2.3.2	Test procedure sequence

Table 8.1.4.2.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add DRB#a associated with a new QoS Flow. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 0B | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 0C | The UE transmits a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |
| 1 | The SS transmits a MobilityFromNRCommand message on NR Cell 1,and embedded RRCConnectionReconfiguration message contains nr-SecondaryCellGroupConfig with the configuration of NR Cell 3 | <-- | MobilityFromNRCommand (RRCConnectionReconfiguration) | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on E-UTRA Cell 1 using the security key derived from the new KeNB and sk-Counter? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 1 | P |
| 3 | The UE transmits an ULInformationTransfer message on the cell specified in the test case. This message includes a TRACKING AREA UPDATE REQUEST message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a DLInformationTransfer message on the cell specified in the test case. This message includes a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | The UE transmits an ULInformationTransfer message on the cell specified in the test case. This message includes a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#b associated with the default EPS bearer in E-UTRA Cell 1 | - | - | 1 | P |
| 7 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on the SCG DRB#a in NR Cell 3 | - | - | 1 | P |

8.1.4.2.1.2.3.3	Specific message contents

Table 8.1.4.2.1.2.3.3-0A: PDU SESSION MODIFICATION COMMAND (step 0A, Table 8.1.4.2.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST associated with the Internet PDU session if available or with the first PDU session |  |  |
| Mapped EPS bearer contexts |  |  |  |  |
| Mapped EPS bearer context |  |  |  |  |
| EPS bearer identity |  | (DRB#a + 4) |  |  |
| Operation code |  | ‘001’B | Create new EPS bearer |  |
| E bit |  | '1'B | Parameters list is included |  |
| Number of EPS parameters |  | ’0001’B | 1 parameter |  |
| Mapped EPS QoS parameters |  | EPC dedicated bearer context of the entry in TS 38.508-1 [4] Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| Authorized QoS rules |  | One entry |  |  |
| QoS rule [1] |  | Reference QoS rule #5 as defined in TS 38.508-1 [4] Table 4.8.2.1-1. | QFI=5 |  |
| Authorized QoS flow descriptions |  | One entry |  |  |
| QoS flow [1] |  | Dedicated QoS Flow | QFI=5 |  |
| NOTE: The mapped EPS bearer will become the Dedicated EPS Bearer after handover. |  |  |  |

Table 8.1.4.2.1.2.3.3-0B: Dedicated QoS Flow (step 0A, Table 8.1.4.2.1.2.3.2-1)

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0101’B | QFI 5 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0010’B | 2 parameters |  |
| 5QI | 5QI 1 | 5QI 1 |  |
| EPS bearer identity | (DRB#a + 4) | EBI related to NR DRB#a |  |

Table 8.1.4.2.1.2.3.3-0C: RRCReconfiguration (step 0A, Table8.1.4.2.1.2.3.2-1)

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-HO-ENDC |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(1,0) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.2.1.2.3.3-0D: RadioBearerConfig-HO-ENDC (Table 8.1.4.2.1.2.3.3-0C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  |  | Comment |  | Condition |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { |  |  |  | 1 entry |  |  |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  |  |  |  |  |  |  |  |
| cnAssociation CHOICE { |  |  |  |  |  |  |  |  |
| sdap-Config SEQUENCE { |  |  |  | SDAP-Config |  |  |  |  |
| defaultDRB |  |  |  | false |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| drb-Identity |  |  |  | DRB-Identity using condition DRB#a |  |  |  |  |
| reestablishPDCP |  |  |  | Not present |  |  |  |  |
| recoverPDCP |  |  |  | Not present |  |  |  |  |
| pdcp-Config |  |  |  | PDCP-Config |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| NOTE: DRB#a will become the Dedicated EPS Bearer and the SCG DRB on NR Cell 3 after Handover. |  |  |  |  |  |  |  |

Table 8.1.4.2.1.2.3.3-1: MobilityFromNRCommand (step 1, Table 8.1.4.2.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromNRCommand::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| mobilityFromNRCommand SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | eutra |  |  |
| targetRAT-MessageContainer |  | DL-DCCH message containing RRCConnectionReconfiguration |  |  |
| nas-SecurityParamFromNR |  | 8 LSB of the downlink NAS COUNT |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.4.2.1.2.3.3-2: RRCConnectionReconfiguration (Table 8.1.4.2.1.2.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.1-8, condition HO-TO-EN-DC(x,0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| mobilityControlInfo | MobilityControlInfo_EN-DC |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and CellGroupConfig_EN-DC |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig_EN-DC |  |  |  |
| NOTE: x is equal to the number of PDU sessions established on NR Cell 1. These are all included in the HO to E-UTRA. |  |  |  |

Table 8.1.4.2.1.2.3.3-2A: CellGroupConfig_EN-DC (Table 8.1.4.2.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition EN-DC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  |  | Comment |  | Condition |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { |  |  |  | 1 entry |  |  |  |  |
| RLC-BearerConfig[1] |  |  |  | RLC-BearerConfig with conditions AM and DRB#a | entry 1 |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| NOTE: DRB#a is the SCG DRB on NR Cell 3 after Handover |  |  |  |  |  |  |  |

Table 8.1.4.2.1.2.3.3-2B: RadioBearerConfig_EN-DC (Table 8.1.4.2.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  |  | Comment |  | Condition |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { |  |  |  | 1 entry |  |  |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  |  |  |  | entry 1 |  |  |  |
| cnAssociation CHOICE { |  |  |  |  |  |  |  |  |
| eps-BearerIdentity |  |  |  | (DRB#a + 4) |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| drb-Identity |  |  |  | DRB-Identity using condition DRB#a |  |  |  |  |
| reestablishPDCP |  |  |  | true |  |  |  |  |
| recoverPDCP |  |  |  | Not present |  |  |  |  |
| pdcp-Config |  |  |  | PDCP-Config |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| NOTE: DRB#a is the SCG DRB on NR Cell associated with the Dedicated EPS Bearer after handover. |  |  |  |  |  |  |  |

Table 8.1.4.2.1.2.3.3-3: MobilityControlInfo_EN-DC (Table 8.1.4.2.1.2.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.5-1, condition HO-TO-EN-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of E-UTRA Cell 1 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for E-UTRA Cell 1 |  |  |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for E-UTRA Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |

Table 8.1.4.2.1.2.3.3-4: SecurityConfigHO (Table 8.1.4.2.1.2.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO-v1530 ::= SEQUENCE { |  |  |  |
| handoverType-v1530 CHOICE { |  |  |  |
| fivegc-ToEPC-r15 SEQUENCE { |  |  |  |
| securityAlgorithmConfig-r15 SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity protection algorithm |  |  |
| } |  |  |  |
| nextHopChainingCount-r15 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.2.1.2.3.3-5: Void

Table 8.1.4.2.1.2.3.3-6:RRCConnectionReconfigurationComplete (step 2, Table 8.1.4.2.1.2.3.2-1)

Derivation Path: 36.508 [7], Table 4.6.1-9, condition HO-TO-EN-DC(1,0)

Table 8.1.4.2.1.2.3.3-7: TRACKING AREA UPDATE REQUEST (step 3, Table 8.1.4.2.1.2.3.2-1)

| Derivation Path: 36.508 [7] clause 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EPS update type |  |  |  |  |
| EPS update type Value |  | '000'B | TA updating | TA_only |
|  |  | ‘001’B | Combined TA/LA updating | combined_TA_LA_without_IMSI |
|  |  | '010'B | Combined TA/LA updating with IMSI attach | combined_TA_LA |
| NAS key set identifier |  |  |  |  |
| NAS Key Set Identifier |  | The value of ngKSI assigned in preamble |  |  |
| Type of security context flag |  | ‘1’B | Mapped security context |  |
| Non-current native NAS key set identifier |  | Not present or any allowed value |  |  |
| Old GUTI |  | Mapped 5G-GUTI |  |  |
| GUTI type |  | Native |  |  |
| Additional GUTI |  | Any value or not present |  |  |
| UE status |  | UE is in 5GMM-REGISTERED state |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | The UE is configured to initiate EPS attach |
| combined_TA_LA | The UE is configured to initiate combined EPS/IMSI attach |
| combined_TA_LA_without_IMSI | The UE is configured to initiate combined attach |