3GPP TSG-RAN WG5 Meeting #90-e	R5-210201

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1341 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCPTT Packet Filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For MCPTT the media control messages shall be mapped to the dedicated EPS bearer too. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New single port packet filter added to the reference dedicated EPS bearer context #5 (Table 6.6.2-1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Media control messages will be mapped to default EPS bearer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 6.6.2	Reference dedicated EPS bearer contexts

The following table defines Reference dedicated EPS bearer contexts. Dedicated EPS bearer context #1 is the default "dedicated EPS bearer context" which is used in the common procedures and test cases where no particular dedicated EPS bearer context is specified.

Table 6.6.2-1: Reference dedicated EPS bearer contexts, first set

| Parameters | Reference dedicated EPS bearer context #1 | Reference dedicated EPS bearer context #2 | Reference dedicated EPS bearer context #3 | Reference dedicated EPS bearer context #4 | Reference dedicated EPS bearer context #5 |
| --- | --- | --- | --- | --- | --- |
| Linked EPS bearer identity | Reference default EPS bearer #1 | Reference default EPS bearer #1 | Reference default EPS bearer #2 | Reference default EPS bearer #2 | Reference default EPS bearer #3 |
| EPS QoS |  |  |  |  |  |
| QCI (Note 1) | 1 (GBR QCI) | 5 (non-GBR QCI) | 2 (GBR QCI) | 1 (GBR QCI) | 65 (Note 8) (GBR QCI) |
| Maximum bit rate for uplink | 384 kbps | 384 kbps (Note 2) | 2104 (Note 7) | 384 kbps | 384 kbps |
| Maximum bit rate for downlink | 384 kbps | 384 kbps (Note 2) | 2104 (Note 7) | 384 kbps | 384 kbps |
| Guaranteed bit rate for uplink | 128 kbps | 128 kbps (Note 2) | 2104 (Note 7) | 128 kbps | 128 kbps |
| Guaranteed bit rate for downlink | 128 kbps | 128 kbps (Note 2) | 2104 (Note 7) | 128 kbps | 128 kbps |
| Maximum bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Maximum bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| TFT |  |  |  |  |  |
| TFT operation code | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" |
| E bit | 0 | 0 | 0 | 0 | 0 |
| Packet filters (Note 5) | 1, 2 | 3 | 5 | 4 | 4, 10 (Note 9) |
| Negotiated QoS |  |  |  |  |  |
| Traffic Class | conversational | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | conversational | conversational | conversational |
| Delivery Order | 'no' | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | ’no’ | 'no' | 'no' |
| Delivery of erroneous SDU | 'no' | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | ’no’ | 'no' | 'no' |
| Maximum SDU size | 150 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 1400 | 150 | 150 |
| Maximum bit rate for uplink | 384 kbps | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 432 kbps | 384 kbps | 384 kbps |
| Maximum bit rate for downlink | 384 kbps | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 432 kbps | 384 kbps | 384 kbps |
| Residual BER | 5*10-2 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 10-5 | 5*10-2 | 5*10-2 |
| SDU error ratio | 10-2 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 7*10-3 | 10-2 | 10-2 |
| Transfer delay | 80 ms | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 130 ms | 80 ms | 75 ms |
| Traffic Handling priority | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Signalling Indication | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Source Statistics Descriptor | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 1 | 0 |
| Maximum bit rate for downlink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Maximum bit rate for uplink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Negotiated LLC SAPI | 3 (Note 6) | See "PDP context3" (Note 6) | - | 3 (Note 6) | - |
| Radio priority | 1 (Note 6) | See "PDP context3" (Note 6) | - | 1 (Note 6) | - |
| Protocol configuration options | - | - | - | - | - |
| Note 1: For all non-GBR QCIs, the maximum and guaranteed bit rates shall be ignored. Note 2: According to TS 24.301, the UE ignores these parameters for a non-GBR QCI. Note 3: Parameters included for UEs capable of UTRAN according to TS 34.123-3 clause 8.10 (table 8.10.1 and 8.10.2). Note 5: This row refers to the reference packet filters defined in the tables below. For each reference dedicated EPS bearer context, a list of reference packet filter numbers is provided. Note 6: Parameter included for UEs capable of GERAN; when value is not provided then value from TS 51.010 subclause 40.5 is applied. Note 7: This value shall reflect the highest possible bandwidth requirement of the H.264 codec to EPS bearer mappings as specified in Appendix E of TS 26.114 [64]. Note 8: QCI 65 is used for MCPTT voice. Note 9: MCPTT: Packet filter #4 for audio stream, packet filter #10 for media control messages. |  |  |  |  |  |

Table 6.6.2-1A: Reference dedicated EPS bearer contexts, second set

| Parameters | Reference dedicated EPS bearer context #6 | Reference dedicated EPS bearer context #7 | Reference dedicated EPS bearer context #8 |  |  |
| --- | --- | --- | --- | --- | --- |
| Linked EPS bearer identity | Reference default EPS bearer #1 | Reference default EPS bearer #2 | Reference default EPS bearer #2 |  |  |
| EPS QoS |  |  |  |  |  |
| QCI (Note 1) | 8 (non-GBR QCI) | 79 (non-GBR QCI) | 66 (GBR QCI) |  |  |
| Maximum bit rate for uplink | 384 kbps (Note 2) | 384 kbps (Note 2) | 384 kbps |  |  |
| Maximum bit rate for downlink | 384 kbps (Note 2) | 384 kbps (Note 2) | 384 kbps |  |  |
| Guaranteed bit rate for uplink | 128 kbps (Note 2) | 128 kbps (Note 2) | 128 kbps |  |  |
| Guaranteed bit rate for downlink | 128 kbps (Note 2) | 128 kbps (Note 2) | 128 kbps |  |  |
| Maximum bit rate for uplink (extended) | 0 | 0 | 0 |  |  |
| Maximum bit rate for downlink (extended) | 0 | 0 | 0 |  |  |
| Guaranteed bit rate for uplink (extended) | 0 | 0 | 0 |  |  |
| Guaranteed bit rate for downlink (extended) | 0 | 0 | 0 |  |  |
| TFT |  |  |  |  |  |
| TFT operation code | "create new TFT" | "create new TFT" | "create new TFT" |  |  |
| E bit | 0 | 0 | 0 |  |  |
| Packet filters (Note 3) | 6, 7 | 8 | 9 |  |  |
| Negotiated QoS |  |  |  |  |  |
| Traffic Class | - | - | Conversational |  |  |
| Delivery Order | - | - | ‘no’ |  |  |
| Delivery of erroneous SDU | - | - | ‘no’ |  |  |
| Maximum SDU size | - | - | 150 |  |  |
| Maximum bit rate for uplink | - | - | 384 kbps |  |  |
| Maximum bit rate for downlink | - | - | 384 kbps |  |  |
| Residual BER | - | - | 5*10-2 |  |  |
| SDU error ratio | - | - | 10-2 |  |  |
| Transfer delay | - | - | 75 ms |  |  |
| Traffic Handling priority | - | - | 0 |  |  |
| Guaranteed bit rate for uplink | - | - | 0 |  |  |
| Guaranteed bit rate for downlink | - | - | 0 |  |  |
| Signalling Indication | - | - | 0 |  |  |
| Source Statistics Descriptor | - | - | 0 |  |  |
| Maximum bit rate for downlink (extended) | - | - | 0 |  |  |
| Guaranteed bit rate for downlink (extended) | - | - | 0 |  |  |
| Maximum bit rate for uplink (extended) | - | - | 0 |  |  |
| Guaranteed bit rate for uplink (extended) | - | - | 0 |  |  |
| Negotiated LLC SAPI | - | - | - |  |  |
| Radio priority | - | - | - |  |  |
| Protocol configuration options | - | - | - |  |  |
| Note 1: For all non-GBR QCIs, the maximum and guaranteed bit rates shall be ignored. Note 2: According to TS 24.301, the UE ignores these parameters for a non-GBR QCI. Note 3: This row refers to the reference packet filters defined in the tables below. For each reference dedicated EPS bearer context, a list of reference packet filter numbers is provided. |  |  |  |  |  |

Table 6.6.2-2: Reference packet filter #1

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 0 1 0 0 0 0 | DL only filter,ID=0 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 | 0 to 7 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 2 Value | 31 160 + EPS Bearer ID - 6 |  |  |
| Component type 3 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 3 Value | 17 | UDP |  |
| Note 1: remoteAddress should be set to the address of an IP server able to send a flow of downlink IP/UDP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-3: Reference packet filter #2

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 2 Value | 61 000 + EPS Bearer ID - 6 |  |  |
| Component type 3 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 3 Value | 17 | UDP |  |
| Note 1: remoteAddress should be set to the address of an IP server able to process a flow of uplink IP/UDP packets received from the UE. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #1. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-4: Reference packet filter #3

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 0 1 0 | Bidirectional filter, ID=2 |  |
| Evaluation precedence | 0 0 0 0 1 1 1 1 | Lowest priority |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Note 1: remoteAddress should be set to the address of an IP server able to process a uplink IP packets from the UE and transmit downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-5: Reference packet filter #4

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 0 1 1 | Bidirectional,ID=3 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-6: Reference packet filter #5

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 0 | Bidirectional,ID=4 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-7: Reference packet filter #6

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 0 1 0 0 0 0 | DL only filter,ID=0 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 | 0 to 7 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 1 | ICMP | remote IPv4 |
| Component type 2 Value | 58 | ICMPv6 | remote IPv6 |
| Note 1: remoteAddress is the address of a network entity and the filter matches for ICMP/ICMPv6 packets sent to the UE with remote address being source address. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-8: Reference packet filter #7

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 1 | ICMP | Ping |
| Component type 2 Value | 58 | ICMPv6 | Ping |
| Note 1: remoteAddress is the address of a network entity and the filter matches for ICMP/ICMPv6 packets sent by the UE with remote address being destination address. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #6. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-9: Reference packet filter #8

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: remoteAddress should be set to the address of an IP server able to process a flow of uplink IP/UDP packets received from the UE. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #1. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-10: Reference packet filter #9

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 1 | Bidirectional filter, ID=5 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-11: Reference packet filter #10

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 0 | Bidirectional,ID=4 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 1 Value | media port | Port used for media control message |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |

<End of modification>