3GPP TSG-RAN5 Meeting #90-e						R5-210202

Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1455 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to SMS test case 18.b1, Annexes A.7.2 and A.7.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. In “Figure 8.8/3GPP TS 24.011: RP-Cause element layout” the extra occurrence of the “Cause value” has been removed by CT1 CR0068 (C1-207121). 2. In clause A.7.2 TP-SCTS is erroneously provided as IE of DELIVERY REPORT TPDU for RP-ACK. See TS 23.040 cl. 9.2.1.2.1a. 3. In clause A.7.8 the MTI of the RP-DATA message (Network to UE) is erroneously provided as ‘000’B. According to TS 24.011 clause 8.2.2 it should be ‘001’B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. In “Figure 8.8/3GPP TS 24.011: RP-Cause element layout” contained in clause 18.b1.2, the extra occurrence of the “Cause value” was removed. 2. In clause A.7.2 TP-SCTS provided as IE of DELIVERY REPORT TPDU for RP-ACK was removed. 3. In clause A.7.8 the MTI of the RP-DATA message (Network to UE) was corrected to ‘001’B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose will remain erroneous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.b1, A.7.2 and A.7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact ! |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF MODIFIED SECTION>

## 18.1b	Mobile Originating SMS / RP-ERROR

### 18.1b.1	Definition

Test to verify that the UE can successfully receive a RP-ERROR message.

### 18.1b.2	Conformance requirement

[TS 24.341, clause 5.3.1.1]:

In addition to the procedures specified in subclause 5.3.1, the SM-over-IP sender shall support the procedures specified in 3GPP TS 24.229 [10] appropriate to the functional entity in which the SM-over-IP sender is implemented. The SM-over-IP sender shall build and populate RP-DATA message, containing all the information that a mobile station submitting an SM according to 3GPP TS 24.011 [8] would place, for successful delivery. The SM-over-IP sender shall parse and interpret RP- DATA, RP-ACK and RP-ERROR messages, containing all the information that a mobile station receiving an SM according to 3GPP TS 24.011 [8] would see, in a SM submission or status report.

NOTE 1:	If the SM-over-IP sender uses SMR entity timers as specified in 3GPP TS 24.011 [8], then TR1M is set to a value greater than timer F (see 3GPP TS 24.229 [10]).

NOTE 2:	If the SM-over-IP sender expects to receive a SM submit report will include the "+g.3gpp.smsip" parameter in the Contact header field when sending a REGISTER request.

[TS 24.341, clause 5.3.1.2]:

When an SM-over-IP sender wants to submit an SM over IP, the SM-over-IP sender shall send a SIP MESSAGE request with the following information:

a)	the Request-URI, which shall contain the PSI of the SC of the SM-over-IP sender;

NOTE 1:	The PSI of the SC can be SIP URI or tel URI based on operator policy. The PSI of the SC can be obtained using one of the following methods in the priority order listed below:

1)	provided by the user;

2)	if UICC is used, then:

-	if present in the ISIM, then the PSI of the SC is obtained from the EFPSISMSC in DF_TELECOM of the ISIM as per 3GPP TS 31.103 [18];

-	if not present on the ISIM, then the PSI of the SC is obtained from the EFPSISMSC in DF_TELECOM of the USIM as per 3GPP TS 31.102 [19]; or

-	if neither present on the ISIM nor on the USIM, then the PSI of the SC contains the TS-Service-Centre-Address stored in the EFSMSP in DF_TELECOM as per 3GPP TS 31.102 [19]. If the PSI of the SC is based on the E.164 number from the TS-Service-Centre-Address stored in the EFSMSP in DF_TELECOM then the URI constructed can be either a tel URI or a SIP URI (using the "user=phone" SIP URI parameter format).

3)	if SIM is used instead of UICC, then the PSI of the SC contains the TS-Service Centre Address stored in the EFSMSP in DF_TELECOM as per 3GPP TS 51.011 [20]. If the PSI of the SC is based on the E.164 number from the TS-Service-Centre-Address stored in the EFSMSP in DF_TELECOM then the URI constructed can be either a tel URI or a SIP URI (using the "user=phone" SIP URI parameter format); or

4)	if neither the UICC nor SIM is used, then how the PSI of the SC is configured and obtained is through means outside the scope of this specification.

b)	the From header, which shall contain a public user identity of the SM-over-IP sender;

NOTE 2:	The IP-SM-GW will have to use an address of the SM-over-IP sender that the SC can process (i.e. an E.164 number). This address will come from a tel URI in a P-Asserted-Identity header (as defined in RFC 3325 [13]) placed in the SIP MESSAGE request by the P-CSCF or S-CSCF.

NOTE 3:	The SM-over-IP sender has to store the Call-ID of the SIP MESSAGE request, so it can associate the appropriate SIP MESSAGE request including a submit report with it.

c)	the To header, which shall contain the SC of the SM-over-IP sender;

d)	the Content-Type header, which shall contain "application/vnd.3gpp.sms"; and

e)	the body of the request shall contain an RP-DATA message as defined in 3GPP TS 24.011 [8], including the SMS headers and the SMS user information encoded as specified in 3GPP TS 23.040 [3].

NOTE 4:	The address of the SC is included in the RP-DATA message content. The address of the SC included in the RP-DATA message content is stored in the EFSMSP in DF_TELECOM of the (U)SIM of the SM-over-IP sender.

NOTE 5:	The SM-over-IP sender will use content transfer encoding of type "binary" for the encoding of the SM in the body of the SIP MESSAGE request.

NOTE 6:	Both the address of the SC and the PSI of the SC can be configured in the EFPSISMSC in DF_TELECOM of the USIM and ISIM respectively using the USAT as per 3GPP TS 31.111 [21].

The SM-over-IP sender may request the SC to return the status of the submitted message. The support of status report capabilities is optional for the SC.

When a SIP MESSAGE request including a submit report in the "vnd.3gpp.sms" payload is received, the SM-over-IP sender shall:

-	if SM-over-IP sender supports In-Reply-To header usage and the In-Reply-To header indicates that the request corresponds to a short message submitted by the SM-over-IP sender, generate a 200 (OK) SIP response according to RFC 3428 [14].

if SM-over-IP sender supports In-Reply-To header usage and the In-Reply-To header indicates that the request does not correspond to a short message submitted by the SM-over-IP sender, a 488 (Not Acceptable here) SIP response according to RFC 3428 [14].

-	if SM-over-IP sender does not support In-Reply-To header usage, generate a 200 (OK) SIP response according to RFC 3428 [14]; and extract the payload encoded according to 3GPP TS 24.011 [8] for RP-ACK or RP-ERROR.

[TS 24.341 clause 5.3.1.3]:

When a SIP MESSAGE request including a status report in the "vnd.3gpp.sms" payload is delivered, the SM-over-IP sender shall:

-	generate a SIP response according to RFC 3428 [14];

-	extract the payload encoded according to 3GPP TS 24.011 [8] for RP-DATA; and

-	create a delivery report for the status report as described in subclause 5.3.2.4. The content of the delivery report is defined in 3GPP TS 24.011 [8].

[TS 24.341 clause 5.3.2.4]:

When an SM-over-IP receiver wants to send an SM delivery report over IP, the SM-over-IP receiver shall send a SIP MESSAGE request with the following information:

a)	the Request-URI, which shall contain the IP-SM-GW;

NOTE 1:	The address of the IP-SM-GW is received in the P-Asserted-Identity header in the SIP MESSAGE request including the delivered short message.

b)	the From header, which shall contain a public user identity of the SM-over-IP receiver.

c)	the To header, which shall contain the IP-SM-GW;

b)	the Content-Type header shall contain "application/vnd.3gpp.sms"; and

c)	the body of the request shall contain the RP-ACK or RP-ERROR message for the SM delivery report, as defined in 3GPP TS 24.011 [8].

NOTE 2:	The SM-over-IP sender will use content transfer encoding of type "binary" for the encoding of the SM in the body of the SIP MESSAGE request.

[TS 24.011 clause 8.2.5.4]:

This element is a variable length element always included in the RP-ERROR message, conveying a negative result of a RP-DATA message transfer attempt or RP-SMMA notification attempt. The element contains a cause value and optionally a diagnostic field giving further details of the error cause.

The coding of the cause value is given in table 8.4/3GPP TS 24.011. The mapping between error causes in 3GPP TS 24.011 and 3GPP TS 29.002 (MAP) is specified in 3GPP TS 23.040. Parameters included in the return error from MAP (e.g. System Failure) are mapped directly into the diagnostic field.

| 8 7 6 5 4 3 2 1 |  |  |
| --- | --- | --- |
| 0 | 1 0 0 0 0 1 0 RP-Cause IEI | 1 octet |
| Length indicator |  | 1 octet |
| 0 ext | Cause value | 1 octet |
| Diagnostic field |  | 1 octet * |

Figure 8.8/3GPP TS 24.011: RP-Cause element layout

Table 8.4/3GPP TS 24.011 (part 1): Cause values that may be contained in an RP-ERROR message
in a mobile originating SM-transfer attempt

| Cause value | Cause | Cause |
| --- | --- | --- |
| Class value | number |  |
| 7 6 5 4 3 2 1 | # |  |
| 0 0 0 0 0 0 1 | 1 | Unassigned (unallocated) number |
| 0 0 0 1 0 0 0 | 8 | Operator determined barring |
| 0 0 0 1 0 1 0 | 10 | Call barred |
| 0 0 0 1 0 1 1 | 11 | Reserved |
| 0 0 1 0 1 0 1 | 21 | Short message transfer rejected |
| 0 0 1 1 0 1 1 | 27 | Destination out of order |
| 0 0 1 1 1 0 0 | 28 | Unidentified subscriber |
| 0 0 1 1 1 0 1 | 29 | Facility rejected |
| 0 0 1 1 1 1 0 | 30 | Unknown subscriber |
| 0 1 0 0 1 1 0 | 38 | Network out of order |
| 0 1 0 1 0 0 1 | 41 | Temporary failure |
| 0 1 0 1 0 1 0 | 42 | Congestion |
| 0 1 0 1 1 1 1 | 47 | Resources unavailable, unspecified |
| 0 1 1 0 0 1 0 | 50 | Requested facility not subscribed |
| 1 0 0 0 1 0 1 | 69 | Requested facility not implemented |
| 1 0 1 0 0 0 1 | 81 | Invalid short message transfer reference value |
| 1 0 1 1 1 1 1 | 95 | Semantically incorrect message |
| 1 1 0 0 0 0 0 | 96 | Invalid mandatory information |
| 1 1 0 0 0 0 1 | 97 | Message type non-existent or not implemented |
| 1 1 0 0 0 1 0 | 98 | Message not compatible with short message protocol state |
| 1 1 0 0 0 1 1 | 99 | Information element non-existent or not implemented |
| 1 1 0 1 1 1 1 | 111 | Protocol error, unspecified |
| 1 1 1 1 1 1 1 | 127 | Interworking, unspecified |
| All other cause values shall be treated as cause number 41, "Temporary Failure". |  |  |

Reference(s)

3GPP TS 24.341[90], clauses 5.3.1.1, 5.3.1.2, 5.3.1.3 and 5.3.2.4, and TS 24.011 [92], clause 8.2.5.4.

### 18.1b.3	Test purpose

1)	To verify that when sending of a Mobile Originating SMS over IMS is initiated, the UE sends a SIP MESSAGE request constructed as described in 3GPP TS 24.341 [90], clause 5.3.1.2; and

2)	To verify that the UE correctly handles reception of a SIP MESSAGE request indicating RP-ERROR.

### 18.1b.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF, and registered to IMS services.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

1)	Sending of a Mobile Originating SMS over IMS is initiated at the UE. The SS waits for the UE to send a SIP MESSAGE request including a vnd.3gpp.sms payload that contains the short message.

2)	The SS responds to the SIP MESSAGE request with a 202 Accepted response.

3)	The SS sends a SIP MESSAGE request to the UE including a vnd.3gpp.sms payload and RP-ERROR message.

4)	The SS waits for the UE to respond to the SIP MESSAGE request with a 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | SIP MESSAGE request | UE sends a SIP MESSAGE request including a vnd.3gpp.sms payload that contains a short message |
| 2 |  |  | 202 Accepted | SS responds with 202 Accepted |
| 3 |  |  | SIP MESSAGE request | SS sends a SIP MESSAGE request including a vnd.3gpp.sms payload and RP-ERROR message |
| 4 |  |  | 200 OK | UE responds with 200 OK |

NOTE:	The default messages contents in annex A are used with condition “IMS security” or “GIBA” when applicable

Specific Message Contents

SIP MESSAGE request (Step 1)

Use the default message “Message for MO SMS” in Annex A.7.3

202 Accepted for SIP MESSAGE request (Step 2)

Use the default message “202 Accepted” in annex A.3.3.

SIP MESSAGE request (Step 3)

Use the default message “Short message submission report for MO SMS” in Annex A.7.4 with the following exception.

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Message-body |  | RP-ERROR message with RP-Cause Data: Length: 2, Length indicator = 1 Extension: not extended Cause value: 38 (Network out of order) |  | TS 24.011 [92] TS 23.040 [93] |

200 OK for SIP MESSAGE request (Step 4)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

| Message-body | A3 | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity=sip:final@conf-factory. appended with px_IMS_HomeDomainName state="full" version="0"> <users> <user entity="any IMPU within the set of IMPUs on ISIM"> <endpoint entity=" Contact URI of the UE"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>34567</label> <src-id>SSRC of UE's RTP packets</src-id> <status>sendrecv</status> </media> </endpoint> </user> </user> </conference-info> |  | RFC 4575 [86] |
| --- | --- | --- | --- | --- |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 TS 34.229-2 [5])) |
| A2 | GIBA (A.6a/1 TS 34.229-2 [5])) |
| A3 | SS sends NOTIFY to indicate that UE is now subscribed to the conference event package |
| A4 | SS sends NOTIFY to indicate that UE’s subscription to conference event is terminated now |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

<END OF MODIFIED SECTION>

<START OF MODIFIED SECTION>

## A.7.2	MESSAGE for delivery report for MT SMS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | MESSAGE |  | RFC 3428 [91] TS 24.341 [90] |
| Request-URI |  | same P-Asserted-Identity URI as received in A.7.1 |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | not checked |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE |  |  |
| tag |  | any value |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same as P-Asserted-Identity URI received in A.7.1 |  | TS 24.341 [90] |
| tag |  | not present |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | any value (but different from the Call-ID values used in preceding requests of this test case) |  |  |
| In-Reply-to |  |  |  | RFC 3261 [15] |
| callid |  | The value of the Call-Id received in the original MT SMS |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | any value |  |  |
| method |  | MESSAGE |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/vnd.3gpp.sms |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Message-body |  | RP-ACK message with RP-User Data including SMS-DELIVERY-REPORT: TP-MTI=‘00’B (SMS-DELIVERY-REPORT) TP-PI=‘00000000’B |  | TS 24.011 [92] |

<END OF MODIFIED SECTION>

<START OF MODIFIED SECTION>

## A.7.8	RP-DATA message (Network to UE)

| Information element | Value/Remark |
| --- | --- |
| RP-Message Type | ‘001’B |
| RP-Message Reference | Any valid value |
| RP-Originator Address | TS-Service Centre Address(default value as defined in E.3.2.14) |
| RP-Destination Address | 0 length address |
| RP-User Data | Any valid value |

<END OF MODIFIED SECTION>